#                                           A d v a n c e d

#                   ����  �����    ����  ��    �� �  �����

#                  �  ��  �    �  ��  �� ��    �� � ������

#                  �    � �����  ��    �� ��  ��  �       �

#                   ����  �   �  �      �  ����   �  �����

#                  Advanced Gravis Computer Technology Ltd.

#

#      Fax (604) 431-5155                   BBS  (604) 431-5927 V32bis

#      Compuserve: GO PCVENB (71333,350)    InterNET EMail: tech@gravis.com

#      FidoNET: 1:153/978                   SBCNET: 13:900/3

#                   UltraMID TSR V1.04 (UMID110.ZIP)

#                       1.0 About these Drivers

#                       2.0 Installation

#                       3.0 Who Supports

#                       4.0 Revision history and Packing List

#                       5.0 Troubleshooting

#                       6.0 Note for Developers

#                 DEVELOPERS: read the developr.txt file

#1.0.0 *************** CHAPTER 1. - ABOUT THESE DRIVERS *******************

#1.1.0 What is UltraMID

================

The Advanced Gravis UltraSound card is a unique sound card in the

sound card industry.  It is the only audio product for the PC that can

produce realistic sounding instruments without requiring an expensive

digital signal processor.  Digital signal processors (DSPs) are

relatively expensive microprocessors that are designed specifically

for working with audio and visual data.  UltraMID is a software version

of a DSP.  It provides an extremely easy way for software developers to

write software for the UltraSound card.

#1.2.0 What does UltraMID do for me

============================

Most of the new games that are released that support the UltraSound

card will use UltraMID for midi music and digital synthesis.  There

are titles being released from almost 100% of the major game comapanies

that use UltraMID.  Each of these games will ship with its own version

of UltraMID.  However many games that are already out on the market

can be adapted to use UltraMID.  Any games that uses the "Audio Interface

Libraries" from Miles Design, Inc., (AIL), or DIGPAK from The Audio

Solution, can use UltraMID.  AIL and DIGPAK drivers have been included

with UltraMID so that you can get older software to work with the

UltraSound card.

#1.3.0 Audio Interface Libraries (AIL) - MIDPAK/DIGPAK

===============================================

The Audio Interface Libraries (AIL), (we know them as "Miles" Drivers)

and DIGPAK are a set of interface drivers which enable software

developers (mainly game developers) to integrate sound capabilities

into their programs.  Once a developer has integrated his program into

the AIL drivers the application will work with all sound cards on the

market who provide AIL drivers. The AIL's provide support for MIDI as

well as the Digital Sound interface of an audio card. These library

drivers allow the developers to use UltraSound's wavetable synthesizer

and allow them to utilize the UltraSound's digital interface to play 8

or 16 bit audio in either mono or stereo mode.

Advanced Gravis & FORTE have recently designed and released to Miles

Design and The Audio Solution Inc. the AIL and DIGPAK routines for the

UltraSound card which provide the mechanism for developers to easily

integrate with, and use UltraSound's features.  Developers who have

the AIL's and/or DIGPAK can get the driver updates and integrate

their programs into the UltraSound.  The AIL/DIGPAK updates will also

be available through the Advanced Gravis BBS or any of our nodes.

Please refer to GRAVNODE.TXT for a complete list. Developers who do

not have the AILs can get them from Miles Design and/or DIGPAK from

The Audio Solution Inc.

Developers who want to take full advantage of UltraSound's advanced

features; and do not want to pay either Miles Design or The Audio

Solution Inc. license fees, Advanced Gravis and FORTE have a stand

alone driver as well.  These drivers use a Terminate and Stay

Resident (TSR) utility called UltraMID.  Developers utilizing this

TSR are able to do digital playback at 8 or 16 bits up to 44.1kHz

sample rate.  Also, this allows programmers to use all of the

UltraSound's 32 digital voices as well as the superior wavetable

synthesizer.

These drivers are another level of support for the UltraSound card

developers.  Both the AIL and DIGPAK interfaces provide a means

for programmers to easily design software for many sound cards using

only one API while Advanced Gravis/FORTE's stand alone TSR allows

developers to take advantage of UltraSound's more advanced features.

#      ....................... END of Chapter 1 ...........................

#2.0.0 ****************** CHAPTER 2. - INSTALLATION ***********************

If you are reading this you have probably already unzipped

all the files into a directory.  Once this is done type "UMINSTALL".

This program will copy ULTRAMID.INI to your ULTRASND\MIDI directory,

and ULTRAMID.EXE to your ULTRASND directory.  It does this by

looking for your ULTRADIR environment variable.  It will also verify

all of your patches to make sure that they are compatible with UltraMID.

Once the install is completed, you can start ultramid by simply

typing "ultramid" at the DOS prompt.

#2.0.1 installing the AIL, or DIGPAK drivers.

If you have a game that uses the AIL or DIGPAK drivers and you

would like to get that game to use UltraMID, read this section.

Now for the tough part. First, find a game that uses the AIL or

DIGPAK/MIDPAK drivers.  A partial list is provided at the end of

this README.  To determine if a game uses the AIL drivers look for

files that end with ".ADV".

FIRST, you should make a backup of the games current .ADV drivers,

just in case something happens.  What we will now do is overwrite

some of these files with the Gravis drivers.

A table is provided below with some guidelines on which files you

should overwrite:

COPY FROM      COPY TO      CONFIGURE THE GAME AS

=================================================

GF1MIDI.ADV    MT32MPU.ADV  Roland

GF1DIGI.ADV    SBDIG.ADV    Sound Blaster

OR

GF1MIDI.ADV    SBFM.ADV     Sound Blaster

GF1DIGI.ADV    SBDIG.ADV

OR

GF1MIDI.ADV    MT32MPU.ADV  Roland

GF166.COM      SBLASTER.COM Sound Blaster (DIGPAK)

By looking at the names and size of the .ADV drivers you should be able

to determine which cards they are for.  For example:

ADLIB    ADV     14693 07-07-92   6:00a  = Adlib

ADLIBG   ADV     16175 07-07-92   6:00a  = Adlib Gold

ARIAXMID ADV     44836 03-26-93  10:26a  = Aria

MT32MPU  ADV     10590 03-16-92  12:00p  = Roland

MULTISND ADV      7042 03-09-93  12:51p  = Turtle Beach

PASFM    ADV     15160 07-07-92   6:00a  = Pro Audio Spectrum

PCSPKR   ADV      8306 03-16-92  12:00p  = PC Speaker

SBFM     ADV     14743 07-07-92   6:00a  = Sound Blaster

SBP1FM   ADV     15153 07-07-92   6:00a  = Sound Blaster Pro

GF1DIGI  ADV      5248 08-14-93   3:49p  = UltraSound Digital

GF1MIDI  ADV     10816 08-14-93   3:50p  = UltraSound MIDI

In general if any .ADV file is > 10K it is the music driver and should

be copied over with GF1MIDI.ADV.  If any .ADV file is < 10K it is the

digital audio driver and should be replaced with GF1DIGI.ADV

Once this is done simply run the games setup and select whatever

sound card that you just over wrote with the Gravis drivers.

For example, if you copied over top of the Sound Blaster and Roland

drivers then you would configure the game for sound blaster roland.

That is it.  If everything is working correctly you should now be

getting UltraSound support in that game.  If not please refer

to the trouble shooting section in chapter 5.

#2.1.0 Hints and Tricks

================

- In theory you could copy over top of ANY of the .ADV drivers as

long as you know which sound card to configure the game for.

- Some programs do not use the regular naming convention for .ADV

drivers, for instance Ultima Underworld II by Origin.  The

convention for UW2 is DM*.ADV, and DD*.ADV.  The DM is for

MIDI and the DD is for digital audio.  Since we can't tell by

looking at the file name which sound card these drivers are for

you may just want to copy GF1DIGI.ADV overtop of ALL DD*.ADV

files and GF1MIDI.ADV overtop of all DM*.ADV drivers.

#  REMEMBER TO BACKUP YOUR ORIGINAL .ADV DRIVERS

- In theory, a game should be looking in its sound driver directory

for any files with a .ADV extention, then present the user with a

list of the sound card drivers found.  For instance, in Spell

Caster 2000 you simply have to copy the GF1*.ADV drivers into

its directory.  It will then detect these drivers and you may choose

ULTRASOUND from its setup menu. This is the only game so far that

I've been able to find which does this.

- (06/22) Origin uses a slightly modified version of the AIL drivers.

As such their games may sound slightly off or not work at all with

our drivers. An example of this is Strike Commander, which will not

work at all with the standard AIL interface.

- We have found some games that use both the AIL drivers and access the

sound hardware directly.  An example of this type of game is EYE OF

THE BEHOLDER.  Since these type of games do not rely soly on the AIL

drivers and try to access the hardeware directly they may not work

with these drivers.

#2.2.0 Command Line Switchs

====================

ULTRAMID by default will try to preload patches for all MIDI songs.

This will give you the best quality sound but may not be compatible

with all games.  If you start ULTRAMID but can't get the game to

make any music, try reloading ULTRAMID with the -C parameter.

e.g. lh ultramid -c -m100 -nultramid.ini -dc:\game\

The above example will load UltraMID high, preload a patch library, sets

the master volume at 100, looks for an .INI file named Ultramid.ini and

tells UltraMID to look in the C:\game directory for the configuration

file.

-C    - Preloads a 256K, 512K 768K or 1024K patch library, depending

on how much memory you have on your UltraSound card. UltraMID

automatically detects how much memory is on your card.  This

option does not sound as good as patch cacheing but should

work with all games.

-F    - Unload ULTRAMID from memory

-Mxxx - [1-127] Sets the master volume for music.  In some games the

music may over power the speech.  If so you can use this

switch to adjust the master volume as required. The default

volume is 127.

-Nfilename

- Tell UltraMID not to use ultramid.ini, but look for an

initialization file named "filename"

-Dpath

- Use an alternate path for finding the ultramid configuration

file and for finding patches. The path should include a

trailing backslash (e.g.) -Dc:\game\

-q    - quiet.  Don't display any messages.

#2.3.0 Limitations of AIL and DIGPAK

=======================

The Audio Interface Libraries and DIGPAK each only give access to

ONE STEREO digital channel.  Since the UltraSound is capable of 32

independent digital channels this poses quite a limitation.  The

file UMDOC.ZIP on the Advanced Gravis BBS has detailed technical

documentation on how to write programs that use UltraMID.  UltraMID will

let you play multiple simultaneous digital audio tracks as well as

general midi music.

#2.4.0 Support

=======

These AIL, MIDPAK/DIGPAK drivers are intended for developers who

will be releasing future programs that use the AIL drivers.  We

are releasing these to the general public so that you may attempt

to patch older games to include direct UltraSound support.

Since this is our first public release of these drivers we have

provided a file called FEEDBACK.TXT, which you may fill out and send

back to us with your comments.  For example, how did you get it to

work with game 'XYZ'.  Tips you may have for others, etc...

#2.6.0 General Information

===================

- We currently support V3.2 of the DIGPAK standard.

- We currently support V2.0 of the AIL standard.

- You may have heard the names 'John Miles' and 'John Ratcliff' and

wondered who they are.  John Miles is Miles Design, the designer and

creator of the Audio Interface Libraries.  John Ratcliff is The

Audio Solution (MIDPAK/DIGPAK). They are two seperate companies who

work together towards the same goal.  A standard interface for

sound cards.

- Documentation on how to program directly for ULTRAMID will be made

available in the future by Forte and Advanced Gravis.

- If you are interested in AIL or MIDPAK/DIGPAK you may contact Audio

Solutions at any of the numbers provided later on in this README.

- ULTRAMID will look for your ULTRADIR environment variable to locate

your MIDI directory.  ULTRAMID.INI must reside in your MIDI directory.

- Tech NOTE: Developers: Please use a multiple of 32 bytes for sound

buffers.  The Gravis UltraSound card can only DMA to 32 byte

boundaries; so to reduce UltraMID overhead, a multiple of

32 bytes will mean no polled I/O.

- (06/22) Sierra and Lucas do NOT use either the AIL drivers or DIGPAK.

Sierra has drivers available for their general midi games that use

UltraMID.

- (06/22) If possible try copying overtop of the SCC1 or general midi

drivers.  As some games will load up a different (usually better)

music score.  An example of this is the 7th Guest.

#      ....................... END of Chapter 2 ...........................

#3.0.0 ****************** CHAPTER 3. - WHO SUPPORTS ***********************

#3.1.0 The Audio Solution

==================

The following is a list obtained from the Audio Solution BBS

(PRODUCTS.LST). It is a little out of date but does include some

information on which game publishers are licensed to use

these drivers.

THE AUDIO SOLUTION

P.O. Box 11688

Clayton, MO 63104

Order/Information: (314)-567-0267 9am-5pm Central Mon-Fri

Technical Support: (314)-926-0232 9am-5pm Central Mon-Fri

BBS:               (314)-939-0200 24 hours 7 days.

MILES DESIGN, INC.

6702 Cat Creek Trail

Austin, TX, 78731

(512) 345-2642 / FAX (512) 338-9630 / BBS (512) 454-9990

DIGPAK/MIDPAK COMPLETE PRODUCT LIST

(as of 1/3/93)

This list does not reflect all products produced by product line

licensees; since we are not always informed of those products as

they are released.  It also does not reflect the products in

development (over 100) which use either DigPak and MidPak.

Product Name            Publisher                    Use

===========================================================================

<unamed>                Epyx                         DigPak+MidPak

ATC/Tracon              Wesson International         DigPak

Animal Quest            Alive Software               DigPak+MidPak

Battle Chess 4000       Interplay                    DigPak

Battle Cruiser 3000AD   Three-Sixty Pacific          DigPak

Beyond Shadowgate       ICOM Simulations             DigPak+MidPak

Bilou                   Concepteva                   DigPak

BodyLink                Stragem                      DigPak

Chess Maniac 5,000,001  Spectrum Holobyte            DigPak+AIL

Chessmaster 3000        The Software Toolworks       DigPak

Contraption ZAck        Presage Software Co. Inc.    DigPak+MidPak

Creepers                Destiny Software             DigPak+MidPak

Cribbage Master         Ninga                        DigPak

Cyberstrike             Simutronics                  DigPak

DNA Parrot              T&t Research                 DigPak

Dinosaur Adventure      Knowledge Adventure          DigPak

Fun Univ Ntwk Intfc     Fun Univ Netwk               DigPak+MidPak

Gateway to the Savage   Strategic Simulations Inc.   DigPak

Gateworld Trilogy       Home Brew Software           MidPak

GemStone III            Simutronics                  DigPak

Grandmaster Chess       IntraCorp                    DigPak+MidPak

Guardians of Eden       Access Software              MidPak

Guzzle Puzzles          Redwood Games                DigPak

Inspector Gadget:       Azeroth Publishing           DigPak+MidPak

KidDesk                 Edmark Corporation           DigPak

Layout for DOS          Objects, Inc                 DigPak+MidPak

Le Ponctueur            C.R.A.P.O., Inc.             DigPak

Magic Crayon            Alive Software               DigPak+MidPak

Marvin The Moose        Milliken Publishing          DigPak

Math Sequences          Milliken Publishing          DigPak

Math Zone               Milliken Publishing          DigPak

Mechwarrior II          Activision                   DigPak

MicroLeague Baseball IV Microleague Sports Assoc.    DigPak+MidPak

MicroLeague Football II Microleauge Sports Assoc.    DigPak+MidPak

Microleage Cards        Microleage Sports Assoc.     DigPak

Millie's Math House     Edmark Corpoartion           DigPak

Milliken Storyteller    Milliken Publishing          DigPak

Monopoly Deluxe         Virgin Games                 DigPak+MidPak

Mutanoid Math Challenge Legacy Software              DigPak+MidPak

Mutanoid Word Challenge Legacy Software              DigPak+MidPak

My Paint                Saddleback Graphics          DigPak

Oregon Trail Deluxe     MECC                         DigPak+MidPak

Pickle Wars             Redwood Games                DigPak+MidPak

Places to Play          Magnetic Images              DigPak

Pools of Darkness       Strategic Simulations Inc.   DigPak

Ragarok (Valhalla)      Norsehelm Productions        DigPak

Realms of Avarton       Kram                         DigPak

Return to Zork          Activision                   DigPak

Shadow President        DC True                      DigPak

Sights & Sounds         Macmillan/McGraw Hill        DigPak

Solitaire's Journey     Quantum Quality Productions  DigPak+MidPak

Space Adventure         Knowledge Adventure          DigPak

Space Chase 1,2,3       Safari Software              MidPak

Spaceward Ho PC         Presage Software Co. Inc.    DigPak+MidPak

Stepping Stones Bonus   Compu-Teach, Inc.            DigPak+MidPak

Stickybear Townbuilder  Optimum Resource             DigPak

Switch Progressions     Cooper, R.J. & Assoc.        DigPak+MidPak

Switch Quik             Cooper, R.J. & Assoc.        DigPak+MidPak

T-Zero                  Dennis Cunningham            DigPak+MidPak

Terminator 2029         Bethesda Softworks           DigPak

The 7th Guest           Virgin/Trilobyte             DigPak+AIL

Tom Landry Football     Merit Software               DigPak+MidPak

VGA Jigsaw              Alive Software               DigPak+MidPak

Waterford Mental Math   Waterford Institute          DigPak+MidPak

Wheel of Fortune        Gametek                      DigPak+MidPak

WorldAtlas              The Software Toolworks       DigPak+MidPak

Product line licensees of DigPak and/or MidPak.  Product line licensees

have the rights to include DigPak and MidPak in any product produced by

their company.

Electronic Arts

Activision

Spectrum Holobyte

The Software Toolworks

SSI, Strategic Simulations Inc.

Milliken Publishing Company

Commodore Computer

Virgin Games

Compu-Teach

Knowledge Adventure

Gametek

Product line licensees to AIL (Mile Design's Audio Interface Library) which

uses the same MIDI drivers as MIDPAK.

Activision

Strategic Simulations

Electronic Arts

Origin Systems

Spectrum Holobyte

Virgin-Games

Knowledge Adventure

Bethesda Softworks

Interplay

Microprose

Gametek

Impressions

Mindscape UK

#3.2.0 Other Products Found to use these Drivers

=========================================

Great Naval Battles

Tony La Rusa Baseball II

War of Two Worlds

Spell Caster 2000

Trump Castle III

Buzz Aldrin's Race To Space

Clash of Steel - World War II Europe 1939-45

SimCity Classic

SimFarm

Dark Sun

Kids Zoo

SSN Sea Wolf

#      ....................... END of Chapter 3 ...........................

#4.0.0 *************** CHAPTER 4. - ABOUT THIS UPDATE *******************

#4.1.0 Revision History

================

V1.10 - Updated copyright message

V1.09 - 16 bit dma bug was corrupting sound card memory

V1.08 - envelope problem causing short percussion fixed

V1.07 - extra interrupts in envelopes being generated causing clamped notes

V1.05 - digital buffering improved

V1.03  - (09/15/93) MIDI in support for sequencers added

V1.02a  - (09/02/93) Digital Audio improved

V1.02  - (09/01/93) All notes off midi message fixed

V1.01  - (08/31/93) This UltraMID offers much faster patch loading than

the previous version, and is about 3K smaller.

It also can load almost twice as many patches as

it used to in the same amount of PC memory.  (You

are still bound by the size of your UltraSound card

however.)

This release also fixes several small bugs.

1) DMA bug while loading patches.

2) ULTRAMID.INI problems.

3) Noises during patch loading.

New Features:

1) Automatically removes SBOS.

2) Can split up its code and data so that if you

have fragmented upper memory or not enough

upper memory, UltraMID can still load a

portion of itself high.

PS: REMEMBER TO RUN the UMINSTAL.EXE program.

V1.0   - (06/25/93) Yes, a version 1.0 release

V0.97B - (06/22/93) Fifth Beta - Fixed DMA bug.

Fixed patch loading bug.

Fixed xmidi multi-sequence bug.

Fixed digital hang bug.

Fixed problem with 512K patch library

V0.96B - (06/17/93) Fourth Beta - Fix DIGPAK buffering

#4.2.0 Packing List

============

ULTRAMID.EXE     AIL TSR - Required for drivers

GF1MIDI.ADV      UltraSound MIDI AIL Driver

GF1DIGI.ADV      UltraSound Digital Audio AIL Driver

GF166.COM        UltraSound DIGPAK Driver

UM200.INI        Patch Map for 2.00 - 2.05 UltraSound software releases

UM206.INI        Patch Map for 2.06 UltraSound software releases

UM206a.INI       Patch Map for 2.06 UltraSound software releases

README           This README file

GV.EXE           Gravis Document Viewer

INSTAL.EXE       Installs the correct .INI file and UltraMID.EXE in your

ULTRASND directory.

FEEDBACK.TXT     Beta Test Feedback Form

GRAVNODE.TXT     List of Gravis BBS Nodes

SDKORDER.TXT     Gravis SDK Order form

PUPDATE.EXE      Used to fix .PAT files if necessary. READ Troubleshooting

DEVELOPR.TXT     Note to Developers

#      ....................... END of Chapter 4 ...........................

#5.0.0 *************** CHAPTER 5. - TROUBLESHOOTING ***********************

I copied the drivers on top of other drivers, but now the game says that

it can't find the sound card:

1) Most games require running a set-up program to tell it that you

have changed sound cards.  Try finding such a program and running

it.  It most likely will not show Gravis or Ultrasound in the list

of available drivers, so you will have to pick the sound cards which

you copied the AIL or DIGPAK drivers to.

2) Some games use modified AIL or DIGPAK drivers.  Most of the games

from Origin have been modified and therefore are not compatible with

these drivers.

3) Some games only use the AIL or DIGPAK drivers for certain

operations. For the rest of them operations it may try to access

the sound hardware directly.  These type of games will not work

with UltraMID and our AIL/DIGPAK drivers.

I have sucuessfully installed the drivers for a game, but sometimes the

music seems strange, as if its missing notes, or completely quiet:

1) Restart UltraMID with the -c option to preload instruments.  Most

music will take more than 256K of memory. 768K to 1Mb is the

optimal setup for the UltraSound card. (.e.g.)

C> ultramid -f

C> lh ultramid -c

2) If you have replaced any of your patches with non-Gravis patches,

the size may be different. The patch sets in ULTRAMID.INI are

configured so that your UltraSound card will be loaded with as

many patches as will fit into its available memory. Leaving 8K

for digital audio.  This means, if you try and load a patch

that is 4K bigger than the original, than there will only be a

4K buffer left instead of an 8K, and the drivers may not be

able to load correctly.

3) Your patches may be out of date.  A program called pupdate.exe is

available which will repair old or bad patches.  doo.pat is an

example.  run pudate like this:

pupdate -check_patch -force_update *.pat

in your ultrasnd\midi directory.  Pupdate will show you incorrect

patches and ask if it should update them.  Answer yes to all of

your patches EXCEPT BLANK.PAT.  BLANK.PAT is meant to be silent

and should not be corrected.  Pupdate will also create backups of

all of your patches as it modifies them.  acpiano.pat will become

acpiano.pa1.  acpiano.pa1 will become acpiano.pa2, etc.  The

install program uminstal.exe will check all of your patches during

the install.

Playback of digital audio sounds choppy.

1) Your disk or CD may be too slow, and the sound card plays sound faster

then your disk can retrieve data.

2) The UltraSound cannot play interleaved stereo data, and must use

your pc to seperate the left and right channel data before playback.

Slower 286 or 386 machines may not be able to keep up with high

speed, high fidelity audio.

3) The game is using audio buffers which are too small for

the UltraSound to effectively "double buffer" the data.  The

technique called "double buffering" provides clear sound without

clicks or pauses in audio, and smooth animations in video.

Optimal data sizes for UltraMID are multiples of 2K for stereo

data and multiples of 4K for mono data.  Some audio cards that

uses the DMA hardware for playback may call this technique

backfill DMA.  The UltraSound only uses the DMA hardware for data

transfers; the GF1 actually produces the sound.

When I run ULTRAMID I get a 'error loading patch' message.

1) You must be running the patches that came with the V2.x software.

This includes V2.0, and the latest V2.06 disk sets.

2) A patch may either be missing or corrupt. If you are unable to

determine which one simply reinstall the patches.

The games says that it can't run because there is not enough memory.

1) If you are using an Expanded Memory Manager (emm386, qemm, bluemax,

386max, etc) you can load the driver into high memory to provide

more conventional memory for your game.  If you Expanded Memory

manager supports the dos LH command, make sure the following

line is in your config.sys:

DOS=HIGH,UMB

If you are using DOS 5 or DOS 6, you can use the LH command to

load UltraMID into high memory.  For example: LH ultramid -m100 -c

will load ultramid into high memory, set the master volume to

100 (out of 127), and preload as many patches as will fit into

your UltraSound card.  If you are using a memory manager that does

not support the LH command (386max), then make sure the following

line is in your config.sys:

DOS=HIGH

NOTE: 386MAX uses its own command to load programs into upper

memory instead of LH.  Try something like the following:

c:\386max\386load prog=c:\ultrasnd\ultramid.exe

2) If you cannot use an Expanded Memory Manager but can use an

extended memory manager (himem.sys), then also include the

DOS=HIGH line in your config.sys so that DOS will be loaded

into upper memory.

3) If you can't fit UltraMID into high memory, try loading other

drivers or TSR's into high memory using DEVICEHIGH= instead

of DEVICE= in your config.sys and LH in your autoexec.bat files.

4) DOS 6 comes with a utility called MEMMAKER that will help you

optimize the use of memory on your machine.  QEMM comes with

a utility called optimize to help you optimize your memory.

BLUEMAX and 386MAX also have programs to help you maximize

the use of expanded and extended memory.

5) If there are other drivers in your system that you don't need

to play the game, try making a floppy disk with just the programs

you need to get your machine running.  For example,

FORMAT /s a:

will format a disk in the "A" drive and copy MS-DOS to it.  Then

create a minimal config.sys and autoexec.bat such as:

CONFIG.SYS:

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.exe ram

DOS=HIGH,UMB

SHELL=C:\DOS\COMMAND.COM C:\DOS\ /E:768 /p

files=20

buffers=20

LASTDRIVE = D

AUTOEXEC.BAT:

PATH C:\DOS;C:\ULTRASND;C:\UTILS

SET ULTRASND=220,1,1,11,7

SET ULTRADIR=C:\ULTRASND

SET BLASTER=A220 I7 D1 T1

LH C:\MOUSE\MOUSE

LH ULTRAMID -m100

rem if you have the memory available, and you are not using

rem the -c option, Smart Drive improves the performance

rem for loading patches tremendously.  I have  8Mb on my

rem machine, so I usually set up a 4Mb disk cache.

LH C:\DOS\smartdrv /Q 4096 0 c

rem If you have dos 6.2, use the following smartdrv instead:

LH C:\DOS\smartdrv /X /Q 4096 0

These examples are for experienced DOS users, and probably won't

work with your machine without some modification.  Please consult

your DOS manual or a book that covers memory management to get more

hints on how to manage PC memory.

6) If you can't get UltraMID to load into high memory and you

don't need an EMS memory, then load emm386 with the option noems.

This will give you 64K more room for UltraMID to load.

#      ....................... END of Chapter 5 ...........................

#                                  THE END