01

ESC  -  Emergency Shutdown.

This  function key causes the reactor to  execute

an  Emergency  SCRAM.   The rods are sent to  the

bottom,  and  the make-up feed pump speed is  in-

creased  to  100% and the make-up  feed  flow  is

directed to the primary loop.   The primary drain

valve  is shut and the emergency cooling pump  is

started.   This function key will safely shutdown

the  reactor  and minimize damage to the  reactor

core.  It should be used in any emergency.

02

F1  -  SCRAM.

This function key shuts down the reactor by send-

ing  the rods to the bottom and by  shutting  the

primary drain valve.  This function key should be

used  while conducting a normal shut-down of  the

reactor.  Care must be taken to prevent damage to

the  reactor  core due to low  pressurizer  level

when using this function key.

03

F2  -  Primary Drain Valve.

This  function  key  controls the  primary  drain

valve which is used to maintain pressurizer level

within the required range on 50-85%.   This  fun-

ction  key alternately opens and shuts the  drain

valve.

04

F3  -  Turbine Trip Valve.

This  function key controls the position  of  the

turbine  trip  {bypass}  valve which is  used  to

prevent  liquid water from entering the  turbine.

This  valve  should be open  {Turb  Trip  warning

(Yellow)  on} whenever heat exchanger temperature

is  below  100C.   Care should be  taken  not  to

exceed  the condenser pressure/temperature limits

when operating this valve.

05

F4  -  Generator Field Control.

This  function key controls the  generator  field

and  speed control system.   The generator  field

must  be  on for production of power.   When  the

generator field is on, the turbine speed is auto-

matically regulated to 3,600 RPM to maintain  the

electric  current frequency at 60 Hz.   This fun-

ction  key  provides the only direct  control  of

turbine speed.

06

F5  -  Automatic Control.

This  function  key controls  the  automatic  rod

height  control system which is designed to main-

tain  the  heat exchanger  temperature  near  the

control set point temperature.   This system fun-

ctions  only  above 500C reactor temperature  and

will  maintain the temperature within  the  range

specifed  by  the Bias setting below the  control

set point temperature.

07

F6  -  Automatic Set-point.

This  function  key controls  the  automatic  set

point   computer.    This  computer  adjusts  the

control set point and primary and secondary cool-

ant  pumps to maintain the production rate  above

the demand.   This computer should generally only

be used when the demand is above 1700 KW.

08

F7  -  Pressurizer Heaters.

This  function key controls the pressurizer  hea-

ters which are used to maintain a steam bubble in

the pressurizer.   The steam bubble creates pres-

sure  on the primary loop which prevents  boiling

in the reactor core,  which could cause fuel ele-

ment failure or meltdown.    The pressurizer hea-

ters  must be on any time the reactor temperature

exceeds 100C.    Primary coolant pressure must be

at  least 2X reactor core temperature to  prevent

damage to the reactor core.

09

F8  -  Lube Oil Pump.

This  function key controls the operation the the

turbine and generator lubricating oil circulation

pump.   This  pump provides high pressure oil  to

the bearings of the turbine and  generator.   The

Lube  oil pump must be on whenever the turbine is

operating.

10

F9  -  Condenser Circulating Pump

This   function   key  controls   the   condenser

circulating  pump.   This pump  provides  coolant

flow  for the condenser.   The exhaust steam from

the  turbine  is condensed into  water,  and  the

condensate  is pumped to the steam generator  for

recycling.   This pump should be operating  when-

ever the secondary temperature is above 212F.

11

F10  -  Auxiliary Power.

This  function key controls the  auxiliary  power

source  which  is used to provide electricity  to

the  reactor plant whenever the generator is  not

functioning.   Auxiliary power is required during

start up and shutdown of the reactor  plant,  and

should be on at all times.

12

Left Arrow  -  Plant Display.

This  function  key swithes the  display  between  the

graphic  and  numeric displays.   The numeric  display

shows the current value of the specific system values.

This display also shows the last 5 log entrys,  if the

Autolog-Screen  function  is  selected.   The  graphic

display  shows  a schematic of the plant and  a  chart

displaying  the  last  30 minutes  of  the  four  most

important indicators.   Pumps,  valves, and levels are

indicated  on the schematic.   Pumps that are off  are

indicated  by  an  '+',  pumps that  are  running  are

indicated  by an 'O',  and pumps that have failed  are

indicated  by an 'X'.   Shut valves are indicated by a

solid  white circle.   Open valves are indicated by  a

black stripe indicating the direction of flow  through

the valve.  Fluid levels are indicated by white blocks

in the various vessels and storage tanks.  Turbine and

generator  speeds  are indicated by the angle  of  the

throttle  vanes  in the symbols.   A  horizontal  vane

indicates  speed  0,  and  a verticle  vane  indicates

normal operating speed.

13

Up Arrow  -  Alarm Silence.

This  function key switches the audible alarm  on  and

off.   The alarm sounds whenever there is a warning or

danger indication on the alarm panel.

99

########################################################################################################################################################################################################################################