SSTFMAT Program Version 3.36.2            24 Jan 91

RE: Notes on Using the SSTFMAT.MO Configuration File

for Magneto Optical Devices

The Program SSTFMAT.EXE will try to identify MO Devices thru the

use of a User Editable File called SSTFMAT.MO.  The Device Vendor ID

and Product Name are compared with the Text in this file and the

Specific Symbol (S,H,M,R) following a match will let the program

know how to handle that device.

The SSTFMAT.MO File looks like this:

-------Start of File------

SONY    SMO-C501-00     S

HP      S6300.650A      H

MaxOptixTahiti 1        M

RICOH   RO-5030E        R

-------End of File--------

As New MO Devices are Released, the User may add those Devices Names to

the MO Compatibility of the Formatter Program by Adding the Device's

Vendor ID and Product Name in ASCII to this File.  Each Identification

is followed immediately (ie. no Spaces except those from the Dev Name)

by an "S", "H", "M", or an "R" depending on the way the Device handles

the below actions.

Vendor ID            |   SONY  |   HP    |  RICOH  | MAXOPTIX

|   (S)   |   (H)   |   (R)   |   (M)

|         |         |         |

-----------------------------------------------------------------

Page Header Size     |    0    |    0    |    8    |    8

(Bytes)              |         |         |         |

-----------------------------------------------------------------

Mode Sense/Select    |   01h   |   01h   |   01h   |   01h

Pages Uses           |   20h   |   20h   |   20h   |   03h

-----------------------------------------------------------------

Format Mode for      |   03h   |   03h   |   01h   |   N/A

Defect Management    |         |         |         |

-----------------------------------------------------------------

Format CDB           |   03h   |   03h   |   00h   |   00h

(3rd Byte)           |         |         |         |

-----------------------------------------------------------------

Defect List Length   | 3 bytes | 3 bytes | 3 Bytes | 4 Bytes

(4 Bytes per element)|  used   |  used   |  used   |  used

-----------------------------------------------------------------

Reports Interleave   |   NO    |   NO    |   NO    |   YES

-----------------------------------------------------------------

Reports ARRE         |   NO    |   NO    |   YES   |   NO

-----------------------------------------------------------------

(NOTE: For our Purposes here, Sony and HP styles are the same and

may be used interchangeably.  Or if you don't know your device's

parameters but know the vendor name, it may be easier to use the

device ID you are fimilar with).

If a New Sony MO Device with the following Inquiry Data was Released

Vendor ID    = "SONY   "

Product Name = "SMO-Q501-99     "

and it happened to match the previous SONY Specification in the Above

Table, you could add it to your MO Config File as such:

-------Start of File------

SONY    SMO-C501-00     S

HP      S6300.650A      H

MaxOptixTahiti 1        M

RICOH   RO-5030E        R

SONY    SMO-Q501-99     S

-------End of File--------

From this point onward, your SSTFMAT Program will now recognize this

Device as an MO Device with and treat it according to its particular

specifications.

If the MO Device does not match one of the above, it will not work

with all options and will be treated by the program as simply another

DASD Device.

Dept of Engineering

Columbia Data Products, Inc.