VOICE BLASTER ADDENDUM

April 2, 1993

Thank you for purchasing the VOICE BLASTER SOFTWARE kit from

COVOX, Inc.  In order to help you through the initial steps in

setting up the VOICE BLASTER we have included the following

information.

You should have the following items available....

1.   COVOX E/Q module & headset.

2.   2 high density disks marked Disk 1 & Disk 2.

DISK 1 includes

Install programs for

Voice Blaster Software for Windows 3.1

Voice Blaster Software for DOS

Covox Audio Toolkit for DOS

DISK 2 includes

Turtle Beach WAVE editor for Windows 3.1

Audio File voice annotation software for Windows 3.1

3.   VOICE BLASTER owners manual

4.   Warranty card

By sending in (or phoning in) your warranty card information

along with the serial number of your software, you will receive

FREE OF CHARGE the following items....

1.   Toll free technical support (1-800)

2.   Quarterly news letter on new sound technologies

INSTALLING THE VOICE BLASTER SOFTWARE

STEP 1 (Hardware and Software)

Turn your computer and peripherals off.  Remove the E/Q module from

the box and attach it to the parallel printer port on your

computer. (The E/Q module will not interfere with the normal

operation of your printer.) If applicable, attach your printer to

the E/Q module.

STEP 2 (Hardware Setup)

Connect the Red male (3.5mm), microphone plug from the Covox headset

into the female (3.5mm), jack of the Covox E/Q module.  Next, connect

the male Black (3.5mm), plug of the Covox E/Q module to the MIC IN

jack of your sound card.

STEP 3 (START UP FOR DOS)

Place disk 1 in your disk drive and type INSTALL.  Follow the

screen prompts to install the VOICE BLASTER software and the

Covox Audio File Toolkit for DOS onto the hard drive.

STEP 4  (DOS)

To install the Windows 3.1 software, follow directions in STEP 7.

STEP 5  (DOS)

Proceed to the VOICE BLASTER for DOS subdirectory (Default is

C:\COVOX\VB40) and type VBCONFIG.  For more information on

VBCONFIG consult the VBCONFIG addendum.

STEP 6  (DOS)

After you have configured the VOICE BLASTER software you may run

the program by typing VB [Enter Key].  Refer to the owners manual

for more information on running VOICE BLASTER Software for DOS.

You may now also start the Covox Audio Sound Toolkit by

typing STK.EXE at the DOS prompt.

STEP 7  (WINDOWS)

If you wish to install the Windows software onto your computer,

you must first start Windows 3.1.

STEP 8  (WINDOWS)

A.  Voice Blaster

Select the option RUN under FILE and proceed to change

to the \VB40WIN Sub-directory on disk 1 and follow the

prompts.

B.  Wave for Windows 3.1

Select the option RUN under FILE and Proceed to change

to the \WAVE Sub-directory on disk 2.

C.  AudioFile for Windows 3.1

Select the option RUN under FILE and Proceed to change

to the \AUDIO /Sub-directory on disk 2.

STEP 9 (FOR ALL COVOX STUFF)

Send us your warranty card.

STEP 10 (FOR ALL COVOX STUFF)

HAVE FUN !!!!!!!!!!!!