c1stripe.doc

$Date:$

c1stripe is a utility for striping tapes with SMPTE timecode, using

the Yamaha C1 music computer.  The utility also lets you read

timecode off a tape, displaying the time and format.

This file describes how to use c1stripe.  The utility is very simple,

so you may not even need to read this.  If you encounter trouble,

however, please check here before calling for technical support.

We've include some tips about using timecode in general, like

avoiding noise reduction and using edge tracks.

c1stripe is Copyright (c) 1993 by Twelve Tone Systems, Inc.  All

rights reserved.

--- RUNNING THE PROGRAM

Like Cakewalk Professional/C1 3.0, c1stripe uses the CAPI driver

supplied on the Cakewalk Professional distribution diskette.  Make

sure you have CAPI loaded before running c1stripe.  If you forget,

c1stripe will let you know.

NOTE: CAPI is provided by us solely for use with c1stripe.  Cakewalk

Professional 5.0 does NOT use CAPI.  Running CAPI during a session

where you don't need to run c1stripe won't interfere with using

Cakewalk Professional 5.0, but it will waste a little memory.

To start c1stripe, simply enter its name at the DOS command prompt:

c1stripe

--- SCREEN LAYOUT

The program screen is divided into two main sections.  The larger

portion on top is simply a menu.  It shows what commands are

available.  The key to press to select a command is displayed in

parentheses () before the command.

The bottom portion is the "status window".  The top line of the

status window shows some parameters related to striping a tape:

The current time, which is updated while you are striping.

The offset (not included in the current time display).

The format.

A general status indicator.

The bottom line of the status window shows some parameters related to

reading timecode off a tape.

The current time, which is updated while you are reading.

The format.

--- STRIPING A TAPE

When you start c1stripe, the format is set to 30 frame non-drop and

the offset is set to 00:00:00:00.

* If you require another format, press F to be prompted to enter a

new format.

* If you require another offset, press O to be prompted to enter a

new offset.

Note that your changes will appear in the status window.

To begin striping, start the tape deck in record mode.  Allow a

little leader, then press S to start striping.  When you have enough

timecode laid down, press S again to stop striping.

PROBLEMS?

- Did you remember to connect the "Tape Out" jack on the back of the

C1 to the input of your tape deck?

- Did you remember to turn on the record mode on the tape deck?

- Did you have a nice strong level set on the tape deck (but not so

strong the needle is off the meter)?

- Did you remember to turn OFF noise reduction?  Noise reduction

circuitry like Dolby and dbx usually wreaks havoc with timecode of

any sort.  Make sure NR is disabled while recording and playing back.

If you have a mult-track cassette deck, see if it has special

features for timecode tracks.

- Did you put timecode on an edge track?  Use track 1 or track 4 (if

you have a 4-track deck).  Avoid putting an audio part with strong,

pulsing dynamics on the track next to the timecode track.

--- READING A TAPE

If you wish, you can check a tape by reading timecode off it.

Press R to start reading, and start your tape deck.  When the tape

reaches the section with timecode, this should register about a

second later in the status window.  You should see a value appear for

the format and you should notice the hh:mm:ss:ff display rolling

over.

Press R again to stop reading.

Problems?

- Did you remember to connect the output of the tape deck to the

"Tape In" jack on the back of the C1?  Is the output from the track

that has timecode?

- Do you have the volume turned up on the tape deck?

- Is noise reduction disabled?  See "Problems?" under "Striping."

- Did you use an edge track?  See "Problems?" under "Striping."