Workstation for OS/2 Issues

Remote Workstations Not Supported

The files needed to install and use remote

workstations are not included in this release. The

RIPL files will be available at a later date.

Frame Type Default Is Different

The default frame type for Ethernet ODI drivers (such

as NE2000.SYS) has changed. In NetWare 3.x and

NetWare 2.x, the default for Ethernet drivers was set

to Ethernet_802.3. In NetWare 4.0, the drivers

default to Ethernet_802.2.

Routers on the network must also support the frame

type or your workstation often might not get a

connection. Some routers currently on your network

might not support the new Ethernet_802.2 default.

Your workstation might not be able to connect to a

network expecting Ethernet_802.3 if you use the

Ethernet_802.2 default.

To eliminate a potential conflict, define both frame

types (Ethernet_802.2 and Ethernet_802.3) on your

network.

For the workstation, define frame types in NET.CFG.

Include a line similar to the following, replacing

NE2000 with the name of your ODI driver:

link driver ne2000

frame ethernet_802.2

frame ethernet_802.3

The first frame defined is the only one used for the

initial Get Nearest Server request. Therefore, if you

have some servers that are using only one frame type,

such as Ethernet_802.3, put that frame type first.

That way your workstation can make a default

connection to those servers.

For more information about frame types, see Chapter

5, "Network Boards and Drivers," of NetWare

Workstation for OS/2.

Logging In From Global DOS Not Supported

Don't log in from a global DOS session. Instead, log

in from an OS/2 session and then switch to the DOS

session.

Except for this login difference, you can still set

up drive mappings for global sessions as instructed

in Chapter 3, "Network Access From Virtual DOS and

MS Windows" of NetWare Workstation for OS/2.

Login Utilities No Longer Shipped with the OS/2 Requester

This version of the NetWare Requester for OS/2 does

not include any NetWare utilities. Previous versions

of the requester included a few utilities (Login.exe,

map.exe, etc.) which were copied to the OS/2

workstations hard drive. This enabled OS/2 users to

login to NetWare servers which did not have the OS/2

utilities installed on them.

While NetWare 3.11 included the OS/2 utility

diskettes, it did not have them automatically

installed to the SYS:LOGIN\OS2 and SYS:PUBLIC\OS2

areas on the server. Later releases of NetWare

automatically install the OS/2 utilities so all OS/2

workstations on the network will have full access to

them.

The OS/2 workstations therefore do not need a copy of

these utilities on the hard drive to gain access to

the network. If you lose connection to the file

server or attach to a file server without OS/2

utilities NWTOOLS.EXE, in the Novell Icon on the OS/2

desktop, will allow you to attach and map to servers

without access to OS/2 utilities.  Another option is

to copy the basic utilities such as LOGIN.EXE, and

MAP.EXE to your local disk.

Editing Login Scripts for DOS Sessions

If a user logs in from a private DOS session running

NETX.EXE, a NetWare 3.11 user login script will

execute if it exists.

This login script is called LOGIN and is located in

the SYS:MAIL\userID directory. You can edit it with

a text editor or a bindery utility from previous

versions of NetWare such as SYSCON.

If the server is a new NetWare 4.0 server rather

than

an upgraded server, login scripts and the

\MAIL\userID directory don't exist.

Login scripts in NetWare 4.0 are properties of the

user object. You can create the login script on a

local drive and use the /S option when you log in.

For example, create the LOGIN.DOS login script on

the

C:\ drive and type the following to log in:

LOGIN /S C:\LOGIN.DOS

Run NetWare 4.0 Utilities in DOS_STARTUP_DRIVE Sessions

In a private DOS session booted from a real DOS

kernel

(i.e., a session with DOS_STARTUP_DRIVE set) running

NetWare 4.0 DOS Requester, you may receive errors

running some of the utilities.

Solve the problem by being logged in from an OS/2

session whenever you use a utility from the

DOS_STARTUP_DRIVE session.

You must be logged in to the same server from the

OS/

2 session as you are logged in to from the

DOS_STARTUP_DRIVE session.

For example, suppose you logged in to server SALES

from the DOS_STARTUP_DRIVE session. For some

utilities to function, you would also need to log in

to server SALES from an OS/2 session.

As long as your OS/2 session is logged in, your

utilities will function properly. This condition

particularly applies to the PCONSOLE, NETUSER,

FILER,

NDIR, and NPRINT utilities.

Request Retries Default Incorrect

The Workstation for OS/2 manual and online help

incorrectly document the default for the "NetWare

Requester request retries" NET.CFG setting. The

correct default is 10, not 20.

If you are running slower ODI drivers (lower than 10

Mbits/sec), then change the default to 20, this

helps

to minimize transmission errors.