Chapter 6

Please note: The information contained in this documentation is preliminary and subject to change without notice.

Concepts of Line Devices

Using Full Telephony is, in essence, using its two device classes: the line device class and the phone device class. Any physical telephonic device recognized by TAPI can be abstracted into one of these two classes, and can thus be treated in a consistent and reliable manner.

This chapter describes the role that TAPI's line functions play and explains how developers can use them. Because there are many functions that work similarly for both for line devices and phone devices, you will notice similarities between the material in this chapter and descriptions of phone devices in later chapters.

## What is a Line Device?

A line device is a physical device such as a fax board, a modem, or an ISDN card that is connected to an actual telephone line. Line devices support telephonic capabilities by allowing applications to send information into a telephone network. A line device contains a set of one or more homogeneous channels that can be used to establish calls.

Within TAPI applications, a line device is the logical representation of a physical line device. Although "line" often connotes something with two endpoints, it is possible to abstract a line device to a single point because TAPI views it only as a point of entry to the line that leads to the switch.

µ §

Lines are used as entry points for Telephony applications

Although the three lines in the preceding illustration are composed of different hardware and used for different functions, they are abstracted to the same device type and governed by the same rules. Note that the telephone represents not a phone device but a line device used for voice calls. When using this line device for incoming or outgoing calls, the application would also need to open and control an instance of the phone-device class, which is described in detail in later chapters.

TAPI requires that every TAPI-capable line device support all of Basic Telephony. If an application needs to use capabilities beyond those of Basic Telephony (namely Supplementary or Extended Telephony), it must first determine the line device's capabilities, which can vary according to network configuration (client versus client-server), hardware, service-provider software, and the telephone network. The function lineGetDevCaps returns the Supplementary and Extended capabilities of a given line device in a data structure of the type LINEDEVCAPS.

## Relationships Between Lines, Channels, and Addresses

In POTS, exactly one channel exists on a line, and this is used exclusively for voice. With ISDN, at least three (and as many as 31) channels can exist on a line simultaneously. Currently, most TAPI functionality involves POTS applications that handle a single line using its single channel because ISDN hardware is not yet widespread. In POTS. an application that wants to transmit data would communicate over one line, and a voice application (such as the Call Manager provided with TAPI) would communicate over another line—both of these applications could use the same line, but if so, not at the same time.

In general, one line has exactly one address (telephone number). In cases where lines carry two or more channels, each channel usually has its own address, which means that a line has as many addresses as it has channels. TAPI assigns Address IDs to these different addresses to make it easier to manipulate them.

The following illustration shows the usual relationship between lines, channels, and addresses. The number of addresses (and thus Address IDs) is equal to the number of channels carried by the line. Other kinds of lines such as PRI-ISDN (not shown in the illustration) can carry even more channels, and thus supply more usable addresses to the application.

µ §

The assignment of addresses and Address IDs to line devices

### Multiple Addresses on a Single Channel

Some installations support the assignment of more than one address to a single channel. On POTS lines, multiple addresses are made possible by various systems, such as "distinctive ringing" or "DID" (direct inward dialing), which are extra-fee services provided by the telephone company.

Many large corporations use DID for incoming calls. Before a call is connected, its destination extension number is signaled to the PBX, which causes the extension to ring instead of the operator's phone. An example of distinctive ringing in a private home would be if the parents used one address, the children another, and a fax machine a third. Because only one line connects the house to the telephone network, all phones ring when a call appears. With distinctive ringing, the people know who the incoming call is meant for, and the fax machine answers its calls by recognizing its own ringing style.

In ISDN, the various B channels do not normally have separate addresses. Because these B channels are usually on the same address, it is the service provider (and not the application or a person who has dialed the number) that assigns calls to these channels.

### ISDN Subaddresses

Subaddressing is a capability provided on ISDN lines that allow more information than just a single telephone number to be used when dialing. This additional information may specify an individual telephone extension to ring or, in a computing environment, a particular application to be alerted. Other parameters that can be passed can describe the required aspects of a requested modem connection, such as rate and timing.

## Address Configurations, Functions, and Messages

Each line device is assigned one or more addresses. An address corresponds to a telephone directory number, and it is actually assigned twice: First, by the telephone company at the switch, and second, by the user while configuring the local system. If a telephone number is changed at the switch, the user will normally need to assign the new number at the local system, although some systems may be sophisticated enough to perform the reassignment without human control.

After addresses have been assigned to lines, TAPI assigns address IDs to addresses. An address ID is a number between 0 and the number of addresses on the line minus one. Because each address depends on its line to exist, the address's ID is meaningful only in the context of the associated line device. For this reason, an address name consists of not only the address ID, but also an identifier of the line.

### Address Configurations

The relationship of an address to a line (and to other local addresses) is known as its configuration. The network or switch can configure address-to-line assignments in several ways. The main types of address configurations recognized by TAPI are:

1.	Private. The address is assigned to one line device only. An inbound call for this address is offered (the switch informs the desktop computer of an incoming call) at one line device only.

2.	Bridged. A bridged address is a single address assigned to more than one line device. An incoming call on a bridged address will be offered on all lines associated with the address. The network of lines connected together is known as the "bridge." Different variations of bridged behavior are possible:

·	Bridged-Exclusive. Connecting one of the bridged lines to a remote party causes the address to appear "in use" to all other members of the bridge.

·	Bridged-New. Connecting one of the bridged lines to a remote party does not preclude the other lines from using the bridged address to answer or make calls. However, a new call appearance is allocated to another of the connected lines.

·	Bridged-Shared. If one line is connected to a remote party, other bridged lines that use the address automatically enter into a multi-party conference call on the existing call.

3.	Monitored. The line provides an indication of the busy or idle status of the address, but the line cannot use the address for answering or making calls.

#### Address-Related Functions and Messages

Different line devices may have different capabilities, and so may their addresses. Switching features and capabilities may be different for different addresses. An application calls the function lineGetAddressCaps to determine the telephony capabilities of each address, and then receives this information in a data structure of the type LINEADDRESSCAPS.

(In a similar way, an application can call LineGetDevCaps for a line device to determine the number of addresses assigned to the line, as well as other information.)

TAPI's device-query capability and status and event reporting mechanisms give an application the information it needs to manage the different kinds of bridged-address arrangements. For example, the application can determine whether a bridged station has answered a call by tracking the status changes and call-state event changes on the address. (Call states are explained later in this document.)

Normally, addresses on a line device are identified by their address IDs. However, TAPI lets applications that make outgoing calls use alternate address types for the originating address, such as dialable format (see below), or naming mechanisms specific to a given service provider. This could be accomplished via API extensions that are based on switch-assigned station IDs. A useful function for this purpose is lineGetAddressID, which retrieves the ID of an address specified in an alternate format.

## Address Formats

Each address ID corresponds to a directory number through which incoming calls can be placed to the address. Similarly, making outbound calls typically requires that a directory number—the address—be supplied to identify the party being called.

### Storing Numbers in Electronic Address Books

Many users choose to dial people, fax machines, bulletin boards, and other entities by selecting their names from an address book. The actual number that is dialed depends on the geographic location of the user and the way the line device to be used is connected. For example, a desktop computer may have access to two lines, one connected to a PBX, the other to the telephone company's central office. When making a call to the same party, different numbers may have to be used. Or, a user may make calls from a portable PC and want to use a single, static address book even when calling from different locations or telephony environments. TAPI's address translation capabilities let the user inform the computer of the current location and the desired line device for the call. TAPI then handles any dialing differences, requiring no changes to the user's address book. An application uses the function lineTranslateAddress to convert an address from the canonical address format to the dialable address format (see the next section).

A related topic is the handling of international call progress, a procedure driven by DTMF (Dual Tone Multiple Frequency) tones. The call-progress tones involved in making a call may include all tones needed to complete the call via the path leading from the originating switch and ending at the remote party's switch. Because the meanings of call-progress tones typically vary from country to country, the service provider must know what call progress to follow when making an international outbound call.

### The Canonical Address Format and the Dialable Address Format

To help manage making calls with directory numbers, TAPI recognizes two address formats: canonical address format, and dialable address format.

#### Canonical Addresses

The canonical address format is intended to be a universally constant directory number. For this reason, numbers in address books are best stored using canonical format. A canonical address is an ASCII string with the following structure:

+ CountryCode Space [(AreaCode) Space] SubscriberNumber 	| Subaddress ^ Name CRLF ...

The components of this structure are given in the following table:

+	Equivalent to ASCII Hex (2B). It indicates that the number that follows it uses the canonical format.

CountryCode 	A variably sized string containing one or more of the digits 0-9. The CountryCode is delimited by the following Space. It identifies the country in which the address is located.

Space	is exactly one ASCII space character (0x20). It is used to delimit the end of the CountryCode part of an address.

AreaCode 	A variably sized string containing zero or more of the digits 0-9. AreaCode is the area code part of the address and is optional. If the area code is present, it must be preceeded by exactly one ASCII left parenthesis character (0x28), and be followed by exactly one ASCII right parenthesis character (0x29) and one ASCII Space character (0x20).

SubscriberNumber 	A variably sized string containing one or more of the digits 0-9. It may include other formatting characters as well, but shall not include any of the dialing control characters described in the Dialable Address Format:

AaBbCcDdPpTtWw*#!,@$?;

The subscriber number may also not contain the left parenthesis or right parenthesis character (which are used only to delimit the area code), nor may it contain the “|”, “^”, or CRLF characters (which are used to begin following fields). Most commonly, non-digit characters in the subscriber number would include only spaces, periods (“.”), and dashes (“-”). Any allowable non-digit characters which appear in the subscriber number will be omitted from the DialableString returned by the lineTranslateAddress function, but will be retained in the DisplayableString.

| 	ASCII Hex (7C). If this optional character is present, the information following it up to the next + | ^ CRLF, or the end of the canonical address string is treated as subaddress information, as for an ISDN subaddress.

Subaddress 	A variably sized string containing a subaddress. The string is delimited by + | ^ CRLF or the end of the address string. During dialing, subaddress information is passed to the remote party. It can be such things as an ISDN subaddress or an email address.

^	ASCII Hex (5E). If this optional character is present, the information following it up to the next CRLF or the end of the canonical address string is treated as an ISDN name.

Name 	A variably sized string treated as name information. Name is delimited by CRLF or the end of the canonical address string and may contain other delimiters. During dialing, name information is passed to the remote party.

CRLF 	is ASCII Hex (0D) followed by ASCII Hex (0A), and is optional. If present, it indicates that another canonical number is following this one. It is used to separate multiple canonical addresses as part of a single address string (inverse multiplexing).

For example, the canonical representation of the main switchboard telephone number at Microsoft Corporation would be:

+1 (206) 882-8080

#### Dialable Addresses

The dialable address format describes a number that can be dialed on the given line. A dialable address contains part addressing information and is part navigational in nature. Any input string which does not begin with a “+” character is presumed to be not in canonical format and therefore in dialable address format, and is returned to the application unmodified. A dialable address is an ASCII string with the following structure:

DialableNumber | Subaddress ^ Name CRLF ...

where:

DialableNumber	digits and modifiers 0-9 A-D * # , ! W w P p T t @ $ ? ; delimited by | ^ CRLF or the end of the dialable address string.

Within the DialableNumber, note the following definitions:

0-9 A-D * # 	ASCII characters corresponding to the DTMF and/or pulse digits.

! 	ASCII Hex (21). Indicates that a flash is to be inserted in the dial string.

P p	ASCII Hex (50) or Hex (70). Indicates that pulse dialing is to be used for the digits following it.

T t	ASCII Hex (54) or Hex (74). Indicates that tone (DTMF) dialing is to be used for the digits following it.

,	ASCII Hex (27). Indicates that dialing is to be paused. The duration of a pause is device specific and can be retrieved from the line's device capabilities. Multiple commas can be used to provider longer pauses.

W w	ASCII Hex (57) or Hex (77). An uppercase or lowercase W indicates that dialing should proceed only after a dial tone has been detected.

@ 	ASCII Hex (40). Indicates that dialing is to "wait for quiet answer" before dialing the remainder of the dialable address. This means to wait for at least one ringback tone followed by several seconds of silence.

$ 	is ASCII Hex (24). It indicates that dialing the billing information is to wait for a "billing signal" (such as a credit card prompt tone).

? 	is ASCII Hex (3F). It indicates that the user is to be prompted before continuing with dialing. The provider does not actually do the prompting, but the presence of the "?" forces the provider to reject the string as invalid, alerting the app to the need to break it into pieces and prompt the user in-between.

; 	is ASCII Hex (3B). If placed at the end of a partially specified dialable address string, it indicates that the dialable number information is incomplete and more address information will be provided later. ";" is only allowed in the DialableNumber portion of an address.

| 	is ASCII Hex (7C), and is optional. If present, the information following it up to the next + | ^ CRLF, or the end of the dialable address string is treated as subaddress information (as for an ISDN subaddress).

Subaddress 	is a variable sized string containing a subaddress. The string is delimited by the next + | ^ CRLF or the end of the address string. When dialing, subaddress information is passed to the remote party. It can be an ISDN subaddress, and email address, etc.

^	is ASCII Hex (5E), and is optional. If present, the information following it up to the next CRLF or the end of the dialable address string is treated as an ISDN name.

Name 	is a variable sized string treated as name information. Name is delimited by CRLF or the end of the dialable address string. When dialing, name information is passed to the remote party.

CRLF 	ASCII Hex (0D) followed by ASCII Hex (0A). If present, this optional character indicates that another dialable number is following this one. It is used to separate multiple canonical addresses as part of a single address string (inverse multiplexing).

The function lineTranslateAddress is used to translate an address from canonical format to dialable format. An application cannot use this function to dial a number but it can use it to display for verification a number that could be dialed. Also, from the country code and area code it can compute the local time at the destination address.

When using lineTranslateAddress, the application specifies the line device and a canonical address, and the function returns the dialable number and the country code. Because the line device may have specific dialing requirements, it is part of the context needed for an accurate translation.

The user's geographic location also plays a role in address translation. This is done via a small control application that is provided as part of Windows. The Subaddress and Name fields, if present in the address, are unmodified by the translation. Alphabetic characters, as in 1-800-FOR-TAPI, are not translated due to the different standardizations in use in different countries.

Although an application can use dialable addresses returned by lineTranslateAddress, it is not limited to them. It is free to compose its own dialable numbers.

## Initialization and Shutdown of Line Functions

For an application to use any of TAPI's 25 basic line functions or 30 supplementary line functions, it needs a connection to TAPI.DLL, through which it can receive messages. The application establishes this connection with a process known as initialization, using either the lineInitialize or the phoneInitialize function. One of the parameters passed to TAPI in these function calls is a pointer to the application's callback function. After initialization, TAPI uses this pointer to notify the application of the asynchronous completion of functions.

#### Notes about Initialization

·	The initialization functions are not device-related. When an application calls an initialization function, TAPI does not act on an actual line or phone device or an abstraction thereof.

·	The first time an initialization function is called in a telephony session, TAPI also sets up the telephony environment. Among the tasks it performs are loading TAPI.DLL and loading the device drivers specified in TELEPHON.INI. In addition, the communication link described above is established between TAPI and the calling application. After environment-related tasks are completed, calling an initialization function does no more than pass a pointer to a callback function.

·	The INIFILECORRUPT error can be returned whenever TAPI.DLL determines that the TELEPHON.INI contains an invalid entry. When this error occurs (in lineInitialize and phoneInitialize, or another function), the user should identify and resolve the problem. It may be necessary to rebuild the entire INI file, which can be done through the Telephony Control Panel.

For example, the LINEERR_NODRIVER ("the driver was not installed") error indicates either that a service provider that was previously installed can no longer be found or that some subsidiary component of a service provider (such as a vxd) cannot be found. When this error is encountered, the application should advise the user to correct the problem with the Driver Setup function within the Telephony Control Panel.

·	Although each application needs only one callback function, nothing prevents it from calling an initialization function more than once to specify other callback functions.

·	The functions lineInitialize and phoneInitialize are the only two callbacks defined in TAPI. This means that they are the only two functions that can pass the pointer of a callback function to TAPI. All subsequent TAPI functions, when completing asynchronously, result in calls to this callback function. (There is one exception: In Simple Telephony, the function tapiRequestMediaCall uses Windows messages to return call statuses to the application that is placing the call.)

·	Both lineInitialize and lineShutdown (and the corresponding phone functions) operate fully synchronously. That is, these functions always return a success or failure indication, never an asynchronous Request ID. They cannot finish asynchronously because, at the time they are called, TAPI has no pointer to use for sending messages to the application. Not until they complete is this pointer (to a callback function) available to TAPI.

Upon completion, the lineInitialize function returns two pieces of information to the application: an application handle and the number of available line devices.

·	The application handle represents the application's usage of TAPI. That is, to TAPI, it represents the application. TAPI functions that use line or call handles (explained later in this document) do not require the application handle, because this handle is derived from the specified line, phone, or call handle.

·	The second piece of information returned by lineInitialize is the number of line devices available to the application via TAPI. Line devices are identified by their device identifier (device ID). Valid device IDs range from zero to the number of line devices minus 1. For example, if lineInitialize reports that there are two line devices in a system, the valid line-device IDs are 0 and 1.

Once an application is finished calling TAPI's line functions, it invokes lineShutdown and passes its application handle to terminate its usage of TAPI. This allows TAPI to free any resources assigned to the application.

## Calls

Unlike line devices and addresses, calls are dynamic. A call represents a connection between two (or more) addresses. One address is the originating address (the caller), which is the address from which the call originates, and the other is the destination address (the called), which identifies the remote end point or station with which the originator wishes to communicate.

Zero, one, or more calls can exist on a single address at any given time. A familiar example of multiple calls on a single address is call waiting: While having a conversation with one party, a subscriber with call waiting is alerted that another party is trying to call. The subscriber can flash the phone to answer the second caller (which automatically places the first party on hold), and then toggle between the two parties by flashing. In this example, the subscriber has two calls on one address on the line. Since the person at the telephone handset can be talking to only one remote party at a time, only one call is active per line at any point in time. The telephone switch keeps the other calls on hold. With a line able to encompass more than one channel, different configurations can allow multiple active calls on a line at the same time.

TAPI identifies a specific call by means of the call's handle, and TAPI.DLL assigns call handles as required. One call handle exists for every call owned or monitored by an application, and an application can obtain call handles in a number of well defined ways. Certain TAPI functions create new calls. As they do so, they return any new call's handle to the application. Sometimes, call handles are provided unsolicited in callbacks made to the application from TAPI.DLL, as is the case with inbound calls or calls being handed off by other applications.

As mentioned above, for every call, one handle exists per application. This means that different applications with handles to the same call use different handles for it, which limits the scope of a call handle to a single application. To function properly, TAPI uses unique call handles provided by the application. In addition, the service provider may assign a unique call ID to a call (unrelated to the call's handle), which is used to track the call across transfers. Whether or not a service provider can assign call IDs to calls is a device capability.

The privileges of an application for a given call are maintained by TAPI.DLL and are not the property of an application's handle for the call. Resources such as memory are allocated dynamically for each call for each application that is given a handle to the call. These resources are not automatically deallocated when the call is dropped since the application may still find it useful to extract information from the call (such as for logging purposes). Therefore, applications must dispose of their call handle when they have finished using it.

## Version Compatibility

Over time, different versions may exist for TAPI.DLL, applications, and service providers for a line or phone. New versions may define new features, new fields to data structures and bit fields, and so on. Version numbers therefore indicate how to interpret various data structures.

#### Version Negotiation

To allow optimal interoperability of different versions of applications, versions of TAPI.DLL itself, and versions of service providers by different vendors, TAPI provides a simple, two-step version negotiation mechanism for applications. Two different versions must be agreed on by the application, TAPI.DLL, and the service provider for each line device. The first is the version number for Basic and Supplementary Telephony and is referred to as the API version. The other is for provider-specific extensions, if any, and is referred to as the Extension version. The format of the data structures and data types used by TAPI's basic and supplementary features is defined by the API version, while the Extension version determines the format of the data structures defined by the vendor-specific extensions.

Version negotiation proceeds in two phases. In the first phase the API version number is negotiated, and in the second phase the Extension version is negotiated. If the application does not use any API extensions, it skips the second phase and extensions are not activated by the service provider. If the application does wish to use extensions, but the service provider's extensions (the extension ID) are not recognized by the application, the application should skip the negotiation for Extension version as well. Each vendor has its own set of legal (recognized) versions for each set of extension specifications it distributes.

The function lineNegotiateAPIVersion is used to negotiate the API version number to use. It also retrieves the extension ID supported by the line device, returning zeros if no extensions are supported. With this function call, the application provides the API version range it is compatible with. TAPI.DLL in turn negotiates with the line's service provider to determine which API version range it supports. TAPI.DLL next selects a version number (typically, although not necessarily, the highest version number) in the overlapping version range that the application, the DLL, and the service provider have supplied. This number is returned to the application, along with the Extension ID that defines the extensions available from that line's service provider.

If the application wishes to use the extensions defined by the returned Extension ID, it must first call lineNegotiateExtVersion to negotiate the extension version to use. In a similar negotiation phase, the application specifies the already-agreed-upon API version and the extension version range it supports. TAPI.DLL passes this information on to the service provider for the line. The service provider checks the API version and the extension version range against its own, and selects the appropriate extension version number, if one exists.

When the application later calls lineGetDevCaps, it returns a set of new device capabilities for the line, revised according to the results of version negotiation. It contains the selected API version number, the extension version number and extension ID, the line's device capabilities consistent with the API version, and the line's device-specific capabilities consistent with the extension version. The application must specify both of these version numbers when it opens a line. At that point, the application, the DLL, and the service provider are committed to using the agreed-upon versions. If device-specific extensions are not to be used, the Extension version should be specified as zero.

Note that in an environment where multiple applications open the same line device, the first application to open the line device selects the versions for all future applications that wish to use the line. (Service providers do not support multiple versions simultaneously.) Similarly, an application that opens multiple line devices may find it easier to operate all line devices under the same API version number.

Different switch vendors have different names for address bridging, such as "multiple appearance directory number" (MADN), "bridged appearance," or "shared appearance."

To dial through the PBX, for example, the PC may need to dial '9' to "get out," or a different prefix may be needed for a call made through the central office.