ALL ABOUT OUR GS/GM MIDIFILES

IN THE BEGINNING there was MIDI madness. Every synthesizer  manu-

facturer  had  a different scheme for program  assignments,  drum

note assignments, polyphony for multi-timbral use, MIDI channels,

velocity  curves,  note-stealing routines,  note  ranges,  octave

designation, sound names, and other MIDI mysteries. Now  however,

most of these irregularities have been addressed by the  develop-

ment of General MIDI (or General Standard as Roland says. For our

purposes, the two are equivalent). Simply put, General MIDI  says

that Slap Bass 1 shall forever be on program change #37 and  that

the  Kick drum shall be on note #36. This being the case,  I  can

orient  our  songs to automatically play back on a  Roland  Sound

Canvas  (or any Roland product with the GS logo) and they  should

sound about the same on a Yamaha TG100, Korg 03RW, Kawai  G-Mega,

or any other General MIDI tone module, synth or sound card. Also,

it  provides you with a ready-to-play sequence that can  be  used

"as  is" or freely edited just like any other sequence.  So  keep

reading to understand more fully what I've done to our songs  for

instant playback, or you can skip it and just play them.

THE SCOOP

All  songs  come as Standard MIDIFile type 1 and use  16  tracks.

Each  track is labeled using the same names as the sounds in  the

Sound  Canvas. If necessary we can provide type 0  MIDIFiles  but

that  means you get no track names if they're loaded onto a  com-

puter  sequencing  program.  Track 1 is used as  a  title  track.

Tracks  2  through 16 play the musical parts on MIDI  channels  1

through 15 with the drums on track 11 set to MIDI channel 10. The

musical  tracks  generally follow this order:  Bass,  Pianos  and

Organs, Guitars, Synths, Strings and Brass, then specialty sounds

and  effects.  Partial reserve or dynamic  voice  allocation  can

affect  this  order so it's just a guideline. Unused  tracks  are

unnamed  and have "vanilla" control settings on them to  indicate

they are not needed.

TOTAL CONTROL

All  parts have the following control data embedded in the  first

bar: Bank Select CC0 set to 0 (if you've been messing around with

the  SC-55  Variation  tones this  resets  to  Capital),  Program

Change, Bender reset to 0, Volume CC7, Panning CC10, Reverb CC91,

Chorus CC93, and Pitchbend Range set (CC100, CC101, and CC6).  If

pitchbend range is set to 12 it will be returned to the default 2

at the end of the song. No program changes are embedded  anywhere

other than the first bar so if you reassign a part to a different

sound  or  synth  you won't get an  unpleasant  surprise  halfway

through  the song. If you want to setup the sequence   for  some-

thing other than General MIDI just delete all controllers in  the

first bar and insert your own controller data.

ESTHETICS

I have tried my best to balance not only volumes (CC7) but veloc-

ity levels as well. For example, E.Piano sounds best in a certain

velocity  range and my work reflects this subjective edit on  all

parts.  Also,  I have given acoustic consideration  in  selecting

panning  values. That is, strings and pads are spread out  across

the  stereo  image,  guitar solos are  panned  away  from  guitar

rhythms;  and  recurring hits, themes, and  rhythmic  motifs  are

generally opposite each other. The same approach also applies  to

REVERB and CHORUS.

THE BEAT

Let's  talk  about the drums. The drums are on track  11  and  by

convention on MIDI channel 10. Why 10? Because Roland's MT-32 had

the  drums  stuck  on MIDI channel 10 and could  not  be  changed

except  by Sysex. It's merely a holdover. Most of the  songs  use

the  STANDARD  kit (PG1) but some use the POWER  kit  (PG17).  In

either  case,  the  program change is embedded in bar  1  and  is

transmitted  while  a  countoff is being played  on  the  Maracas

sound. I've tried to scale the velocities for a balanced perform-

ance and they should need little tweaking. If you want to  fiddle

with  individual  drum sounds you'll have to split notes  out  to

separate tracks. Just remember to merge them back together if you

use a Roland Sound Brush because it will not play a MIDIFile 1 of

more  than  16  tracks. Of course you could save the  song  as  a

MIDIFile 0 and separate tracks on the same MIDI channel would get

merged together anyway (Yamaha MDF-2 users: MIDIFile 0 only).  On

a  few  songs, I've used pitch bend to alter the texture  of  the

drums.  That's why the drum track (and all tracks) begin  with  a

"reset bender to 0" message. As for the MIDI note assignments for

drums,  they were getting standardized even before  General  MIDI

and in reality they're just Roland numbers anyway.

SO PLAY, ALREADY!

So what do you have to do to play our GS/GM songs? Nothing except

have  your  Sound Canvas or other General MIDI module set  to  GM

Mode.  I  decided not to transmit the GM Mode  command  from  the

sequence  because you may have edited global parameters or  indi-

vidual  sounds  to your liking and neglected to save  them  to  a

suitable  storage medium. The reset command would wipe them  out.

Also, I've got a bad feeling that some manufacturers are going to

put  their General MIDI tone bank on some "back burner bank"  and

the GM reset command will select their trademark patches instead.

So I'm leaving it out for now.  Besides, I work from the  default

settings so why not start from there. You can always change  set-

tings later and store a Sysex setup to reflect your customizing.

LAST WORD

General  MIDI is not perfect. But it's a great first step  toward

stabilizing the sharing of MIDI data and is ideal for  commercial

libraries  like TRAN TRACKS MIDI Sequences. Even if you just  use

our GS/GM sequences to get started and then turn them inside  out

or  whatever, you'll get the idea of what the programmer  (that's

me) intended in the first place. I believe you'll be pleased  and

sometimes  amazed  at how good the songs already  sound.  If  so,

we'll be here to help you with more songs and service.

Tran Whitley

TRAN TRACKS INC.

TEL 201-383-6691      FAX 201-383-0797      ORDER 800-473-0797