README.DOC - Fax Software for OS/2

Copyright (c) 1992, 1993 by Keller Group Inc.

All rights reserved.

March 8, 1993

Topics contained in this README.DOC file:

MODEM CONFIGURATION SETTINGS

NEW FEATURES

MODEM CONFIGURATION SETTINGS

----------------------------

Tested modems are listed below.  Unless the manufacturer has changed

the product since the time of our testing, you should use the

following information to determine correct Options/Configure values

for your fax hardware.  (Many other modems will also work, but the

vendors have not provided us with evaluation units for testing.)

MANUFACTURER  MODEL                          CONFIGURATION

Adtech Micro  Externally Yours! (ROM v2.2)   Class-2, might need 'No DLE esc'

AT&T Paradyne DataPort Fax Modem             Class-1

BIT Blitzer   XM124S/RFAX                    Class-2

Brooktrout    TR112 (with OS/2 BTLIB 1.04+)  uses FaxBT.adp as Fax.adp

Complete PC   Complete Modem Plus            Class-1

CompuCom      Speedmodem Combo (ROM v2.77)   Class-2

Dataplex      DPX-223-55 2400/9600 rom5.42   Class-2

Dataplex      DPX-225-01 14,400 (ROM 1.00)   Class-2

Digicom (DSI) Scout Plus, Spirit II          Class-1

Dove          DoveFax (ROM v4.2)             Class-1

Dr. Neuhaus   FURY 9600, 9600 TI, 14.4 TI    Class-2

Dr. Neuhaus   FURY 2400 POCKET/FAX, PC/FAX   Class-2

Dr. Neuhaus   FURY 2400 PC JUNIOR            SendFax

Dr. Neuhaus   FURY 2402 TI/FAX               SendFax

Everex        EverFax 24/96 (COM1/2 only)    Cls2,Alt bit order,Xon recv start

GVC           FAXMODEM 9624Vbis              Class-1 or Class-2

Hayes         Optima 24/96                   Class-1

Hayes         Optima 96/96                   Class-1

Hayes         Optima 14.4/14.4               Class-1

IBM           L40SX laptop modem             Class-1, Disable BPS stepdown

IBM           ThinkPad Fax Modem             Class-1

Image Comm    TWINCOM 24/96                  Class-2

Image Comm    TWINCOM Commuter 24/96         Class-1

Intel         SatisFAXtion classic           uses FaxIS.adp as Fax.adp

Intel         SatisFAXtion/200               uses FaxIS.adp as Fax.adp

Intel         SatisFAXtion/400  (internal)   uses FaxIS.adp as Fax.adp

Intel         SatisFAXtion/400e (external)   Class-1

Intel         SatisFAXtion/100               Class-1

Megahertz     96/96 FAX/Modem (P296FMV)      Class-1, Special cmd \J0\Q1

MICC          9642                           Class-2

MultiTech     MultiFaxPC MT932               Class-2, Alt bit order

MultiTech     Multi Modem MT932EAF           Class-2, Alt bit order

MultiTech     Multi Modem II                 Class-2, Alt bit order

MultiTech     Multi Modem V.32 Fax           Class-2, Alt bit order

NetComm       M4F or M7F (ROM rev K)         Cls-2, Special cmd +FDCC=,3,0,2,0

Practical     2400FX96                       Class-2

Practical     2400 SendFAX                   SendFax

Practical     14400FXSA V.32bis              Class-1, Check TCF signal

Practical     14400FXMT V.32bis              Class-1 or Class-2

Prodigy       Eagle (9600 data, 96/48 fax)   Class-1

QuickCom      Spirit II                      Class-1

Supra         SupraFAXmodem V.32bis          Class-1 or Class-2

TwinCom       TwinCom v1.13C                 Class-2

UDS           FasTalk Fax32                  Class-1

USRobotics    Courier HST Dual Std w/Fax     Cls-1,Spec cmd &H2,Recv EOP delay

USRobotics    Sportster 14,400               Cls-1,Spec cmd &H2,Recv EOP delay

USRobotics    Worldport 2496 V.42bis         SendFax

USRobotics    Worldport 9696 MNP5 FAX        SendFax

USRobotics    Worldport 9696 V.42 w/Fax      Class-1, Special cmd &H2

USRobotics    Worldport 14400 V.42 w/Fax     Class-1, Special cmd &H2

ZyXEL         U-1496 (ROM 5.05 tested)       Class-2

Zoltrix       ZX1896 or Zofax 96/24          SendFax

Zoltrix       9624 Class 2 (regular)         Class-2

Zoltrix       Class 2 with V.42 MNP          Class-2, might need 'No DLE esc'

Zoom          FaxModem FC 9624 (internal)    Class-2

Zoom          FaxModem FX 9624 (external)    Class-2

Zoom          V32bis (ROM 1.56D tested)      Class-1, 9600bps, Spec cmd &C1&D3

Class 1 modem note: For most Class 1 modems, using the 'Check TCF

signal' on the Options/Configure screen will provide enhanced speed

downgrading.  This may improve the quality of received faxes.

The Everex EverFax 24/96 internal is supported, but because the board

does not allow IRQ selection, it must generally be used as COM1 or COM2.

Some users have reported that the following modems work

best when used in Class-1 mode, 9600bps, Special cmd of &C1&D3.

Boca          Bocamodem 14.4Kbps (ROM v1.27) ROM update was requested

Modems that failed our compatibility testing:

Calculus      EZ-FAX                         Proprietary fax commands

Complete PC   certain models                 Proprietary fax commands

DSI           digicom 9624LE                 SendFax non-spec

Formosa       MoFax MF-96P                   SendFax train failure

Frecom        Fax96                          Non-standard chip set

Intel         Connection CoProcessor         Proprietary fax commands

Omnitel       9624+S/R                       SendFax recv failures

Omnitel       Omnitel v1.10                  Class-2 recv failures

NEW FEATURES

------------

The following is a partial summary of new features that are included

in this release:

1.16 - When a received fax terminates in an error status, pages that

were received before the error are retained.

1.16 - Text line spacing is now 120% of the current font point size.

Previously, line spacing was derived from the font, so the number of

lines per inch in a given point size could vary based on the font.

1.16 - Added a "Disable BPS stepdown" check box on the

Options/Configure screen for the L40SX modem.  This fax modem is

still unable to send to fax machines that require stepping down to

slower baud rates, but if this check box is used, the program will

retry without stepping down, which may allow the machines to

communicate in some circumstances.

1.16 - Enhanced capturing with the Utilities/Capture to clipboard

command.  Colored icons, including OS/2 buttons such as

minimize/maximize, now map to a pattern rather than turning black.

1.16 - Added support for the Intel Hand Scanner with SatisFAXtion

boards (see the section in this README.DOC file for details).

1.22 - For the Printer Driver Configuration screen: The program now

sets the fax directory for the printer driver, so the "Fax directory"

field has been removed.  The "No log entry" action has been removed.

A "Tones" field has been added to enable/disable the beeping tones

during printer driver operation.

1.22 - The Send Fax screen now includes a "To" field that shows a

list of all selected destinations.  You can remove a destination by

double-clicking on its line.

1.22 - The "Cover bitmap" pull-down list on the Fax Send screen now

includes BMP files from your fax directory and your working directory.

1.22 - Current call sending is now started by pushing the "Send

current" button on the Send Fax screen.  If you select multiple

destinations, the Send button changes to indicate the number of

destinations.

1.22 - The "Fax local id" field on the Options Settings screen now

supports alphanumeric characters.  This allows you to send text as

your fax id.  (This only works if your fax hardware allows it... some

fax hardware will only accept numerics.)

1.22 - The field widths in the log display have been adjusted to

compensate for OS/2 system font changes in the OS/2 2.0 Service Pack

update (and OS/2 2.1).

1.22 - For the IBM L40SX laptop and other machines with a "suspend"

mode, the program can now receive a fax when the machine is

"resumed".  This did not work in previous releases because the L40SX

fax modem is reset by the suspend/resume procedure.  If the program

is left in receive mode, it will now reinitialize the modem and be

able to answer fax calls when the machine resumes.  Since a suspended

machine is automatically resumed by an incoming call, this means that

a suspended machine can receive faxes.  After the fax has been

received, the L40SX can automatically suspend itself again after a

time delay (which is set on the L40SX by the L40's "PS2 SET POwer off

minutes" command).

1.22 - Also of interest to L40SX users, the "Disable BPS Stepdown"

check box on the Options/Configure screen now applies only to

sending.  The L40SX fax modem can correctly receive at slower baud

rates, but does not correctly support sending at speeds slower than

9600 baud.

1.23 - Tab and cursor key support is improved in the INSTALL program.

1.23 - Alt-F4 now works as a short-cut key for closing the application.

1.23 - Cover sheets now use proportional fonts for enhanced

appearance.  The order of preference (using the first one that is

available on your system) is Helvetica, Times Roman, then Courier.

1.23 - A "Utilities/Edit phone books" command has been added to

permit phone book editing without having to use the Fax/Send command.

1.23 - The application now shuts down fax modems upon program exit,

thereby leaving the modem reset for use by data comm programs even if

you were in fax receive mode.

1.23 - Current call sending has been enhanced.  If a cover sheet is

enabled, you will be prompted for the name and company so that this

information can be included on the cover sheet.  The application will

then construct the fax document and provide a pop-up dialog box when

it is ready for you to dial the call.  This allows the application to

immediately begin sending upon your request.

1.23 - For LAN workstation use, the workstation slave adapter is now

called the workstation adapter and is significantly modified as a

result of improvements for the enhanced LAN version of the product.

1.23 - Enhanced printer support for DOS and Windows printing is

included.  The printer object's job properties can now be either

normal or fine resolution when printing from DOS or Windows (rather

than requiring fine resolution), so you can create either normal or

fine resolution fax documents from DOS and Windows applications.  For

DOS printing, the IBM Proprinter X24 emulation is improved to support

underlining and fine-tune the character spacing.

(WordPerfect 5.1 DOS users... When printing to the fax printer device

from WordPerfect using WordPerfect's IBM Proprinter X24 printer

driver, you can now generate very high-quality output.  Try printing

WordPerfect's printer test file for an example.  If you need even

more font support for printing fax documents from WordPerfect DOS,

you might want to consider the PrimeType product as discussed in the

"DOS PRINTING" section above.)

1.26 - Phone book displays now show the fax number on the main

selection list and on the Utilities/Edit phone books list.

1.26 - For DOS printing, the Proprinter X24 emulation now shows

emphasized and double-strike characters in Bold.  Note that the

Proprinter X24 printer does not provide italic.

1.26 - Enhanced Class-1 support, including support for new Practical

Peripherals 14400FXSA v1.21 ROMs, Megahertz and new USR Worldports.

Improvements for USR HST receiving, ZyXEL sending, and more...

1.26 - The "Recv EOP delay" check box (for Class-1 only) is added to

the Fax modem Options/Configure screen.  Use this option for

USRobotics Courier and Sportster Class-1 modems to correct a USR

quirk when receiving from certain fax machines and modems.

1.26 - The "Check TCF signal" check box (for Class-1 only) is added

to the Fax modem Options/Configure screen.  Use this option for

enabling interpretation of TCF data for enhanced negotiation of

automatic speed downgrading.  This helps overcome problems with

receiving noisy faxes if you telephone company uses a line

concentrator (which may require speed downgrading to 7200bps).

1.27 - Revised INSTALL program.

1.27 - Support for 16550 UARTs.  The 'Use HW FIFO' setting on the fax

modem Options/Configure screen allows the FMD.SYS driver to take

advantage of the high-performance buffering features of UART (serial

I/O chips, used for your COM port) that are compatible with the

NS16550A chips.

1.28 - For LAN version, added support for DTMF/Touch-Tone automatic

routing (in addition to DID automatic routing) with Brooktrout

boards.

1.28 - Fixed current call sending for SatisFAXtion boards.

1.28 - The printer driver now supports the FAX.GGS file for providing

printer driver commands to the next print job.  See the section above

called "ADDITIONAL SUPPORT FOR AUTOMATIC SENDING" for details.  This

can be used to provide fax sending and/or fax merge features from

other applications that cannot otherwise insert the printer driver

commands directly into the printer data stream.

1.28 - The printer driver now supports a configuration item for

selecting the number of gray levels.  For applications that send a

color image to the printer, the printer driver can convert the colors

to "gray" patterns.  Using gray areas will increase fax transmission

time, but can provide very nice looking graphics.  Experiment with

various settings on the printer driver configuration screen to see

what works best for your images.

1.28 - A program (FxRcv.EXE) is now provided on the distribution

diskette as a alternative way of turning receive mode on and off.

See the section above called "CONTROLLING RECEIVE MODE FROM COMMAND

LINE/COMMAND FILE".  This can be used in command files for

temporarily taking the fax program out of receive mode so that you

can make a data communications call with a fax modem.

1.29 - (LAN version) - Private mode, which provides LAN support for

user privacy and routing, now supports up to 5000 workstations per

server.  This was unintentionally limited to only 200 workstations in

previous beta test releases.

1.29 - Paste from Clipboard now provides color metafile support.

1.29 - "Color filled area" printing is further enhanced in the

printer driver.  You can now configure the printer driver for either

"Black & white", "Fax 8-level" or "System dithered" processing.

"Black & white" forces all colors to black.  "Fax 8-level" maps

colors to patterns that are custom-designed to allow efficient fax

transmission and support resizing and printing without aliasing.

"System dithered" uses standard system color dithering which can

produce nice looking images, but the images are less efficient for

fax transmission and the gray areas may alias (distort) when resized

or printed.  Experiment with various settings on the printer driver

configuration screen to see what works best for your images.

1.29 - For some strange reason, the OS/2 Enhanced Editor opens and

closes the printer driver when you select the printer.  This formerly

caused an "empty" print job.  The printer driver now ignores print

jobs that don't actually print anything.