ToneLoc v0.99

Reference Manual

by

Minor Threat & Mucho Maas

This is the reference manual.  You should read this AFTER reading

TL-USER.DOC unless:

1. You are already familiar with ToneLoc, or

2. You want to get started quickly

------------------------

COMMAND LINE OPTIONS

------------------------

Here are the command line options for ToneLoc:

ToneLoc  [DataFile] /M[Mask] /X[ExMask] /R[Range] /D[ExRange] /C[Config]

/#[Number] /S[StartTime] /E[EndTime] /H[Hours] /Q

/T[-] /K[-]

ToneLoc must ALWAYS be run with at least one parameter, and if you

only use one parameter it MUST be the dialing mask to use.  If you only

give ToneLoc one parameter, the first 8 characters of the dialing mask

will also be the data file name.  So if you run "TONELOC 555-XXXX", the

mask will be 555-XXXX and the data file will be 555-XXXX.DAT.

/M - Dial Mask.  Use this parameter if you want to use a data file-

name other than your dialing mask.

/X - Exclude dial mask.  This tells ToneLoc to -NOT- dial certain

numbers.  For example, if your local Police department has

phone numbers ranging from 555-5000 to 555-5099, and you were

using the dial mask 555-XXXX, use the exclude mask 50XX. This

mask applies to the active part of the dial mask (in this case

XXXX) and therefore should be no more than 4 characters long.

If an exclusion mask is longer than 4 characters, ToneLoc will

ignore it.  Up to 9 exclude masks are allowed at once, but all

exclusions must be sub-sets of the dialing mask.

/C - Config File.  This tells toneloc to use a config file other

than the default, TONELOC.CFG.  You may change this for different

types of scans, but you'll probably want to make different

config files for different kinds of scanning or for different

kinds of modems.  For example, you could have a config file,

TONES.CFG, that is configured to scan for tones, and another

called CARRIERS.CFG configured to scan for carriers.

/S - StartTime.  Tells ToneLoc to wait until a certain time to begin.

You may either use military time or end the time with a "p" for

P.M.  Example:  5:30p and 17:30 are the same.

/E - EndTime.  Tells ToneLoc to STOP dialing at a certain time.  Use

the same notation as the /S parameter above.

/H - Hours.  Tells ToneLoc to dial for a certain number of hours

after starting, and then exit.  "/H5:30" would mean 5 hours,

30 minutes.

/Q - Quiet.  Tells ToneLoc to suppress ALL sound.  The speaker

and all sounds will be turned off.

/# - Number. Tells ToneLoc to dial a certain number of times,

then exit. "/#:5700" would mean ToneLoc makes 5700 dials.

/T - Sets the scan mode to look for tones, overriding whatever is

specified in the config file.  /T- will look for everything

_except_ tones.

/K - Sets the scan mode to look for carriers, overriding whatever is

specified in the config file.  /K- will look for everything

_except_ carriers.

These parameters may appear in any order, EXCEPT the first parameter

MUST always be the datafile, and if you do not specify a dial mask with

/M it MUST ALSO be the mask.

-----------------

WHILE DIALING

-----------------

While ToneLoc is scanning, there are several keys that do various

things.  Here they are:

A - Blanks the screen totally black, and turns off the cursor.

Press 'A' again to unblank it.

B - Blanks the screen, but instead of making it black, it loads

the optional Alternate Screen file set in the config file.

Preess 'B' again to unblank it.

C - Logs the current number as a "Carrier".  Say you're scanning

for tones, but you hear a carrier, you can press 'C'.

F - Logs the current number as a "Fax".  Useful for when you

hear a fax machine while scanning.  A fax machine sounds like

a 300 baud modem underwater.

G - Logs the current number as a "Girl".  Useful if you hear

a really cute-sounding babe saying "hello big boy".

J - Pressing 'J' will Jump to a DOS shell.  Upon returning,

ToneLoc will redial the last number.

K - Allows you to enter a custom note for the current number.

N - This will "Note" the current number by putting a special entry

in the logfile.  It looks like this: "* Noted *".  Use this if

you want to remember this number for later.

P - Pauses scanning until another key is hit.

Q - Quits scanning and exits AFTER the current dial is finished.

R - Redials the current number.

S - This toggles the modem speaker ON or OFF while dialing, but

ToneLoc will wait until the current dial is completed before

toggling the speaker.  If you press 'S' again before the

speaker is toggled, the speaker won't be toggled.

T - Logs the current number as a "Tone".  Say you're scanning for

carriers, but you hear a PBX dialtone, you can press 'T'.

V - Logs the current number as a "VMB", in case you hear a VMB

system during a scan.

X - This will extend the current dial time by 5 seconds.  The

meter will reflect the time addition.

Y - Logs the current number as a "Yelling Asshole".  Useful if you

hear some dick yelling "I KNOW ABOUT YOU HACKERS AND YOUR

2500 BAUD MODEMS!"

0-9 - These are the modem volume changing keys.  They will send

the corresponding string in the config file to the modem.  They

work like <S>peaker toggle.

[Spacebar] - Aborts the current number and goes on to the next one.

[Escape] - Pressing Escape will abort the current dial and

exit ToneLoc.

----------------------

CONFIGURATION FILE

----------------------

ToneLoc uses a configuration file, TL.CFG.  Before running ToneLoc,

you need to run TLCFG to create this file. The configuration program has

extensive on-line help, so instead of us pointlessly duplicating

information here, please use it.

TLCFG can be run in monochrome mode by entering "TLCFG /B".

-----------------

DESQview INFO

-----------------

ToneLoc runs most excellently under DESQview.  It will detect

DESQview, and if DESQview is found, screen writes are directed to the

screen buffer so your screen isn't trashed.  ToneLoc also releases

unneeded timeslices to DESQview.  Because of this, ToneLoc uses VERY

LITTLE CPU time, so you can run it in background with minimal slowdown.

I tested ToneLoc with two windows open.  One was a DOS window sitting

at the prompt idle (doing nothing).  The other was ToneLoc scanning.

ToneLoc used about 10% of the CPU time, and the idle DOS window used

about 88% (with about 2% DESQview overhead).

The file "TL-PIF.DVP" is included with ToneLoc 0.98.  We were

too lazy to dig it up for this version.  To add ToneLoc to your DESQview

setup, do the following:

1. Run DESQview

2. Choose "Add a Program"

3. Type 'O' for other

4. Enter the path where ToneLoc is found

5. Move the cursor bar to "ToneLoc" and hit [Space Bar]

6. Press [Enter].

ToneLoc will be added to the list of applications on your DESQview menu.

To run ToneLoc, choose "TL" from the application menu.  You will be placed

at the DOS prompt with "ToneLoc " already typed.  Just finish the command

line and press enter.

---------------

FOSSIL INFO

---------------

ToneLoc no longer requires a FOSSIL driver.  However, if the

built-in SERIAL routines don't work, or you are using a very strange

com port, like a digiboard, you may need to use the FOSSIL routines.  The

FOSSIL routines aren't any less reliable or slower, but the FOSSIL

routines require an external FOSSIL driver program to be loaded.  The

two most popular FOSSIL drivers are X00 and BNU.  Just call your local

PD board and search for "FOSSIL", "BNU", or "X00".  ToneLoc works with

both.

--------------------------------------------------------------------------

If this doc file sucks, please us know how to improve it.

-- Minor Threat

-- Mucho Maas