File Name: README.DOC (Host PC)

File Date: 8/30/93

Host PC Revision: I/NET 7700 Revision 2.11

==================================================================

Welcome to I/NET 7700!

Revision 1.00 of I/NET 7700 includes all of the existing S7000

Revision 4.60 features as well as the following enhancements:

*    Commercial Ethernet LAN support for up to a total of 63

Operator Stations utilizing Western Digital Ethernet cards and

LANTASTIC Netbios operating system software.

*    Support for both Report and Event printers at each Host

Operator Station.  Printers can be either serial or parallel.

DOS driven Report printers as well as DOS, BIOS, and Interrupt

driven Event printers are supported.

*    Host Message Routing editor allowing users to steer messages

to different Commercial LAN Host Operator Stations during

occupied and unoccupied hours.

*    Transparent operation of any Host or its controllers from any

other Host amongst all Commercial LAN Host Operator Stations,

including global (tapless) Host Operator Stations.

*    Enhanced Page, Library Symbol, and Docutrend Report copy

features between Commercial LAN Host Operator Stations.

*    User defined path names for System Messages, DCU or PCU Save

files, Pages/Symbols/PCX files, Docutrend Online cell data,

and Docutrend Archived cell data.

*    Support for up to 1100 unique Host Passwords per System

allowing individual selection of all editor and Command Line

functions.  A "Pre-Sign On" function has also been added for

DCU and PCU Passwords.

*    New Function key prompt line on the bottom of all non-graphic

CRT displays allowing the user to access Help Screen

information, silence audible alarms, or "recall" text entries.

If more than 18 lines of text appears on the CRT display a

"Page Up/Down=Continue" indication also appears.

*    Context sensitive online Help Screens available by pressing

the F1 key from any system prompt.  Also an interactive

Conversion Coefficient Calculator has been incorporated in the

DCU and PCU Station Parameters Help Screen.

*    Adjustable Host Screen Saver (from 0 to 60 minutes).

*    Enhanced Graphic System Pages and System Pages editor

consisting of:

-   Support for both EGA or VGA monitors as defined in the

Setup Program.

-   Use of local PCX files as System Page "backdrops".

-   A new color bar prompt has been added allowing the user

to view the actual text or graphics color being selected.

-   A "Floating Arrow" cursor has been included for the

selection of items on a System Page or in the System Page

editor.

-   A "Zoom" feature, with a "Pop-up window" selection

screen, has been added allowing the user to directly

define or access DCU or PCU editors from a Summary

Display, System Page, or the System Page editor.

-  Graphic "Icons" have been included for all Points, Point

Extension editors, Pages, and DDC Lines.  From a System

Page or the System Page editor, the user can "zoom"

through these icons to the individual editors they

represent.  The editor can then be added or modified as

desired.  This feature allows the user to generate DCU or

PCU databases "online" from the System Page editor.

-  Home page designations can be assigned for all alarm

points on the System Page.  When an alarm occurs, the

point address and it's Home Page number will appear in

the Alarm Summary display.

-  A "Save? Yes/No/Continue" prompt is now received when

exiting the System Page editor.

-  A "Highlight handles? No/Yes" prompt has been added for

the System Page editor Move and Delete operations.

-  From a System Page points can be controlled from "Pop-up

window" displays.

*    New Host Graphics Page Print editor.

*    New Host Offline database editor for all DCU and PCU types.

*    Prompt selections made by typing in the first letter of the

desired choice eliminating the need to use the Enter or Return

key.

*    Twelve point Host Operator Station Trend Log.

*    Inclusion of Docutrend Online 90% full, 95% full and cell data

lost messages in the Alarms Summary.  An audible alarm can now

be generated in addition to displaying the third party *ALM*

icon.

*    Automatic and Manual Docutrend to DIF cell data conversions.

*    Third Party Multiple Report Generation support allowing the

Host to automatically perform Docutrend to DIF conversions

from data in multiple Hosts/multiple cells, exit to DOS,

execute a pre-defined batch file (using or storing the

converted DIF data into a separate program), and then re-start

the I/NET 7700 Host software at daily, weekly, monthly,

yearly, or scheduled intervals.

*    Automatic updating of Host Operator Station EVENTS and ALARMS

files as messages or alarms are received.

*    Online Host Operator Station Disabled Point and Offnormal

Point Summaries.

*    Addition of All_Yes and All_No prompt choices in the Review

Messages and Host Password editors.

*    Host Operator Station Automatic DCU or PCU Saves at 2:00 a.m.

each day on user selected direct connect and AD/AA DCU's or

PCU's, provided a programming change has been made.

*    New Host ATS editor allowing the user to define Host Automatic

Time Schedules and send these schedules to multiple DCU or PCU

resident Master ATS discrete output and control points.

*    Support for up to 100 (00-99) "logical" system links.

*    Adjustable controller LAN baud rates between 9600 baud and

19,200 baud.

*    Online Host, Link, Station, and Unitary Controller

Configuration Summaries displaying each device's address,

type, name, revision level and revision date accessible from

the System Page prompt line.

*    Support for Priority and Critical alarm relay outputs in Model

B7801R Host taps.

*    Remote telephone numbers now stored in the Host Network

Configuration file allowing the use of any of up to 16

available AD/AA taps when communicating with a remote

controller LAN.  (Names, rather than phone numbers, can now be

used for 78040/41 and 78050/51 taps.)

*    New Tap Save and Tap Restore functions for the Model 78060 and

78061 AD/AA taps.

*    Support for the Model 7716 Process Control Unit, including the

capability to download firmware to the PCU from any Host

Operator Station.

*    New Message Priority and Message Mask parameters in all Tap,

DCU, and PCU Configuration/Status editors to control the

generation and reporting of system messages.

*    Bit offset addresses from 00 to 09 for all input and output

points in all DCU's and PCU's.  All points which share the

same point (PP) address are scanned at the same rate.

*    Support for LAN, Link, and System Global points and Indirect

points in lieu of Reference points in all DCU's and PCU's.

*    Adjustable momentary discrete output and control points in the

7700 DCU, 7740 DCU, and the 7716 PCU.

*    Independent, Master, and Slave ATS Schedules for DCU or PCU

resident discrete output and control points.

*    Automatic Time Schedule Sunrise/Sunset commands for DCU and

PCU discrete output and control points.

*    Adjustable Optimized Start and Optimized Stop parameters (from

0 to 480 minutes) for discrete output and control points in

all DCU's and PCU's.

*    Demand Control in all Model 7760 Unitary Controller

Interfaces.

*    New Julian Date, Old Data, Average, and Time to Start

calculated point operators in all DCU's and PCU's.

*    Perpetual accumulators in DCU and PCU Runtime editors

resetable at user defined intervals.

*    Full floating point PID module output ranges in all DCU's and

PCU's.

*    Automatic and Adaptive Tuning of PID and Floating DDC modules

for automatic development of a module's Mode (direct/reverse

acting), Sample Interval, Proportional Band, Reset Interval

and Rate Interval parameters.

*    New Save remainder prompt automatically displayed when

disconnecting from a DCU or PCU provided the user has made an

editor change and not previously performed a Station Save.

*    Enhanced diagnostic data on the second page of all DCU and PCU

Configuration/Status editors.

*    "Cold Starting" of all DCU's and PCU's upon a change of

address from the Hand Held Console without the need to reset

or power down the controller.

*    S7000 to I/NET 7700 System Page/Library Symbol conversion

program and DCU Save file conversion program.

*    Offline Commercial LAN exerciser program.

*************************************

I/NET ETHERNET QUALIFICATIONS

*************************************

The "LAN" mode of the Shell's Setup Editor allows qualified PC

nodes to interchange data as follows:

1)   Any node may display the graphic pages and point data from any

other node.

2)   Any node may view DCU, PAGE, LOG, and CONFIGURATION SUMMARIES

from any other node.

3)   Any node may access all editors of any other node (HOST, TAP,

or DCU).

4)   Any node may create Docutrend Reports using data from any

other node or nodes.

5)   Any node may route any of its Point, DCU, Tap and Sign On/Off

Messages to any other node or nodes.

Up to 5 nodes on various 80286/80386 Ethernet LAN arrays have been

thoroughly tested/verified in all modes of the above types of data

interchanges.

NOTE that heavy simultaneous activity on any 2 (or more) of the

above 5 types may cause degradation of data transfer and

necessitate possible PC reboot.

CSI is currently stress-testing the system with greater than 5

nodes and is improving these data interchange capabilities in a

modified driver due for release in the next few months.  The Host

Backup Station Software is not being implemented with this release

and is also due for incorporation at the same time.

==================================================================

Released Revision 1.10 of I/NET 7700 includes the following

enhancements:

*    Support for the Memory Interface Processor (MIP) Board in all

I/NET 7700 DCU's and Taps.

*    Compatibility with MS-DOS Revision 5.0.  Note, that the

program SHARE.EXE cannot be memory resident during the

installation process.

*    Support for the GM78041, GM78051, and GM78061 AD/AA Taps has

been incorporated into the standard 78041, 78051, and 78061

AD/AA Taps.  The external modems are now placed in the "Auto-

Answer" mode by their respective taps.  Therefore, separate

firmware loads for Domestic and International versions of

these AD/AA taps are no longer necessary.

and the following corrections:

1.   Line two and three of the Host PC display are no longer

deleted when exiting Help Screen number 503 (System Page

Editor).

2.   Host PC no longer displays ASCII characters when exiting Help

Screen number 5401 (Creating/Expanding Docutrend or Message

files).

3.   The "7300_DCU" has been changed to read "7730_DCU" in all

editors and displays.

4.   The "7800_DCU" has been changed to read "7720_DCU" in all

editors and displays.

5.   Docutrend Runtime, Consumption, and Demand cell records,

simultaneously received from multiple points within the same

controller, are no longer being tagged with descending dates.

6.   Docutrend cell records, simultaneously received from multiple

points and stored at the Host PC, are no longer being tagged

with the same station address.

7.   Docutrend Override Billing cell records from multiple points

within the same 7750 Building Manager are now being correctly

stored, with their proper values, by the Host PC.

8.   Indirect DO and DC output points, controlled by Override Zones

in the 7750 Building Manager, are no longer being tagged as

"Old Data" when the DCU passes through midnight.

9.   The Install program now correctly allows 8 character program

directory names to be used.

10.  On a commercial LAN, Host Station numbers greater than 32 now

work correctly.

11.  The background driver is being correctly re-started after a

Backup or Restore is performed from the Shell menu.

12.  The background driver is being correctly stopped and re-

started anytime the Docutrend RWONLN file is being created or

expanded from the Shell menu.

13.  The SETUP.TXT and SETUP.TXS files are now correctly being

included on the Text Conversion Diskette.

*************************************

I/NET DOS 5.0 QUALIFICATIONS

*************************************

I/NET 7700 Revision 1.10 is now supported by CSI as a DOS 5.0

Application.  The more traditional DOS 3.3 is also supported

although DOS 4.X is still not recommended.  Please note that I/NET

7700 Revision 1.10 has not been thoroughly tested using many of the

advanced DOS 5.0 utilities such as the High Memory Manager, the

Upper Memory Manager, the Expanded Memory Manager, the RamDisk, the

Cache, etc.  If you plan to make use of these utilities, you must

use the same cautions and techniques as with other DOS Applications

and you must expect the same sorts of incompatibilities as with

other DOS Applications.  In particular, the CSI "Background Driver"

is now implemented as a "TSR" (Terminate and Stay Resident) program

and may have conflicts with other "TSR's".  CSI strongly recommends

the use of DOS 5.0 to provide only the traditional services

(COMMAND.COM, IBMBIOS, Format, Copy, etc) unless you are

particularly expert in the use of the more advanced services, and

unless you are prepared to spend long hours working out the details

of their implementation.  You should never attempt execution of

advanced DOS 5.0 services with executable I/NET 7700 modules.  For

instance, using the Upper Memory Manager to load the CSI

"Background Driver" into the Upper Memory Block is not recommended.

The results of this implementation will be indeterminate and may

cause significant problems during day to day operation.

==================================================================

Released Revision 1.11 of I/NET 7700 includes the following

enhancements:

*    Changes to the PID, AO-PID, and DO-PID module/extensions

providing new "P-Only Operation" in the 7716 PCU, MIP7700, and

MIP7740 software loads as well as the 7200 UC EPROM load.

and the following corrections:

1.   Correction for problems encountered when adding Indirect

Points in the Host Offline Data Base and Resident I/O Editors.

Note that a new error message will now be received in the

Resident I/O Editor if a user attempts to enter a global

address from a non-existent link to an indirect point.

2.   Correction of Data Base Convert Program problems associated

with Indirect Points.  Note that as a result of this change,

all DCU's that were previously converted to I/NET 7700 and

restored, must be re-saved after I/NET 7700 Revision 1.11 is

installed.

3.   Correction of other Data Base Convert Program problems

resulting in incorrect DM Point addresses, DC Point Time to

State values, Indirect AI/AO Broadcast Change Count values,

and Indirect DI/DA Number of bit entries in the converted DCU

Save file.  Note that Reference DM Points are converted to

Indirect DI Points because the information required to convert

them does not exist in the S7000 Reference Data Base.

4.   Correction of a Page Convert Program problem caused when

conversions of previously corrupted PGGRF files were

performed.

5.   Correction for the problem where Indirect Points with

questionable states/values ("old data") in conjunction with

LAN reconfigurations or power fluctuations resulted in the

loss of existing database programming in the 7716 PCU,

MIP7700, MIP7740, MIP7760, and MIP7770 software loads.

6.   Correction for the problem where analog point Conversion

Coefficient entries incorrectly jump by a value of eight in

all DCU software and EPROM loads.

7.   Correction for a problem in the MIP7760 software load where

enabling the scanning of non-existent UC's caused the UCI to

drop off the controller LAN.

==================================================================

Released Revision 1.12 of I/NET 7700 includes the following

corrections:

1.   A 7716 PCU directly connected to a Host Operator Station, now

correctly appears as a "7801 Hst/Lnk/LAN Tap" when a Link

Configuration Summary is performed.

2.   The 7771 ICI MODBUS Configuration entry has been removed from

the parameter selection menu of the Host Data base print

editor.

3.   On all system messages received at a Host Operator Station

directly connected to a 7716 PCU, the DCU Station name is no

longer limited to seven characters.  User's initials are now

also printed on all manually initiated system messages.

4.   From a remote Host Operator Station on an commercial LAN, when

a Software Restore is performed on a 7716 PCU, the system will

correctly download the 7716.BIN software file under all

conditions.

5.   Within the Station Software Save editor, the text "Station -

7416" has been changed to "Station - 7716".

6.   Time Zone entries are now being correctly saved by the DCU's

and PCU's.

7.   The Connect Line (Line 2) of the CRT display no longer

indicates that the user remains connected to a station when

the Offline Database editor is exited.

8.   In the Demand editor, toggling the Emergency and Override Shed

level entries between "Constant" and "Point" no longer result

in erroneous "Constant" values.

9.   When defining a number of actions to be skipped in the DCU

Event Sequence editor, the prompt "Enter Value (1-64)" has

been corrected to read "Enter Value (1-18)".

10.  Restoring a Save file to a 7760 UCI, with a MIP board; no

longer causes a LAN reconfiguration to occur.

11.  The database conversion program (DBCNV45.EXE) message

"Converting Rev 4 to Rev 5" has been corrected to read

"Converting S7000 Rev 4 to I/NET Rev 1".

12.  After the Messages prompt choice has been selected to review

System Events/Messages, the displayed information is no longer

incorrectly "blanked" by the receipt of a Docutrend data

upload request message.

13.  The Database Print editor now prints 7760 UCI Demand Control

information correctly.

14.  The software Watch Dog Timer now works correctly in the 7716

PCU.

15.  Analog and Discrete Sample Docutrend data is now correctly

received and stored by a Host Operator Station directly

connected to a 7716 PCU.

16.  External discrete hardware inputs on the 7716 PCU are now

being scanned at 100ms intervals.

17.  The Host Operator Station no longer allows the user to

incorrectly "zoom" to an output point which is being

controlled by a 7750 DCU Zone.

18.  In the Host Password editor, the Station Software entry is no

longer incorrectly toggled in conjunction with the DCU MODBUS

entry.

19.  The Offline Database editor now works correctly on two station

7750 DCU's.

20.  Multi-link dial tap functions now work correctly on a Host

Operator Station configured as "Direct" (communicating with a

78040/1 AD/AA Host Tap).  Note that as a result of this

change, SETUP.EXE must always be used to define all System and

Hardware links.  This is true regardless of the desired Host

type (Direct, Dial, LAN).

21.  The "Failsafe Command" entry in the Two-Position DDC module is

no longer incorrectly changed to "0" when a Station Restore is

performed or when power is reset on the DCU.

22.  The Host Operator Station now correctly saves all Demand Loads

when a DCU Save is performed.

23.  Exiting the DDC Automatic Tuning editor prior to its

completion, no longer causes invalid Setpoint entries to

appear in the PID and Floating module editors.

24.  DDC modules now honor external analog point "Offset Count"

entries.

25.  A LAN reconfiguration no longer results in multiple erroneous

Station Lost/Restore messages being received from a 7716 PCU

when directly connected to a Host Operator Station.

26.  Demand, Consumption, Runtime, and Override cell data from

multiple points can now be correctly stored within the same

Docutrend cell at a Host Operation Station.

27.  In the Database print editor, non-existent DCU Action Messages

no longer cause other Action Messages to be printed

incorrectly.

28.  The Network Definition editor now correctly "pages down" to a

new page when more than 18 controllers are added.

29.  The DCU Event Sequence editor no longer displays an erroneous

"." command in the list of prompt choices.

30.  When ESC is pressed to return to the Index Page from a

previously displayed System Page, the cursor now correctly

returns to the prompt choice "Page".

31.  From a complex System Page, a prompt choice of "Point" or

"Page" no longer stops the I/NET program and returns the user

to the DOS prompt.

32.  Sixteen character DCU or Link names can now be used.

33.  On converted 7760 UCI Save files, performing an Offnormal or

Disabled Point Summary no longer causes the UCI to lose its

existing database.

34.  In the PID module, the AO-PID extension, and the DO-PID

extension, the Ramp Limit entry now works correctly.

35.  DCU Master and Slave ATS output points are now controlled

properly.

36.  When a Docutrend Archive is performed, the Archive file is now

correctly named "YYMMDDNN.01".

37.  A manual Docutrend Report can now be successfully generated by

pressing "ENTER" from the system prompt rather than pressing

"ESC".  Pressing "ESC" correctly aborts the generation of the

Docutrend report.

38.  The DCU Dynamic Data Upload editor now works correctly.

39.  When the DCU Dynamic Upload editor is selected from the main

edit menu, a new prompt now appears alerting the user that a

"Dynamic Data Upload request has been initiated, press any key

to continue."

40.  The Station Software Save editor now correctly saves

information to the file named "7716.BIN" rather than

"7416.BIN".

41.  Points or Lines displayed on a System Page with their "Number

of Fractional Digits" set to zero are no longer displayed with

six decimal digits.

42.  Host ATS schedules now control both Master and Slave DCU

points correctly.

43.  External analog points, used as inputs to DDC modules, falling

below 768 counts no longer cause erroneous outputs to be

issued by PID, Floating, and Two-Position DDC modules.

44.  When the Docutrend On-line file is being re-sized via the

Shell Menu, the new RWONLN file size will be correctly

expanded to the desired number of sectors.

45.  After a Docutrend On-line file has been re-sized via the Shell

Menu, the new RWONLN file will correctly store cell

information in the expanded portion of the file.

46.  After a Docutrend RWONLN file has been re-sized via the Shell

Menu, the "Online 90% Full", "Online 95% Full", and "Online

Data Lost" messages are now correctly generated by the Host

Operator Station.

47.  Lengthy Save files can now be restored to 7760 UCI's with MIP

boards installed.

48.  Docutrend Event Cells will now correctly store system

messages.

49.  "DCU Sign-on" messages are now correctly reported back to a

Host Operator Station by the Model 7803 or MIP7803 LAN tap.

==================================================================

Released Revision 1.13 of I/NET 7700 includes the following

correction:

1.   "Delay before Make" and "Interval" DDC Relay modules can now

be added to PCU's and DCU's with MIP boards installed.  Also,

the "Delay before Make" Relay modules no longer function as

"Standard" Relay modules and the "Interval" Relay modules no

longer function as "Delay before Break" Relay modules.

==================================================================

Released Revision 1.14 of I/NET 7700 includes the following

corrections:

1.   Dial Link taps (78050, 78051, MIP78050, and MIP78051) will now

operate successfully at 9600 baud.

2.   Docutrend Variables with their "Number of Fractional Digits"

entries set to "0" now appear correctly within a Docutrend

report.

3.   Trend Plots of discrete points no longer "slant" up or down.

They are also displayed correctly even when the displayed data

is being "compressed" onto the same Trend Plot screen.

4.   The Scan Rate for all point types defined in the Resident I/O

editor of a 7750 Building Manager are no longer limited to 10

or 60 seconds.  Entries from 1 to 255 seconds are now

accepted.

5.   DCU's with MIP boards installed as well as the 7716 PCU no

longer advance by one day of the week following a

microprocessor reset.

6.   After deleting a Demand Load from a DCU and performing a DCU

Save, a DCU Restore can now be successfully performed.

7.   From the Offline database editor, the user can now delete any

demand loads from the DCU Demand extension.

8.   User can now zoom to a trend plot from a point or trend icon

on a system page even when not connected to a DCU.

9.   A new text file UPGRADE.DOC has been added to the Program

directory on I/NET7700 A/B Diskette #1.  This file contains

information on upgrading from S7000 Revision 4.00 (or higher)

to I/NET 7700 as well as information on upgrading from I/NET

7700 Revision 1.00 to I/NET 7700 Revision 1.10 (or higher).

This file will be periodically upgraded as major revisions of

I/NET 7700 are released.

==================================================================

Welcome to I/NET 7700 Revision 2.00!

Revision 2.00 of I/NET 7700 includes all of the existing I/NET 7700

Revision 1.14 features as well as the following enhancements (refer

to the file named UPGRADE.DOC for upgrade instructions):

*    Support for the Model 7780 Distributed Lighting Control Unit

(DLCU).

*    Default DCU Save files for the model 7780 DLCU for use with

the Software Restore editor.

*    Support for the Model 7718 Process Control Unit.

*    New high speed pulsed inputs (up to 20 pulses per second) in

the 7718 PCU.

*    Changes in the Multi-link AD/AA design allowing one Host PC

and AD/AA Host tap to support up to 100 System links each with

up to 64 remote sites.

*    New Maintenance Management Software package designed to be

installed on a separate Host PC on the commercial LAN.

*    New Backup Station Software package designed to be installed

on a separate Host PC on the commercial LAN.  This Backup Host

can provide support for up to eight Host Operator Stations.

*    New optional PRS232 modem board for the model 7716 PCU, 7718

PCU, and 7780 DLCU providing onboard 78035 and 78061 tap

functions.

*    Automatic Checksum calculation every 10 minutes in the 7716

PCU, 7718 PCU, and 7780 DLCU.

*    Individual scan rates for all points with separate bit offset

(BB) addresses in all DCU types.  Note that for every unique

scan rate entered, four bytes of DCU memory will be used.

*    Selectable Restart Control Actions (None/Trip/Close/Reinforce)

for all DO and DC points in all DCU types.

*    New Event Sequence Zero in all DCU types allowing the user to

predictably control DO, DC, and AO points following a DCU

power failure.

*    1440 Trend Samples per point address in all DCU types.

*    New offline Page Point Conversion Utility allowing the user to

quickly change the link (LL), station (SS) or point (PP)

address of a point or a group of points on a graphic system

page.  The user can also change the link or station address of

DDC lines and DDC modules.

Released Revision 2.00 of I/NET 7700 also includes the following

corrections:

1.   The Docutrend to DIF editor now allows up to sixty-four

entries.

2.   Help screens have now been provided for the Host Network

Configuration Editor.

3.   To enhance the operator interface during a Backup or Restore

operation, a new message now appears prior to the reloading of

the background driver program.

4.   The Third Party DOC to DIF conversion did not convert all

Docutrend records when a conversion was attempted on a single

cell which contained multiple records.

5.   External Discrete Input points in all controller types were

not being globalized.

6.   Each time a PID or Floating module was copied, the last three

tuning parameters (Maximum Overshoot, Target Damping, and

Noise Band) were incorrectly changing on the target module.

7.   Link numbers, from 00-99, can now be entered in all AD/AA LAN

taps.

8.   Manually adding two consecutive points from a DCU to a system

page while connected to a different DCU caused the cursor to

be incorrectly positioned onto the screen.

9.   When a Save file was restored to a 7716 PCU, or any other DCU

with a MIP board, via the Software Restore Editor, indirect

points did not automatically begin to update.

10.  All DCU's on the controller LAN were simultaneously changing

their address to the same incorrect value. The problem

occurred in situations where numerous errors in communications

were occurring due to power fluctuations, improper LAN

installation, lightning strikes, etc.

11.  The user could not select a Page ICON from a graphic System

Page after a DCU Summary had been performed.  The prompt

"Enter/Select Page Number:" correctly appeared; however, none

of the page ICON's on the System Page were highlighted (for

selection by the user) and a constant audible error was

sounded anytime the user attempted to move the mouse.

12.  The system prompts which appear when "Page", "Point", or

"Zoom" are selected from the main prompt line on a graphic

System Page have been changed.

13.  The Trend Plot Editor has been removed from the Offline

Database Editor.

14.  If an external discrete point was displayed on a page as a

graphics point (pair of library symbols) and the point changed

state, the Host would lock-up.

15.  Adding a Trend Sampling extension to a DC or DM point caused

the DC point's Time To State (TTS) entry to be ignored.  The

same problem also occurred anytime a Trend Sampling extension

was added to any other point type with the same address as

either the DC or DM point.

16.  The Host PC locked-up (displaying a permanent "Please Stand

By" message) when exiting the Offline Database Editor, if the

user was editing a multiple station Save file and added a

point which is resident in any station other than the base

station of the DCU.

17.  When Runtime extensions were being printed, the DCU point

addresses were incorrect.

18.  Within a database print, Discrete Monitor addresses appeared

incorrectly if the point resides in a different station than

its associated Discrete Control point address (only possible

within a multi-station DCU).

Also, released Revision 2.00 of I/NET 7700 includes the following

known problems:

*    The Look-up table parameters entered for a 7716 are not

getting printed out correctly in a database print.

*    In the Software Restore Editor, a 771603 DCU cannot be

downloaded as a 7716 without receiving a "Response error 1"

message.  The user can successfully download the device as a

7803 tap.

*    The wink off time is not being honored for an 7750 OB wink

point.

*    If the user changes the switch settings on a 78061 tap (EPROM

version), power must be cycled twice before the modem will get

initialized.

*    Site numbers cannot be entered in a 7716, 7718, or 7780 via

the HHC.

*    After an operator has made entries in the Manual Tuning

portion of the 7760 UCI Tuning editor, the Input_Output plot

for that extension will not display the correct values.

*    "Momentary Output Duration" values are not being included in

a database printout for DO/DC points.

*    Purging alarms from the second page of an Alarms file causes

the remaining alarms to scroll from the top to the bottom

instead of from the bottom to the top.

*    During an All_S/W and All_D/B download of several controllers,

it has been observed that not all of the DCU Save files are

successfully restored.  Once the problem appears, the editor

will begin to download the Save file and abort at

"Configuration".  This results in the correct DCU name being

successfully downloaded to the DCU, but none of the point or

extension information.

*    The default timeout parameter for a 7806/61 Dial tap needs to

be modified from 30 to 60 seconds to ensure adequate time for

successful connection of the Optima series modems at 4800 and

9600 baud.

*    The user cannot zoom to a UC extension which is attached to a

DO point with a non-zero bit offset.

*    PID modules (without Reset and Rate defined) in the EPROM

versions of the 7700 and 7740 DCU's only do not work correctly

when ramp limit is used.  The ramp limit correctly limits the

module output on the first module output, but then the module

fails to change its output again.

*    An "@" sign is not appended to beeper numbers as they are

being dialed by the 78060/61 AD/AA LAN taps.

=================================================================

Released Revision 2.00 of I/NET 7700 also includes the following

corrections which were not included in the README.DOC file (dated

11/13/92):

1.   Action Messages #17 through #33 are now being saved properly

when a DCU Save is performed.

2.   Analog and Discrete Sample cell records stored immediately

prior to midnight are no longer stamped with the incorrect

date and time.

=================================================================

Released Revision 2.01 of I/NET 7700 includes the following

corrections:

1.   When using the LANEX.EXE program with a large number of Host

Operator Stations on an EtherNet LAN, unusually long keyboard

response times (6 to 10 seconds between keystrokes) are no

longer encountered.

2.   The LANEX.EXE program no longer runs session traffic at

unpredictable rate on a "quiet" Commercial LAN.

3.   Host Operator Station PC's on an EtherNet LAN which are also

directly connected to a 7716 PCU (configured as 771601) no

longer intermittently display point values as "???????" on

graphic System Pages or Summary displays ultimately requiring

the user to press ESC and re-display the desired Page or

Summary.

4.   Host Operator Stations on an EtherNet LAN, no longer lock-up

when executing simultaneous bi-directional FRESTORE.EXE

commands.

5.   Host Operator Stations on an EtherNet LAN no longer lock-up

when one Host Operator Station is removed.

6.   Host Operator Stations on an EtherNet LAN no longer

permanently display point values "????????" on graphic System

Pages or Summary displays requiring the user to disconnect and

re-connect to the DCU.

7.   Host Operator Stations on an EtherNet LAN no longer lose their

ability to display messages.  When the problem occurred, the

"MSG" icon correctly appeared; however, when "Message" was

selected the Host Operator Station "locked-up".

8.   Host Operator Stations on an EtherNet LAN no longer stop

polling their associated taps when not displaying a graphic

System Page or Summary requiring the user to cycle power on

the model 7801 Host tap.

9.   The user no longer must cycle power twice on a Model 78061

AD/AA LAN tap to correctly initialize the tap's external modem

after changing the switch settings on the tap.  The same

problem was also corrected in the Model 7810 Printer Tap.

10.  The default timeout parameter for Model 78060/61 or

MIP78060/61 Dial taps was changed from 30 to 60 seconds.  This

ensures adequate time for successful connection of the Optima

series modems at 4800 and 9600 baud.

11.  To fully support the use of various pager systems, several

additional phone number characters have been added to the

78060/61 and MIP78060/61 Tap editor.  These additional

characters are as follows:

"@"- detects a 5 second period of silence.  This replaces the

need for numerous commas.

";"- causes an immediate hang up.  This should be used at the

end of every pager number dialed.  Note that this

character is only valid at the end of a telephone number

to be dialed.

"!"- issues Hook flash.  This forces the phone to go "on-hook"

for 1/2 second.

"W"- waits for dial tone.

These characters have also been added to the 7716, 7718, and

7780 DCU LAN tap editor which is displayed when any of these

DCU types is configured as 77XX06.  The addition of these

characters also resulted in a change to the MIPBOOT, 7716BOOT,

7718BOOT, and 7780BOOT firmware.

12.  When a MIP78060/1 is returned to boot control, having phone

numbers entered in the tap which do not fit entirely in NOVRAM

memory, no longer causes all the phone numbers within the tap

(including those in NOVRAM and RAM memory) to be erased.  This

same problem could also occur in the 7716, 7718, or 7780 DCU's

when configured as 77XX06 and therefore a correction in the

7716, 7718, 7780 downloadable binary files and the MIPBOOT,

7716BOOT, 7718BOOT, and 7780BOOT firmware has been made.

Also note, after RAM memory in an AD/AA LAN tap or DCU is

lost/cleared, any portion of a phone number that was in the

tap or DCU NOVRAM memory will remain in the tap or DCU even if

it is an incomplete portion of the phone number.

13.  When calling out to a pager, the 78060/61 AD/AA LAN taps no

longer continually call out to the same pager number every 60

seconds, even though the first call to the pager was

successfully received.  Correcting this problem also resulted

in a change to the 7716, 7718, 7780 downloadable binary files

and the MIPBOOT, 7716BOOT, 7718BOOT, and 7780BOOT firmware.

14.  The 78060/61 and MIP78060/61 tap phone number mask positions

for calls to pagers are now being honored correctly.

Correcting this problem also resulted in a change to the 7716,

7718, 7780 downloadable binary files and the MIPBOOT,

7716BOOT, 7718BOOT, and 7780BOOT firmware.

15.  When a MIP78060/1 is returned to boot control, all NOVRAM

resident tap telephone numbers are now successfully being

dialed out.  Previously, the second, third, and

subsequent group's telephone numbers were being dialed

out, but not correctly received by a Host PC.

16.  An Automatic DCU save is now only performed on a DCU that

contains a "DCU Last change" date which is more recent than

the current "DCU Save file" date.

17.  Adding an "Alarm Delay Time" to an Internal Discrete Alarm

(DA) point addressed at LLSSPP08 or LLSSPP09 no longer causes

the point to never go into alarm.

18.  Revision 1.XX 7760 UCI Save files are now correctly converted

by the Database Convert Program.  Previously a "Response error

2" error message and Retry/Abort prompt was received when the

"History Parameters" portion of the converted Save file was

being restored to the Revision 2.00 UCI.

19.  The correct DCU type is now displayed by the Database Convert

Program.

=================================================================

Released Revision 2.02 of I/NET 7700 includes the following

corrections:

1.   The 78060 Tap would occasionally stop dialing out to report

alarms.  If power was cycled on the tap, the device would

resume dialing out after an undetermined period of time.  This

problem only required a change in the 78060 tap (EPROM version

only).

2.   Automatic DCU Saves, DCU Time Synchronization, and Host ATS

functions now work correctly with MIP78040/1 AD/AA Host Taps

and MIP78050/1 AD/AA Link Taps.  Note that this problem was

fixed in the Host PC, not in the taps.

3.   Under certain circumstances, "old" global points (controlled

via a calculated point extension) in one DCU caused their

associated indirect points in other DCU's to permanently be

displayed as "old".  This most often occurred when a global

point in one DCU was being controlled with a calculated point

and one of the calculated point parameters was an indirect

point from the same DCU.  Also, the global point was

broadcasting its state/value to another indirect point in a

second DCU.  This problem has been fixed in all DCU types.

Note that as a result of this change, the state/value of any

global point is now broadcast onto the controller LAN anytime

the point changes from a normal state/value to "old" or from

"old" back to a normal state/value.

4.   The Alarm Inhibit function in all DCU types has now been

corrected such that Discrete Alarm (DA) points addressed from

0000 through 0309 now correctly generate an alarm if they

change to the alarm state while inhibited and the alarm period

expires.  This problem has been fixed in all DCU types.

5.   DCU/PCU's no longer incorrectly generate a "Return to Normal"

message when a point goes into alarm, and then returns to

normal during the alarm inhibit period.  This problem has been

fixed in all DCU types.

6.   DCU/PCU's no longer lockup as a result of simultaneous editing

from multiple Host Operator Stations.  This problem has been

fixed in all DCU types.

=================================================================

Released Revision 2.10 of I/NET 7700 includes the following change:

1.   A new entry has been added to the 7716 and 7718 PCU resident

PID, Floating, and Two-Position DDC module editors.  This new

entry "Input Filter? (Y/N)" allows the user to individually

enable/disable the automatic filtering of a Process Variable

input on any selected PID, Floating, or Two-Position DDC

module within the 7716 or 7718 PCU.

Also, released Revision 2.10 of I/NET 7700 includes the following

correction:

*    Corrections have been made to the following Online Help

Screens:

#2102 (Resident I/O Points)

#2201 (Calculated Points)

=================================================================

Released Revision 2.11 of I/NET 7700 includes the following

enhancements:

*    A new CHKONLN.EXE program has been placed in the RW

subdirectory of INET7700A/B diskette number 2 which can be

used to repair Docutrend RWONLN files with corrupted sectors.

RWONLN sector problems can cause the Host PC to stop

collecting Docutrend cell data or can corrupt previously

collected cell data.  Note this utility corrects sector

problems with Docutrend Online (non-archived) RWONLN cell data

only.

Type "CHKONLN/S" followed by the "ENTER" key to display the

current RWONLN file statistics.  Type "CHKONLN/F" followed by

the "ENTER" key to fix the RWONLN file.

With the release of this utility, SPR-760 and SPR-774 have

been closed.

*    Support for CMOS NOVRAM memory in the 7716BOOT, 7718BOOT, and

7780BOOT firmware.

Released Revision 2.11 of I/NET 7700 includes the following

corrections and/or changes:

1.   Indirect Points which reference non-existent global points in

other DCU's no longer cause constant LAN reconfigurations.

2.   Indirect Points continue to work correctly even if the DCU

containing their associated global point is restored via the

Station Restore Editor.

3.   DCU's with more than 50 indirect and/or global points now

correctly broadcast/update their global/indirect point values.

4.   Previously, controllers with single point databases (driven by

calculated point extensions) stopped working properly

following a DCU reset or power failure.  This problem has been

fixed in all DCU types.

5.   External PWM Analog Output points no longer "lock-up"

following a Station Restore to a 7700, 7740, 7716, or 7718

PCU.

6.   771606, 771806, and 778006 phone numbers are no longer

corrupted if the user escapes from the Site Tap Configuration

Editor, enters and escapes from the DLCU Configuration/Status

Editor, followed by re-entering the Site Tap Configuration

Editor.

7.   7806X and MIP7806X taps no longer reconfigure the controller

LAN when the Link Tap Configuration Editor is modified.

8.   A default link number of "**" no longer appears for newly

added phone numbers in the AD/AA Link Tap Configuration

Editor.

9.   External modems connected to AD/AA LAN Taps and AD/AA Host

Taps no longer remain connected after transmitting an "Upload

Request" to a Host PC whose background driver is not running.

10.  When calling out on a Multi-link AD/AA Host PC (with more than

one System Link defined per Hardware Link) the system now

correctly "rolls-over" to other AD/AA Link Taps on the same

Host LAN.

11.  A 771603 PCU can now be successfully downloaded by selecting

the PCU as a 7716 PCU or 7803 Tap from the Host Software

Restore Editor.

12.  The local Host PC keyboard no longer "locks-up" following an

edit session from a remote Host PC on the same commercial LAN.

13.  Host Passwords are now correctly equalized between all Host

PC's on the same commercial LAN.

14.  Transient Docutrend cells now work correctly.

15.  In the Docutrend Cell Inquiry Editor, Transient cells are no

longer archived.  If attempted individually, an error message

will be received.  If "Select_all" is used, the system will

skip over any existing transient cells.

16.  The manual and automatic scaling of the X-axis in a Docutrend

XY Plot is no longer off by a factor of 5.

17.  Deleting all of the data from a Docutrend cell and attempting

to manually add new cell data without escaping from the editor

no longer results in corruption of the Cell Inquiry Editor.

18.  Weekly automatic Docutrend reports are now correctly printed

on Sundays rather than Tuesdays.

19.  The operation of the Docutrend Lookup Table and 7716/7718

Lookup Table editors has been changed allowing the user to

"add" a value prior to the first line of the table.

20.  A new "Add Delete Modify" prompt has been added to the

7716/7718 Lookup Table editors.

21.  In the first Host Trend Log Report (at the "Print begin" time)

valid sample data will now appear for points provided that the

editor's Sample Interval and Print Interval are set the same.

22.  A communication error occurring during an "All_S/W" and

"All_D/B" restore to a group of DCU's no longer causes the

editor to abort the database download of all subsequent DCU's

resulting in lost DCU point and extension information.

23.  After an operator has made entries in the Manual Tuning

portion of the 7760 Tuning Editor, the Input-Output Plot for

that extension now displays the correct values.

24.  When selecting a controller from the second page of the

Connect Screen, the user is no longer connected to a

controller on the first page of the screen.

25.  Purging alarms from the second page of the Alarms Summary

screen no longer causes the remaining alarms to incorrectly

scroll down (rather than up) the screen.

26.  "ATS-mstr failed" and "Special day lost" messages are now

correctly reported on a Host PC configured as "Direct".

27.  A new prompt is now displayed when the user exits the Manual

Tuning portion of the Direct Digital Control editor (or UC

editor) giving the user the option of automatically updating

these values in the PID or Floating module (or UC extension).

28.  From a System Page or the Page Editor, the user can now zoom

to a UC extension which is attached to a DO point with a non-

zero bit offset address.

29.  When accessed via Zoom, the Trend Plot editor now correctly

displays the address of the point being displayed.

30.  An operator who is password protected from all system

functions except the ability to display pages can no longer

use Zoom to access the Event Definition, Trend Sample, Trend

Plot, or DDC icons.

31.  The user is now given the option to "Update the save file?

Y/N" for a DCU when exiting the Offline Database Editor.

32.  When copying Lighting Circuits from the second page of the

Lighting Control editor, the editor no longer becomes

corrupted after pressing the PgUp key.

33.  The correct controller address is now displayed on line two of

the graphic system page when the user zooms to a controller

different from the one he is currently connected to.

34.  In the System Page editor, text added to the first line of the

page can now be deleted without having to exit and re-enter

the editor.

35.  When editing System Pages 10 through 1023, lengthy page names

(displayed on line 1) no longer wrap around to line 2 of the

CRT display.

36.  In the System Page Editor, the Control and Home keys can now

be used to insert blank lines past lie 30.

37.  When adding AO-PID, DO-PID, AO-FLT, or DO-FLT extensions to a

UCI/UC, the default "Input High Limit" and "Output High Limit"

values are now correctly set to 100.

38.  Model 7750 DCU save files are now successfully converted by

the DBCNV46.EXE database conversion program.

39.  The Installation Program now correctly limits the Data File,

Message, DCU Save File, Docutrend, and Docutrend Archive

subdirectory pathnames to 16 characters to prevent Host PC

lockup problems.

40.  The word "Archive" is now spelled correctly in the

Installation Program menu.

41.  The "Edit...All_Points" prompt in the Graphic Page Point

Conversion Editor is no longer overwritten with incorrect

characters when the user escapes and re-enters the editor.

42.  When editing lines or modules in the Graphic Page Point

Conversion Editor, the user is now allowed to select

"All_Stations" from the prompt.

43.  In the Graphic Page Point Conversion Editor, the name of a

previously modified page no longer remains on line 1 of the

CRT display when a subsequent page is selected for

modification.

44.  7716 and 7718 PCU Look-up Table entries now appear correctly

in a database printout.

45.  "Momentary Output Duration" values now appear correctly in a

database printout for DO/DC points.

=================================================================