************************************************************

U.S. ROBOTICS

The Intelligent Choice in Data Communications

************************************************************

************************************************************

------------------------------------------------------------

REFERENCE GUIDE

for

Sportster High Speed Modems

with V.42 bis

------------------------------------------------------------

************************************************************

Manual Revision Date:  4/1/94

Copyright 1994 by U.S. Robotics, Inc.

8100 North McCormick Blvd.

Skokie, Illinois 60076-2999

All Rights Reserved

************************************************************

TABLE OF CONTENTS

************************************************************

SPORTSTER CONFIGURATION

Factory Configuration (&Fn and Yn)

Permanent Configuration Templates (&Fn)

Programmable Templates (Yn)

Customizing Your Defaults (&Wn)

Using AT Commands

Resetting the Modem (Z)

PLACING CALLS

Dial (D)

Cancel Dialing

Dial Command Options

Dial Type-Pulse or Tone

Adaptive Dialing (X2-X4)

Wait for Another Dial Tone (W)

Pause (,)

Slash (/)

Dial and Return to Command Mode (;)

Dialing Letters (")

Transferring Calls (!)

Wait for an Answer (@)

Reversing Originate/Answer Frequencies (R)

Dialing Stored Numbers (DSn)

Redialing

Dial the Last Dialed Number (DL)

Re-execute the Last Command (A/)

Answering

Suppressing Auto Answer

Manual Answer

Points to Remember

Disconnecting and Hanging Up

Escape Code Operations (+++)

Modem Response to +++

Returning Online (On)

Hanging Up (Hn)

Break Handling (&Yn)

International Calls

ITU-T/Bell Answer Sequence (Bn)

Guard Tone (&Gn)

Pulse Dial Make/Break Ratio (&Pn)

ITU-T V.21-300 bps

ITU-T V.23-1200/75 bps

TEMPORARILY CHANGING A SETTING

Tone Dialing

Speaker

Volume Control (Ln)

Speaker Control (Mn)

Local Echo

Command Mode Local Echo (En)

Online Local Echo (Fn)

S-Registers

Result Codes

Response Modes (Vn)

Quiet Mode (Qn)

Result Code Options (Xn)

Additional Result Code Options (&An)

Error Control (&Mn)

Auto Answer with Error Control

Data Compression (&Kn)

Data Rate Commands

Software Requirements

Serial Port Rate (&Bn)

Connection Rate (&Nn)

Flow Control

Warning on Software Flow Control

Software Flow Control Precautions

Transmit Data Flow Control (&Hn)

When it is Required

Received Data Flow Control

Hardware Control (&Rn)

Software Control (&In)

Serial Operations

Carrier Detect (&Cn)

Data Terminal Ready (&Dn)

Data Set Ready (&Sn)

GETTING HELP

Inquiries (In)

S-Register Query (Sr?)

Stored Phone Number Query (&Zn?)

Last-Dialed Number Query (DL?)

Help Screens

Stop/Restart Display

Cancel Display

Basic Command Set ($)

Ampersand Command Set (&$)

Dialing (D$)

S-Register Functions (S$)

TESTING THE MODEM

Testing with &T

Dial Test-S16=2

OTHER OPERATIONS

Fax Operations

Voice/Data Communications

TECHNICAL SPECIFICATIONS

Compatibility

The Serial Interface

Default Settings

MODEM CONCEPTS

How Modems Work

Modem Configuration

Line Travel

Digital Data

Flow Control

Error Control

ITU-T V.42 Handshaking

MNP Handshaking

Data Compression

Flow Control

Online Fallback/Fall Forward

Throughput Guidelines

Achievable Throughput Statistics

************************************************************

SPORTSTER CONFIGURATION

************************************************************

The Sportster is preconfigured at the factory so you should

be able to connect the modem, set up your communications

software, and be ready to send and receive data.  Many users

won't need to change the default configurations, but if you

want to, this guide will tell you how.  Many of the

functions described below can be handled by your

communications software, but you also have the power to

place your computer in Terminal mode (via your

communications software) and control the modem directly.

NOTE:  Unless otherwise indicated, settings listed as

"default" are based on those set for the modem when it is

shipped.

============================================================

FACTORY CONFIGURATION (&Fn and Yn)

============================================================

The Sportster is preconfigured at the factory with three

permanent and two programmable templates.

============================================================

Permanent Configuration Templates (&Fn)

The following permanent templates come with your Sportster.

The default template consists of the most reliable settings,

but these settings may not work for all users.

&F0    This template does not include performance features,

such as a fixed serial port rate or hardware flow

control.  What it does offer is compatibility with

non-typical computers or software that cannot handle

flow control and other features.

&F1    Default.  This template sets the modem to

hardware flow control, a fixed serial port rate, the

highest level result codes, and the most complete

result-code set.

The &F1--Hardware Flow Control template is

recommended for all IBM-compatible computers, as long

as your communications software supports hardware

flow control, a fixed serial port rate, and the

advanced result-code subset.

NOTE:  We also recommend that users of Macintosh

computers invest in a Hardware Handshaking cable

(instead of the standard cable), use this template,

and use software that supports hardware flow control.

&F2    This is a software flow control template.  We

recommend you do not use software flow control, and

thus, this template.  However, if you use a

Macintosh, see the note above.  In addition, some

computers are limited to three-wire cables that do

not support hardware flow control signals.  If this

is the case, review the warning on software flow

control later in this guide.

To load the &F2--Software Flow Control template and

save it as your power-on/reset default, enter the

following command:

AT&F2&W <Enter>

NOTE:  Refer to the "Programmable Templates" section

for more information on selecting and modifying

templates to save as your start-up configuration.

If you need a low performance template because of data-

transfer difficulties, flip DIP switch 7 ON and reset the

modem (ATZ command), or load the &F0--Low Performance

template by entering the following command:

AT&F <Enter>

NOTE:  When the modem receives the above command, it assumes

you want to load the &F0 template because there is no

position number indicated.  It is the same to enter AT&F as

to enter AT&F0.

This template allows you to communicate with non-typical

systems that cannot handle flow control and other features.

============================================================

Programmable Templates (Yn)

In addition to the &Fn permanent configuration templates,

two programmable templates are provided.  Your programmable

templates are located in nonvolatile, random access memory

(NVRAM).

The Sportster is shipped to load NVRAM template 0 (or Y0)

for its power-on/reset default settings.

Possible NVRAM templates:

Y0    Loads the NVRAM template 0 (stored with template

&F1--Hardware Flow Control settings).  Default.

Y1    Loads the NVRAM template 1 (stored with template

&F2--Software Flow Control settings).

If you want to switch the power-on/reset default NVRAM

template from Y0 to Y1, enter the Yn command, then reset the

modem with the Z command.

AT Y1 Z <Enter>

To customize other defaults you want to store in NVRAM as

part of your customized power-on/reset template, use the &Wn

command (below).

WARNING:  The Yn command, which selects template settings to

use as the power-on/reset default, may select different power-

on/reset default settings than those currently loaded (if you

altered the settings during the current session).  Before

changing a template, you may want to use option 4 of the

Inquiry (I) command to display the current settings.

current settings.

AT I4 <Enter>

The Y setting on the ATI4 screen indicates if Y0 or Y1 is the

power-on/reset default.  If you want to check all the NVRAM

settings in both Y0 and Y1 templates, use option 5 of the

Inquiry command (I).

AT I5 <Enter>

============================================================

CUSTOMIZING YOUR DEFAULTS (&Wn)

============================================================

When the modem is shipped, it reads template 0 in NVRAM as

the power-on/reset default.  Template 0 (Y0) is initially

loaded with &F1 permanent configuration settings.  Although

you cannot alter the &F1 template, you can add, delete, or

modify the settings stored in your NVRAM templates.  It is

more likely, however, that you will want to keep your power-

on/reset default settings and simply change a setting for a

current session.  Refer to "Temporarily Changing a Setting"

in this guide for more information.

If  you want to modify your default configuration settings,

use the &Wn command.  Possible settings are:

&W0    Modifies the NVRAM 0 template (Y0).

&W1    Modifies the NVRAM 1 template (Y1).

Typically, users prefer the default settings stored in NVRAM

when the modem is shipped, but you may set up different

power-on/reset defaults.  For instance, if your system

supports hardware flow control, you can set your Y1 template

to the settings for hardware flow control so both templates

set the modem for hardware flow control.  (The Y0 template

is shipped with hardware flow control settings.)

AT &F1 &W1 <Enter>

NOTE:  Yn is unique because it writes itself to NVRAM-you do

not need to use the &Wn command.

Similarly, if your system only supports software flow

control, you can set your Y0 template to the settings for

software flow control so both templates set the modem for

software flow control.  (The Y1 template is shipped with

software flow control settings.)

AT &F2 &W <Enter>

You may also specify the entire configuration in a single

command string that ends with the &Wn command.  The

following example sets the modem for the current session and

stores the configuration in NVRAM 1 (or Y1).

AT &K3 &W1 <Enter>

After sending a configuration to NVRAM, you can change any

setting just for the current session, as in the following

example.  The NVRAM configuration remains intact.

AT &K3 <Enter>

But if you want the new setting to be a default, write it to

NVRAM at the same time, as in the following example, which

saves the setting to NVRAM template 0.

AT &K3 &W <Enter>

If you've sent the modem commands to change settings

throughout your session and want to save your current

configuration, send just the &Wn command.  The current

settings are then written to NVRAM 0 (or Y0) in the example

below.

AT&W <Enter>

If you cannot use hardware flow control and need to use

Xmodem-type file transfer protocols, use either Y0 or Y1 for

a no flow control configuration with no error control.

AT &F &M &W <Enter>

or

AT &F &M &W1 <Enter>

============================================================

USING AT COMMANDS

============================================================

To send commands directly to your modem, first put your

computer in Terminal mode.  Some communications programs do

this automatically upon loading.  Others require you to

display a communications terminal screen, press a Function

key, or perform some other operation.  If necessary, refer

to your communications software documentation for

instructions.

1.   Type all commands in either upper case (AT) or lower

case (at), but not a combination (At).

2.   All commands except A/ (re-execute last command) and

+++ (escape code) are preceded by the AT prefix and are

executed with the Enter/Carriage Return key (<Enter>).

3.   Command string length = 40 characters, maximum.  The

modem doesn't count the AT prefix, Carriage Return

character, or spaces.  In a dial string, it counts--but

doesn't act on--punctuation such as hyphens and

parentheses.

4.   A missing numeric parameter is assumed to be zero, as

in the command to hang up; ATH <Enter> is the same as

ATH0 <Enter>.

============================================================

RESETTING THE MODEM (Z)

============================================================

If you've changed several current settings and want to reset

to your power-on defaults, type the following command.

ATZ <Enter>

The modem reads its DIP switch settings and resets to its

NVRAM defaults (DIP switch 7 OFF) or the &F0--Low

Performance template (DIP switch 7 ON) settings.

NOTE:  Use the ATZ command if you've changed the position of

DIP switches 1-7 while the modem is on, so that the modem

can read the new settings.  The only other way to initiate a

new setting for switches 1-7 is to turn the modem off and on

again.

************************************************************

PLACING CALLS

************************************************************

NOTE:  Unless otherwise indicated, settings listed as

"default" are based on the hardware flow control template

stored in NVRAM when the modem is shipped.

============================================================

DIAL (D)

============================================================

To dial a phone number and place a call without using your

software's dialing directory, first put the computer in

Terminal mode.  Then type the AT and D commands, the number

you wish to connect with, and press Enter.  Spaces in our

command examples are ignored by the modem and are only

included for readability.

ATD 1234567 <Enter>

Unless you lowered your modem's speaker volume, you will

hear the modem go off hook and dial the telephone number,

followed by a series of handshaking signals.

The Dial command string may include up to 40 characters.

The modem counts but ignores punctuation characters such as

parentheses and hyphens.  It does not count spaces, the AT

prefix, or the Carriage Return key (<Enter>) required to

execute the command.

The modem also executes any other commands or options

included in the command line.  The following Dial command

example instructs the modem to turn off the speaker (M0),

and dial (D) the phone number (1234567) using tone dialing

(T).

AT M0 DT 1234567 <Enter>

You may want to review the many options on the next several

pages that are available for tailoring Dial strings.  The

most typically used are the Dial Type--Pulse or Tone, and

the Pause options--comma, slash, and W.

============================================================

Cancel Dialing

To cancel Dial command execution, press <any key>.  If you

inadvertently hit a key on the keyboard while the modem is

dialing, the call is canceled.  If this occurs, type the A/

command, which re-executes the last command you entered.

============================================================

Dial Command Options

You can modify the dial string according to the needs of the

connection.  Listed below are options available to you when

entering the Dial string.

-----------------------

Dial Type-Pulse or Tone

-----------------------

The modem defaults to pulse (rotary) dialing.  To have the

modem use tone dialing, which includes the asterisk (*) and

pound sign (#), use the T command.

Dial type commands may be included in the Dial string (ATDT

number) or, issued separately (ATT or ATP).  However, if

you'll always use tone dialing, write tone dialing to NVRAM

as the modem's default and/or use adaptive dialing,

described below.

NOTE:  You can switch from one dial type to another within a

dialing sequence.  The modem remains set to the last dialing

type instruction until it is reset (ATZ command), or it

receives a different dial-type command.

--------------------------------

Adaptive Dialing (X2 through X4)

--------------------------------

When any of the X2 through X4 (default) result code options

is in effect and you do not issue a dial type in the Dial

string, the Sportster defaults to pulse dialing.  If you

issue a tone dial command,  the Sportster will default to

Hunt dialing, beginning with tone and adapting to pulse when

necessary.  For instance, if the phone company's central

office does not have tone-detection equipment, the modem

automatically reverts to pulse dialing.

------------------------------

Wait for Another Dial Tone (W)

------------------------------

This command is useful in situations where you must wait for

a second dial tone before continuing to dial.  For example,

if you need to dial for an outside line, as in the following

example, the Sportster continues to dial as soon as it

detects the next dial tone.

ATD9W1234567 <Enter>

NOTE:  This command executes only if result code option X3

or X4 (default) has been issued.  If the modem is set to X2

or lower, the modem interprets the W as a comma (two-second

pause).

---------

Pause (,)

---------

A comma causes a two-second delay in the dial sequence.  The

following example contains four-second delays at several

points.

ATDP 9,,7654321,,55555,,1 312 1234567 <Enter>

The first four-second pause is to access an outside line

after dialing 9, but you may wish to use the W option, just

described.  The second pause is to make sure the remote

system is ready for the user's account number, and the

third, to delay before dialing the long-distance number.

Such pauses, however, may not be necessary.  Experiment and

use delays only as required.

---------

Slash (/)

---------

A slash (/) can be used in any command string to make the

modem pause for only 125 milliseconds.  Some users find it

helpful to have the shorter delay of a series of slashes,

rather than the 2-second comma pause.

-----------------------------------

Dial and Return to Command Mode (;)

-----------------------------------

If your phone is plugged into the modem, you can use this

option to have the modem Auto Dial a telephone rather than a

modem.  The Sportster dials, remains off hook, and returns

the OK message, indicating it is in Command mode.

For example, to have the modem place a voice call, enter the

Dial command with a semicolon.

ATDT5551234; <Enter>

When the modem returns the OK result, pick up your phone

receiver so you can talk to the other party, and send the

command that hangs up the modem.

ATH <Enter>

Similarly, if you can call a recorded weather or other

service, have the modem Dial, listen to the recording over

the modem's speaker and, when you are finished, instruct the

modem to hang up.

-------------------

Dialing Letters (")

-------------------

Quotation marks are used to make the modem dial

abbreviations and acronyms used as phone numbers, such as

DIAL USR (the U.S. Robotics Sales Department's 800 number).

This option is called Quote mode.  Quotation marks are

inserted at the beginning of the alphabetic string.

ATDT "DIAL USR <Enter>

NOTE:  If you are including another command after the dial

string, use closing quotation marks before the additional

command.

ATDT "DIAL USR",,,1234<Enter>

----------------------

Transferring Calls (!)

----------------------

This command is used for modems installed where other modems

share the phone line.  The modem flashes the switch-hook.

That is, it goes off hook 0.5 seconds, on hook for 0.5

seconds, and off hook again to dial the specified extension.

The following example includes instructions to return to

Command mode (;) and to hang up (H).

ATDT !1234;H <Enter>

----------------------

Wait for an Answer (@)

----------------------

Some online services answer the phone and return a tape-

recorded request for information before processing

transactions.  In such instances, the @ command can be used

in the Dial string to tell the modem to detect at least one

ring, wait for five seconds of silence at the other end of

the call, and then continue to execute the Dial string.

To use the @ command, set the modem result code option to X3

or X4 (default).  If the modem is set to X2 or lower, the

modem returns an ERROR message when encountering the @

character in a command string.

In the next example, the modem is set to the X3 result code

option and dials a banking service.  Each occurrence of @ in

the example indicates a five-second wait for silence.  That

is, for taped requests from the bank for a password (12345),

an account number (6789), and a transaction code (2).  The

transaction code might indicate, for example, a request for

an account balance.

ATX3 DT5551234 @ 12345 @ 6789 @ 2 <Enter>

If the necessary conditions do not occur--no rings, or no

following five seconds of silence--the modem times out as it

normally would (after 60 seconds).  It then sends the

message NO ANSWER to the screen and aborts the call.

------------------------------------------

Reversing Originate/Answer Frequencies (R)

------------------------------------------

This command allows calls to an originate-only modem (a

modem set up to send only).  It reverses the modem's

originate/answer frequencies, forcing the Sportster to dial

out at the answer frequency.  The command follows the Dial

command, before or after the phone number.

AT D1234567R <Enter>

AT DR1234567 <Enter>

============================================================

Dialing Stored Numbers (DSn)

You can store up to four frequently used telephone numbers

in nonvolatile, random-access memory (NVRAM).  Use the &Zn=s

command to store telephone numbers, where n is the position

of the number in NVRAM (0-3) and s is the stored number.

The following command string stores the telephone number

(4441212) at position 1 in NVRAM.

AT&Z1=4441212 <Enter>

To dial a stored number, use the DSn command, where n is the

position of the number in NVRAM (0*3).  In the first

example, the modem dials the phone number stored at position

0.  In the second, the modem dials the phone number stored

at position 3.

ATDS0 <Enter>

ATDS3 <Enter>

You can also store a partial dial sequence.  If you often

call a set of phone numbers and only their last three or

four digits differ, it might be useful to store the other

digits.  The following example stores a partial phone number

at position 1.

AT&Z1 = 9W1 616 123 <Enter>

Once you have stored the partial phone number in NVRAM, use

the DSn command to dial the partial number, placing a slash

(/), then the remainder of the number after the DSn command.

The slash separates the DSn number from the remainder of the

string, yet maintains the dial command..

AT DS1/4567 <Enter>

NOTE:  Do not include modem settings in the &Zn=s string.

If the call requires modem settings, insert them in the

command string before the DSn command.  For example, &K3

(selective data compression) is inserted before the Dial

command to call the number stored at position 0.

AT&K3 DS0 <Enter>

This establishes &K3 as the current setting.  To return to

default data compression mode after the call, issue the

following command.

AT&K1 <Enter>

============================================================

REDIALING

============================================================

The most frequent reason for redialing is receipt of a busy

signal.  The Sportster modem provides two redialing options,

as follows.

============================================================

Dial the Last-Dialed Number (DL)

When you want to redial a number, enter the DL command.  The

modem dials the last-dialed number, which it has stored in a

special buffer.

ATDL <Enter>

To display the number stored in the last-dialed buffer, use

the following query.

ATDL? <Enter>

To write the last number dialed to NVRAM, use &Zn=L where n

is the position in NVRAM.  The following example stores the

last-dialed number at position 3.

&Z3=L <Enter>

If a number is already stored at position 3, that number is

overwritten with the last-dialed number.

============================================================

Re-execute the Last Command (A/)

Another option for redialing is to enter the A/ command.

This command does not require the AT prefix or a Carriage

Return.

A/

When the modem receives a command, it stores it in its

command buffer until it receives the next AT command.  Note

that if you've sent the modem an additional command since

the Dial command, A/ repeats that command instead of

redialing.

============================================================

ANSWERING

============================================================

Your Sportster modem is shipped with DIP switch 5 ON, Auto

Answer suppressed.  To set the modem to automatically answer

incoming calls, do one of the following.

Before powering on the modem...

*  Set DIP switch 5 OFF.  When you power on, the modem

answers incoming calls on the first ring.

If you want the modem to answer after several rings, set

DIP switch 7 OFF.  Specify the ring you want the modem to

answer on in NVRAM (default=1 ring).  The valid range is

1-255 rings.  Power off the modem and power it on again

with its new DIP switch and NVRAM settings..

During a current session...

*  Issue the following command to have the modem answer on

the fourth ring:

ATS0=4 <Enter>

When the modem senses a call, it sends the RING result code

to your screen, goes off hook, and sends the remote modem a

high-pitched answer tone.  If there is no Carrier Detect

within 60 seconds, the modem hangs up.  If the connection is

made, the modem returns a CONNECT result code.  When the

call is disconnected by you or the remote user, the modem

hangs up and returns the NO CARRIER code.

============================================================

Suppressing Auto Answer

To disable Auto Answer, set DIP switch 5 ON before powering

on the modem, or set the modem to answer on zero (or no)

rings during the current session.

ATS0=0 <Enter>

NOTE:  The S-Register setting S0=0 cannot be saved to NVRAM.

============================================================

Manual Answer

If you've disabled Auto Answer but are expecting an incoming

data call, use the Manual Answer command.  Send the modem

the following command after the RING result code appears on

your screen.

ATA <Enter>

When the call is disconnected, the modem returns to its

original state, Auto Answer disabled.

------------------

Points to Remember

------------------

1.  You can set the modem to receive calls when you're not

at your computer.  Load your communications software as

you normally do, and set the modem to Auto Answer.  Also

set your software's file-save function to save incoming

messages and/or files.

2.  If you've attached your phone so it can be used for

conventional calls, disable Auto Answer when you are not

expecting incoming data calls.  Otherwise, your modem

may answer the phone before you do, greeting a voice

caller with a high-pitched, irritating answer tone.

============================================================

DISCONNECTING AND HANGING UP

============================================================

The commands outlined below describe how you can instruct

the modem to disconnect the call and hang up the line.

============================================================

Escape Code Operations (+++)

Once the modem is online to another system, the only command

it recognizes is an escape code of three plus symbols that

forces the modem into Online-Command mode.  Issue the

escape command in the following sequence.

*  Wait one second after sending the last item of data.

*  Type three plus symbols (+++).

*  Wait one second before typing any data.

Do not type the AT prefix or a Carriage Return.  The guard

time of one second before and after prevents the modem from

misinterpreting the presence of the +++ symbols in the

transmitted data stream.

If necessary, the character used in the escape code or the

duration of the guard time can be changed by resetting

Register S2 or S12.  See the "Quick Installation Guide" for

more information.

---------------------

Modem Response to +++

---------------------

When the modem receives +++, by default it enters Online-

Command mode and returns the OK result code.  It maintains

the connection and is ready to act on commands.  You may

then choose one of the following options.

*  Issue commands to the modem and then return it online.

*  Hang up the modem.

Alternatively, if you want the modem automatically to hang

up on receiving the +++ escape code, set Register S14 to 1

and write the command to your NVRAM templates.

ATS14=1 &W &W1 <Enter>

The modem disconnects and sends the NO CARRIER result code

to the screen.

============================================================

Returning Online (On)

After forcing the modem into Online-Command mode with the

escape code sequence, you can issue commands and then toggle

the modem back online with the O command, as in the

following example.

AT Q1 O <Enter>

O0   Return online (normal).

O1   Return online and retrain.  If errors occur during a

non-ARQ connection, try this option.  The modem returns

online and requests that both modems resynchronize.

============================================================

Hanging Up (Hn)

At its default setting, the escape code forces the modem

into Online-Command mode but leaves the line open.  To hang

up the modem, issue the following command once the modem

sends the OK result code.

ATH <Enter>

H0   Hang up (go on hook).

H1   Go off hook.

============================================================

Break Handling (&Yn)

This command lets you send a Break to abort data transfer

without disconnecting from the phone line interface.

&Y0  Destructive, no Break transmitted:  the modem clears

the data from its transmit buffer (all data is lost)

but does not transmit the Break to the remote modem.

&Y1  Destructive, expedited:  the modem clears the buffer

and immediately sends a Break to the remote modem.

Default.

&Y2  Nondestructive, expedited:  the modem retains data in

the transmit buffer, but immediately sends a Break to

the remote modem.

&Y3  Nondestructive, unexpedited (send Break in sequence):

the modem transmits any buffer data received before the

Break, sends the Break, and then sends any subsequent

input from the computer.

NOTE:  If the call is under MNP5 data compression,

destructive Breaks cause both modems to reset their data

compression tables.  When transmission resumes, the modems

build new tables, and the result is lower than normal

throughput.

============================================================

INTERNATIONAL CALLS

============================================================

The following commands apply to international calls.

============================================================

ITU-T/Bell Answer Sequence (Bn)

To answer international calls, use the default B0 setting.

B0   Default.  This setting is required for V.32 or higher

modems to answer V.32 or higher calls.  It also selects

the ITU-T V.25 answer sequence, used outside of North

America.

B1   This setting selects the Bell 2225 Hz answer tone used

in the U.S. and Canada and may be used when the remote

modem operates at only 2400 bps or lower.  However, if

the modem is set to B1, it won't be recognized by V.32

or higher modems and can't answer calls at 9600 and

above.

============================================================

Guard Tone (&Gn)

The United Kingdom and some Commonwealth countries use phone

switching systems that require answering modems to send an

1800 Hz guard tone after they send an answer tone.  Some

other European phone networks require a 550 Hz guard tone.

Guard tones are not used in the United States or Canada.

&G0  No guard tone, U.S./Canada.  Default.

&G1  550 Hz guard tone follows answer tone.  Requires B0

setting.

&G2  1800 Hz guard tone follows answer tone, United Kingdom

and some Commonwealth countries.  Requires B0 setting.

============================================================

Pulse Dial Make/Break Ratio (&Pn)

This command sets the ratio of the off-hook/on-hook

(make/break) interval for pulse dialing.  The default sets

the modem for use in North America.  The ratio must be

changed if the modem is used to dial out in the United

Kingdom and some Commonwealth countries.

&P0  Make/break ratio, U.S./Canada:  39%/61%.  Default.

&P1  Make/break ratio, United Kingdom, some Commonwealth

countries:  33%/67%.

============================================================

ITU-T V.21-300 bps

Select this option to call or answer overseas modems at 300

bps.  In V.21 mode, the modem answers Bell 103/V.21 calls,

but only originates V.21 calls.  This option is enabled in

Register S27 with the following command.

ATS27=1 <Enter>

============================================================

ITU-T V.23-1200/75 bps

Some United Kingdom and European systems require a 1200-bps

speed with a 75-bps back channel.  This option is enabled in

Register S34 with the following command.

ATS34=8 <Enter>

************************************************************

TEMPORARILY CHANGING A SETTING

************************************************************

When you change a setting, the modem retains it until you do

one of the following.

*  Change the setting again.

*  Issue the ATZ command to reset the modem.

*  Turn the modem off and power it on again.

The parameters described in this section are organized in

the order of their likely use.  That is, those you are most

likely to use appear at the beginning of the section, and

those you are least likely to use appear toward the end.

============================================================

TONE DIALING

============================================================

The modem defaults to pulse (rotary) dialing.  To have the

modem use tone dialing, which includes the asterisk (*) and

pound sign (#), use the T command.

ATDT 4445555 <Enter>

To have the modem always use tone dialing, use the T and &W

commands:

AT T &W &W1 <Enter>

============================================================

SPEAKER

============================================================

The modem's speaker enables you to monitor the dial-connect

process.  There are several ways to make use of this

feature.  After the Sportster modem dials a number, it waits

up to 60 seconds for a high-pitched answer tone from the

other modem, immediately followed by data signals, called a

"carrier."  These signals must occur before a data link is

established.

At the default X4 setting, the modem sends your screen the

NO CARRIER message after 60 seconds.  If you listen to the

speaker and realize you have received a voice answer, you

can respond immediately, instead of waiting for the modem to

time out, by pressing any key on the keyboard.  This cancels

the call.

You can also hear if dialing is proceeding too quickly for

the system.  Terminate the call (press any key) and retype

the Dial command, but insert a comma (,) or a couple of

slashes (/), to have the modem pause during the dialing

process.

============================================================

Volume Control (Ln)

The following options allow you to adjust the speaker volume

if you have an internal modem.  You can adjust an external

modem's volume via its volume switch.

L0   Low.

L1   Low.

L2   Medium.  Default.

L3   High.

============================================================

Speaker Control (Mn)

This command disables the speaker entirely or sets the

speaker to monitor different segments of the dial-connect

sequence.

M0   Disables the speaker entirely so that you don't hear

the modem go off hook, dial, etc.

M1   The speaker is on until Carrier Detect.  Default.  You

can monitor call progress until the Sportster detects

the remote modem's carrier signals, or until the 60-

second timeout and result code display.  At Carrier

Detect, the modem disconnects the speaker and data-

transmission sounds are suppressed.

M2   The speaker is on continuously, including during data

transmission.

M3   The speaker doesn't go on until after the last digit is

dialed, then goes off at Carrier Detect.

============================================================

LOCAL ECHO

============================================================

Local echo is the display of what you type at the keyboard

and online echo display of data the Sportster transmits to

another modem.  The En command controls the display of your

typed commands, when the Sportster is in Command mode.  The

Fn command controls the display of data when your modem is

online to another system.

============================================================

Command Mode Local Echo (En)

The power-on/reset default for command mode local echo is

set with DIP switch 4.  The Sportster modem is shipped with

DIP switch 4 OFF, enabling local echo of your typed

commands.

Use the En command to control the local echo for a current

session, independent of the switch setting.  This command

may not be stored in NVRAM.

E0   Command mode echo OFF.  The modem does not display

keyboard commands.

E1   Command mode echo ON.

NOTE:  If double characters appear on the screen, both the

modem's and software's local echo are on.

============================================================

Online Local Echo (Fn)

This command causes the modem to display a copy of the data

it is transmitting to another system.  Many systems,

however, return a copy of received data, which is called a

remote echo.  If the modem's online echo is ON and there is

also remote echoing, double characters appear on the screen.

In some microcomputer documentation, the term "duplex" is

applied to online local echoing, although the term is not

technically accurate.

F0   Online echo ON.  Sometimes called "half duplex."  As

the modem transmits data to a remote system, it also

sends a copy of the data to the screen.

F1   Online echo OFF.  Sometimes called "full duplex."

Default.

============================================================

S-REGISTERS

============================================================

The S-Registers are used to set various timing parameters

and to redefine selected ASCII characters and other

configuration options.  Refer to the "Quick Installation

Guide" for a summary of S-Register functions and setting

options.

Users typically require the default values.  To change a

setting, use the ATSr=n command, where r is the register and

n is a decimal value from 0-255 (unless otherwise

indicated).

The example below sets the modem's test timer (S-Register

18) to 10 seconds.

ATS18=10 <Enter>

The modem does not perform a value-range check.  Some values

you select may not work with some equipment, and you'll have

to readjust the settings.

Use ATSr? to display the contents of a register, as follows.

ATS18? <Enter>

============================================================

RESULT CODES

============================================================

Four commands control the result codes that the modem

returns to the screen.

Vn   Numeric/verbal response mode

Qn   Display/suppress all result codes

Xn   Result code options

&An  Display/suppress additional result code options

============================================================

Response Modes (Vn)

Result codes are sent to the screen in words (Verbal mode)

or numbers (Numeric mode).

The power-on/reset default for response modes is set with

DIP switch 2.  The Sportster modem is shipped with DIP

switch 2 OFF, enabling Verbal mode.

Use the Vn command to select verbal or numeric result codes

for a current session, independent of the DIP switch

setting.  This command may not be stored in NVRAM.

V0   Numeric mode.  Numeric result codes are followed by a

Carriage Return but no Line Feed, as in the following

example, where a 3 is returned (for NO CARRIER).

ATD1234567 <Enter>

becomes

3TD1234567 <Enter>

V1   Verbal mode.  Verbal responses are preceded and

followed by a Carriage Return and a Line Feed, as shown

below.

ATD1234567 <Enter>

NO CARRIER

============================================================

Quiet Mode (Qn)

Enable/suppress the display of result codes.

The power-on/reset default for response modes is set with

DIP switch 3.  The Sportster modem is shipped with DIP

switch 2 ON, to display result codes.

Use the Qn command to control the display for a current

session, independent of the DIP switch setting.  This

command may not be stored in NVRAM.

Q0   Result codes are displayed.

Q1   Result codes are suppressed (made quiet).

Q2   Result codes are suppressed only in Answer mode.

============================================================

Result Code Options (Xn)

You have five options, X0 through X4, for selecting the

result-code set best suited to your applications, as shown

in the tables below.

X0   Basic set, returns the first five codes (0-4) in the

following table.

X1   Extended set, codes 0-5, 10, 13 and 18, that adds rate-

specific CONNECT messages to the Basic set.  This set

also includes all &An result codes.

X2-4 The default is X4.  These options include the X1 set,

call-progress codes 6 or 7, and 8, and the functions

listed in the following table.

NOTE:  By default, the modem is also set to &A3, which

selects additional results that report the protocols used in

the connection.

Result Code Options

Setting

Result Codes              X0      X1      X2      X3      X4

0/OK                      o       o       o       o       o

1/CONNECT                 o       o       o       o       o

2/RING                    o       o       o       o       o

3/NO CARRIER              o       o       o       o       o

4/ERROR                   o       o       o       o       o

5/CONNECT 1200                    o       o       o       o

6/NO DIAL TONE                            o               o

7/BUSY                                            o       o

8/NO ANSWER                                       o       o

10/CONNECT 2400                   o       o       o       o

13/CONNECT 9600                   o       o       o       o

18/CONNECT 4800                   o       o       o       o

20/CONNECT 7200*                  o       o       o       o

21/CONNECT 12000*                 o       o       o       o

25/CONNECT 14400*                 o       o       o       o

43/CONNECT 16800+                 o       o       o       o

85/CONNECT 19200+                 o       o       o       o

91/CONNECT 21600+                 o       o       o       o

99/CONNECT 24000+                 o       o       o       o

103/CONNECT 26400+                o       o       o       o

107/CONNECT 28800+                o       o       o       o

-----------------------------------------------------------

Functions

-----------------------------------------------------------

Adaptive Dialing                          o       o       o

Wait for 2nd Dial Tone (W)                        o       o

Wait for Answer (@)                               o       o

Fast Dial                                 o               o

* V.32 bis and V.FC modems only

+ V.FC modems only

NOTE:  More CONNECT messages indicate an error control

connection, the call modulation, or the error control and

data compression used.  See Additional Result Code Options

(&An), after this section.

Result Code Definitions

Result Code          Meaning

0/OK                 Command has been executed.

1/CONNECT            Connection established with another

modem; if set to X0, connection may be

300-28.8K bps; if X1 or higher,

connection is at 300 bps.

2/RING               Incoming ring detected.

3/NO CARRIER         Carrier detect has failed or carrier

has been dropped due to disconnect.

4/ERROR              Command is invalid.

5/CONNECT 1200       Connection established with another

modem at 1200 bps.

6/NO DIAL TONE       Dial tone not detected during the

normal 2 seconds, set in Register S6.

7/BUSY               Busy signal detect; modem hangs up.

8/NO ANSWER          After waiting 5 seconds for an answer,

modem hangs up; returned instead of

NO CARRIER when the @ option is used.

10/CONNECT 2400      Connection at reported rate.  Same

for results of 4800 (18), 9600 (13),

7200 (20), 12K (21), 14.4K (25), 16.8K

(43), 19.2K (85), 21.6K (91), 24K (99),

26.4K (103), 28.8K (107).

NOTE:  Connections over 9600 bps apply

to V.32 bis and V.FC modems only.

Connections over 14.4K bps apply to V.FC

modems only.

Adaptive (Hunt)      The modem attempts to use tone

Dialing              dialing and, if that doesn't work,

reverts to rotary (pulse) dialing.

Wait for Another     The modem continues dialing as soon as

Dial Tone (W)        it detects another dial tone.

Wait for an          The modem continues dialing when it

Answer (@)           detects 5 seconds of silence on the

line.

Fast Dial            The modem dials immediately on dial-

tone detect, instead of waiting the

normal 2 seconds set in Register S6.

============================================================

Additional Result Code Options (&An)

Use this command to enable/disable one of the following sets

of error control, modulation, or protocol result codes.

If you encounter software problems, it may be because your

software expects to find different result codes from those

the modem is sending.  (The modem defaults to &A3.)  Review

your software documentation or try &A2, &A1, or &A0.  You

may need to call your software's technical support to find

out which &An setting they support.

&A0  ARQ (error control) codes are disabled.  This setting

does not affect an error control connection; the modem

returns standard CONNECT messages if result codes are

enabled.

&A1  ARQ codes are enabled.  CONNECT/ARQ is displayed if the

modem is set to X0 and the connection is 1200 bps and

above.  The remaining results indicate connection rate

and require X1 or above.  If your software cannot

handle the ARQ codes, select &A0.

&A2  V.32 modulation codes are enabled.  These results

require a setting of X1 or higher.  If your software

cannot handle the added modulation information, select

&A1 or &A0.

&A3  Protocol codes are enabled.  Default.  Error control

protocols reported are LAPM or MNP.  When the call is

not under one of those protocols (and ARQ is not

included in the result code), the modem reports NONE,

for no protocol.

If the modems are using data compression, the type of

compression-V42BIS or MNP5-is added to the result code.

If your software cannot handle the added protocol

information, select &A2, &A1, or &A0.

Although these codes will return numeric identifiers if

DIP switch 2 is ON or you've set the modem to V0, they

are the same numeric identifiers used for &A2 result

codes.  If you want &A3 protocol indicators, use Verbal

mode (V1), and not Numeric mode (V0).

============================================================

ERROR CONTROL (&Mn)

============================================================

By default, the Sportster first attempts a connection using

V.42 (LAPM) error control and, if that doesn't succeed, it

attempts an MNP connection.  If that doesn't succeed, the

modem tries to connect without error control.

Error control is possible at rates of 1200 bps and above.

The following options are available.

NOTE:  ARQ is U.S. Robotics' term for error control

protocols that feature error detection and automatic

retransmission of corrupted blocks of data.

&M0  Normal mode, no error control.  Because of the nature

of phone-line channels, this is never recommended for

calls above 2400 bps unless you're using an error-

correcting file-transfer protocol.

&M1  Reserved.

&M2  Reserved.

&M3  Reserved.

&M4  Normal/ARQ mode.  Default.  If the remote modem doesn't

recognize the Sportster's error control signals--V.42

or MNP--the modem operates in Normal mode, as though it

were set to &M0.

&M5  ARQ mode.  If the remote modem doesn't recognize the

error control request--V.42 or MNP--the Sportster hangs

up.

Always set the Sportster for error control, &M4 (default) or

&M5, for calls at speeds over 2400 bps.  Most users

communicating with V.42- or MNP-compatible modems will want

error control at 2400 and 1200 bps, as well.

If you're dialing out and have trouble connecting, it may be

because the remote modem doesn't have either MNP capability,

and it misinterprets your modem's MNP error control request.

If you know the remote modem doesn't support MNP, disable

MNP handshaking by setting Register S27 to 16 (ATS27=16).

The Sportster still connects, but without MNP error control.

To reset the modem for normal operations after the

call, issue the ATZ (reset) command, or set Register

S27 to zero.

============================================================

Auto Answer with Error Control

When set to &M4 or &M5 and a call comes in, the modem goes

off hook and responds to received error control signals.  If

the Sportster doesn't receive those signals and is set to

Normal/ARQ mode (&M4), it answers the call in Normal mode

(&M0).  If it doesn't receive the signals and is set to ARQ

mode (&M5), it hangs up.

============================================================

DATA COMPRESSION (&Kn)

============================================================

When data compression is enabled, the transmitting modem

detects redundant units of data and recodes them into

shorter units of fewer bits.  The receiving modem

decompresses the redundant data units before passing them to

the receiving computer.

Compression does not occur unless the modems can establish

an error control connection.

If the Sportster makes a V.42 connection, it negotiates V.42

bis compression.  If V.42 bis is not feasible, the

connection remains under error control, but without data

compression.

If the Sportster makes an MNP connection, it negotiates for

MNP Level 5 (MNP5) data compression.  If the remote modem

does not have MNP5 capability, the connection remains

under MNP error control, but without compression.

&K0  Data compression disabled.

&K1  Auto enable/disable.  Default.  The modem enables

compression if the serial port rate is fixed (&B1) and

disables compression if the serial port rate follows

the connection rate (&B0).  (Compression offers no

throughput advantage when serial port and connection

rates are equal, and may even degrade throughput.)

&K2  Data compression enabled.  Use this setting to keep the

modem from disabling compression.

&K3  Selective data compression.  The modem negotiates only

for V.42 bis compression, and disables MNP Level 5

(MNP5) compression.  See the following note.

NOTE:  MNP5 compression is not useful when transferring

already compressed files, such as the .ZIP files downloaded

from Bulletin Boards and 8-bit binary files that appear

already compressed to the modem.  MNP5 tends to add data to

these files, and throughput over the phone link degrades.

V.42 bis compression detects when data is already compressed

and turns off until it detects that compression will be

advantageous.  The special &K3 setting allows the best

throughput for already compressed and 8-bit binary files.

============================================================

DATA RATE COMMANDS

============================================================

The &Bn and &Nn commands control data rates at the serial

port and phone line interfaces.

The Sportster defaults to a fixed rate at the serial port

interface (&B1) to keep the serial port rate higher than the

connection rate.  You'll get greater throughput regardless

of the connection rate.  Your software, however, must

support this feature and you must set your software to use a

fixed rate.

For the phone line, however, we recommend variable

connection rates (&N0).  This lets the modem switch its

connection rate to match the rate of a remote modem, no

matter what that rate is.  If the connection rate is fixed,

for example, at 9600 bps (&N6), the modem only connects with

modems operating at that rate.  Of course, if your

application requires connections with modems at only one

rate, you may wish to set the modem to a fixed connection

rate.

---------------------

Software Requirements

---------------------

Both variable and fixed serial port rates require

communications software support.  Most communications

programs support variable or fixed rates.  Check your

software manual.  Set your software BEFORE making calls.

============================================================

Serial Port Rate (&Bn)

Initially, the modem's serial port rate depends on your

software setting.  The modem uses the rate at which it

receives the AT command to determine the serial port rate;

after that it depends on its &B setting.

&B0  Variable serial port rates.  The modem switches its

serial port rate to follow the connection rate

established with a remote modem.  If your software has

Auto Baud Detect, turn it on.  The software will switch

its serial port rate to match the connection rate.

&B1  Fixed serial port rate.  Default.  The modem detects

its serial port rate from your last AT command and

maintains it, whatever the connection rate.

Set your software to 115.2K, 57.6K, 38.4K, or 19.2K bps,

then set the modem to &B1 for the best throughput.  Some

programs require that you turn off Auto Baud Detect to

fix the serial port rate.  Others use the term Serial

Port Lock (yes or no).

The serial port rate MUST EQUAL OR EXCEED the

connection rate.  Also, this setting requires

flow control, preferably hardware (&H1), to avoid

buffer overflow.

&B2  Fixed for ARQ calls/Variable for non-ARQ calls, Answer

mode only.  When the modem goes off hook and answers in

ARQ mode, it shifts its serial port rate to the one

written to NVRAM, for example, 38.4K bps.  In non-ARQ

mode, it acts as if set to &B0 when answering, and

switches its serial port rate to match the call's

connection rate.

This option is designed for installations such as

Bulletin Boards that receive calls from a wide variety

of modems, ranging from the very slow to those with the

Sportster 's advanced design.

============================================================

Connection Rate (&Nn)

Use the &Nn command to select variable or fixed rates at the

phone-line interface.  Variable rates let the modem connect

with a variety of remote modems, while fixed rates limit

calls to one connection rate.

&N0  Variable rates.  Default.  The Sportster negotiates

with the remote modem for the highest possible

connection rate, depending on the capabilities of the

remote modem.  This is the recommended setting.

&N   Fixed rate.  The Sportster only connects if the remote

1-8  modem is operating at the same rate.  If not, the

Sportster hangs up.

The fixed rate options are as follows.

&N1  300 bps      &N5  7200 bps (V.32 bis/V.FC only)

&N2  1200 bps     &N6  9600 bps

&N3  2400 bps     &N7  12K bps (V.32 bis/V.FC only)

&N4  4800 bps     &N8  14.4K bps (V.32 bis/V.FC only)

By fixing the connection rate, you can filter out calls at

other than a specific rate, for security or other reasons.

============================================================

FLOW CONTROL

============================================================

The modem uses either hardware or software flow control to

manage the amount of data stored in the buffers, thus

preventing buffer overflow.

The Sportster defaults to hardware flow control.  This type

of flow control is performed between the modem and computer

with serial signaling.

Software flow control uses the standard ASCII Transmit OFF

(XOFF) character, Ctrl-S, and the Transmit ON (XON)

character, Ctrl-Q.

ASCII definitions are as follows.

XON    Ctrl-Q      ASCII 17 Decimal, 11 Hex

XOFF   Ctrl-S      ASCII 19 Decimal, 13 Hex

NOTE:  Both your software and computer must support the flow

control you select.  All 100% IBM PC-compatible computers

support hardware flow control, as do Apple computers

equipped with a Hardware Handshaking cable.  However,

hardware flow control requires software support.  Refer to

your software documentation to see what your software

supports.

When you have determined the type of flow control your

system supports, be sure to set your software appropriately

BEFORE transferring files.

============================================================

Warning on Software Flow Control

In ordinary operations, the only characters the modem

recognizes during a call are the three plus symbols (+++) of

the escape code that return it to Command mode.  But when

software flow control is enabled, the modem or computer also

looks for Ctrl-S or Ctrl-Q characters.  IF THESE CHARACTERS

OCCUR IN A FILE OR AS PART OF A PROTOCOL, it reads them as

XON/XOFF characters and acts on them.

For example, XON/XOFF characters occur in binary files, and

are used by Xmodem-type protocols.  They may also come from

the remote system.  An XON from the remote system, after

your modem has sent an XOFF, can result in buffer overflow.

Software flow control may prove satisfactory if you're only

transferring text files; however, you may lose data if

XON/XOFF characters occur in the data stream from other

sources.

To select software flow control, write the &F2 configuration

template to Y1.

AT &F2 &W <Enter>

---------------------------------

Software Flow Control Precautions

---------------------------------

If you can't use hardware flow control and are doing Xmodem-

type protocol or binary file transfers, select a protocol

that performs error correction, and turn off the modem's

error control (AT&M0).

============================================================

Transmit Data Flow Control (&Hn)

This type of flow control regulates data your computer

transmits to the modem for transmission over the phone link.

The modem monitors its Transmit Data buffer as data comes

in.  If the buffer nears 90% capacity, the modem signals

the computer to stop transmitting.  When it has sent enough

data over the link to empty half the buffer, it signals the

computer to resume transmitting.

-------------------

When it is Required

-------------------

Transmit Data flow control should be enabled in the

following situations.

*  You're using error control (any allowable rate above

300 bps), with or without data compression.  If data is

resent due to errors, a continuous stream of data from

the computer could overflow the modem's buffer.

*  The serial port rate is higher than the connection rate.

For example, the serial port rate is 38.4K bps and the

connection rate is 14.4K bps.

NOTE:  Set your software to either hardware or software flow

control.  Some programs also require that you turn off the

type you are not using.

&H0  Transmit Data flow control disabled.

&H1  Hardware flow control.  Default.  Your computer and

software must support Clear to Send (CTS).  The modem

drops the Clear to Send (CTS) signal to the computer

when its buffer nears 90% capacity, and starts sending

CTS again when the buffer is about half full.

&H2  Software flow control.  Your software must support

XON/XOFF signaling.  The modem sends an XOFF to the

terminal when its buffer nears 90% capacity, and sends

an XON when the buffer is about half full.  Default in

the &F2-Software Flow Control template.

&H3  Use both hardware and software flow control.  Select

this option if you're not sure what your computer

supports.

NOTE:  If possible, use hardware flow control.  See "Warning

on Software Flow Control," earlier in this guide for

details.

============================================================

Received Data Flow Control

Two commands--&Rn (hardware) and &In (software)--control the

flow of received data passed by the modem to your computer.

Because computers handle incoming data more quickly than the

modem receives it over the phone line, most applications

won't need this.

----------------------

Hardware Control (&Rn)

----------------------

When your computer drops its Request to Send (RTS) signal,

the modem stops passing along received data.  The computer

sends RTS again when it is ready to receive more data.

Your computer and software must support RTS.  You cannot use

this type of flow control, however, if your software

requires a constant RTS signal.

NOTE:  Use only for ARQ (error control) calls, because the

V.42 and MNP protocols control the data flow across the

phone link.  During non-ARQ calls, however, there is no way

to signal the remote modem to stop sending data.  If your

modem stops passing data to your computer and the remote

modem keeps sending, the Received Data buffer will overflow.

&R0  Reserved.

&R1  The modem ignores Request to Send (RTS).  This setting

is required if your software does not support RTS.

&R2  Hardware flow control of received data enabled.

Default.  The modem passes received data to your

computer only on receipt of the RTS signal.

----------------------

Software Control (&In)

----------------------

When you send the modem a Ctrl-S (XOFF) command from the

keyboard, the modem stops passing received data to your

computer.  When you send a Ctrl-Q (XON) command, it resumes.

NOTE:  Because of the risk of data loss, &I1-5 are

recommended only if your data does not have XON/XOFF control

characters.  See "Warning on Software Flow Control" for

details.

&I0  Disables XON/XOFF flow control of received data.

Recommended for non-ARQ calls.

&I1  The modem acts on your typed Ctrl-S/Ctrl-Q commands and

passes them to the remote computer.  Not recommended

for non-ARQ calls.  Use in ARQ mode only, but keep in

mind that XON/XOFF characters sent to the remote

computer may interfere with XON/XOFF signaling between

it and the remote modem.  &I2 is preferred for ARQ

calls.

&I2  The modem acts on your XON/XOFF commands, but removes

them before sending data to the remote computer.

Default in the &F2--Software Flow Control template.

Recommended setting for ARQ mode, only.  It ensures

that the remote computer does not confuse its modem's

XON/XOFF characters with yours.

If the call is not in ARQ mode, there is no flow

control on the phone link.  If you send an XOFF to your

modem and it stops passing data, it has no way to tell

the remote computer and modem to stop sending for a

while, and your modem's buffer may overflow.  See &I5

for another alternative.

&I3  Host Mode.  Applies only to modems attached to HP

mainframes using the ENQ/ACK protocol.  ARQ mode only.

&I4  Terminal Mode.  Applies only to modems attached to HP

system terminals using the ENQ/ACK protocol.  ARQ mode

only.

&I5  Enables phone link flow control when the connection is

not under error control.  Both modems must use &I5.  In

ARQ mode, the modem operates as if set to &I2.  It acts

on XON/XOFF commands, but does not pass them on to the

remote system.

In non-ARQ mode, the modem acts as if set to &I0.  It

does not look for local XON/OFF commands, but does look

for any XON/XOFF characters coming in over the phone

link from the remote computer.  The modem acts upon

them and drops them from the data stream.

Operators can signal the other modem to stop sending

and control phone link data flow to keep their modem's

buffer from overflowing, if both are set to &I5.

============================================================

SERIAL OPERATIONS

============================================================

The parameters described below are directly affected by the

serial connection and DIP switch settings, and apply to

external modems only.

============================================================

Carrier Detect (&Cn)

Like DIP switch 6, this command controls Carrier Detect (CD)

signaling from the modem to the computer.

The power-on/reset default for response modes is set with

DIP switch 6.  The Sportster modem is shipped with DIP

switch 6 OFF, disabling override for normal operations.

Use the &Cn command to control Carrier Detect for a current

session, independent of the DIP switch setting.  This

command may not be stored in NVRAM.

Check your communications software manual to find the

required setting.

&C0  CD override, CD always ON.

&C1  Normal CD operations.  The modem sends a CD signal when

connecting with another modem and drops CD upon

disconnecting.  Most communications software programs

require this setting.

============================================================

Data Terminal Ready (&Dn)

Like DIP switch 1, this command controls the Data Terminal

Ready (DTR) signaling from the computer to the modem.

The power-on/reset default for DTR is set with DIP switch 1.

The Sportster modem is shipped with DIP switch 1 OFF,

disabling override for normal operations.

Use the &Dn command to DTR for a current session,

independent of the DIP switch setting.  This command may not

be stored in NVRAM.

Check your communications software manual to find the

required setting.

&D0  DTR override, modem ignores DTR.

&D1  Reserved.

&D2  Normal DTR operations.  The computer or terminal must

send a DTR signal or the modem won't accept commands.

Dropping DTR terminates a call.  Most communications

software programs require this setting.

============================================================

Data Set Ready (&Sn)

The modem sends your computer a Data Set Ready (DSR) signal.

(Data Set is industry jargon for modem.)  Few, if any,

commercial communications programs require the modem to

control DSR (&S1).  Leave the modem set for DSR override

(&S0), unless you know that your installation requires a

different setting.

&S0  DSR is always ON (override).  Default.

&S1  In Originate mode, the modem sends DSR after dialing,

when it detects the remote modem's answer tone.  In

Answer mode, it sends DSR after sending an answer tone.

************************************************************

GETTING HELP

************************************************************

Checking the Help screens is the first step to recovery.

These screens give you the information you'll need to

identify a problem.  Once a problem is identified, it can

easily be solved.

============================================================

INQUIRIES (In)

============================================================

The Inquiry command has eight options.  The most commonly

used options display the following information.

ATI4  Current settings

ATI5  NVRAM settings

ATI6  Link diagnostics

I0   The modem returns a product code.  If you have a

problem and call U.S. Robotics' Technical Support

Department, you may be asked for this product code.

I1   The modem performs a checksum of its read-only memory

(ROM) and returns the result to the screen.  This

function is used only in factory testing.  The modem

should always read the same number.

I2   The modem performs a test of its random-access memory

(RAM) and returns either the OK (0) or ERROR (4) result

code, followed by OK when the test is completed.  You

may want to use this command as a checkpoint if the

modem appears to be malfunctioning.

I3   The modem returns a product identification string.

I4   The modem displays its current configuration.

I5   The modem displays the stored phone numbers and two

templates (Y0 and Y1) stored in nonvolatile random

access memory (NVRAM).  Activate the second screen by

pressing any key.

I6   During a connection, the modem monitors and stores

information about link operations.  When the call is

ended, you can request a diagnostic summary.

For calls under data compression, the number of

characters sent may be less than the number of octets

sent, due to buffering operations.

Most terms used in the display are self explanatory

except for the following:

Octets:  Compressed data units.  If the number of

octets is greater than the  number of

characters sent, the modems probably used MNP5

compression on an already compressed file, and

the result was expanded data.

Blers:   Errors in data and protocol blocks.  If there

were many block errors, your receiver may have

experienced problems on the line.

Blocks Resent:  These represent blocks the remote modem

resent due to the previous category, Blers.

Link Timeouts:  Protocol detection problems;

communications were severed momentarily and

the modems probably recovered.  This does

not indicate the retry timeout.

Link Naks:  Negative acknowledgments (one or more

blocks).

Data Compression:  Indicates the type of data

compression negotiated for the call (V42BIS or

MNP5) or NONE.  A V42BIS response includes the

size of the dictionary and the maximum string

length used, for example, 2048/32.

Fallback:  Enabled/Disabled:  indicates whether or not

the modems negotiated online fallback during

the connection sequence.

Protocol:  Indicates the error control protocol

negotiated (LAPM, MNP, NONE).

Speed:   The last rates at which the  receiver/

transmitter were operating before

disconnecting.

Disconnect Reason:  Possible reasons the modem hung up

are as follows:

*  DTR dropped--The DTE dropped the Data

Terminal Ready signal, terminating the

call.

*  Escape code--The operator sent the modem

the +++ escape code.

*  Loss of carrier--The modem detected loss of

the remote modem's carrier and waited the

duration specified in Register S10 (default

is 0.7 seconds).

*  Inactivity timeout--The modem detected no

activity on the line for the duration

specified in Register S19 (default is 0,

timer disabled).

*  MNP incompatibility--The modem is set to

&M5 and the remote modem does not have MNP

compatibility, or there was an MNP

negotiation procedure error.

*  Retransmit limit--The modems reached the

maximum of 12 attempts to transfer a data

frame without error.

*  LD received--The remote modem sent an MNP

error control Link Disconnect request.

*  DISC--The remote modem sent a V.42

Disconnect frame.

*  Loop loss disconnect--The modem detected a

loss of current on the loop connecting

it with the telephone company central

office.  This usually occurs because the

remote modem has hung up:  the central

office drops current momentarily when

there is a disconnect at the other end of a

call.  Unless Register S38 is set higher

than 0, the modem immediately hangs up at

loop loss.

*  Unable to retrain--After several attempts,

disturbances on the phone line prevented

the modems from retraining

(resynchronizing), and they could no longer

transmit or receive data.

*  Invalid speed--The modem is set to &N1 or

higher, for a fixed link rate, and the

remote modem is not operating at the same

rate.

*  XID timeout--The modems failed to negotiate

the V.42 Detection (XID Exchange) phase.

*  SABME (Set Asynchronous Balance Mode

Extended) timeout--The modems failed this

part of V.42 link negotiation.

*  Break timeout--Incompatible processing of a

Break signal occurred.

*  Invalid codeword--The modem received an

invalid V.42 bis (compression) frame.

*  A rootless tree--The modem received an

invalid V.42 bis (compression) frame.

*  Illegal command code--The modem received an

invalid V.42 bis (compression) frame.

*  Extra stepup--The modem received an invalid

V.42 bis (compression) frame.

I7   The modem returns a product configuration.  If you have

a problem and call U.S. Robotics' Technical Support

staff, you may be asked to read this screen.

============================================================

S-REGISTER QUERY (SR?)

============================================================

This command allows you to view the contents of a particular

S-Register, as in the following example that requests the

contents of Register S0 ("On what ring will the modem

answer?").

ATS0? <Enter>

============================================================

STORED PHONE NUMBER QUERY (&Zn?)

============================================================

At this command, the modem returns the phone number stored

in NVRAM at position n, as in the following example that

includes a sample modem response.

AT&Z3? <Enter>

5551234

============================================================

LAST-DIALED NUMBER QUERY (DL?)

============================================================

At this command the modem displays the number stored in the

last-dialed number buffer.

ATDL? <Enter>

============================================================

HELP SCREENS

============================================================

Sportster modems provide four Help screens:  summaries of

the basic AT command set, extended ampersand (&) command

set, Dial command options, and S-Register functions.

--------------------

Stop/Restart Display

--------------------

The following command stops the display.  Hold down the

Control key and type S.

<Ctrl>-S

To restart the display, use the same command or press <any

key>.

--------------

Cancel Display

--------------

Either of the following commands cancels the display.

<Ctrl>-C

<Ctrl>-K

============================================================

Basic Command Set ($)

At AT$, the Sportster displays a screen that shows a partial

summary of the command set.  A second screen, activated by

pressing any key, shows the remaining commands.

============================================================

Ampersand Command Set (&$)

At AT&$, the Sportster displays a screen that shows a

partial summary of the extended ampersand command set.  A

second screen, activated by pressing any key, shows the

remaining command set.

============================================================

Dialing (D$)

At ATD$, the Sportster displays the Dial command summary.

============================================================

S-Register Functions (S$)

At ATS$, the Sportster displays a screen that shows a

partial summary of the S-Register functions.  A second

screen, activated by pressing any key, shows the remaining

registers.

************************************************************

TESTING THE MODEM

************************************************************

Modem testing is available with the &Tn command and Register

16 (Dial test).  All loopback testing conforms to ITU-T

Recommendation V.54.

NOTE:  Only one test can be performed at a given time.  If

you send a test command while the modem is in Test mode,

you'll receive an ERROR message.

============================================================

TESTING WITH &Tn

============================================================

The tests supported through the &Tn command include Analog

Loopback, Digital Loopback, and Remote Digital Loopback.

You can key in your own data during testing, or use the

modem's internal test pattern and error detector.

Always disable error control before testing.  If the modem

is detecting errors and retransmitting the affected data,

your test results will be invalid.

============================================================

Ending a Test--&T0, S18

Issue the &T0 command to terminate a test.  Alternatively,

set Register S18 to a specified number of seconds, for

example, S18=10.  When the 10 seconds are up, the modem

automatically ends the test and returns to Command mode.  If

the test was Analog Loopback, the &T0 command hangs the

modem up.  If the test was Digital or Remote Digital

Loopback, issue an ATH command to hang up the modem, or an

ATZ command to hang up the modem and reset it to its

defaults.

NOTE:  If you use the S18 test timer, but in the process of

testing you issue an ATZ command, S18 resets to zero and the

timer is disabled.  Also, you cannot store a value for S18

in nonvolatile memory;  its power-on and reset default is

always zero.

============================================================

Analog Loopback-&T1, &T8

This test checks the operation of the modem's transmitter

and receiver.

---

&T1

---

This AL option requires that you type data you can later

verify at your screen.

1.  The modem should be on hook in Command mode.  If you

wish, set Register S18 as a test timer.  For example,

insert S18=10 in the following command string before

&T1.

AT &M0 S18=10 &T1 <Enter>

The modem disables error control, sets the timer, enters

Analog Loopback (AL) mode, and sends a CONNECT message.

2.  Type recognizable data so that you can verify it when

it is looped back to the screen.

3.  End the test.  If you set S18, the modem automatically

stops the test at the timeout, exits AL mode and

responds OK.

If you didn't set Register S18, wait one second and type

+++ to end the test and return the modem to Command

mode.  When the modem responds OK, type AT&T0 to

terminate AL mode and hang up the modem.  (Typing ATH

has the same effect.  Typing ATZ also has the same

effect, but also resets the modem and restores your &M

default.)

The modem responds OK.  If the modem sends an ERROR

message, you have issued an invalid command.

4.  If there were no errors, reset the modem to &M4, for

error control, unless you've issued the ATZ reset

command.

NOTE:  If the modem is in Online-Command mode--that is,

still connected to a remote modem--and you send it an

&T1 or &T8 command, it drops the call, enters AL mode,

sends a CONNECT result and waits for loopback

characters.

---

&T8

---

This AL option causes the modem to send an internal test

pattern to its transmitter and loop it back to the receiver.

An internal error detector counts any errors and, when the

test is ended, sends the number of errors or 000 (no errors)

to the screen.

Since you don't type anything during this test, and the

modem does not send anything to the screen, this option

verifies only the modem.  If there are no errors but your

problem continues, it may be at the computer interface.

1.  The modem should be on hook in Command mode.  If you

wish, set Register S18 as a test timer.  For example,

insert S18=10 in the following command string before

&T8.

AT &M0 S18=10 &T8 <Enter>

The modem disables error control, sets the timer, and

enters AL mode.  The modem sends its internal test

pattern to the transmitter, and loops the pattern back

to the receiver.  You will not see any data on your

screen.

2.  End the test.  If you set S18, the modem automatically

stops the test at the timeout.  If you didn't set

Register S18, type AT&T0 to end the test.  Or use ATH

or the command that resets the modem, ATZ.  Both of the

latter end the test and hang up the modem.

When the modem hangs up, it returns a three-digit code,

followed by OK.  A code of 000 indicates no errors were

found.  A code of 255 indicates 255 or more errors.  An

ERROR message indicates that you issued an invalid

command.

3.  If there were no errors, reset the modem to &M4 for

error control unless you issued the ATZ command.

============================================================

&T2

This option is reserved.

============================================================

Digital Loopback-&T3

If your modem has passed the Analog Loopback (AL) test, this

test can help you locate a problem with a remote modem or

the telephone channel.

1.  Set the modem to &M0 to disable error control.

Establish a connection with the remote modem.

2.  Bring the modem back to Online-Command mode with the

+++ escape code.  Then send it the AT&T3 command.  The

modem enters Digital Loopback (DL) mode.

3.  The remote user should type a short message.  It will

be looped back by your modem's transmitter for

verification on the remote screen.  You will not see the

message or any other data.

4.  When the remote user has completed the test,  issue the

AT&T0 command to end the test.  If you wish, return the

modem online (ATO) to resume Data mode.  Or type either

ATH or the command that resets the modem, ATZ.  The

latter two commands end the test and hang up the modem.

The modem responds OK.  If the modem sends an ERROR

message, you have issued an invalid command.

============================================================

&T4, &T5

The &T4 option grants a remote modem's request for a Remote

Digital Loopback test.

The &T5 option cancels &T4 and your modem will not recognize

a request for a Remote Digital Loopback test from a remote

modem.  This is the default so that your modem isn't subject

to another user calling and tying up your modem without your

permission.

============================================================

Remote Digital Loopback-&T6, &T7

This test, like the Local Digital Loopback test, verifies

the condition of both modems and the phone link.

The request for and granting of Remote Digital Loopback

testing requires that both modems use ITU-T V.54 standard

signaling.  The test MUST be performed at 2400 or 1200 bps.

If the remote modem does not have the capability or is not

set to respond, you will get an ERROR result code.

---

&T6

---

This RDL option requires that you send keyboard data to the

modem and verify it when it is returned over the phone lines

and to your screen.

1.  Set your software to 2400 or 1200 bps.  Set the modem to

&M0.  If you wish, set the S18 timer.

Establish a connection with the remote modem.  If you

haven't already done so, arrange with the remote user to

cooperate with your testing.  The remote user should set

his or her modem to acknowledge the RDL request, for

example, AT&T4.

2.  Bring the modem back to Online-Command mode with the

+++ escape code.  Send it the AT&T6 command.  The modem

enters RDL mode.

3.  Type a short message.  It will be looped back to your

modem by the remote modem and to your screen for

verification.  (The remote user will not see your data.)

4.  End the test.  If you set Register S18 the modem

automatically ends the test when the test timeout is

reached.  If you didn't set S18, type AT&T0 to end the

test.  If you wish, return the modem online (ATO) to

resume Data mode.  Or send either ATH or the command

that resets the modem, ATZ.  The latter two commands end

the test and hang up the modem.  The modem responds OK.

If you issue an invalid command, the modem sends an

ERROR message.

Data errors indicate a problem with the remote modem or

the phone link.  If you have not performed analog loop-

back testing with your modem, the problem may also lie

with your modem.

5.  Reset the modem to &M4 unless you used the reset

command, ATZ.

---

&T7

---

This test option causes the modem to send an internal test

pattern through the Remote Digital Loopback.  An internal

error detector counts any errors and, when the test is

ended, sends the number of errors or 000 (no errors) to the

screen.

You don't need to type anything during this test.  The modem

sends only its final error count to your screen.

1.  Set your software to 2400 or 1200 bps.  Set the modem

to &M0.  If you wish, set the S18 timer.

Establish a connection with the remote modem.  If you

haven't already done so, arrange with the remote user to

cooperate with your testing.  The remote user should set

his or her modem to acknowledge the RDL request, for

example, AT&T4.

2.  Bring the modem back to Online-Command mode with the

+++ escape code.  Then send it the AT&T7 command.  The

modem enters RDL mode.  The modem sends its internal

test pattern to the remote modem, which loops it back to

your modem.  You will not see the data on your screen.

3.  End the test.  If you set S18, the modem automatically

stops the test when the timer times out.  If you didn't

set Register S18, type AT&T0 to end the test.  After you

view the test results (next paragraph), return the modem

online if you wish (ATO) and resume Data mode.  Or send

either ATH or the command that resets the modem, ATZ.

The latter two commands end the test and hang up the

modem.

When you terminate the test, the modem returns a three-

digit code, followed by OK.  A code of 000 indicates no

errors were found.  A code of 255 indicates 255 or more

errors.  If you issue an invalid command, the modem

sends an ERROR message.

If you've performed an Analog Loopback and know your

modem is working properly, errors indicate a problem

with either the phone connection or the remote modem.

4.  Reset the modem to &M4 unless you've sent it the ATZ

reset command.

============================================================

DIAL TEST-S16=2

The Dial test is used for factory-testing the frequencies of

tone dialing values.  When S-Register 16 is set to 2 and a

single tone is dialed (for example, ATD7 <Enter>), the modem

continues to transmit that tone until you press Enter again.

************************************************************

OTHER OPERATIONS

************************************************************

============================================================

FAX OPERATIONS

============================================================

============================================================

Fax Modem Guidelines

Fax operations require facsimile-compatible software that

can send or receive Group III faxes.  Follow the

instructions in your fax software manual.

The modem's normal operating mode is Data mode.  If your fax

software is typical, it automatically switches the modem to

Fax mode when you run the program, and resets the modem to

Data mode when you exit the program.

If you have a problem, however, and think the modem may be

in the wrong mode, you can use one of the following AT

commands to manually switch the modem.

Most users will never need to use these commands.

AT+FCLASS=0 <Enter>   (Switch to Data mode)

AT+FCLASS=1 <Enter>   (Switch to Class 1 Fax mode)

AT+FCLASS=2.0 <Enter> (Switch to Class 2.0 Fax mode)

If you are not sure whether the modem is in Data or Fax

mode, type the following command.

AT+FCLASS? <Enter>

The modem returns a 0 to indicate Data mode, a 1 to indicate

Class 1 Fax mode or a 2.0 to indicate Class 2.0 Fax mode.

NOTE:  Whenever the fax modem is reset using the ATZ

command, toggling the DTR signal, or turning the power off

and on, the modem will be set to Data mode.

============================================================

Fax Mode Flow Control Setting

Many facsimile software products use software flow control

when the modem is in Fax mode.  To allow compatibility with

software products that use software flow control by default,

U.S. Robotics fax modems switch to software flow control

when entering Fax mode.

For the best information on modem settings, see your Fax

software manual.

============================================================

FCC Notice

FCC part 68, rules regarding fax operation, has been amended

as follows:

Telephone facsimile machines--identification of the

sender of the message:  It shall be unlawful for any

person within the United States to use a computer or

other electronic device to send any message via a

telephone facsimile machine unless such a message clearly

contains, in a margin at the top or bottom of each

transmitted page or on the first page of the

transmission, the date and time it is sent and an

identification of the business, other entity, or

individual sending the message and the telephone number

of the sending machine or of such business, other entity,

or individual.  Telephone facsimile machines manufactured

on and after December 20, 1992 must clearly mark such

identifying information on each transmitted page.

============================================================

A Note to Programmers

If you want to know more about the supported fax commands,

refer to the standard for the Service Class 1 fax protocol.

ANSI/EIA/TIA-578-1990 (EIA-578)

Asynchronous Facsimile DCE Control Standard

November, 1990  Approved:  October 22, 1990

You can obtain a copy of this standard by contacting Global

Engineering Documents, at 1-800-854-7179.

For more information on supported Class 2.0 fax commands,

refer to the standard for the Service Class 2.0 fax protocol.

ANSI/EIA/TIA-592-1993 (EIA-592)

Asynchronous Facsimile DCE Control Standard

May, 1993

============================================================

VOICE/DATA COMMUNICATIONS

============================================================

When you install the Sportster, you have the option of

plugging your phone into the second modular jack of the

modem so it's available for voice calls.  You can also use

the phone to set up communications with another user whose

modem uses the AT command set, such as a U.S. Robotics or

U.S. Robotics-compatible modem.

============================================================

Preparation

First call the other user to establish the parity, word

length, and number of Stop bits the other person's modem

accepts.  Then turn the call over to your modems, as

follows.

============================================================

Procedures

1.  Without hanging up the phone, have your modem go off

hook in Originate mode.  Type the following command.

ATD <Enter>

NOTE:  Be sure the modem is NOT set to X2 or X4, or it

will return the NO DIAL TONE result code and hang up.

2.  The other party should have the remote modem go off

hook in Answer mode.  The following command is used to

do this.

ATA <Enter>

3.  Now both of you can hang up your respective phones

while the two modems establish the data link.  They will

maintain the link until one of you gives your modem a

command to disconnect.

Either party's device can be the originate or answer

modem; it doesn't matter who made the phone call.  But

one modem must enter Originate mode and the other Answer

mode.  You and the other party, therefore, must agree on

which command, ATD or ATA, you will each use.

************************************************************

TECHNICAL SPECIFICATIONS

************************************************************

============================================================

COMPATIBILITY

============================================================

The Sportster modem conforms to the following standards,

ensuring compatibility with a wide base of installed modems.

ITU-T is the new signifier of what was formerly the CCITT

international standards body.

V.Fast Class         28.8K/26.4K/24K/21.6K/19.2K/16.8K/14.4K/

12K/9600/7200/4800 bps

(V.FC only)

ITU-T V.32 bis       14.4K/12K/9600/7200/4800 bps

(V.32 bis/V.FC modems only)

ITU-T V.32           9600/4800 bps

ITU-T V.22 bis       2400 bps

Bell 212A            1200 bps (also ITU-T V.22)

ITU-T V.23           1200 bps with 75 bps back channel

(Some United Kingdom and European

phone systems)

Bell 103             300 bps (ITU-T V.21 optional)

ITU-T V.42           LAPM error control, 1200 bps and higher

ITU-T V.42 bis       Data compression, 1200 bps and higher

MNP                  Levels 2, 3 and 4 error control, level 5

data compression, 1200 bps and higher

ITU-T V.54           Analog, digital, and remote digital

loopback testing

------------------------------------------------------------

Fax Modems

------------------------------------------------------------

TIA/EIA-578          Service Class 1 Asynchronous

Facsimile DCE Control Standard

ITU-T V.17           14.4K/12K bps

ITU-T V.29           9600/7200 bps

ITU-T V.27 ter       4800/2400 bps

ITU-T V.21           300 bps

============================================================

THE SERIAL INTERFACE

============================================================

The serial interface information below applies only to

external modems.

============================================================

Description

The serial interface is a standard developed by the

Electronic Industries Association (EIA).  It defines the

signals and voltages used when data is exchanged

between a computer and a modem or serial printer.

The entire standard covers many more functions than are used

in most data communications applications.  Data is

transmitted between the devices over a shielded serial cable

with a 25-pin male (DB-25P) connector to the modem and a 25-

pin, 9-pin, 8-pin, or custom-built connector to the

computer.

NOTE:  FCC regulations require the use of shielded cable

when connecting a modem to a computer to ensure minimal

interference with radio and television.

============================================================

Pin Assignments

Pin assignments are factory-set in the Sportster modem to

match the standard DB-25 assignments in the following table.

DB-9 connectors for IBM PC/AT-compatible computers should be

wired at the computer end of the cable as shown in the DB-9

column.

Serial Interface Pin Definitions

Signal Source

DB-25    DB-9    Circuit   Function         Computer -- Modem

1       *       AA       Chassis Ground          Both

2       3       BA       Transmitted Data      Computer

3       2       BB       Received Data          Modem

4       7       CA       Request to Send       Computer

5       8       CB       Clear to Send          Modem

6       6       CC       Data Set Ready         Modem

7       5       AB       Signal Ground           Both

8       1       CF       Carrier Detect         Modem

12       *       SCF      Speed Indicate         Modem

20       4       CD       Data Terminal Ready   Computer

22       9       CE       Ring Indicate          Modem

If you're using a Macintosh computer, ask your dealer for

the correct modem cable; we recommend a Hardware Handshaking

cable.

Macintosh 8-Pin DIN

Signal Source

DB-25    MAC        Function                Computer -- Modem

20/5*     1         Output Handshake*        Computer/Modem*

4         2**       Input Handshake              Computer

2         3**       Transmit Data Negative***    Computer

7         4         Ground                          Both

3         5         Receive Data Negative          Modem

--        6         Transmit Data Positive****       --

--        7         Not connected                    --

--        8         Receive Data Positive            --

*     Adds CTS capability when in Hardware mode.

**    Hardware handshaking lines.

***   To do this, you must ground pin 6.

****  To do this, you must ground pin 8.

NOTE:  A three-wire interface consists of Receive, Transmit,

and Ground wires and does not support hardware flow control.

Systems requiring three-wire interfaces must use software

flow control.  If your system doesn't support software flow

control, use no flow control but be sure to use an error-

correcting protocol.

============================================================

Minimum Requirements

Some computer equipment supports only a few of the serial

signal functions set in the Sportster modem.  The minimum

required for the modem to operate is as follows.

Minimum Required Pins

DB-25      DB-9       8-Pin

Pin        Pin        DIN           Function

2          3          3            Transmitted Data

3          2          5            Received Data

7          5          4            Signal Ground

20          4          1            Data Terminal Ready*

* Required if DIP switch 1 is OFF for normal DTR operations,

override disabled.

---------------------------------

Additional Flow Control Functions

---------------------------------

If your computer and software support Clear to Send (CTS)

and you wish to use Transmit Data hardware flow control

(&H1), Pin 5 (DB-25) or Pin 8 (DB-9) is required.

If your computer and software support Request to Send (RTS)

and you wish to use Received Data hardware flow control

(&R2), Pin 4 (DB-25) or Pin 7 (DB-9) is required.

-------------------------------------------------

For 115.2K, 57.6K and 38.4K bps Serial Port Rates

-------------------------------------------------

Your software and computer must support the 115.2K, 57.6K or

38.4K bps rate.  Make sure the serial cable is shielded.

Cables are normally six feet long, but longer lengths are

possible.  If you encounter problems with signal degradation,

try a shorter cable.

If you decide to build your own cable, use a

low-capacitance cable.  To further minimize the capacitance,

connect only those functions (pins) that your application

requires.

============================================================

DEFAULT SETTINGS

============================================================

============================================================

Data Format

Both your software and the remote system must use the same

10-bit data format.  If you don't know the setup of the

remote computer's modem, phone ahead to find out what

combination of word length, parity, and Stop bit is

required.

Set your communications software to the required scheme.

Some communications programs use a kind of shorthand for

formats, such as 7-E-1 or 8-N-1.  The modem detects the

format from the AT prefix of the next command it receives

from your keyboard or from your software.

Allowable Data Formats

Word         Parity          Stop

Length        (1 Bit)         Bits

7          Even, Odd,         1

Mark, Space

7            None             2

8            None             1

============================================================

Template Settings

You can create one or two customized configurations and

store one of them at a time in nonvolatile random-access

memory (NVRAM) as your power-on/reset default using the

&Wn command.  As long as DIP switch 7 is OFF when you power-

on or reset the modem, your defaults are loaded into the

modem's random-access memory (RAM).  To view your NVRAM

settings, use the ATI5 command.

The Sportster modem is preconfigured in the factory for the

&F1--Hardware Flow Control template settings in NVRAM as Y0,

and the &F2--Software Flow Control template settings in

NVRAM as Y1.

Tables on the next pages list the settings of the permanent

configuration templates &F1 (default), &F2, and &F0, as well

as

parameters you can modify and store in the NVRAM

configuration templates.

&F1--Hardware Flow Control Template

Factory Default

Feature                              &F1  Settings

ITU-T/Bell Answer Sequence           B0  ITU-T sequence

Online Echo                          F1  Online Echo OFF

Speaker Control                      M1  Speaker ON until

CONNECT

Pulse/Tone Dialing                   P   Pulse Dialing

Result Code Options                  X4  All Result codes

ARQ Result Codes                     &A3 All protocol codes

enabled

Serial port Rate                     &B1 Fixed serial port

rate

Guard Tone                           &G0 No guard tone

Transmit Data Flow Control           &H1 Hardware flow

control

Modem Testing                        &T5 Deny remote digital

loopback

Received Data Hardware Flow Control  &R2 Enabled

Received Data Software Flow Control  &I0 Disabled

Data Compression                     &K1 Auto enable/disable

Error Control                        &M4 Auto select

Connection Rate                      &N0 Variable connection

rate

Make/Break Ratio                     &P0 U.S./Canada ratio

Volume Control (internal)            L2  Medium volume

Data Set Ready (DSR)                 &S0 DSR always on

Break Handling                       &Y1 Break clears

buffer; break

then goes to

remote modem

The following parameters are changed via your communications

software:

Stored Phone Numbers                &Zn=s

Word Length                         8 bits*

Parity                              None*

Serial port Rate                    19.2K bps*

* Initial Settings; match software settings of subsequent &W

commands.

The &F2 and &F0 templates largely resemble the &F1 template.

The tables below list only those settings that differ from

the &F1 template.

&F2--Software Flow Control Template

Feature                             &F2  Settings

Transmit Data Flow Control          &H2  Transmit data

software

flow control

Receive Data Flow Control           &R0  Received data

hardware

flow control

disabled

&I2  Received data

software

flow control

enabled

&F0--Low Performance Template

Feature                             &F0  Settings

Result Code Options                 X1   Basic subset

ARQ Result Codes                    &A1  ARQ codes enabled

Serial port Rate                    &B0  Variable serial

port rate

Transmit Data Flow Control          &H0  Disabled

Receive Data Flow Control           &R1  Disabled

The following parameters are changed via your communications

software:

Word Length                         7 bits*

Parity                              Even*

Serial port Rate                    9600 bps*

* Initial Settings; match software settings of subsequent &W

commands.

NVRAM S-Register Options

NVRAM S-Register Options                  Template Settings

S0*  Number of rings to answer on                   1

S2   Escape code character                          43

S3   Carriage Return character                      13

S4   Line Feed character                            10

S5   Backspace character                            8

S6   Dial wait-time, sec.                           2

S7   Carrier wait-time, sec.                        60

S8   Dial pause, sec.                               2

S9   Carrier Detect time, 1/10th sec.               6

S10  Carrier loss wait-time, 1/10th sec.            7

S11  Tone duration, spacing, msec.                  70

S12  Escape code guard time, 1/50th sec.            50

S13  Bit-mapped functions                           0

S14  Bit-mapped functions                           0

S15  Bit-mapped functions                           0

S19  Inactivity/hang up timer                       0

S21  Break length, 1/100th msec.                    10

S22  XON character                                  17

S23  XOFF character                                 19

S25  DTR recognition time, 1/100th sec.             5

S27  Bit-mapped functions                           0

S28  V.21/V.23 fallback delay, 1/10th msec.         8

S34  Bit-mapped functions                           6

S38  Disconnect wait time, sec.                     0

NOTE:  Bit-mapped registers have up to eight functions.  See

instructions under S13 in Appendix A of the "Quick

Installation Guide."

* The valid range of rings that can be stored in NVRAM for

S0 is 1-255.  S0=0 cannot be stored in NVRAM.  Regardless of

the NVRAM setting, DIP switch 5 must be OFF for the modem to

be in Auto Answer mode at power-on/reset.

************************************************************

MODEM CONCEPTS

************************************************************

============================================================

HOW MODEMS WORK

============================================================

Modem is a term based on the concept of MOdulation and

DEModulation.  A modem modulates (converts) digital data

(computer information) to analog data (fluctuations in tones

carried over a copper telephone wire).  The information is

carried over a telephone network until it reaches its

destination, where another modem demodulates the analog

signals and converts them back to digital data so the

computer there can use the information.

This ability to use the telephone network for quick,

inexpensive data exchange is a powerful tool used by

businesses and individuals worldwide to expand

business and personal networks.

============================================================

MODEM CONFIGURATION

============================================================

Modems come in all shapes and sizes and their ability to

communicate is based on the protocols they use, or rules

they follow to perform operations in identical ways.  They

may be preset or reset physically (DIP switches) and

logically (communications software) to best communicate with

the modem they are transferring information to and receiving

information from.

Much of this is done automatically by the modems when they

initially contact each other.  The calling modem contacts

the answering modem and introduces itself.  The modems

communicate via a series of signals to identify the

appropriate protocol and speed for efficient data transfer.

The answering modem either accepts the call or rejects the

call.  This transaction is called a "handshake."

Successful handshaking results in what is called "carrier."

When modems establish carrier, your modem sends a Carrier

Detect signal to your computer, indicating that the modems

are ready to transfer data.  If they fail to connect, your

modem sends your computer a No Carrier message.

============================================================

LINE TRAVEL

============================================================

Poor line quality may cause a decrease in efficient data

transmission.  In order to ensure the data sent and received

is reliable, error control was introduced by modem

manufacturers.  The modems check each data block received,

and if something went wrong between locations, the receiving

device instructs the sending device to resend the affected

block.

Modems send information at different rates, measured in bits

per second (bps).  Today, the figures can be staggering.  In

the most optimal situation, the Sportster can exchange data

as fast as 115,200 bps.

In most cases, though, the speed relies heavily on the

ability to adapt to line conditions at high speed.  This

adaptability is the most important feature of the Sportster.

============================================================

DIGITAL DATA

============================================================

Modems send data via asynchronous communication.  The

smallest data unit sent is made up of a defined word length

(7 or 8 bits each), a Start bit (a 0 that indicates where

the data unit begins), and one or two Stop bits.

Parity bits were the typical method of controlling errors

before cyclic redundancy check (CRC) error correction,

described below under :Error Control."  A parity bit is

either a 1 (odd parity) or a 0 (even parity), depending upon

whether the data segment has an odd or even number of binary

digits.  Some systems allow mark parity (parity is always

1--odd) or space parity (parity is always 0--even).  Parity

bits are used less often now that CRC is common.

The setting 8-N-1 (word length=8, parity=None, stop bits=1)

has become the most common data format in data

communications.  Both computers involved in a data transfer

must use the same parity, word length, and number of Stop

bits or connection isn't possible and garbage characters

will display.  The software must first be set the same on

the computers at both ends of the data transfer before the

modems can operate effectively.  A simple phone call

to determine the settings at the other end can clear this up

quickly and easily.

The requirement to specify parity setting, even if it is

None (*-N-1), assures that users with older systems can

still communicate with newer modems.

============================================================

FLOW CONTROL

============================================================

Another important aspect of modem communications is flow

control, which manages the amount of data stored in buffers.

Buffers are used to store information temporarily before it

is passed on to a computer or modem.  Flow control is used

to prevent buffer overflow.  The system uses either hardware

or software (control characters) flow control.  U.S.

Robotics recommends the use of hardware flow control,

because actual data may be mistaken for the control

characters used in software flow control and the data may be

distorted.

============================================================

ERROR CONTROL

============================================================

Error control protects the integrity of data transferred

over phone channels and is available for calls at 1200 bps

and above.  It can be disabled, although high-speed calls

(above 2400 bps) should always be under error control.  The

operations defined in an error control protocol include the

following.

*  Establishment of compatibility

*  Data formatting into blocks

*  Error detection through Cyclic Redundancy Checking (CRC)

CRC is based on algorithms that calculate a value for an

entire block of data.  The CRC value attached to each

block sent must match the receiving modem's calculation.

If not, the remote modem sends a negative acknowledgment

to the sending modem.

*  Positive acknowledgment of error-free blocks and

negative acknowledgment of corrupted data blocks

*  Retransmission of corrupted data blocks

Always set the Sportster for error control, &M4 (default) or

&M5, for calls at speeds over 2400 bps.

The Sportster is set at the factory to &M4, causing it to

try for an error control connection and, if that isn't

possible, to proceed with the call in Normal mode.  The

modem first tries for a V.42 connection, then an MNP

connection.  The information below is based on the

Sportster's setting of &M4.

============================================================

ITU-T V.42 Handshaking

The exchange of signals between two devices in order to

establish a communications link is called handshaking.

ITU-T V.42 includes a two-stage handshaking process.

*  A Detection phase that is based on an exchange of

predefined characters.

*  LAPM (Link Access Procedures for Modems) Negotiation.

In this phase, the modems identify their capabilities

concerning maximum data-block size and the number of

outstanding data blocks allowed before an acknowledgment

is required.

============================================================

MNP Handshaking

This protocol is supported by the ITU-T V.42 Recommendation.

It was originally developed by Microcom, Inc., and is now in

the public domain.

MNP handshaking begins with an MNP Link Request sent by the

calling modem.  If the remote modem doesn't recognize the

request, error control isn't possible.

============================================================

Data Compression

If the modems successfully establish a V.42 connection, they

also negotiate for V.42 bis data compression.  If they

successfully establish an MNP connection, they negotiate for

MNP5 data compression.

Modems using V.42 bis compression negotiate the following

options.

*  Dictionary size--that is, the amount of memory available

for compression table entries.  (Entries are codes

devised for redundant data.  The data is packed into

shorter data units, called code words, and unpacked by

the receiving modem.)

Possible sizes are as follows.  U.S. Robotics modems use

11-bit, or 2048-entry dictionaries, but drop down if the

remote modem uses a 512- or 1024-entry dictionary.

Bits        Entries

9           512

10          1024

11          2048

*  Maximum string length of each entry.

As the dictionary fills, the modem deletes the oldest unused

strings.  V.42 bis compression is more efficient than MNP5

compression, in part because it dynamically deletes unusable

strings.  In addition, it works better with files that are

already compressed.  These include .ZIP files downloaded

from many Bulletin Boards and 8-bit binary files, which

appear to the modem to be compressed.

MNP5 compression should not be used with such files because

it adds data to them, which lessens throughput.  (The

additional data is stripped when the file is decompressed by

the remote modem.)  When transferring such files, it's best

to set the modem to &K3.  This allows V.42 bis compression

to work dynamically with the compressed data, but disables

MNP5.

============================================================

Flow Control

Flow control of data from the computer is required under

error control for two reasons.

1.  The transmitting modem buffers a copy of each frame it

transmits to the remote end until it is acknowledged by

the receiving modem.

2.  If errors are encountered, the transmitting modem must

resend the corrupted data.  This retransmission

activity, combined with the steady stream of data from

the computer, can overflow the buffer.

============================================================

Online Fallback/Fall Forward

Under error control, if a disturbance on the phone line

causes an error to a data block, the receiving modem replies

with a negative acknowledgment.  In response, the

transmitting modem retrieves a copy of the original data

block from its Transmit buffer, and every block it sent

after that block, and retransmits them.  This keeps

the data error-free and in sequence.

However, there is a retransmit limit:  the modems hang up if

line disturbances are so severe that one of the modems has

retransmitted the same block of data twelve times without a

positive acknowledgment.

Because high-rate calls are more vulnerable than

transmissions at 2400 bps and below, V.32 bis/V.FC modems

risk reaching the retransmit limit and hanging up.  To

prevent this, one of the modems requests that they fall back

if necessary.  When line conditions improve, the modems fall

forward to the next higher rate, up to the link rate of the

call.

Online fallback/fall forward is defined in V.FC and ITU-T

recommendation V.32 bis for modems.  The Sportster 9600,

however, is a V.32 modem.  V.32 modems fallback to 4800 bps

and stay at that rate.

============================================================

THROUGHPUT GUIDELINES

============================================================

The following guidelines should help you to make the most of

your modem's advanced performance features.  In many

instances, experimentation and experience will indicate what

works best for your applications.

Throughput is the volume of user information transferred per

second, without Start and Stop bits and other overhead

information.  You'll obtain optimal throughput under the

following conditions.

1.  Your communications software supports a fixed serial

port rate higher than the connection rate (for example,

setting your software to lock into the 38.4K bps rate,

and retaining the default &B1 setting).

2.  The call is under data compression.

3.  You're transmitting text files.  Throughput is higher

for text files than other types of files, such as .EXE

or .COM binary files.

4.  File transfer may be slowed down by a file-transfer

protocol.  Many non-text files require a file-transfer

protocol, but throughput results vary.  Certain public

domain file-transfer protocols, for example, have the

following effects.

Kermit     Newer versions of Kermit support packets up

to 9K and a sliding window design to

eliminate turnaround delay.  With earlier

versions, however, throughput may be severely

reduced due to short block lengths (possibly

under 128 bytes) and acknowledgment

turnaround time.

Xmodem     Throughput may be reduced if your version

uses short block lengths, for example 128

bytes.  Some versions use blocks of 1K byte,

which is much better, although overhead

(error control protocol information) still

affects overall throughput.

Ymodem     This protocol is similar to Xmodem with 1K

byte block lengths, and allows multiple files

to be sent in one transfer.

The above protocols further reduce throughput during error-

control (ARQ) connections.  The accuracy of the data is

checked twice, by the file-transfer protocol and the modem.

To avoid redundancy, disable modem error control by setting

the modem to &M0.

The most current version of Zmodem can yield the most

efficiency.  Leave the modem at its error control default

(&M4) and data compression default, &K1.  Zmodem performs

the same kind of compression as V.42 bis; it turns off its

compression if files are already compressed.

An alternative protocol is Ymodem-G, with the modem left at

its error control default, &M4.  Ymodem-G assumes the modems

are handling error control.  Overhead is minimal, with

throughput almost equal to that obtained with no file-

transfer protocol.

However, keep in mind that Ymodem-G is only useful if the

modems are using error control.  In addition, follow this

recommendation only if your machine and software support

hardware flow control.

NOTE:  Both modems must use the same protocol for data

transfer to take place.

WARNING:  If you are using an X-, Y- or Zmodem-type

protocol, do NOT use the modem's software flow control.

============================================================

Achievable Throughput Statistics

The table below indicates the maximum throughput, in

characters per second (cps), that can be expected under the

following conditions:

*  Serial port rate set at 57.6K bps; modem set to &B1

Your software and computer must support 57.6K bps in

order to use that rate.)

*  Connection (link) rate of 14.4K bps (assuming no

protective fallback to a lower speed is necessary)

*  V.42 bis compression negotiated for the call, and the

default size 11-bit, 2048-entry dictionary

*  Straight data (that is, not already compressed, and no

file-transfer protocol)

*  Transmission from a fast (for example, 386) computer

Throughput (cps) if set to 14.4K bps

File Type                            MNP5         V.42 bis

Assembler or Compiler listing        2880         3840

Text file                            2325-2625    3400-5760

Binary file:  .EXE                   2175-2400    2030-2600

Binary file:  .COM                   2100-2250    2050-2300

.ZIP files (common on BBS's)*        1500-1650    1700

Random binary 8-bit*                 1460-1575    1700

* These files are already compressed or appear to the modem

to be compressed.  Additional MNP5 compression causes

throughput lower than what can be expected using MNP without

compression.  We recommend setting the modem to &K3 when

transferring these files, to allow V.42 bis but disable

MNP5.

The following table indicates the maximum throughput, in

characters per second (cps), that can normally be expected

in the same conditions as the previous table, but with a

serial port rate of 38.4K bps.

Throughput (cps) if set to 14.4K bps

File Type                            MNP5         V.42 bis

Assembler or Compiler listing        2880         3840

Text file                            2325-2625    3400-3840

Binary file:  .EXE                   2175-2400    2030-2600

Binary file:  .COM                   2100-2250    2050-2300

.ZIP files (common on BBS's)*        1500-1650    1700

Random binary 8-bit*                 1460-1575    1700