Farallon PhoneNET PC Card III (MCA) Installation Notes

======================================================

1. IMPORTANT NOTES

---------------

To use this MLID, PHNETMC2.COM is the only software driver you need to

run PhoneNET PC in your system.

2. GENERAL INFORMATION

-------------------

The Farallon PhoneNET PC Card III (MCA) uses I/O ports and interrupts.

The card does not use any DMA channels or shared memory.

The Farallon PhoneNET PC Card III (MCA) uses one of 6 I/O ports.  The

card's I/O address can be set by using the reference diskette that comes

with your computer system and .ADF file that comes with the adapter.

The PhoneNET install program allows the following possible values for

I/O ports:

340H, 320H, 2A0H, 260H, 240H, 200H

The Farallon PhoneNET PC Card III (MCA) uses one of 3 IRQs.  The card's

IRQ number can be set by using the same files mentioned above.  The

interrupt numbers allowed are :

3, 4, 5

For more details about configuring Farallon's PhoneNET PC Card III

(MCA) and installing the driver, please refer to the "Farallon PhoneNET

PC Card II User's Guide" that comes with your adapter.