SMC EtherCard PLUS ELITE LAN Adapter Installation Notes

=======================================================

1. IMPORTANT NOTES

---------------

You MUST set jumpers W1 and W2 to 'SOFT' and run the configuration

software EZSETUP.EXE to configure the adapter before you load the

driver SMC8000.COM.  The configuration software is shipped with the

driver on the distribution diskette when you purchase this adapter.

The 'PORT SELECT' jumper on the card must be set to the appropriate

media. Note that not all RAM Base Addresses are available in the

installation program.  If you need to select a different RAM Base

Address from those presented you will need to edit the NET.CFG file

after installation.

2. GENERAL INFORMATION

-------------------

For more details, please refer to the manual that comes with your

adapter.