This file contains important information for PC Board Layout

Tools 386+ that was not included in the printed documentation.

******************   !!!  IMPORTANT  !!!   ***********************

Please read these first three entries BEFORE installing PCB 386+

******************************************************************

------------------------------------------------------------------

Installing PCB 386+ while running 4DOS

------------------------------------------------------------------

Do not run the install program for PCB 386+ if you have 4DOS

loaded as your current operating system. Be sure MS-DOS is the

active operating system on your computer before you run the

install program.

------------------------------------------------------------------

If you choose during installation to update your ORCADESP.DAT

files manually using MERGEDAT

------------------------------------------------------------------

If you choose 'NO' (the default selection) when the install

program asks you if you want your ORCADESP.DAT files updated to

include the new PCB 386+ configurations, you need to run MERGEDAT

when the installation is complete.

The install program places PCB386.MRG in the directory specified

by the ORCADESP environment variable (the ORCADESP directory, if

your directory structure follows the default configuration). If

you want all your ORCADESP.DAT files updated for PCB 386+, you

enter the following command:

MERGEDAT PCB386.MRG/A

This creates the file PCB386.DAT in all your design directories.

To access the new PCB 386+ configurations, copy PCB386.DAT to

ORCADESP.DAT in all of your design directories.

------------------------------------------------------------------

Renaming PCB II configuration files

------------------------------------------------------------------

When PCB 386+ is installed, the PCB II configuration files

PCB.CFG and PCB.BCF are overwritten in the TEMPLATE and TUTOR

design directories by PCB 386+ configuration files that have the

same filenames. If you want to retain the PCB II configuration

files after installing PCB 386+, you need to rename them, such as

PCBII.CFG and PCBII.BCF, before installing PCB 386+.

Copy the RENAMED PCB II configuration files to your other design

directories. If you need to run PCB II after the PCB 386+

configuration files are installed, you can rename PCB.CFG and

PCB.BCF to filenames like PCB386.CFG and PCB386.BCF, then rename

PCBII.CFG and PCBII.BCF to PCB.CFG and PCB.BCF. After you are

done with PCB II, you rename the four configuration files back to

their original filenames.

******************************************************************

******************************************************************

------------------------------------------------------------------

Copying PCB 386+ configuration files to all design directories

------------------------------------------------------------------

After installing PCB 386+ you need to copy the PCB 386+

configuration files PCB.BCF and PCB.CFG from the TEMPLATE or TUTOR

design directories to your other design directories. This must be

done so PCB 386+ will work properly with all of your design

directories.

If you used PCB II with other designs, the PCB II configuration

files PCB.CFG and PCB.BCF are in the other design directories.

You need to rename these configuration files before copying the

PCB 386+ configuration files. See the entry above titled

"Renaming PCB II configuration files" for related information.

------------------------------------------------------------------

*** CAUTION ***  Running incompatible drivers for PCB 386+ while

working in PCB II design directories

------------------------------------------------------------------

When you are working in a PCB II design directory you must be

sure that the configurations are updated so they specify the new

PCB 386+ drivers. PCB II drivers are incompatible with PCB 386+.

If an incorrect driver is configured, information similar to

what is shown below displays, including an error message:

Reading Configuration Data File: PCB.BCF

OrCAD PCB 386+   Version   x100m   28-Jun-93

(C) Copyright 1985-1993  OrCAD, Inc.  ALL RIGHTS RESERVED.

OrCAD is a Registered Trademark of OrCAD, Inc.

HANDLER          Version    V1.00  30-Jun-93

(C) Copyright 1992,1993  OrCAD, Inc.  ALL RIGHTS RESERVED.

Loading "C:\WORK\ROUTE\LQ1000.DRV"

LQ1000 Printer Driver  V4.20

(C) Copyright 1987 OrCAD, Inc.  ALL RIGHTS RESERVED.

----

Attempting to use: "C:\WORK\ROUTE\VGATEST.DRV"          |

ERROR: Driver is not compatible with this version.      |

Please configure a driver which came with this product. |_ error

|  message

MSDOS Error # 10 Bad Environment                        |

ERROR: Unable to initialize Driver Handler - ERROR# 10  |

----

------------------------------------------------------------------

*** CAUTION *** Loading a netlist containing duplicate time stamps

------------------------------------------------------------------

PCB 386+ cannot process a netlist containing duplicate time

stamps. If duplicate time stamps are encountered, an error

message displays.

Versions of SDT prior to SDT 386+ v1.10 are able to create

schematic parts containing duplicate time stamps. The BLOCK Save

command copies sections of a schematic to a buffer. The BLOCK Get

command retrieves the items in the buffer and copies them to the

worksheet. The copied schematic parts have the same timestamp as

the original source parts.

To avoid duplicating time stamps, perform a BLOCK Export of the

section of the schematic you wish to copy, then do a BLOCK Import

of the file into the worksheet. The schematic parts in the

imported file receive new time stamps.

------------------------------------------------------------------

Drawing an outline before printing and plotting your board file

------------------------------------------------------------------

It is very important that you create an outline that surrounds

all board file objects, including dimensions, alignment targets,

and documentation, that you want to appear on the printed or

plotted page. The outline defines the area that is considered for

printing and plotting in PCB 386+. You do not have to include the

outline in your print or plot.

If your board file is not enclosed within an outline, the PC

board outline is used to define the printing limits. Objects

placed outside the board outline, such as alignment targets and

dimensions, may not appear on the print.

You can draw the outline on the Assembly Layer, then add items

like a title block and documentation on the Assembly Layer to

create an assembly drawing for the board file.

------------------------------------------------------------------

Entering numeric values in ESP configuration screens

------------------------------------------------------------------

Do not exceed three decimal places for any numeric values entered

in ESP configuration screens. Unpredictable results may occur if

values with more than three decimal places are entered.

------------------------------------------------------------------

Addendum to "Chapter 3: Transferring from schematic to layout"

in the PC Board Layout Tools 386+ User's Guide

------------------------------------------------------------------

"Chapter 3: Transferring from schematic to layout" in the PC Board

Layout Tools 386+ User's Guide describes how to transfer a design

from OrCAD's Schematic Design Tools to PCB 386+. The procedures

and figures depict version 4.30 of the ESP design environment and

version 1.10 of Schematic Design Tools 386+.

The following figures shown in chapter 3 are unique to ESP v4.30:

*  Figure 3-2.  Design Management Tools screen, Design View

*  Figure 3-4.  The Schematic Design Tools screen.

*  Figure 3-6.  Partial view of the Configure Schematic Design

Tools screen, Library Options section.

*  Figure 3-11. The Configure Update Field Contents screen.

The following menus shown in chapter 3 are unique to ESP v4.30:

*  Configuring INET, Local Configuration

*  Configuring ILINK, Local Configuration

*  Configuring IFORM, Local Configuration

The following changes apply if you use the indicated versions

of SDT:

SDT Release IV and SDT 386+ v1.0X

---------------------------------

In the section "Viewing key fields," there are no preconfigured

key field entries in the Configure Schematic Design Tools screen.

The key fields must be defined by the user.

In the section "Updating key fields," TUTOR386.STF is not selected

from the Files list box. The name of the update file must be

entered in the Update file entry box. Also, Part Field 8 is not

automatically selected in the Field to be updated section of the

Configure Update Field Contents screen. The part field must be

defined by the user.

In the section titled "Checking design integrity," there is no

Check Design Integrity tool to configure and incrementally

execute Cleanup Schematic, Cross Reference Parts, and Check

Electrical Rules. The processors must be selected individually.

SDT Release IV only

-------------------

In the section "Creating the netlist," the "Configuring IFORM"

procedures describe selecting FEDIF.EXE in the Netlist Format

list box. You select <EDIFfilename>.CF or <EDIFfilename>.CCF in

this list box.

------------------------------------------------------------------

Zone information

------------------------------------------------------------------

Displaying fill zones with Show Copper Pour enabled

---------------------------------------------------

If Show Copper Pour is enabled in Global Options, and you delete

a segment from a fill zone, the entire fill zone does not

display. This is because a complete boundary no longer exists for

the copper pour. Follow these steps to display the fill zone:

1.  Select SET to display the Global Options dialog box.

2.  Disable Show Copper Pour, then select OK. The remaining

segments of the fill zone display.

3.  Place a new fill zone segment so it closes the zone.

4.  Select SET to display the Global Options dialog box.

5.  Enable Show Copper Pour, then select OK. The fill zone

displays as a filled area.

Completely closed zones

-----------------------

PCB 386+ checks a zone to make sure it is completely closed. If a

zone is not closed, the following error message displays:

"A zone of this type X is not closed"

The 'X' in the error message is replaced with the type of zone.

Zones created incorrectly

-------------------------

If segments for a zone are placed on different layers, the

following error message displays:

"Zone of this type X has edges with different layer

designations"

The 'X' in the error message is replaced with the type of zone.

Maximum number of autoroute zone edges

--------------------------------------

An autoroute zone can contain a maximum of 100 segments. If the

number is exceeded, the following error message displays:

"Autoroute zone has too many edges"

Using an autoroute zone as a board outline

------------------------------------------

You can use an autoroute zone as a board outline in PCB 386+.

When you autoroute the board, all tracks are routed inside the

autoroute zone outline, even if the shape is an irregular polygon.

Spacing violations may occur if you use an autoroute zone as a

board outline and place copper objects, such as copper edge

connectors, so they violate the copper to copper spacing between

the zone and the connector fingers.

If you use an autoroute zone as a board outline, you should also

consider manufacturing requirements for the board. If tracks and

vias need to be kept a certain distance inside the edges of the

board (at least 50 mils, for example), you may want to draw a

conventional board outline and draw the autoroute zone 50 mils

inside the board outline.

Eliminating copper islands within fill zones

--------------------------------------------

If a fill zone is placed around a group of pads and the pad

isolation produces copper islands within the fill zone, draw

a No Fill zone around the copper islands to delete them. The

No Fill zone outline does not need to exactly match the outline

of the copper island--it can overlap parts of tracks and pads.

Connectivity between pads inside a fill zone

--------------------------------------------

Connectivity is not automatically established between pads in a

fill zone if the zone encompasses the pads and all objects are

assigned to the same net. An 'X' displays inside the pads when

Show Unconnected Objects is enabled in the Global Options dialog

box, indicating that the pads are unconnected.

To establish connectivity between the pads, you need to route the

pads before you place the fill zone.

------------------------------------------------------------------

Engineering Change Order (ECO) layers for PCB 386+

------------------------------------------------------------------

Edit Layout has two board layers that are used for Engineering

Change Orders, or ECOs. You enter engineering change information

on these layers, such as routing revisions or information about

modules that need to be changed or added.

You specify routing changes by drawing with outline segments. Do

not use the ROUTE command to place tracks on the ECO layers. Use

a color that distinguishes the ECO layer from the other layers.

Use PLACE Text to enter text on the ECO layers.

------------------------------------------------------------------

Loading a PCB 386+ board file with a preset zoom scale and

pointer location

------------------------------------------------------------------

The current zoom scale and pointer location is saved when you

save the board file. You can use this feature to automatically

display the area you were previously editing when you reload the

board file. Follow these steps:

1.  Pan and zoom in to the area you want to automatically display

the next time you load the board file.

2.  Place the pointer in the center of the screen. The saved

window uses the pointer location as its centerpoint.

3.  Select QUIT Update Board File or QUIT Write Board File to

save the file and retain the zoom scale and pointer location.

4.  Select QUIT Initialize Board File and select the board file

you just saved. The board file displays with the saved zoom

scale and pointer location.

------------------------------------------------------------------

Working with OrCAD/PCB II board files in PCB 386+

------------------------------------------------------------------

Converting PCB II board files containing multiple zones

-------------------------------------------------------

FROMPCB2.EXE, the utility that converts PCB II board files to

PCB 386+ board files, may not successfully convert PCB II board

files containing multiple zones. The converted board file will

need to be closely examined.

FROMPCB2 netname and module name translations

---------------------------------------------

FROMPCB2 translates netnames to be all lowercase and module names

to be all uppercase.

Translated PCB II files containing unconnected tracks

-----------------------------------------------------

If you intend to modify converted OrCAD/PCB II board files in

PCB 386+, you should be aware that the converted files may have

unconnected tracks. The following procedures describe ways you

can edit unconnected tracks in PCB 386+:

Automatically adding track segments to pad centers

--------------------------------------------------

1.  Load the translated board file into Edit Layout.

2.  Save the board file. Track segments are automatically added

to tracks that end inside a pad, but not at the center.

Determining which tracks are unconnected

----------------------------------------

1.  Load the translated board file into Edit Layout.

2.  Select QUIT Cleanup Stubs, then select OK. All unconnected

tracks are deleted.

3.  Select UNDELETE to restore the tracks, or select SELECTIVE to

display all deleted track segments in their normal layer

colors. In selective-undelete mode, these tracks can be

selected and undeleted.

Methods for connecting unconnected tracks

-----------------------------------------

*  Run the autorouter on the board. The unconnected tracks will

be ripped up and rerouted.

*  Manually repair a track by moving the endpoint so Edit Layout

snaps the segment to the center of the pad.

------------------------------------------------------------------

DRC checking on tracks constructed with arc segments

------------------------------------------------------------------

DRC checking in Edit Layout may report a clearance violation

between a copper object and a track constructed with arc segments

even though no violation exists. To guarantee that there is proper

clearance between arc tracks and adjacent copper objects, follow

these steps:

1.  Select SET to display the Global Options dialog box.

2.  Enable Show Highlight Guard. This displays the copper to

copper spacing for copper objects already on the board.

3.  Enable Show Copper And Guard While Routing. The copper to

copper spacing for the arc segments displays while you route.

4.  Select OK to close the dialog box.

5.  Zoom in on the area you are routing with arc segments and use

the displayed guard bands as a visual guide to determine if

you are routing too close to another copper object.

------------------------------------------------------------------

Deleting DRC violations

------------------------------------------------------------------

Do not select the autoroute option "Delete DRC Violations" in the

Autoroute Options dialog box to delete DRC markers related to

arc segments. Instead, follow these steps:

1.  Correct all DRC violations.

2.  Select GO TO FUNCTION Autorouter.

3.  Place the pointer above and to the left of the corrected

area and select Block.

4.  Move the pointer below and to the right of the corrected area

and select Block End.

5.  Select Spacing/DRC Check Block from the menu. The Finished

dialog box displays, indicating the number of DRC violations.

Note the number of violations, then select OK.

If the number of violations is zero, all DRC markers are deleted.

If DRC violations are still reported for the areas containing arc

tracks, and you know there is adequate clearance, follow these

steps to delete those DRC markers:

1.  Select LAYER to display the Layer dialog box.

2.  Select Comment Layer, then select OK. When Comment Layer is

the active layer you can delete DRC markers without

accidentally deleting other objects.

3.  Place the pointer on the DRC marker and select DELETE.

------------------------------------------------------------------

Determining the filename of the current board file

------------------------------------------------------------------

Before you select QUIT Update Board File to save the current

board file, you can determine the filename it will be saved to

by following these steps:

1.  Select QUIT Initialize Board File. The filename of the loaded

board is highlighted in the Files list box of the Initialize

to Board File dialog box.

2.  Select Cancel to close the dialog box.

------------------------------------------------------------------

Running multiple sessions of ESP under Windows 3.1

------------------------------------------------------------------

You can run multiple sessions of ESP under Windows 3.1 and Windows

for Workgroups, but not under Windows 3.0. SHARE.EXE must be

loaded. Performing local configurations with multiple sessions of

ESP running may produce unpredictable results. You will need to

monitor memory consumption if you use the PCB 386+ autorouter.

------------------------------------------------------------------

Placing a new module in Edit Layout

------------------------------------------------------------------

When you select PLACE Module, then select New and select a

module from the listed library, the module is automatically

placed at the current pointer location. You are actually

performing a move command as you drag the module around the

screen. A module image remains where it was originally placed.

If you press <Esc>, the module is not deleted--it is returned to

its original placed position. You must delete the module if you

do not want it placed on the board.

------------------------------------------------------------------

Downloading a demo Gerber viewer from the OrCAD bulletin board

------------------------------------------------------------------

A demo version of a WISE Gerber viewer is available on the OrCAD

bulletin board. You can download this file from the Customer

Contributed Programs section of the PCB Download area.

The phone number for the OrCAD bulletin board is (503)690-9791.

------------------------------------------------------------------

Selecting an OrCAD icon in Windows

------------------------------------------------------------------

When you install PCB 386+, an OrCAD icon (ORCAD.ICO) is installed

in your ORCADEXE directory. You can use this icon for your OrCAD

applications that are installed in the Windows Program Manager.

Follow these steps to change the icon to ORCAD.ICO:

1.  Select the icon for the OrCAD application you wish to change.

2.  Select File from Program Manager. The menu displays.

3.  Select Properties... to display the Program Item Properties

dialog box. The current icon displays in the dialog box.

4.  Select Change Icon... to display the Change Icon dialog box.

The path for the icon displays in the File Name entry box, and

the current icon displays in the Current Icon select box.

5.  Select Browse... to display the Browse dialog box.

6.  Change the directory to your ORCADEXE directory.

7.  Select ORCAD.ICO in the File Name list box, then select OK.

The dialog box closes and the new icon and directory path

displays in the Change Icon dialog box.

8.  Select OK. The Program Item Properties dialog box displays,

and the new icon displays in the dialog box.

9.  Select OK. The dialog box closes and the new icon displays

for the OrCAD application.

------------------------------------------------------------------

Determining the number of incomplete nets after autorouting

------------------------------------------------------------------

Refer to the statistics in the Conditions dialog box to determine

the number of incomplete nets after autorouting a board. Select

? CONDITIONS from the main menu to display the dialog box.

------------------------------------------------------------------

The number of displayed items in a list box

------------------------------------------------------------------

List boxes that list items the user can create, such as copper

tools, are limited to five displayed entries. If more than five

items are available, the list box displays scroll buttons so you

can scroll the selections.

------------------------------------------------------------------

Mirroring objects in the board editor

------------------------------------------------------------------

Be sure Allow Edits of Module Objects is disabled in the Global

Options dialog box before you block select an area of your board

and attempt to mirror the objects within the block. When

Allow Edits of Module Objects is disabled, modules inside the

block will not mirror with the rest of the objects.

------------------------------------------------------------------

Running MAKELIB on a board file containing multiple copies of a

module

------------------------------------------------------------------

If you create a module library from a PCB 386+ board file using

the MAKELIB utility, be sure you delete any duplicate modules on

the board file before you run MAKELIB. If duplicate module are

present in the board file, duplicate modules are placed in the

module library when you run MAKELIB. Error messages are

encountered when you place a module from the library or load a

netlist.

It is suggested that you create a new board file, then load the

modules that you want grouped into a module library and run

MAKELIB on the new board file.

------------------------------------------------------------------

Working with planes in PCB 386+

------------------------------------------------------------------

Planes: general information

---------------------------

When a layer is enabled as a plane and assigned a net name, the

plane does not display in PCB 386+. You can select a different

display color for the plane layer, but the plane is invisible.

Connectivity and thermal relief is automatically created between

the plane and all thru-hole pads assigned to the same net.

Suggested autorouting procedures are described below for boards

using planes:

1.  Select GO TO FUNCTION, then select Net Property Editor from

the menu. The Edit Net Properties dialog box displays.

2.  Select the net in the Net Names list box that is assigned to

the plane.

3.  Enable Do Not Route Net, then select OK to accept the new

settings and close the dialog box. You do not want the

autorouter to route the selected net since it is already

routed to the plane.

4.  Select GO TO FUNCTION Autorouter.

5.  Select Set to display the Autoroute Options dialog box, then

select autorouting options and select OK.

6.  Select Whole Board, then select Autoroute Whole Board from

the menu to route the remaining nets.

Reporting connectivity to plane layers

--------------------------------------

Performing a DRC check on a net assigned to a plane places a DRC

marker on any pad in the net, such as a surface mount pad, that

is not connected to the plane.

Also, the modify modules utility, MODMOD_.EXE, reports nets

assigned to planes as unconnected.

Maximum number of characters for a netname assigned to a plane

--------------------------------------------------------------

You cannot have a netname containing more than 21 characters if

you intend to assign the net to a plane. If a netname has more

than 21 characters it cannot be selected as a plane in the Copper

Colors/Enables/... dialog box. The netname does not display in the

netname droplist box for a plane layer because the netname becomes

the layer name, and layer names cannot be longer than 21

characters.

------------------------------------------------------------------

Creating a Gerber 274-D tool list

------------------------------------------------------------------

You need to provide a tool list containing Gerber aperture codes

if you plot to the Gerber 274-D photoplotter. Use the following

procedures to create the tool list:

1.  Load the board file into Edit Layout.

2.  Select GO TO FUNCTION Printing and Plotting. The Printing and

Plotting dialog box displays.

3.  Select Driver. The Driver Configuration dialog box displays.

4.  Select Vector Device. The droplist button displays.

5.  Select the droplist button to display the vector device list.

6.  Select Gerber (274-D). The droplist box closes. Gerber format

selections and the Load button in the Tool List box displays.

7.  Select a Gerber format, or accept the default selections.

8.  Leave File as the destination.

9.  Enter <filename>.GBR in the entry box beneath the Files list

box. This specifies the Gerber file that is written to the

disk when you select Begin All or Begin in the Printing and

Plotting dialog box.

10. Select Replace.

11. Select OK. The Printing and Plotting dialog box displays.

12. Set up the pages and page contents for the board layers you

want to photoplot.

13. Select Begin All. The Gerber information for all pages is

written to <filename>.GBR, and a tool list for all board

objects is created in memory.

NOTE: The same tool list can also be created by selecting an entry

in the Page Contents list box and selecting Begin. The

Gerber file that is produced only contains information about

the selected objects, but the resulting tool list still

contains D codes for all objects on all layers because the

entire board file is scanned to produce the tool list.

14. Select Driver. The Driver Configuration dialog box displays,

and the Load, Save, and Clear buttons display in the Tool

List box.

15. Select Save in the Tool List box. The Save Tool List to File

dialog box displays.

16. Enter <filename>.TOL (or the extension you prefer) in the

entry box beneath the Files list box, then select OK. The

tool list is saved to the filename.

17. Delete the .GBR file, or use it as part of your package of

Gerber files that you prepare as you select additional layers

and plot them to new .GBR filenames.

18. Review the tool list file and edit it to delete any D code

entries that are not currently used on the board.

19. Copy the tool list file and Gerber files to a diskette or

tape, then transport the files to the photoplotter.

NOTE: The tool list is retained in memory until you select Clear

from the Tool List box, or until you select QUIT Abandon

Program. You can load additional board files and add the

aperture information for those board files to the list.

------------------------------------------------------------------

EMS error message for PCB 386+ utilities

------------------------------------------------------------------

If you do not have enough EMS memory while running a PCB 386+

utility program, you may encounter an error message similar to

the example shown below:

EMS error, code 8A  from function 0044

EMS handle 5

ERROR (frompcb2), Error accessing EMS, requested

output will be incomplete

The EMS error code, function number, and EMS handle number varies

depending on system conditions. The utility encountering the EMS

memory problem displays in parentheses.

Use the /Page_Size switch for the utility to provide additional

memory for processing. The following utilities use the /Page_Size

switch:

COMPNET_.EXE

FIXTIME_.EXE

FROMPCB2.EXE

MODLOC_.EXE

MODMOD_.EXE

NCDRILL_.EXE

REANNO_.EXE

See Appendix A in the PC Board Layout Tools 386+ Reference Guide

for information about these utilities and the /Page_Size switch.

------------------------------------------------------------------

Renaming previously installed netlist format files when PCB 386+

is installed

------------------------------------------------------------------

When you install PCB 386+, the install program checks your system

for the presence of the following netlist format files:

EDIF.CF

EDIF.CCF

FEDIF.EXE

You must use the netlist format file supplied with PCB 386+ to

create a netlist that is compatible with PCB 386+. When you

install PCB 386+, the install program renames the currently

installed netlist format files using the convention shown below:

EDIF.CF      ------renamed to------  EDIFCF.OLD

EDIF.CCF     ------renamed to------  EDIFCCF.OLD

FEDIF.EXE    ------renamed to------  FEDIF.OLD

------------------------------------------------------------------

Flipping modules

------------------------------------------------------------------

Coordinates for flipped modules

-------------------------------

When you flip a module, PCB 386+ first flips the module, then

repositions it so it is within positive coordinate space.

PCB 386+ will not allow negative module coordinates because it

disrupts some NC drill machines.

Preventing the shifting of flipped modules

------------------------------------------

When you flip a module to the other side of the board the module

may shift its relative position. The shift occurs because the

pointer becomes the centerpoint for the flip function.

To make sure that the module does not shift, select the module

from its center, if possible. Also, the module will not shift if

you block select the module so it is centered within the block.

------------------------------------------------------------------

Specifying a path for the PCB 386+ template file

------------------------------------------------------------------

In the Miscellaneous Options section of the Configure PC Board

Layout screen, you must enter the complete path and filename for

the PCB 386+ template file, ORCADPCB._T_, if the file is located

in a different directory than your startup design directory.

If ORCADPCB._T_ is located in your startup design directory, you

can enter just the filename. PCB 386+ specifies the startup design

directory path.

------------------------------------------------------------------

Retaining switches for utilities after running FIXDAT

------------------------------------------------------------------

Switches are different for PCB II and PCB 386+ utilities. FIXDAT

brings the old configuration settings forward to your new

ORCADESP.DAT file, but the switches may not be functional with

the new versions of the utilities.

------------------------------------------------------------------

Communicating to invalid devices or ports in PCB 386+

------------------------------------------------------------------

PCB 386+ uses various functions which communicate to devices,

such as floppy drives, network drives, and serial/parallel

communication ports. If the device or port is inactive or

invalid you may encounter problems.

For example, you may direct PCB 386+ to write a file to a floppy

drive that does not contain a floppy disk. You could also direct

PCB 386+ to plot a file to the COM port assigned to your mouse.

If you run PCB 386+ from DOS, an error message displays for

either of these cases. The error message describes the status of

the device, such as "Write fault error writing device COM2." The

error message also displays "Abort, Retry, Ignore, Fail?" as

choices of action. Please choose the action carefully.

Selecting "Abort" causes PCB 386+ to halt operation and return to

DOS without saving the current state of your board. You will need

to reset your computer. If at all possible, do not select Abort.

It is a good idea to save your board file before attempting to

access different devices.

If you can correct the problem, such as placing a floppy in the

empty floppy drive, select "Retry" and the process will continue

normally.

If you cannot correct the problem, such as directing PCB 386+ to

print or plot to the COM port that your mouse uses, select

"Fail." This option halts device access, but PCB 386+ continues

operation, displaying a "Printer output error" message. Your mouse

is disabled, so you will need to press <Enter> to dismiss the

error message. You can use the cursor keys and shortcut keys to

move the pointer. Select the commands to save your board file

and quit PCB 386+, then return to DOS. Reload your mouse driver,

if necessary.

If you run PCB 386+ under Windows 3.1 and perform the same

actions, the "Abort, Retry, Ignore, Fail" error message does not

display. However, the "Printer output error" message does display.

Use the arrow and shortcut keys to save the board file and exit

to the Program Manager, then restart PCB 386+. The mouse will be

restored.

------------------------------------------------------------------

Configuring module libraries

------------------------------------------------------------------

If you do not have any module libraries configured in the

Configured Libraries list box of the Configure PC Board Layout

screen, then all module libraries in the Available Libraries list

box are searched. If a module library is configured in the

Configured Libraries list box, then only the configured library is

searched. If the requested module is not found in the configured

library, no other libraries are searched.

------------------------------------------------------------------

Corrections to the PCB 386+ printed documentation

------------------------------------------------------------------

The following entries are corrections to the PCB 386+ printed

documentation:

PC Board Layout Tools 386+ User's Guide

---------------------------------------

The module footprint for U2, U3, and U4 on DEMO.BD1 and

PLACED.BD1 is incorrect. The part is a 22V10, which should be a

narrower footprint. The incorrect module footprint is also in

TUTOR386.MLB, the module library that is used when you load a

netlist in Chapter 6: Placing the TUTOR board.

On page 120, step 10 reads "Select Begin and move the pointer to

the right to (0.0500", 0.2000")."  The step should read "Select

Begin and move the pointer to the LEFT to (0.0500", 0.2000")."

On page 49, the command line example at the bottom of the page

ends with "Go To Editor." The example should be "GoTo."

PC Board Layout Tools 386+ Reference Guide

------------------------------------------

On page 264, the statement "See Chapter 15: Compare Netlists"

should be "See Chapter 10: Compare Netlists."

On page 264, the statement "See Chapter 19: To Schematic"

should be "See Chapter 13: To Schematic."