Asante 2000 and 2100 series Ethernet Adapter Installation Notes

===============================================================

1. IMPORTANT NOTES

---------------

If you use this adapter card, NE2000.COM is the only driver you need.

2. GENERAL INFORMATION

-------------------

The Asante 2000 series boards contain two jumper blocks to allow the

user to configure the Interrupt Line and Base I/O Address.

During this installation, the installer will prompt different Base I/O

Addresses and Interrupt Lines for your selection. In all cases, the

actual jumper settings on the card (or the previously configured software

settings in the case of a MicroChannel card) and the parameters specified

in the INSTALL program must match or the MLID will not work.

The Asante 2001 and 2101 Ethernet Adapters work with Twisted Pair

Ethernet cabling (connected via an RJ-45 connector) or with an external

transceiver (MAU) connected via a DB-15 AUI connector.

The Asante 2003 and 2102 Ethernet Adapters work with thinwire Ethernet

cabling (connected via a BNC coax connector) or with an external

transceiver (MAU) connected via a DB-15 AUI connector.

For more information about installing and configuring your adapter,

please refer to the manual that comes with your adapter.