Allied Telesis AT-1500T Ethernet Adapter Installation Notes

===========================================================

1. IMPORTANT NOTES

---------------

If you use this adapter card, NE2100.COM is the only driver you need.

2. GENERAL INFORMATION

-------------------

The AT-1500T card uses software settings instead of hardware jumpers

to configure the card. You must run the configuration program that

came with your card in order to set it up properly.

During this installation, the installer will prompt different Base I/O

Addresses and Interrupt Lines for your selection. In all cases, the actual

jumper settings on the card and the parameters specified in the INSTALL

program must match or the MLID will not work.

The Allied Telesis AT-1500T Ethernet Adapter only works on Twisted Pair

Ethernet cabling (connected via an RJ-45 connector).

For more information about installing and configuring your adapter, please

refer to the manual that comes with your adapter.