Dayna Communications Inc's LocalTalk PC Card Installation Notes

===============================================================

1. IMPORTANT NOTES

---------------

To use this MLID, DL2000.COM is the only software driver you need to run

PhoneNET PC in your system.

2. GENERAL INFORMATION

-------------------

The LocalTalk PC Card uses I/O ports and interrupts.  The card does not

use any DMA channels, or shared memory.

The LocalTalk PC Card uses one of 6 I/O ports.  The card's I/O address

can be set by changing the Switch Bank S2.  If you choose to install

this card, the install program will show a picture of the S2 Switch

Bank.  Use the arrow keys to make the screen selection match S2 Switch

Bank on the card.  The install program allows the following possible

values for I/O  ports:

340H, 320H, 2A0H, 260H, 240H, 200H

The LocalTalk PC Card uses one of 3 IRQs.  The card's IRQ number can be

set by changing Switch Bank S3.  The following IRQ numbers are allowed:

3, 4, 5

The LocalTalk PC Card also supports polling method (no interrupt).  It

can be set by switching all switches in Switch Bank S3 to the "Down"

position.

For more details about configuring the LocalTalk PC Card and installing

the driver, please refer to the "LocalTalk Cards User's Guide" that

comes with your adapter.