Eagle/Novell NE1000 Ethernet Adapter Installation Notes

=======================================================

1. IMPORTANT NOTES

---------------

If you use this adapter card, NE1000.COM is the only driver you need.

The INSTALL program does not provide all the possible Base I/O Address

and Interrupt Line settings which are specified in the manual that comes

with your adapter. If you need to use a different setting which is listed

in the manual but not shown during installation procedure, you should

choose any setting and continue with the installation. After the

installation is completed you may edit the NET.CFG file to change the

Interrupt Line and Base I/O Address values to suit your need.

2. GENERAL INFORMATION

-------------------

The NE1000 board contains three jumper blocks to allow the user to

configure Interrupt Line, Base I/O Address and Connector Type.

Note that, in all cases, the actual jumper settings on the card and the

parameters  specified in the INSTALL program must match or the MLID will

not work.

The Eagle/Novell NE1000 Ethernet Adapter will work on either thin Ethernet

cabling (connected via a BNC connector) or thick Ethernet cabling

(connected via a DIX connector).

For more information about installing and configuring your adapter, please

refer to the manual that comes with your adapter.