Ungermann-Bass PC-NIU/EX Ethernet Adapter Installation Notes

============================================================

1. IMPORTANT NOTES

---------------

Before you load the driver UBENODI.COM, you MUST run LOADNIU.EXE and

TCPBIOS.COM that come with the adapter card, otherwise the adapter will

not work correctly.

The file LOADNIU.EXE MAY NOT WORK CORRECTLY if you have some other

software loaded in the DOS HIGH or LOW memory.

Please also check the file CONFIG.SYS that comes with the driver diskette,

and the file CONFIG.SYS that your system is using.

2. GENERAL INFORMATION

-------------------

The PC-NIU/EX is an ISA host interface Ethernet Adapter. It only supports

Thick Ethernet cabling (via a DIX connector).

For more details about configuring the adapter card, please refer to

"PC-NIU/EX User's Guide" that comes with the adapter card.