Farallon PhoneNET PC Card II (ISA) Installation Notes

=====================================================

1. IMPORTANT NOTES

---------------

To use this MLID, PHNETPC2.COM is the only software driver you need to

run PhoneNET PC in your system.

2. GENERAL INFORMATION

-------------------

The Farallon PhoneNET PC Card II (ISA) uses I/O ports and interrupts.

The card does not use any DMA channels or shared memory.

The Farallon PhoneNET PC Card II (ISA) uses one of 6 I/O ports.  The

card's I/O address can be set by changing the Switch Bank S2.  If you

choose to install this card the install program will show a picture of

the S2 Switch Bank.  Use the arrow keys to make the screen selection

following possible values for I/O ports:

340H, 320H, 2A0H, 260H, 240H, 200H

The PhoneNET PC Card II (ISA) uses one of 3 IRQs.  The card's IRQ number

can be set by changing the Switch Bank S3.  The following IRQ numbers

are allowed :

3, 4, 5

For more details about configuring Farallon's PhoneNET PC Card II (ISA)

and installing the driver, please refer to the "PhoneNET PC Card II

User's Guide" that comes with your adapter.