README.DOC                                        updated May 4, 1993

Backup Exec for NetWare v3.1

---------------------------------------------------------------------

*********************************************************************

***  YOU ARE NOT FINISHED INSTALLING YOUR BACKUP EXEC NLM SYSTEM  ***

*********************************************************************

READ SECTIONS 1-3 BELOW OF THIS README.DOC FILE FOR INFORMATION

VITAL TO COMPLETING THE BACKUP EXEC NLM INSTALLATION!!!

PRINT A COPY OF THIS README.DOC FILE, WHICH IS LOCATED IN YOUR

SERVER'S SYS:SYSTEM\BKUPEXEC SUBDIRECTORY.  FOR EXAMPLE:

NPRINT [ insert server name ]/SYS:SYSTEM\BKUPEXEC\README.DOC

CAREFULLY FOLLOW THE INSTRUCTIONS IN SECTIONS 1-3 BELOW TO COMPLETE

THE INSTALLATION PROCESS.

=====================================================================

TABLE OF CONTENTS

=================

Section#  Topic

--------  ------------------------------------------------------

1.   Installation

2.   Novell CLIB.NLM

3.   Server Configuration Issues for Backup Exec NLM

4.   Compatibility information for drives and controllers

5.   Limitations

6.   Commonly asked questions

7.   Files on disks

=====================================================================

1. INSTALLATION

============

The Backup Exec NLM software is installed from a DOS workstation

attached to the server.  The NetWare v3.x servers that can

support Backup Exec NLM are limited to those running version

3.11 or greater of NetWare.

The Backup Exec NLM installation process consists of several

steps:

(1)  Install Backup Exec NLM software from a DOS workstation to

a NetWare v3.11 file server.  You must have SUPERVISOR or

equivalent rights to install the Backup Exec NLM software,

so either LOGIN or ATTACH to the desired file server prior

to starting the Install program.

(2)  Update your server's CLIB.NLM to Novell's rev B

maintenance release that is provided on the Backup Exec

NLM Installation diskette (refer to Section 2 below).

(3)  At the file server console, load the Novell INSTALL.NLM

and make any changes required/desired to the STARTUP.NCF

and AUTOEXEC.NCF files (refer to Section 3 below).

(4)  DOWN the file server and power it off.

(5)  Install any new backup related hardware into the file

server, for example: an Adaptec SCSI controller and an

internal tape drive.

(6)  Turn on the file server and restart.

2. NOVELL CLIB.NLM

===============

The Backup Exec NLM utilizes Novell's CLIB.NLM to process many

of its NetWare specific requests.  A maintenance release

version of CLIB.NLM (provided by Novell) is included on the

Backup Exec NLM Installation diskette.  This maintenance release

includes fixes for MANY problems in previous versions of

CLIB.NLM.  Conner Software STRONGLY recommends that you use

this updated version of CLIB!!!

Updating to the Novell rev B maintenance release of CLIB.NLM:

-------------------------------------------------------------

To determine if you wish to use Novell's updated CLIB.NLM,

examine the NOVREAD.ME file on the Backup Exec NLM diskette

and then follow these steps:

(1)  Determine the "Last Updated" date of the CLIB.NLM

file located on your target file server by entering

the following command from a DOS workstation:

NDIR [ insert server name ]/SYS:SYSTEM\CLIB.NLM

(2)  If the Last Updated date is earlier than 09/04/92,

simply copy the NLMs from the Backup Exec NLM

Installation diskette to the SYS:SYSTEM directory of

the target server by entering the following commands

from a DOS workstation:

FLAG [ insert server name ]/SYS:SYSTEM\CLIB.NLM /N

FLAG [ insert server name ]/SYS:SYSTEM\MATH*.NLM /N

NCOPY A:*.NLM [ insert server name ]/SYS:SYSTEM

FLAG [ insert server name ]/SYS:SYSTEM\CLIB.NLM +RO

FLAG [ insert server name ]/SYS:SYSTEM\MATH*.NLM +RO

Using the original Novell CLIB.NLM:

-----------------------------------

If you DO NOT use the provided rev B maintenance release

version of CLIB.NLM, you MUST ALSO load several

Novell patch NLMs:

PATCH311.NLM

PATCHMAN.NLM

CLIBP002.NLM

FAILURE TO LOAD THESE PATCH NLMS WILL CAUSE YOUR SERVER TO

ABEND!!!

These patch NLMs can be obtained from the NetWire forum on

Compuserve.

If you are not using the rev B maintenance release of CLIB,

you could also see any one of the following problems:

a)   Backup and restore operations are NOT supported for

Macintosh folders/files on remote NetWare servers.

b)   Performing operations on remote Server/Volumes which

have the maximum allowable volume name length

(15 characters and over) will abend the Backup Exec

server and possibly hang and/or abend the Remote

server.

c)   During a backup operation of a volume that supports

the Macintosh name space, the Last Updated time will

be reset to 12:00 a.m. (midnight).

3. SERVER CONFIGURATION ISSUES FOR Backup Exec NLM

===============================================

a. STARTUP.NCF Changes Required:

--------------------------------

Servers with greater than 16 megabytes of memory:

-------------------------------------------------

If you have more than 16 megabytes of memory in your file

server, you will need to increase the number of reserved

buffers that are below the 16 megabyte boundary (NetWare

defaults to 16 of these buffers).  Backup Exec will

allocate its tape buffers from this reserved buffer pool,

which makes it necessary to increase the amount of

reserved buffers.

To increase the number of reserved buffers below 16

megabytes, use the following SET command in your

STARTUP.NCF file:

SET RESERVED BUFFERS BELOW 16 MEG = 200

The server must now be downed and re-started for this

change to take effect.

b. AUTOEXEC.NCF Changes Recommended:

------------------------------------

Since Backup Exec NLM requires loading several NLMs prior

to loading any of the Backup Exec NLMs, you may wish to

modify your AUTOEXEC.NCF file to include the following:

For Adaptec SCSI controller installation:

-----------------------------------------

Load the following NLMs in the sequence specified:

load AHAXXXX   (replace XXXX with either:

1540, 1640 or 1740 depending

on the type of Adaptec

controller installed, the

ASPITRAN.NLM ASPI manager will be

loaded)

load CLIB      (CLIB.NLM will load STREAMS.NLM)

mount all      (mount all NetWare volumes)

load AD_ASPI   (Device driver for Adaptec ASPI

manager)

SPECIAL ADAPTEC CONTROLLER NOTE:  If you are

installing an Adaptec SCSI controller

specifically to support tape drive(s), you must

issue the "MOUNT ALL" command following the

loading of the AHAXXXX driver, and prior to

loading any of the Backup Exec NLMs.

For example:

load aha1740

mount all

load ad_aspi

The Adaptec driver determines the devices

connected to the SCSI bus when the NetWare

volumes are mounted.  If the NetWare volumes are

mounted prior to loading the Adaptec driver(s),

the Backup Exec NLMs will report that no tape

drives can be found connected to the controller.

For Maynard SCSI controller installation:

-----------------------------------------

Load the following NLMs in the sequence specified:

load CLIB      (CLIB.NLM will load STREAMS.NLM)

load MS_SCSI   (Device driver for Maynard SCSI

controller)

c. Backup Exec Automatic Backup NLM

-----------------------------------

You may also wish to load the Backup Exec NLM Automatic

Backup NLM from your AUTOEXEC.NCF file.  Simply include

loading the BkupMgr.NLM after loading AD_ASPI or MS_SCSI,

for example:

load CLIB

load AD_ASPI

load BKUPMGR

4. COMPATIBILITY INFORMATION FOR DRIVES AND CONTROLLERS

====================================================

a.   SCSI Controller Support

Vendor         Model          Notes

------------------------------------------------------------

Adaptec        1510           Must disable Fast File Restore

Exabyte 10i not supported

Adaptec        152x           Must disable Fast File Restore

Exabyte 10i not supported

Adaptec        154x

Adaptec        1640

Adaptec        174x

BusLogic       BT542          Use BT311.DSK dated 02/02/93

BusLogic       BT646          Use BT311.DSK dated 02/02/93

BusLogic       BT747S         Use BT311.DSK dated 02/02/93

DPT            PM2011

DPT            PM2012B

Maynard        16-Bit SCSI    Cipher T860DLT not supported

Maynard        8-Bit SCSI     Cipher T860DLT not supported

UltraStor      14F

UltraStor      24F            Exabyte 10i not supported

------------------------------------------------------------

Additional Controller Support notes:

aa. Adaptec Controllers

Backup Exec NLM v3.1 was compatibility tested with

Adaptec's ASW-1440 Version 3.3 NetWare 386 SCSI

Drivers for AHA-154x/1640/174x.  The specific

driver versions follows:

Driver:        Version:       Module Date:

AHA1510.DSK    1.22           11/13/91

AHA1520.DSK    1.22           11/13/91

AHA1540.DSK    2.22           07/26/91

AHA1640.DSK    2.22           07/26/91

AHA1740.DSK    1.20           07/01/91

ASPITRAN.DSK   1.0            07/31/90

Recommended Drive Selection Settings

------------------------------------

Conner Software recommends that, when used with an

Adaptec controller, the SCSI ID for the tape drive

be set at address 2 for the first tape drive.

Addresses 0 and 1 are reserved for hard disks.

Synch Negotiation Changes Required

----------------------------------

If you are using a ArDAT Python DAT or DAT/DC

tape drive connected to an Adaptec 164x or 174x

you must disable the Adaptec controller's Synch

Negotiation (i.e. the Adaptec controller must not

initiate a synch negotiation).  Use the PS/2 setup

or EISA configuration utility provided with your

system to change this option.

Fast File Restore problem with Adaptec 1510/1520

------------------------------------------------

The Adaptec 1510/1520 controllers were tested with

AHA1510.DSK and AHA1520.DSK drivers both dated

12/03/91.  Unlike the drivers for the 154x, 1640 or

174x controllers, Backup Exec's Fast File Restore

feature will not work with these 1510/1520 drivers.

Until the problem is fixed in the supplied Adaptec

drivers, you must disable Backup Exec's Fast File

Restore feature.  Simply edit the BE.CFG

configuration file located in the SYS:SYSTEM\BKUPEXEC

directory on your server and set the parameter

'Use_fast_file_restore' equal to 0.

Recommendation for loading AHA1510.DSK

--------------------------------------

When loading the AHA1510.DSK driver, Adaptec recommends

that you explicitly specify all of the load options

available, for example:

load aha1510 port=aaa verbose=y fixed_disk=off int=i

id=z sync=off

where aaa = port address of 1510 controller card

i   = interrupt setting of 1510 card

z   = SCSI address of the 1510 card

bb.  BusLogic Controllers

Backup Exec NLM v3.1 was compatibility tested with

the following BusLogic SCSI drivers:

Driver:        Version:       Module Date:

BT311.DSK      2.20           02/02/93

BTASPI.DSK     1.0            07/23/92

Using a BT311.DSK dated earlier than indicated

can cause the Backup Exec NLM application to "hang"

during its initialization process.

cc.  DPT Controllers

Backup Exec NLM v3.1 was compatibility tested with

the following DPT SCSI drivers:

Driver:        Version:       Module Date:

PM12NW31.DKS   1.82           02/03/93

PM11NW31.DKS   1.80           05/11/92

DPTDDL.NLM     1.80           05/11/92

The PM2012B and PM2011 controllers were tested with

"3G" firmware and "1J9" BIOS ROM installed.

dd.  UltraStor Controllers

Backup Exec NLM v3.1 was compatibility tested with

the following UltraStor SCSI drivers:

Driver:        Version:       Module Date:

U14_31X.DSK    1.05           01/31/92

U24_31X.DSK    1.01           01/27/92

ASPITRAN.DSK   1.0            07/31/90

b.   SCSI Device Support

Vendor    Drive                    Firmware Certified

------------------------------------------------------

Archive   Viper 250                21247-011

Archive   Viper 2525               -003, -007

Archive   Anaconda 2750            5.06

Archive   Anaconda 2800            ANCDA 2800 28889-900

Archive   Python DAT               1.36, 2.19, 2.66, 2.96

Archive   Python DAT DC            3.58

Archive   Python Turbo DAT DC      28388-XXX 4.28

Archive   Diamondback DAT Loader   4.52 with 2.93D EPROM

Cipher    T860DLT                  430A

Teac      600                      191C, 191D, 239E

Exabyte   8200SX                   2638

Exabyte   8500                     0340, 0415

Exabyte   10i Loader w/8500        2.10 (loader)

Exabyte   4200 DAT                 107

HP        DAT 3547                 7.09

HP        DAT DC 3548              7.09

WangDAT   DAT 3100                 2.0

WangDAT   DAT DC 3200              2.0

------------------------------------------------------

Additional SCSI Device Support notes:

aa.  Archive Python DAT

DO NOT attempt to access a tape created by an Archive

Python DAT DC if your Python DAT drive has firmware

versions 1.36 or 2.19!  In this situation, the Python

DAT drive will appear hung for up to 10 minutes before

returning control to Backup Exec NLM.  This problem

was fixed in the 2.66 firmware revision.

bb.  The WangDAT drives have a switch which enables/disables

Software Eject.  It is set to "DISABLE" by default.  If

the feature is "ENABLED", Backup Exec's Utility "Eject

Tape" feature will operate as expected.  If this switch

is left in the disable position, Backup Exec will only

rewind the tape if the "Eject Tape" function is

selected.

cc.  If you are using an Archive Diamondback DAT Loader,

make sure that a magazine is loaded in the drive prior

to starting BkupMgr.NLM or BkupExec.NLM.  This loader

supports magazines with 4, 8 or 12 tapes inserted and

Backup Exec determines the number of tape slots

supported during initialization.  If a magazine is

not inserted in the drive when Backup Exec initializes

the software will report that no tape drives have been

found.  If this happens, simply exit Backup Exec,

insert the magazine in the Diamondback and re-load

Backup Exec.

5. LIMITATIONS

===========

a.   If you are running Backup Exec NLM with an Adaptec SCSI

controller and the tape drive is powered off during an

operation, you may need to DOWN your file server to

re-use the Backup Exec NLM.  Adaptec's ASPI manager does

not correctly handle this problem and may result in the

Backup Exec NLM appearing to be "hung" trying to complete

an operation.  The file server will continue to function

properly, but you will need to DOWN your file server and

re-boot in order to use Backup Exec NLM again.

b.   The backup performance of Backup Exec NLM suffers greatly

if you attempt to back up a single subdirectory on a remote

server that contains a large number of files (i.e. over

1000).  This problem is due to Novell's CLIB.NLM.

c.   If you attempt to use automatic login on a remote server

with an invalid username, an error will be displayed and

the selected server will disappear from the selection list.

Simply press the <ESC> key twice to force an update of the

device selection list.  The desired server will reappear in

the list, and you can re-attempt the login with a valid

username.

d.   Backup Exec NLM is not able to restore the bindery files

to remote NetWare v3.0 servers.

e.   If you have issued the SECURE CONSOLE command at the

server, the Backup Exec NLM software will not be able to

load the tape driver. This will be fixed in a future

release.  However, you can avoid this problem by using

the MONITOR NLM to lock the console instead of using the

SECURE CONSOLE command.

6. COMMONLY ASKED QUESTIONS

========================

Q:   Whenever I attempt to perform a tape operation, Backup

Exec reports an "Out of Memory" error.  In addition, my

file server reports:

"[date time] 1.1.38 Cache memory allocator exceeded

minimum cache buffer left limit"

on the system console.  Why is this and what can I do to

fix it?

A:   If you have more than 16 megabytes of memory in your file

server, you will need to increase the number of reserved

buffers that are below the 16 megabyte boundary (NetWare

defaults to 16 of these buffers).  If your server is

configured with more than 16 megabytes of memory,

Backup Exec will allocate its tape buffers from this

reserved buffer pool, which makes it necessary to increase

the amount of reserved buffers.

To increase the number of reserved buffers below 16

megabytes, use the following SET command in your STARTUP.NCF

file:

SET RESERVED BUFFERS BELOW 16 MEG = 200

The server must now be downed and re-started for this change

to take effect.

Q:   Why would loading the Backup Exec NLM, slow my server down

more than I'd expected?

A:   NetWare supplies memory for all NLMs from a pool of file

cache buffers.  If you are loading several NLMs on your

file server, your server could reduce the number of

available file cache buffers to its minimum (the default

is 10) greatly reducing server performance.  To protect

the server's performance, use the following SET command

at the fileserver console:

SET MINIMUM FILE CACHE BUFFERS = XXXX

where XXXX is a number from 10 to 1000.  Setting this

value will cause the server to report an out of memory

error when any NLM requests memory that would allocate

from the minimum file size cache.  REMEMBER, use of the

Backup Exec NLM requires ONE MEGABYTE of memory above your

normal operating environment.

Q:   My server constantly reports:

"[date time] Primary Interrupt Controller Detected

a lost Hardware Interrupt"

when I run Backup Exec NLM.  All server and Backup Exec

functions appear to be working correctly. What does this

mean?

A:   This message indicates that the server "thinks" that a

defective controller card is installed in the system, or

that there is an interrupt conflict in the system.

Verify that there are no interrupt conflicts in the system

and configure Backup Exec's settings in the

Options\Hardware\Controller window with specific I/O

Address, Interrupt Number and DMA Channel values instead of

auto-determining these settings.  If the above message

persists, this message can be suppressed by using the

following SET command at the fileserver console:

SET DISPLAY LOST INTERRUPT ALERTS = OFF

Q:   Backup Exec NLM takes a long time to backup a large number

of files from a single directory of a remote server.

Why is this?

A:   Because of a limitation of Novell's CLIB.NLM, the remote

server does not negotiate for large disk transfers of

large directory structures.  The effect on Backup Exec is

that performance suffers when backing up remote servers

with a large number of files in a single directory (see

LIMITATIONS #d above).  The easiest approach to working

around this CLIB.NLM limitation is to split up backups

of problem directories into smaller backups using scripts.

Q:   When I try to use RCONSOLE to control Backup Exec NLM

remotely, I can't use the <+> or <-> keys for Quick File

selections.  Instead of producing the include/exclude

windows, my screen scrolls through the server's console

screens.  How can I create Quick File selections within

Backup Exec while using RCONSOLE?

A:   Novell's RCONSOLE reserves the <+> and <-> keys on the

numeric keypad for stepping through the server's console

screens.  Use the <+> and <-> keys on the main keyboard

to make Quick File selections from within Backup Exec NLM.

Q:   When I attempt to backup my remote servers, I always get

an error message stating that the password supplied does

not match.

A:   This message indicates that Backup Exec NLM can not make

a successful attachment to the remote server and may occur

with the earlier version of Novell's CLIB.NLM.  Check the

AUTOEXEC.NCF file on the local server for the set command:

SET REPLY TO GET NEAREST SERVER = OFF

The default for this set command is ON.  If you have

disabled this option and are running with the earlier

version of Novell's CLIB.NLM, Backup Exec NLM is unable to

attach to remote servers if this option is disabled.  To

correct for this condition, either use the rev B

maintenance release of CLIB.NLM or do not disable this

option.

Q:   I was running the Backup Exec NLM and an "Assertion Failed"

error message similar to this appeared:

Assertion Failed in xxx.c

<Press any key to close window>

What does this mean, and why did it happen?

A:   Backup Exec NLM continuously checks the integrity of the

memory allocated to it.  If Backup Exec finds a memory

corruption situation that could cause the server to abend

(possibly due to another NLM modifying Backup Exec's memory),

the above message is displayed and the Backup Exec NLM is

unloaded.  You need to down the server and re-boot to clear

the condition.  Use <Alt+Esc> to return to your system

console ":" prompt and down the file server.  Bring the

server back up and re-enter the Backup Exec NLM software.

7. FILES ON DISKS

==============

This section outlines the files that can be found on each of

the Backup Exec NLM Installation diskettes.

Backup Exec NLM v3.1 3.5" 1.44MB and 5.25" 1.2MB Diskettes

----------------------------------------------------------

INSTALL.COM         Backup Exec Install program

INSTALL.RES         Resource file for the Install program

_NSTL.EXE           Core Installation program

README.DOC          This file

NOVRDME.DOC         Readme document regarding updated Novell CLIB

FIRMWARE.DOC        Readme file regarding firmware compatibilities

PKUNZIP.EXE         Data extraction utility

@568D.ADF           ADF for Maynard Microchannel SCSI adapter

CLIB.NLM            CLIB.NLM rev B maintenance release

MATHLIB.NLM         MATHLIB.NLM rev B maintenance release

MATHLIBC.NLM        MATHLIBC.NLM rev B maintenance release

BEDISK.1            Compressed data file for software

BE.REQ              Installation control file

1DISKSEQ            Installation control file

BEDISK.END          Installation control file