IBM PS/2 ETHERNET LAN ADAPTER DRIVER

FOR USE WITH DEC's DECnet-DOS and PCSA/PC

INSTALLATION GUIDE

The name of the DECnet device driver for the IBM Ethernet adapters is

IBMDLL.EXE.  This device driver is located in the DECNET directory of

the Ethernet option diskette.  The device driver operates with DEC's

DECnet-DOS or PCSA/PC software.  The device driver is installed after

the system has been configured and DOS has been loaded.

This document describes how to install the IBM Personal System/2

Adapter for Ethernet Networks device driver for a DECnet network.

1. Set the adapter jumpers to match your system configuration.

For Micro Channel adapters, the interrupt, I/O base address,

and RAM base address are established by the system configuration

procedure when the adapter is installed in the machine.

The IBMDLL device driver is installed during power-on according to

the command line specified in the AUTOEXEC.BAT file (described

below).  The user must check that the software settings given on

the device driver command line match the configuration settings

of the adapter.  For more information regarding configuration settings,

refer to the installation guide that comes with each adapter.

2. Insert the adapter in your computer.

3. Turn on computer.

4. Install DEC's software

Follow the appropriate instructions for either DECnet-DOS or

PCSA/PC from Digital Equipment, selecting the "3COM"  option

whenever asked to choose an adapter type.  Specifics for the

hardware are defined below.

Usage of IBMDLL parameters is as follows:

AT bus adapters:

ibmdll [/irq:x] [/io:xxx] [/sm:xxxx] [/size:xx] [database_dir]

Micro Channel adapters:

ibmdll [/slot:x] [database_dir]

All adapters:

ibmdll /u

irq:x   Specifies IRQ level in decimal

(default 3)

io:xxx  Specifies hex I/O base address

(default 280h)

sm:xxxx Specifies hex Shared Memory segment

(default D000h)

size:xx Specifies Shared Memory size in decimal

Kilobytes (default 16)

slot:x  Specifies slot number of adapter (defaults

to slot number LAN adapter is installed in

if only one is installed in machine)

database_dir

Pathname of directory where you can find

DECPARM.DAT file (defaults to \DECNET\

on the current drive)

/u      Unload a previously running copy of dll

4A. For DECnet-DOS: Install the DECnet-DOS network software

using the DIP.EXE installation program supplied on the

DECnet-DOS diskette.  Select ETHERNET communications during the

installation.

Next, copy the IBM PS/2 Ethernet adapter driver IBMDLL.EXE from

this diskette to the \DECNET directory of your system drive.  For

example,

COPY A:\DECNET\IBMDLL.EXE C:\DECNET\IBMDLL.EXE

Next, edit your AUTOEXEC.BAT file to load the IBMDLL driver as

described in step 5.

4B. For PCSA/PC: Have your system administrator create a KEY

diskette for your system.  The KEY diskette should be created

with your DECnet address and your personal startup files.

Next, copy the IBMDLL.EXE from this diskette to the appropriate

directory of your KEY diskette.  For example, with your KEY diskette

in drive A: and this diskette in drive B:, type:

COPY B:\DECNET\IBMDLL.EXE A:\xxxxxx\IBMDLL.EXE

Next, edit the file MSNET.INI on your KEY diskette as described in

step 5A or 5B.  (Make sure to change ALL occurrences of IBMDLL.)

5A. AT bus adapters: You must edit your AUTOEXEC.BAT or

MSNET.INI files to load the IBMDLL.EXE driver and to reflect the

jumper or configuration selections of step 1.  Use the format:

C:\DECNET\IBMDLL /IRQ:irq /IO:base /SM:page /SIZE:size (AUTOEXEC.BAT)

or

IBMDLL /IRQ:irq /IO:base /SM:page /SIZE:size (MSNET.INI)

where,

irq- IRQ level to receive adapter interrupts in decimal, jumper W1 [3 is

factory default)

base- I/O base address in hexadecimal, jumper W1 (280 is factory

default)

page- 4-digit hexadecimal memory segment byte address of the

shared buffer memory, jumper W1 (D000 is factory

default)

size- size of the shared memory on the adapter in decimal (16 is

default)

The actual IBMDLL.EXE defaults are the same as the adapter defaults

shown in parentheses.

For example, the following statement corresponds to the default

factory configuration of the adapter for base I/O and IRQ, and a

16K buffer memory base address of DE000 (which is not the default):

IBMDLL /IRQ:3 /IO:280 /SM:DE00 /SIZE:16

To change the settings from the command line for version 1 of

DECnet-DOS, you may use the version 1 format using "=" instead of

the version 2 format using ":" as the delimiter.  For example,

IBMDLL /IRQ=3 /IO=280 /SM=DE00 /SIZE=8

or

IBMDLL /IRQ:3 /IO:280 /SM:DE00 /SIZE:8

5B. Micro Channel Bus adapters: You must edit your AUTOEXEC.BAT

or MSNET.INI files to load the IBMDLL.EXE driver. Check that the

automated installation procedure has not added command line options

which are not needed for the Micro Channel version.  Use the format:

C:\DECNET\IBMDLL        (AUTOEXEC.BAT)

or

IBMDLL                  (MSNET.INI)

For Micro Channel adapters, IRQ, IO, SM, and SIZE should not be

specified.  If specified, these parameters will be ignored and an

error message will be generated).  The actual values used are those set

by the system configuration procedure for the adapter.

If you have more than one IBM PS/2 Ethernet Micro Channel LAN

adapter installed in your system, you must tell IBMDLL which one to

use.  The adapter is referenced by the slot in which it is installed,

using the /SLOT parameter.

For example, if you have an IBM Ethernet adapter in slot 5, and

another IBM Ethernet adapter in slot 3, and you wish to run DECnet

using the adapter in slot 5, use:

IBMDLL /SLOT:5

Even though you can specify SLOT when a single adapter is installed,

you should let SLOT default.  Then if you later move the adapter to

another slot, you will not need to change AUTOEXEC.BAT or MSNET.INI.

6. Restart the system.