CONFIGURATION PARAMETERS

ODI2NDI has no command line parameters but supports three

NET.CFG keywords and five PROTOCOL.INI keywords. NET.CFG

is the configuration file for the Netware Requestor envi-

ronment and PROTOCOL.INI is the configuration file for

the NDIS environment.

The supported NET.CFG keywords are:

o   NODE ADDRESS

o   FRAME

o   PROTOCOL

The supported PROTOCOL.INI keywords are:

o   DRIVERNAME

o   BINDINGS

o   NETADDRESS

o   TOKEN-RING

o   TOKEN-RING_SNAP

Perform the following steps to manually install the ODI2NDI

mapper for OS/2

1) Create an ODI2NDI directory on your workstation. Copy all the

files from the ODI2NDI\OS2 directory on the option diskette

to the ODI2NDI directory on the workstation.

2) The following updates must be made to config.sys:

a) Add the ODI2NDI.OS2 DEVICE= statement after the Netware

Link Support Layer(LSL.SYS) device driver, but before

the IPX(IPX.SYS) device driver.

DEVICE=*:\ODI2NDI\ODI2NDI.OS2

b) Make sure the PROTMAN.OS2 DEVICE= statement appears

prior to the LSL.SYS DEVICE= statement.

c) If you are using token ring and want source routing support,

add Netware's ROUTE.SYS device driver after the ODI2NDI.OS2

device driver but before IPX.SYS.

d) If not already present, add the appropriate NDIS MAC

driver for the LAN adapter, for example IBMTRDB.OS2.

The NDIS MAC driver can be loaded anytime after

PROTMAN.OS2 is loaded.

3)  ODI2NDI.OS2 has its own section in PROTOCOL.INI.  It's section

is like any NDIS protocol stack section, it has the following

keywords:

"DRIVERNAME ="  - required parameter, must be set to ODI2NDI$

"BINDINGS ="    - required parameter, must be set to the

bracketed section name of the MAC ODI2NDI

will bind to.

"NETADDRESS="   - required parameter, a 12 digit hex number

specifying the LAN adapters universal or

locally administered address.

"TRACE="          optional parameter, 4 digit hex number

specifying trace event activation for

problem determination.

In addition to the above keywords, there are two keywords that

describe which frame header ODI2NDI will build.  The default

frame type keyword if none is specified is "TOKEN-RING"

regardless of actual LAN type.  The six frame type keywords

are:

TOKEN-RING =

TOKEN-RING_SNAP =

To select a particular frame type set it equal to "yes". Defaults to Token-Ring.

Example:

[ODI2NDI_nif]

DriverName = ODI2NDI$

Bindings = IBMTRDB_NIF

NETADDRESS = "4000xxxxxxxx"    <---- This is the universal or local adapter address

TOKEN-RING = "yes"             <---- This is the active frametype

TOKEN-RING_SNAP = "no"

TRACE = 0x0                    <---- This is optional keyword for activating

trace events.

4)  ODI2NDI supports three NET.CFG keywords NET.CFG is the configuration file

for the Netware Workstation environment.

The supported NET.CFG keywords are:

o   NODE ADDRESS       ; must be configured

o   FRAME              ; defaults to TOKEN-RING

o   PROTOCOL           ; defaults to IPX

ODI2NDI reads the NET.CFG file first, and then reads the

PROTOCOL.INI file. The keywords in the PROTOCOL.INI will

override the keywords in NET.CFG. The NET.CFG file and

keywords are optional. The PROTOCOL.INI file is required.

There must be an ODI2NDI section, a DRIVERNAME and a

BINDINGS keyword. The instructions that follow will detail

the necessary configuration information.

A locally administered address must be configured for the

ODI2NDI mapper for OS/2. This address can be configured

through either the NODE ADDRESS keyword in the NET.CFG

file or through the NETADDRESS keyword of the PROTOCOL.INI

file. All NDIS Protocol Stacks that bind to the same NDIS

MAC Driver as the mapper will also need to be configured

for this same LAA to function properly.

Example:

LINK DRIVER macname            ; macname is the bracketed section name ODI2NDI will

bind to.

NODE ADDRESS 4000xxxxxxxx

FRAME TOKEN-RING