DigiWARE Release Notes P/N 93000114H

for

X* DOS Driver V 4.1.0

Operating System Release  2.0.0 - 6.0

Date 03-11-94

1.  Enhancements

- The configuration utility, xidoscfg.exe will add the

proper device= statement to the config.sys file.

2.  Bug Fixes

- The "LEAVE MEMORY ENABLED" default option has been changed to

leave memory disabled after xidos5.sys is loaded.

- Fixed bug in xidoscfg.exe.  During the configuration of

MC2/Xi card, the memory size indicated was always 64k,

regardless of the Jumper (J1) setting.  The memory size

will now be reported correctly (either 32K or 128K).

- Fixed big in xidos5.sys.  The initialization of channel

parameters had been transposed, such that channel 1 would be

initialized with channel 8 parameters and channel 2 with

with channel 7, etc.  As long long as all channels had

the same parameters, this would have no effect.

============================================================================

DigiWARE Release Notes P/N 93000114H

for

X* DOS Driver V 4.0.9

Operating System Release  2.0.0 - 6.0

Date 09-01-93

1.  Enhancements

-  Expanded EBIOS support in INT 14h Function 0h, Initialize a

Port. See Manual Errata.

2.  Limitations

-  The driver's INT 14h interface is not 100% compatable with the

the following DOS drivers and their supported boards:

xapcm232.sys - PC/X*, xim232mc.sys - MC/Xi, and xim232.sys -

COM/Xi. If you are using an INT14h application for an earlier

driver, compare the driver's INT 14h specifications to

determine if any of the incompatabilities may affect your

application.

3.  Bug Fixes

-  Fixed bug in xidoscfg.exe. On the MC/2e board, when the IRQ

was disabled using the POS configuration utility, xidoscfg.exe

would display an erroneous IRQ number on the driver config-

uration screen.

4.  Manual Errata

-  Please note that the DOS driver files are no longer found in

the root directory of the release diskette, but located in

the DOS directory. Any manual references to the files being

located in the root directory is in error.

- Expanded EBIOS Support

Under the EBIOS specification, the highest baud rate

accessible is 19200. The function was enhanced to provide

access to 38400, 57600 and 115200 baud rates. For the

programming interface see INT14.TXT. This enhancement was

made to permit access to higher baud rates from INT 14h

based communication programs that only provided selections

of up to 19200. The driver was changed so that when EBIOS

support is selected from the XIDOSCFG.EXE configuration

utility, the following baud rate substitutions are made :

110      ->     19200

150      ->     19200

300      ->     38400

600      ->     57600

1200      ->    115200

2400      ->      2400

4800      ->      4800

9600      ->      9600

So, for example, to access 57600 support, select EBIOS

support in XIDOSCFG.EXE and select 600 baud in your

communication program.