DigiWARE Release Notes P/N 93000779G

for

NetWare 3.X AIO Device Driver V 1.0.6

Software Manuals   P/N's 92000115C & 92000139A

Date 09-17-93

1.  Enhancements

- The Transmit buffer of the PC/16i and PC/16e has been

increased from 1023 bytes to 1024.

2.  Limitations

- The AIO driver supports a maximum of 7 PC/X* boards in ISA

and EISA computers. These boards may be configured with 64k

or 128k of dual-ported RAM and may reside at host memory

addresses 80000h, 90000h, A0000h, B0000h, C0000h, D0000h or

E0000h.

- The driver supports a maximum of 4 32k MC/Xi boards in MCA

computers. These boards may reside at host memory addresses

C0000h, C8000h, D0000h, D8000h.

- The driver also supports up to 2 64k or 7 8k MCA 2Port ( MC/2e )

boards between C0000h and DFFFFh.

- The following baud rates are not supported : 2000, 3600 and

7200 baud.

- Mark and Space Parity are not supported.

- The Deadman Timer function is not supported.

- Adjustabled buffer sizes are not supported.

- While hardware handshaking is active, RTS can not be overridden.

3.  Bug Fixes

- Flow control bugs found by the new Novell IMSP testing

software have been fixed.

- The notSupportedMask has been changed to support the

externalControl variable.

- The AIOPORTCAPABILITES structure has been fixed.

- Fixed bug in numbering boards in a MCA system with mixed

MC/2e and MC/Xi boards.

4.  Manual Errata

-  Refer to AIODGXI.DOC for updated information.