DigiBoard

6400 Flying Cloud Drive

Eden Prairie, MN 55344

(612) 943-9020

Installation Guide

and

Reference Manual

for

DigiCHANNEL PC/Xem Novell AIO Driver

DBI P/N 92000115B

DigiBoard, DigiCHANNEL, PC/Xem is a trademark of DigiBoard.  NetWare,

NetWare Access and Novell are registered trademarks of Novell, In-

corporated.  All other brand and product names are the trademarks of

their respective holders.

Copyright (c) Digi International, Inc 1993

All Rights Reserved

Information in this document is subject to change without notice

and does not represent a commitment on the part of DigiBoard.

DigiBoard provides this document "as is", without warranty of any

kind, either expressed or implied, including, but not limited to,

the particular purpose.  DigiBoard may make improvements and/or

changes in this manual or in the product(s) and/or the program(s)

described in this manual at any time.

This product could include technical inaccuracies or typographical

errors.  Changes are periodically made to the information herein;

these changes may be incorporated in new editions of the publication.

RESTRICTED RIGHTS:

For non-U. S. Government use:

These programs are supplied under a license.  They may be used,

disclosed, and/or copied only as permitted under such license

agreement.  Any copy must contain the above copyright notice and

this restricted rights notice.  Use, copying, and/or disclosure

of the programs is strictly prohibited unless otherwise provided

in the license agreement.

For U.S. Government use:

Use, duplication, or disclosure by the Government is subject to

restrictions as set forth in sub-paragraph (c)(1)(ii) of the

Rights in Technical Data and Computer Software clause of DFARS

52.227-7013.

1.0  Introduction

1.1  Overview

DigiBoard's support for Novell compatible asynchronous

applications is provided through a universal driver

conforming to the "AIO" (Asynchronous Input/Output)

specification.  Use the AIO driver when DigiBoard hardware

is being installed in the file server (or machine with run-

time NetWare 3.X).  When DigiBoard hardware is installed in

a machine other than the server (for instance NetWare Access

Server v1.2) use the specific driver for that application.

AIO drivers are loaded on the file server from the system

console (or RCONSOLE) command line or the AUTOEXEC.NCF file.

Installation is simply a matter of copying of the driver

files AIODGXEM.NLM and AIODGMEM.NLM.

1.2  Supported Boards

The DigiBoard AIO driver supports up to eight EISA boards, seven

ISA boards or four MCA boards, each supporting up to 4 EBI modules

for a maximal driver configuration of 512 ports.

1.3  Unsupported Hardware Features

The driver does not use interrupts from the DigiChannel PC/Xem.

When configuring the MCA Xem using the POS configuration utility

the board's 'Interrupt Vector' should be set to 'disabled'.

2.0  Installation

To install the DigiBoard AIO driver, simply copy the relevant

files to your file server (although the driver can be loaded

from the file server floppy, this will not be a typical way to

set-up a system).  The DigiBoard AIO driver files (AIODGXEM.NLM

and AIODGMEM.NLM) should both be copied to the system directory

of the file server, typically called F:\SYSTEM.  Copy the files

from a workstation to the file server with the commands:

COPY A:\AIODGXEM.NLM F:\SYSTEM

and

COPY A:\AIODGMEM.NLM F:\SYSTEM

2.1 Loading the Driver

The user supplies information to the driver through a set of command

line parameters at load time. Some or all of the parameters are

optional, depending on the type of board you are using. On ISA and

EISA bus servers, the driver must be loaded for each board. On MCA

bus servers, the driver may be loaded for each board individually or

it may be loaded once to support all boards in the server. The load

command has the following general format :

load 'drivername' { necessary and optional parameters }

The specific format depends on the type of PC/Xem and/or the bus

type of the computer.

The command line syntax for the driver supporting these boards

on ISA, EISA and MCA machines is shown below. Parameters specified

in brackets ( [] ) are optional.

ISA boards in ISA machines :

load aiodgxem port=xxx mem=xxxxx [name=bname] [node=0]

[altp[=mask]]

ISA boards in EISA machines :

load aiodgxem slot=x port=xxx mem=xxxxx [name=bname]

[node=0] [altp[=mask]]

EISA boards in EISA machines :

load aiodgxem slot=x mem=xxxxx [name=bname] [node=0]

[altp[=mask]]

MCA boards :

load aiodgxi [port=xxxx mem=xxxxx [name=bname]] [node=0]

altp[=mask[,mask[,mask[,mask]]]]]

When the 'port=' and 'mem=' parameters are not used, the

driver will load support for all boards found in the system

at supported memory addresses.

A description of the parameters follows.

port=xxx

This parameter is the board's 3 digit hex I/O port address.

For example: port=324.

When used with MCA boards, the 'mem=' parameter must also be

used.

slot=x

This parameter is the board's EISA slot number.

For example: slot=3.

mem=xxxxx :

This parameter is the 5 digit hex starting address of the

board's dual-ported memory.

For example: mem=d0000.

When used with MCA boards, the 'port=' parameter must also be

used.

name=bname :

This parameter is optional and permits specification of a 10

character name to identify the board.  bname may consist of

any combination of alphanumerical characters.  If no name is

specified, one will be assigned by the driver. Following is a

list of the board's I/O port addresses or slot number and the

associated default names and board numbers.

For boards in ISA machines :

Port         Default     Board

Address      Name        Number

-------      --------    ------

104h         DigiXem00     0

114h         DigiXem01     1

124h         DigiXem02     2

204h         DigiXem03     3

224h         DigiXem04     4

304h         DigiXem05     5

324h         DigiXem06     6

For boards in EISA machines :

Slot         Default     Board

Number       Name        Number

-------      --------    ------

1          DigiXem00     0

2          DigiXem01     1

3          DigiXem02     2

4          DigiXem03     3

5          DigiXem04     4

6          DigiXem05     5

7          DigiXem06     6

8          DigiXem07     7

For MCA boards :

Port         Default     Board

Address      Name        Number

-------      --------    ------

108h         DigiXem00     0

118h         DigiXem01     1

128h         DigiXem02     2

208h         DigiXem03     3

228h         DigiXem04     4

308h         DigiXem05     5

328h         DigiXem06     6

node=0 :

This parameter must be used if you are using AIO.NLM version

1.0.2 or earlier. These versions of AIO need sequential board

numbers based on the load order of the driver. (The first board

loaded will be assigned #0, the second board will be assigned

#1 and so on.) Using the node=0 parameter informs the driver

to assign board numbers sequentially starting at 0. If this

parameter is excluded, the driver will assign board numbers

based on the I/O port address as defined above. The parameter

is effective only when used on the first driver load and sets

the method by which subsequently loaded boards will be numbered.

altp[=mask]:

The standard wiring for the RS-232 RJ-45 connectors found on the

Xem modules places the modem control lines RI (Ring Indicator)

and DCD (Data Carrier Detect) on pins 1 and 10, respectively,

which makes them inaccessible to an 8-pin RJ-45 plug.  While RI

is seldom used, DCD is necessary for most modem applications.

To facilitate the use of 8-pin RJ-45 plugs, a software feature

called "ALTPIN" is supported in this device driver. When ALTPIN

is active, DCD is swapped with DSR (Data Set Ready), which is on

pin 1 of an 8-pin RJ-45 plug. To take advantage of this option,

the cabling must route the DCD line to pin 1.

This parameter is optional and allows you to specify that you

want to activate the driver's ALTPIN option. When this parameter

is entered as just "altp", all ports on the board will operate

with the ALTPIN option active. When entered as "altp=mask" the

driver uses the hex mask to determine which ports will use the

ALTPIN feature.

For example : altp=f0

F0 (hex) = 11110000 (binary)

||||||||

|||||||+----Port 1

||||||+-----Port 2

|||||+------Port 3

||||+-------Port 4

|||+--------Port 5

||+---------Port 6

|+----------Port 7

+-----------Port 8

This will activate ALTPIN on ports 5-8.

The mask can be up to 16 hex digits long to activate ALTPIN on

up to 64 ports.

altp[=mask[,mask[,mask[,mask]]]]] :

When autoloading support for all boards in a MCA server, four

masks may be specified, one for each possible board.

For example:

altp=1,8,10,80

1 (hex) = 00000001 (binary)

8 (hex) = 00001000 (binary)

10 (hex) = 00010000 (binary)

80 (hex) = 10000000 (binary)

||||||||

|||||||+----Port 1

||||||+-----Port 2

|||||+------Port 3

||||+-------Port 4

|||+--------Port 5

||+---------Port 6

|+----------Port 7

+-----------Port 8

In this example the first port on the first board,

the fourth port on the second board, the fifth port

on the third board and the eighth port on the fourth

board will use ALTPIN.  When autoloading, the driver

uses the MCA bus's POS (Programmable Option Select)

capabilities to determine the MC/Xi boards that are

installed in your server, starting with slot 1.  The

first ALTPIN mask will activate ALTPIN on the board

in the lowest numbered slot, the second mask for the

next lowest slot, and so on.

IMPORTANT!  IRQs on MC PC/Xem must be set to 'Disabled' using the

POS configuration utility found on the reference disk-

ette.

3.0  Error Messages

The following error messages may occur while the driver is

parsing the parameters.

*Usage Error: AIODGXEM port=xxx mem=xxxx [name=bname] [node=0].

*Usage Error: AIODGXEM slot=x [port=xxx] mem=xxxx [name=bname] [node=0].

These errors will appear if you load the driver with no

parameters on ISA and EISA bus servers, respectivley.

*Usage Error: First parameter not 'port='.

The driver expects the first parameter to be port=xxx on ISA machines,

but this was not the case.

*Usage Error: First parameter not 'slot='

The driver expects the first parameter to be slot=x on EISA machines,

but this was not the case.

*Error: Invalid Port Address 0x310.

This message indicates that an invalid port address was specified.

*Error: Port Address 0x310 is already supported.

This  message indicates that the driver is already supporting a Digi-

CHANNEL board at this port address.

*Error: Invalid Slot 10

This message indicates that an invalid slot number was specified while

loading the driver on a EISA computer (slots 1-8 are valid).

*Error: Slot 2 is already supported.

This  message indicates that the driver is already supporting a Digi-

CHANNEL board in this slot on an EISA computer.

*Usage Error: Second parameter not 'mem='

The driver expects the second parameter to be mem=xxxxx in ISA

machines, but this was not the case. The driver also expects this

to be the second parameter in EISA machines if the 'port=' parameter

was not the second parameter (when using an EISA board).

*Usage Error: Third parameter not 'mem='

The driver expects the third parameter to be mem=xxxxx in EISA machines

if 'port=' was the second parameter (specified for ISA boards), but

this was not the case.

*Error: Invalid Memory Address 0xF0000.

This message indicates that an invalid memory address was specified.

*Error: Memory Address 0xC0000 is already supported.

This message indicates that the driver is already supporting

a DigiCHANNEL board at the specified memory address.  This message

will only be seen on MCA bus servers since boards cannot be 'stacked'

at the same memory addresses in these machines.

*Error : Failed POS Hardware Registration.

This message indicates that the MCA POS register registration was

not accepted by Netware.

*Error: AllocateResourceTag failed.

This message indicates that the driver could not allocate resources

from Netware. Unload unnecessary NLM's and reload the driver.

*Error: AIORegisterDriver failed.

This message indicates that Netware failed to permit registration of

the driver.

*Error: AllocateSemiPermMemory returns NULL.

This message indicates that the drivers request to allocate memory

failed.  Unload unnecessary NLM's and reload the driver.

No usable boards found. / Installed boards already supported.

This message appears only on MCA servers and indicates that no boards

were found when POS was checked, or that the driver is already support-

ing the board(s) that were found.

The following error messages may occur while the driver is

performing some initial tests on the DigiCHANNEL board.

Board 4: Port 310 Mem D0000 Failed Hardware Registration.

This message indicates that Netware failed to accept the driver's

attempt to register the board's hardware resources.

Board 4: Port 310 Mem D0000 Port 310 not found.

This message indicates that the driver could not find a board with

port 310h.  Check to insure that you have specified the correct I/O

port or that you have properly configured the board's port address.

Board 4: Port 310 Mem D0000 Hardware Reset Fails.

This message indicates that the driver could not place the board into

a reset state.  This is most likely the result of an I/O port address

conflict or a faulty board. Be sure that there are no other I/O ports

in your system at the indicated I/O port address.

Board 4: Port 310 Mem D0000 Memory not found error x.

This message indicates that the driver could not find  the board's

memory.  x indicates the number of memory errors found in a 256 byte

block.

Board 4: Port 310 Mem D0000 Memory error x.

This message indicates that the driver has found the board's memory,

but it appears to be faulty.  x indicates the number of errors found

in a 256 byte block.

Board 4: Port 310 Mem D0000 Memory fails disable.

This message indicates that the board's memory could not be disabled.

This could be the result of a memory overlap with some other memory in

the system, or a faulty board. Ensure that no memory conflict exists.

The following messages may occur during the drivers attempt to initialize

the board.

Board 4: Port 310 Mem D0000 BIOS fails initialization.

This message indicates that the board's on-board BIOS did not init-

ialize properly.

Board 4: Port 310 Mem D0000 FEPOS Fails Initialization.

This message indicates that the on-board operating system did not

initialize successfully.

Board 4: Port 310 Mem D0000 FEPOS finds 0 ports.

This message indicates that the on-board operating system initialized

successfully but found no ports. Check to see that the Xem modules are

connected to the host adapter card.

Board 4: Port 310 Mem D0000 Command failure during Initialization.

This message indicates that the on-board operating system encountered

a failure during initialization of the asynchronous channels.