DigiWARE Release Notes P/N 93000137B

for

NetWare 3.X AIO Device Driver for Xem

Software Manual    P/N 92000137A

Date 09-17-93

1.  Enhancements

- The driver now supports 1024 byte Transmit buffers on the

256k Xem host adapter at 64 ports.

2.  Limitations

- The AIO driver supports up to eight EISA boards, seven ISA

boards or four MCA boards, each supporting up to 4 EBI modules

for a maximal driver configuration of 512 ports.

- In ISA and EISA computers these boards may be configured to

reside on 32k boundaries in the host memory address range from

80000h - EFFFFh.

- In MCA computers these boards may be configured to reside on

32k boundaries in the host memory address range from C0000h -

DFFFFh.

- The following baud rates are not supported : 2000, 3600 and

7200 baud.

- Mark and Space Parity are not supported.

- The Deadman Timer function is not supported.

- Adjustabled buffer sizes are not supported.

- While hardware handshaking is active, RTS can not be overridden.

3.  Bug Fixes

- Flow control bugs found by the new Novell IMSP testing

software have been fixed.

- The notSupportedMask has been changed to support the

externalControl variable.

- The AIOPORTCAPABILITES structure has been fixed.

4.  Manual Errata

-  None.