MACHXL 1.3 RELEASE NOTES

=============================

Table of Contents

-----------------

MACHXL version 1.3 now supports the MACH435 and three JTAG devices --

the MACH445, MACH465 and MACH355.

NOTE:

If you are a current MACHXL v1.2 user, just read sections A, C-F,

and K. If you are a MACHXL v1.1 user, read sections A-M.  If you are a

MACHXL v1.0 or a new MACHXL user, read all sections of this README.DOC file.

A. New in MACHXL versions 1.2 and 1.3

B. System configuration and installation

C. Device support and device specific information

D. Iteration between partitioning and placement/routing

E. JTAG programming support from the MACHXL menu

F. Custom BSDL file for in-circuit programming

G. SETF of clock and data in the same vector

H. Improvements in iterative manual partitioning

I. Reporting of partitioning failures

J. Execution speed of MACHXL v1.2 and v1.3

K. Miscellaneous changes

L. Known problems with MACHXL version 1.3

M. Software Trouble Reporting

N. Known problems with the MACHXL manual, document rev 1.1 11/93

O. Portions of Version 1.1 README

1. CHANGES FROM MACHXL VERSION 1.0 SOFTWARE

2. NOTES ON THE PARTITIONING LIMITS CONTROL FILE <DESIGN_NAME>.LIM

3. DEVICE CAPACITY AND THE PARTITIONER

4. RESERVING PINS

5. BACK ANNOTATION AND FLOATING PINS/NODES

6. SIMULATION NOTES

7. ADDITIONAL WORDS ON INTERPRETING THE NUMBER OF

USED AND AVAILABLE MACROCELLS IN THE FITTER REPORT FILE, .RPT

A. New in MACHXL versions 1.2 and 1.3

1. New in MACHXL v1.3

- Full support, including JEDEC generation and 5-volt in-circuit JTAG

programming support (see Sections E and F), for the MACH355

device.  See section C for more information.

- A new fitter option for iteration between partitioning and

placement/routing is available.  See section D.

- Chain file editing support has been added to the MACHXL

DOWNLOAD submenus.  See section E.

- Limited testing shows that MACHXL can be run as a DOS

application under Windows NT. See section B4.

- A SETF of an output-enabled pin will not inhibit generation

of the JDC file by the simulator. See section K1.

2. New in MACHXL v1.2

- JEDEC generation for MACH445 and MACH465 devices was turned on.

See section C for more information.

- A MACH435 device may be cross programmed as a MACH445 device; see

section C.

- 5-volt in-circuit JTAG programming support for the MACH445 and

MACH465 devices is included. See Sections E and F.

- The handling of SETF of a clock and data in the same vector

is explained in greater detail in section G.

- For advanced users, MACHXL supports iterative

manual partitioning without rerunning all programs.  See section H.

- When partitioning fails, the reported "percent completion" will

usually be higher than in MACHXL v1.1. See section I for

clarification on how partitioning failures are reported in

MACHXL v1.2 and v1.3.

- The execution speed of several MACHXL routines has improved;

see section J.

- The Miscellaneous section, K, contains information on many small

changes to MACHXL.

B. System configuration and installation

1. See the MACHXL Software User's Guide for instructions

on installing and configuring MACHXL software. No special

configuration is required for DOS 5 or DOS 6. MACHXL software

does support extended memory managers. 8 MB or more of extended

memory is recommended.

2. Parallel Port for programming JTAG devices via cable.

During installation you will select a PC port for programming

MACH devices with 5-volt in-circuit programmability and JTAG

via cable. To the question,

Which parallel port will be used for programming via cable?

you should respond with lpt1, lpt2 or lpt3.

The MACHXL installation software will not validate your choice or

check for its existence. The default is the "lpt1" port. The port

selection may be changed after installation via Download|Program

Via Cable|Program Device|Communication Port. See section E.

The JTAG cable cannot be used with some software keys. Remove

any software keys from the parallel port before attaching the

JTAG cable to the parallel port of your PC. Attach the other end

to the 10 pin header on your JTAG board (see MACHPRO document ).

3. MACHXL software does run as a DOS application under Windows.

The paragraphs below explain how to create a Windows icon for MACHXL.

For Windows NT, change all references below ( in items a through g)

from MACHXL.PIF to MACHXLNT.PIF.

a. First, install the MACHXL software.

b. Start the Windows File Manager.

c. Drag the file MACHXL.PIF from the \MACHXL\DAT directory

to the Program Manager window of your choice. This will

leave a place holder icon.

d. Select this new place holder icon by clicking

the mouse button once.

e. Modify the properties of the MACHXL place holder icon

by pressing ALT+ENTER. Enter the path to the file

MACHXL.PIF. For example, if MACHXL is installed on the

C drive in the directory MACHXL, type

"C:\MACHXL\DAT\MACHXL.PIF".

f. Select "Change Icon", then enter the drive and location

of MACHXL and its icon in the \MACHXL\DAT directory.

For example, if MACHXL is installed in C:\MACHXL, type

"C:\MACHXL\DAT\-AMD.ICO".

g. The MACHXL software icon (a green AMD logo) will be displayed.

OK the change.

NOTE 1: if you have installed MACHXL software anywhere other

than C:\MACHXL you must also update the MACHXL.PIF file.

Use the PIF editor to change the path under "Program Filename"

to correctly represent your own path to the

MACHXL\EXE\MACHXL.EXE file. Save the corrected MACHXL.PIF file.

NOTE 2: When you start MACHXL software for the first time

using the Windows icon, you will end up in the c:\ or in

the c:\windows directory. Once you have changed to a design

directory, every subsequent invocation of MACHXL will place

you in a design directory because MACHXL remembers your

last working directory.

For more information on creating Windows icons, see your

Windows manual.

4. Limited testing shows that MACHXL v1.3 software runs as a DOS

application under OS/2 and under Windows NT. However,

AMD cannot guarantee support in these environments.

For Windows NT, the proper invocation of MACHXL is

MACHXL -NT

from a DOS prompt.

5. MACHXL v1.3 software has not been tested in and is not

supported in networked environments.  However, if you will be

installing MACHXL in a networked environment, make certain that

users run MACHXL software from a directory that they have

read/write access to.

6. MACHXL version 1.3 software is not supported under DR-DOS.

C.   Device support and device specific information

1. In this release, JEDEC generation is turned on for the MACH355,

MACH435, MACH445 and MACH465 devices.

2. Fitting and partitioning MACH465 designs.

A common cause of routing failure in the MACH465 is reaching

the block fanin limit of the device, 34, in one or more

blocks. To correct this problem, reduce the block fanin

slightly for block(s) where the fanin limit is reached on

the first fitting pass. Look at the partitioning fanin in

the Block Partitioning Summary of the .RPT file to see if

the block fanin limit is a potential problem. If it is,

create a partitioning control limits file (.LIM) and restart

the fitting process. (See page 243 and Appendix C of the

MACHXL revision 1.1 User's Guide.)

Setting the new MACH Fitting Option

Iterate between partition & place/route?     Y/N

to "Y" should reduce the need to control partitioning via

the .LIM file.

3. MACH355 pinout

The MACH355 pinout is not in the MACHXL revision 1.1 Software

User's Guide. There are 96 IO pins for MACH355. In relative

order (0 to 15), the I/O pins within the six MACH355 blocks are:

129 130 131 132 133 134 136 137  // block A

139 140 141 142 144   1   2   3

20  21  24  25  26  27  28  29  // block B

19  18  17  16  14  13  11  10

52  51  50  49  48  47  45  44  // block C

42  41  40  39  37  36  35  34

57  58  59  60  61  62  64  65  // block D

67  68  69  70  72  73  74  75

89  88  85  84  83  82  81  80  // block E

90  91  92  93  95  96  98  99

124 123 122 121 120 119 117 116  // block F

114 113 112 111 109 108 107 106

There are 20 GROUND pins for MACH355:

7 15  23 30  38  46  54  55  63  71

79 86 94 102 110 118 126 127 135 143

There are 16 VCC pins for MACH355:

4 12 22  33  43  53  56  66

76 87 97 105 115 125 128 138

There are 6 JTAG ( or TEST control ) pins for MACH355:

5 31 32 77 103 104

There are 2 input-only pins for MACH355:

6 ( II5 )  78 ( II2)

There are 4 clock/input pins in the MACH355:

8 ( MACHXL default clock pin ), II0

9                               II1

100                             II3

101                             II4

Rules For Associating Node Numbers to I/O Pins

GLOBAL NODE, a phantom node, is node number 1.

The number of the first OUTPUT NODE layout/feedback

associated with the IO pin at the row-column location

( Blk_Num, Rel_Pin_Num ) of the IO pin array above is

2 + 16 * Blk_Num + Rel_Pin_Num

where Blk_Num and Rel_Pin_Num are zero-based with a

Blk_Num of 0 representing block A and Rel_Pin_Num

of 0 representing the first pin in the block.

There are no input nodes.

4. Cross Programming MACH435 Designs to MACH445 Device.

The L field fuses of MACH435 and MACH445 JEDEC files are

identical. Therefore, a MACH435 JEDEC file can be converted

to a MACH445 JEDEC file which can then be used to program

a MACH445 device. This is called cross programming.

To cross program a MACH435 design to a MACH445 using AMD's

MACHXL and MACHPRO software, use MACH445 when asked for "Part

Name" and the name of the MACH435 JEDEC file when asked

"JEDEC file for the specified operation" on the Create/Edit

configuration file menu (see section E).

Note that the pinout will change if a MACH435 JEDEC file is

cross programmed as a MACH445 device. When restricted to

IOs and dedicated inputs or input/clocks the pin mapping from a

MACH435 to MACH445 device is a one-to-one relationship

as shown below:

There are 64 IOs for both MACH435 and MACH445.

BLOCK A-435  3,  4,  5,  6,  7,  8,  9,   10

BLOCK A-445  93, 94, 95, 96, 97, 98, 99, 100

BLOCK B-435  19, 18, 17, 16, 15, 14, 13, 12

BLOCK B-445  12, 11, 10,  9,  8,  7,  6,  5

BLOCK C-435  24, 25, 26, 27, 28, 29, 30, 31

BLOCK C-445  19, 20, 21, 22, 23, 24, 25, 26

BLOCK D-435  40, 39, 38, 37, 36, 35, 34, 33

BLOCK D-445  38, 37, 36, 35, 34, 33, 32, 31

BLOCK E-435  45, 46, 47, 48, 49, 50, 51, 52

BLOCK E-445  43, 44, 45, 46, 47, 48, 49, 50

BLOCK F-435  61, 60, 59, 58, 57, 56, 55, 54

BLOCK F-445  62, 61, 60, 59, 58, 57, 56, 55

BLOCK G-435  66, 67, 68, 69, 70, 71, 72, 73

BLOCK G-445  69, 70, 71, 72, 73, 74, 75, 76

BLOCK H-435  82, 81, 80, 79, 78, 77, 76, 75

BLOCK H-445  88, 87, 86, 85, 84, 83, 82, 81

There are 2 input-only pins for both MACH435:   41   83

& MACH445:   54    4

There are 4 clock/input pins for both MACH435:  20   23   62   65

& MACH445:  13   18   63   68

There are 8 ground pins for MACH435:  { 1, 11, 22, 32, 43, 53, 64, 74 }

There are 16 ground pins for MACH445:

{ 1,  2, 16, 17, 29, 30, 40, 41, 51, 52, 66, 67, 79, 80, 90, 91 }

There are 6 VCC pins for MACH435: {  2, 21, 42, 44, 63, 84 }

There are 8 VCC pins for MACH445: { 14, 15, 39, 42, 64, 65, 89, 92 }

There are 6 JTAG pins for MACH445: { 3, 27, 28, 53, 77, 78 }

with no counterpart in MACH435.

D. Iteration between partitioning and placement/routing

A new MACH Fitting Option has been added which increases the chance of

fitting success and improves block partitioning through automatic

balancing. Setting the new option

Iterate between partition & place/route?     Y/N

to the default value of "Y" improves the Fitter's capability to

balance partitioning of your design across blocks and fit the design.

During this phase, the Fitter looks for suitable partitions. Once it has

found one, it passes that partition on for placement and routing in a

block. In past releases of MACHXL, the partitioning engine would attempt

to completely fill a block before moving to the next block.  These

partitioned blocks were sent to the placement and routing

engine, where it would take as long as needed to fit the partition.

If it could not place and route that partition, the fitting process

would fail.

MACHXL v1.3 improves the chance of fitting as well as providing

more balanced partitioning solutions.  The partitioner now packs blocks

less tightly, improving the likelihood of successful placement and

routing for a partition. If a placement cannot be found for a partition,

the Fitter now attempts another partition, continuing to iterate

between the partitioning and placement & routing until a fit is

found or time runs out.  As a result, the Fitter is more likely to find

a solution and provide a more balanced result automatically, without

having to manually control partitioning from the .LIM file (see

section O2).

However, if the fitting time limit ("Run Time Upper Bound in 15

minutes") is set too low, a design which fit in previous

MACHXL releases may no longer fit. If that happens, increase the

time limit.

E. JTAG programming support from the MACHXL menu

The JTAG programming software, MACHPRO, and related data files

are automatically loaded during the MACHXL installation.

Supported JTAG devices are MACH355, MACH445 and MACH465.

To accommodate the JTAG programming process, the

DOWNLOAD submenu structure has been expanded as follows:

DOWNLOAD

* Download to programmer

* Program via cable ...

- View configuration file

- Create/Edit configuration file

File: file name                    Current  #

Mode: ...                          Total    #

+  Design description                      (optional)

+  Part name:                              (part name)

+  Instruction code:                       (mode)

Program device

Verify device

Read device

Read user code

Read Jtag ID

Bypass mode

+  No.of bits in instruction register      (#)

+  JEDEC file for the specified operation: (filename)

+  Programming result file:                (filename)

+  Specify Manufacturer option:            Y

>  Turn on Program Security fuse 1? N

>  Turn on Program Security fuse 2? N

>  Set the IO pins to be:           Z

- Program device

+ Display status messages               Y/N  (Y default)

+ If error, continue processing chain   N/Y  (N default)

+ Erase device if verification fails    Y/N  (Y default)

+ Communication port:                   lpt1

( choices are lpt1, lpt2, lpt3 )

- Review JTAG results...

+ Review JTAG status

+ View/edit output file(s) (file name)

"Download to programmer"

This option does not apply to devices MACH355, MACH445 and MACH465

which feature JTAG.

WARNING: If you are running MACHXL software as a DOS application

under Windows or Windows NT or OS2, do not select this option.

It will lock your system.

"Program via cable"

This option applies only to MACH devices with JTAG

and to the MACH435 when cross programmed to a MACH445.

"Program via cable: View configuration file" option

The configuration file is also called a JTAG scan path file

or a JTAG chain file. No changes can be made to the configuration

file while VIEWing it.  If <design>.CHN exists, it will be

displayed as the file to VIEW.  If <design>.CHN does not exist,

then <none> is displayed and you should supply a name.

Multiple parts may be programmed from a single chain

file.  The order of the chain entries dictates a multiple-part

programming sequence.  Lines not commented out with a semi-colon

are complete "command lines" or chain entries for JTAG

programming software.

"Program via cable: Create/Edit configuration file" option

This invokes the chain file editor to create or modify the

specified configuration file, <design>.CHN.  You can

select an alternate existing or new .CHN file by

supplying another file name.

The menu screen that appears next, "JTAG CHAIN FILE EDITOR"

is used to create and edit the chain entries for the JTAG

programming software.  Previously existing chain entries are

extracted from the .CHN file and can be displayed in the

JTAG CHAIN FILE EDITOR.  Once you have finished editing the chain

file, the editor rewrites the chain file, appending the new

version to the bottom of the chain file.  Previous entries

are automatically commented out by the chain file editor by placing

a semicolon in the first column.  Consider the example <design>.CHN

file below:

;EDIT VERSION ===> Thu May 26 03:53:47 1994

;'my first chain entry' MACH445 p 6 test.jed;

;'my second chain entry' MACH445 p 6 test2.jed / -s 1 -o Z

;EDIT VERSION ===> Fri May 27 12:25:55 1994

'my first chain entry' MACH445 p 6 test.jed;

'my second chain entry' MACH445 p 6 test2.jed / -s 1 -o Z

'new chain entry for 3rd device' MACH465 p 6 myfile.jed

Note that an EDIT VERSION date stamp separates the different

versions in the .CHN file and that older chain entries are

commented out by adding a semi colon in front of them.

If you wish, use a text editor to delete old chain entries that

have been commented out.

The JTAG CHAIN FILE EDITOR has four modes of operation: (S)elect,

(D)elete, (C)reate, and (Q)uit.  You are prompted to select a

mode by a query at the bottom of the screen.

Use (S)elect when making changes to entries in an existing

chain file.  Choosing (S)elect prompts you for the number of the

entry in the chain file you wish to edit.  Use <F10> after typing

the number of your choice.  You must select the chain entry you

wish to edit first.  Once selected, note that the entry for "Mode:"

in the upper left corner changes to "UPDATING".  Use the up and down

arrow keys to move around the form.  (See "JTAG CHAIN FILE EDITOR menu

choices" below for more information.)  Once you confirm

your new choices for the selected chain entry with <F10), mode changes

to "UPDATED".  The CHAIN FILE EDITOR will automatically update the

chain file for the changed entries.  To discard your changes, pressing

<ESC> will "ABORT" your changes.

Choosing (D)elete will delete the chain entry currently selected.

To delete a chain entry other than the one currently being edited,

(S)elect that chain entry first.

Choosing (C)reate allows you to create a new chain file or add

new entries to an existing chain file.  Note that the "Mode:"

changes to "NEW" when (C)reate is selected.  Use the up and down

arrow keys to move around the form.  (See "JTAG CHAIN FILE EDITOR

menu choices" below for more information.)  Once you confirm

your choices for the selected chain file with <F10), the mode will

change to "ADDED".  The CHAIN FILE EDITOR will automatically append

the new chain entries to the end of the chain file.  To place a new

chain entry inbetween existing chain entries, use (C)reate to create

the new entry first, then use the standard text editor to move

the chain entry when you leave the chain file editor.

Choosing (Q)uit exits the menu-driven JTAG chain file editor,

placing you in the chain file in your preferred text editor. You

may reorder chain file entries, make any last minute changes, or

add comments at this time.  When you quit the text editor, you

are returned to the MACHXL menu.

> "JTAG CHAIN FILE EDITOR" menu choices

------------------------------------------------------------------

|                                                                |

|                                                                |

| File: TEST2.CHN                          Current: 1            |

| Mode: WAITING                            Total:   3            |

|                                                                |

| Design description (optional):           my first chain entry  |

| Part name:                               MACH445               |

| Instruction code:                        Program device        |

| No. of bits in instruction register:     6                     |

| JEDEC file for the specified operation:  test2.jed             |

| Programming result file:                 test2.out

| Specify Manufacturer option:             N                     |

|                                                                |

------------------------------------------------------------------

File: the name of the chain file you are editing

Mode: indicates the mode of operation -- UPDATING, ADDED, WAITING,

UPDATED, NEW, or ABORTED.

Current: the number of the chain entry you are editing

Total:   the total number of chain entries in the chain file

Design description: an optional descriptive field

Part name: the part being added to the chain file. May be filled

in with a supported MACH JTAG device (press <F2>), or

type the name of another device.

Instruction code: press <F2> to choose from

Program device

Verify device

Read device

Read user code

Read Jtag ID

Bypass mode

The first five choices can only be used with

MACH devices with JTAG; only "Bypass mode" can

be used with other devices.  "Read device" reads

a device pattern into a JEDEC file; "Read user"

code reads the user signature into an output

file.  "Read Jtag ID" reads the device ID code

into an output file.

No. of bits in instruction register: supply the appropriate value,

from 0 to 9.  Use 6 for

the MACH445, MACH465 and

MACH355 devices.

JEDEC file for the

specified operation: the name of the JEDEC file. Required for

Program device, Verify device, and Read

device.

Programming result file: An output file name is required for

Read user code and Read JTAG ID.  The

default is *.OUT.

Specify Manufacturer

option:              choose Y or N. Choosing Y brings up another menu.

Turn on security fuse 1:  N

Turn on security fuse 2:  N

Set IO pins to be:        Z

The default for both the security fuses is "N".

The choices for IO pins are "Z", "0", "1", and

"X"; the default is "Z".

"Program via cable: Program device" option

When selected, a submenu of programming options is displayed

for confirmation:

----------------------------------------------

| Display Status Messages:             Y     |

| If error, continue processing chain: N     |

| Erase device if verification fails:  Y     |

| Communication port:                  lpt1  |

----------------------------------------------

The default values for status messages, error handling, and

failure to verify are shown above.  The JTAG programming

port (lpt1, lpt2, or lpt3) is set during installation.  If these

settings are changed, they become the new defaults.

"Review JTAG results: Review JTAG status"

Use this option to review the status messages issued during the

last JTAG device programming.

"Review JTAG results: View/edit output file(s)"

JTAG programming results are redirected to the output file(s)

specified in the JTAG chain file.  Use this option to view

these output file(s).  Warning: these output files may contain

lines longer than 80 characters; do not use the standard

MACHXL editor to alter this file.

F. Custom BSDL file for in-circuit programming

If a JEDEC file is generated for a MACH device with JTAG, a

custom BSDL file <current_design.BSD> is also generated.

The customized BSDL file can be used to improve the efficiency

of the automatic test vector generation program in boundary-scan

test hardware and software systems such as ASSET by Texas

Instruments and Victory by Teradyne.

In addition to the information contained in the device-specific

but design-independent standard (.BSM) BSDL file -- like device

pinout, instruction codes, data registers, and the layout of the

boundary scan registers -- the customized BSDL file contains

design-specific implementation data such as the USERCODE,

which I/Os and inputs are not used and which I/Os are used only

as inputs.

For more information on the BSDL file, refer to either the

MACH3XX/4XX JTAG Specification, or the Boundary-Scan

Handbook by Ken Parker (ISBN: 0-7923-9270-1).

G. SETF of clock and data in the same vector

A signal appearing in a SETF statement will be treated by the

simulator as a clock when it is the default clock or it

i)  occurs on the right-hand-side of only CLKF equations; and

ii) none of the literals in its right-hand-side of its

defining equation (if it has one), appear on the

left-hand-side of a non-CLKF equation.

"Treated as a clock" by the simulator means that U/D values will

be assigned to clock in the JEDEC test vector and execution of

these values will be delayed until after the SETF is applied

to data or to signals that do not satisfy both i) and ii).

Example:

Assume for the following SETF command

> SETF A1 CLOCK1 A4 CLOCK3

that CLOCK1 satisfies both i) and ii) and CLOCK3 does not.

The equivalent order of execution of the above command in

the simulator will be:

> SETF A1 A4 CLOCK3

> SETF CLOCK1

One JEDEC vector will be output with 1/0 for A1 and A4

with U/D for CLOCK1. U/D will be assigned to CLOCK3 if it

is a physically dedicated clock pin, otherwise 1/0 will

be assigned to it.

H. Improvements in iterative manual partitioning

MACHXL version 1.1 introduced the <design_name>.GRP file. This

file, created by the Partitioner, records the partitions created

to the point of failure or success. This partial or complete

partition contains not only user-defined GROUP MACH_SEG statements,

but also GROUP MACH_SEG statements that the Partitioner defined on

its own as a result of finding successful partitions. (This

information can also be derived from the .RPT file.)

MACHXL versions 1.2 and 1.3 software can now process GROUP MACH_SEG

statements in the .PLC  file.  This allows you to change the

pinout of an existing design, or repartition a design using GROUP

MACH_SEG statements by just rerunning the Fitter, rather than

rerunning all programs. This technique is recommended only

for advanced users.

To use this feature, you modify the <design_name>.GRP file

and then incorporate the appropriate GROUP MACH_SEG statements

in the <design_name>.PLC.  GROUP statements from the design

file and the PLC file are merged when you select "Use placement

data from: Last successful placement" and do one of the following:

* modify the pin placements in the .PLC file, or

* delete the pin/node statements from the .PLC file, or

* select 'Float pins, nodes' in the Handling of

Preplacements on the MACH Fitting Options menu.

Under the RUN menu, choose "Rerun Fitter" rather than "Rerun

All Programs" from the "Compile" option to see the impact of

the changes. During processing, priority will now be given to

GROUP MACH_SEG statements in the .PLC file.

I. Reporting of partitioning failures

1. Percent completion

When partitioning fails, the percent completion will usually be

higher than in the previous release of MACHXL software. This is

because input-only signals that are freely mixed between blocks

are counted as partitioned even though their real assignment to

blocks will be deferred until placement and routing.

If all non-input-only signals are partitioned successfully, then

100% success is reported in the .RPT and .LOG files.

If some non-input-only signals fail partitioning, then percent

completion reported for partitioning is calculated as:

(100 * # of signals placed in blocks) / (all signals)

where:

denominator = # of signals listed in the SIGNAL SUMMARY section

of the .RPT file

numerator   = # of signals with block assignments (not a '?')

as listed in the Block column of the SIGNAL SUMMARY

section of the .RPT file.  Note that signal names

that appear twice (once as a node and once as a pin)

with exactly the same name are counted only once.

All other signal pairings, including X and RN_X

are counted twice.

2. When Partitioning fails

When preplaced and block-restricted signals fail to pass

the partitioner's constraint checking, unpartitioned signals

are only reported in the .RPT file. They are not reported

in the .LOG file.

However, if partitioning fails AFTER the constraint checks for

preplaced & block-restricted signals, unpartitioned signals

will be displayed in both the .LOG and .RPT files as shown:

The following signals remain to be partitioned

( excluding pins used only as inputs )

IREG170_                            IREG171_

IREG172_                            IREG173_

IREG174_                            IREG177_

O35_                                O37_

R52_ pair O37_                      R56_ pair O35_

J. Execution speed of MACHXL versions 1.2 and 1.3

A number of changes to MACHXL software since ver. 1.1 have resulted in

changes in execution speed to several MACHXL modules.

1. The Simulator may run up to twice as fast.

2. The Parser may run up to twice as fast when not run as a DOS

application under Windows.

3. The execution speed of the Fitter is relatively unchanged.

However, in some cases the Fitter will run faster as a

result of an improved placement routine.

4. The Partitioner has been enhanced to improve the likelihood

of finding a fit. Therefore, designs which did not previously

fit will now continue to partition past the point of failure

of earlier releases of MACHXL software.

K. Miscellaneous changes and enhancements

1. Prior to MACHXL 1.3, a SETF of an output-enabled pin would

provoke an error message ( #3270 ) which inhibited generation

of the JEDEC ( .JDC ) file. In MACHXL 1.3, the SETF is

ignored with a warning ( #2310 ); the internally generated

( rather than SETF'd ) value for the pin is used in equation

evaluations; and the JEDEC file is generated.

2. Out-of-memory situations should no longer occur while parsing

the design file when MACHXL is run outside the Windows environment.

If an out-of-memory condition for the Parser should occur under

Windows, exit MACHXL, and recompile the design while running

MACHXL from outside the Windows environment.

3. Signal names greater than 14 characters (or vectored signals

greater than 9 characters) that are truncated and modified

to make them unique by the Parser are now recognized by

the Back Annotate program and are handled accordingly. The

truncated signal names are used in the report files and

in all LOG file output from programs MACHBPP through MACHFITR.

4. If you attempt to use a .MXL  file with menu options settings

and other information from a different version of MACHXL

other than version being used, the menu will issue a warning message:

"The version number (v#.#) in the .MXL file belonging

to the current design, 'TEST.MXL', is different from

the version number of this MACHXL program (current version #')."

The software will offer to convert the MXL file to the current

version by asking you "Are you sure? Y/N". Responding

with "Y" will assign default values to any new options found

in versions 1.2 & 1.3 of MACHXL that were not available under

previous releases.  Responding with "N" will de-select the

current design and make no changes to the MXL file associated

with that design file.

FYI: The version number of the MXL file can be found in the

first line of the MXL file.

5. The 'Run|Other operations...|Disassemble from...|Intermediate'

menu option which disassembles the intermediate .TRE file

now allows you to supply input and output file names.

6. The selection list for the Run mode under COMPILATION OPTIONS

has been made circular so that at the end of the selection

list, pressing the down arrow takes you to the top of the

list and vice-versa. In addition, the selection option 'Run

All Programs' has been moved to the top of the list.

7. The <F10> key can now be used to approve the file selected

when browsing a list of file names.

8. The <F9> key will now take you to a DOS prompt, just like the

'Go to system' option from the File menu. To return to MACHXL,

type "exit" after a DOS prompt.

L. Known problems with MACHXL version 1.3

1. Do not use the 'Download|Download to Programmer' option when

MACHXL is invoked under Windows or OS/2. It will lock your system.

2. MACHXL MXL files should be of the same version or less than the

version of MACHXL being run.  If it is less than the version

of MACHXL being run (version 1.3), you should convert the

MXL file as discussed in section K.4.

When the version number of a MXL file is larger than the

version of MACHXL being run, the menu may be unable to

process the MXL file.  If you have encountered this problem,

try deleting all the .MXL files in the root directory, deleting

the <design>.MXL file and the SETUP.MXL file in your current

directory.  This will force MACHXL to create its own MXL default

files. Note that you will need to reset your own preferences.

3. The startup syntax for state machines will be errored out in the

MACHEXP program. Please read the STARTUP.DOC file in the

\MACHXL\DOC subdirectory.

4. Out of memory messages

If you encounter an out of memory message, check to see that you

have the recommended minimum memory configuration of 8 MB.

Other situations that might occur:

a. When the parser is invoked as a DOS application under

Windows, Windows NT or OS2 and the current design has a

large number of or very complex CASE statements, the

Parser may run out of memory.  If this happens, try

running MACHXL directly from DOS.

b. If the Minimizer MACHMIN reports an out of memory condition, try

rerunning the design using fast minimization (found in the Logic

Synthesis menu). This problem usually occurs when the logic

synthesis options "Optimize Registers for D/T-type" is set to

"Change all to D-type" or "Change  all to T-type",

"Use Automatic Gate Splitting" is set to "Y", and "Ensure

polarity after minimization is" is set to "Best for device".

The solution is to reduce the number of minimization options

selected. "Ensure polarity after minimization is: Best for

device" is the most memory intensive of these options.

c. If the Minimizer reports a 64K segment limit condition, it has

encountered an equation that has a large number of product

terms (i.e. 25 to 30) where each product term has only one

or two unique signal names. A sparse Karnaugh map results,

which is unlikely to reduce further. To correct the

problem, try turning on fast minimization or manually gate

splitting the offending equation.

M. Software Trouble Reporting

To report a MACHXL software problem such as a general protection

fault or internal error, refer to the file \MACHXL\README.BUG or

to Appendix D of the MACHXL revision 1.1 Software User's Guide.

N. ERRORS IN THE MACHXL revision 1.1 SOFTWARE  USER'S GUIDE

1. Page 45-46. The explanation of the Logic Synthesis option

"Ensure polarity after minimization is: Best for device" is

incorrect. On page 45, replace the "Y" in "If the

22V10/MACH1xx/2xx S/R Compatibility option in the MACH Fitting

Options form is set to 'Y'" with "N". On page 46, replace the

"N" in "If the 22V10/MACH1xx/2xx S/R Compatibility option in

the MACH Fitting Options form is set to 'N'" with "Y".

2. Page 118. The comment in the third signature example on page

118 should read "hex result is" instead of "binary result is".

3. Page 143. In the second paragraph, change the "N" in "When

the '22V10/MACH1xx/2xx S/R Compatibility?' option is set to

'N', the only.." to "Y".

4. Page 215. The first sentence on page 215 of the MACHXL version

1.1 Software User's Guide should read "Equations of the form

'rpin1 = rpin1 + ... +   rpin N' will inhibit the discard of

pins 'rpin1, ... rpinN' which are otherwise not used in the

design file".

5. Page 275. In the example at the bottom of the page, delete

"PAIR C2" from the NODE statement. It should read "NODE ? B2 REG".

6. Page 276. In the first sentence and the accompanying example of

TCO, C2 should be replaced with B2.

7. Page 277. In the last sentence and accompanying example of TCR,

B2 should be replaced with C2.

8. The first sentence on page 215 of the MACHXL revision 1.1

Software User's Guide should read "Equations of the form

'rpin1 = rpin1 + ... + rpin N' will inhibit the discard of

pins 'rpin1, ... rpinN' which are otherwise not used in the

design file".

***********************************************************************

***********************************************************************

O.  Portions of Version 1.1 README.

***********************************************************************

***********************************************************************

1. CHANGES FROM MACHXL VERSION 1.0 SOFTWARE

a. Selecting "Best for Device" for the Logic Synthesis option

"Ensure polarity after minimization is..." no longer changes

the polarity of equations.  See page 45 of the version 1.1

MACHXL User's Guide.

b. Two new fitting options enhance the Fitter's speed and

robustness. "Reduce routes per placement" speeds fitting

by limiting the number of routing attempts per placement.

"Reduce non-forced global clocks" gives you more control over

the placement and number of single literal floating clock signals.

See page 58 of the version 1.1 MACHXL User's Guide.

c. More information is presented in the Fitter report file (.RPT)

when partitioning fails. See pages 278-280 of the version 1.1

MACHXL User's Guide.

d. Partitions, both user-defined and system-defined can be found

in the <design_name>.GRP file. See note 4 below.

e. The reasons for classifying a clock as global or non-global

are now summarized in the .LOG file. See pages 238-239 of

the version 1.1 MACHXL User's Guide.

f. Advanced users will find the Place and Route Data file

(<design_name>.PRD) generated by the Fitter useful for

unplaceable and unroutable designs. See pages 256 to 271 of

the version 1.1 MACHXL User's Guide.

g. The partitioning control limits file (.LIM) is a control file

you can create that specifies the maximum number of macrocells

and logic array inputs available to a design. See Appendix C of

the version 1.1 MACHXL User's Guide.

h. The functionality of active-low T flip-flops has changed. SET and

RESET are now always swapped regardless of the setting of

"22V10/MACH1xx/2xx S/R Compatibility" MACH Fitting Option.

i. Overlap of single literals between SET and RESET is now avoided

when the MACH Fitting Option "Set/Reset treated as DON'T CARE"

is set to "Y". See page 214 of the version 1.1 MACHXL Software

User's Guide.

2. NOTES ON THE PARTITIONING LIMITS CONTROL FILE <DESIGN_NAME>.LIM

The partitioning limits control file <design_name>.LIM is a

user-defined file that specifies the maximum number of macrocells

and logic array inputs available to a design. For an explanation

of .LIM files, see Appendix C of the MACHXL revision 1.1 Software

User's Guide.

The partitioning limits control file is only read by the partitioner

during fitting; it is not used during design rule checking. Design

rule checking uses the architectural limits of the device, not

any additional constraints imposed by the designer.

3. DEVICE CAPACITY AND THE PARTITIONER

a. When partitioning fails, the number of clusters or macrocells available

is more likely to be the problem than the number of product terms

available. Because the clock and Set/Rest product terms for asynchronous

macrocells are not included in the "PT" column in any section of

the .RPT file, it may appear that the Partitioner has problems

recognizing the number of product terms available to each block,

even though the real problem may be the number of clusters or

macrocells available. (See pages 278-279 of the MACHXL revision

1.1 User's Guide for a discussion of partitioning failures.)

b. When partitioning fails in MACHXL versions 1.1, 1.2 and 1.3,

the number of macrocells used is reported in the Block

Partitioning Summary, but the number of PT clusters used

is not. This figure is easily derived from the Block

Partitioning Summary.

This is a change from MACHXL version 1.0. In MACHXL version 1.0,

the "Macrocells Used" column in the Block Partitioning Summary

was used to report clusters used rather than macrocells used.

c. During partitioning, a shortage of clusters does not always occur

before a shortage of macrocells.  This situation can occur when

there are many signals driven by a single product term which is

implemented through the XOR and product term steering.

4. RESERVING PINS

Because the fitters in MACHXL version 1.1, 1.2 and 1.3 recursively

discard unreferenced pins and nodes, the method for reserving

pins is different than in MACHXL version 1.0. (See pages 214-215

of the MACHXL version 1.1 Software User's Guide.)

5. BACK ANNOTATION AND FLOATING PINS/NODES

The back annotation program, MACHBAK will usually issue an error

message if a pin or node is preplaced in the design file but then

is floated as a result of selecting a fitting option. The solution

is to edit the design file, replacing pin and node numbers with a "?"

or to run "Change all to floating" before running back annotation

from the Run:Other menu. (See page 62 of the MACHXL version 1.1

Software User's Guide.)

6. SIMULATION NOTES

The simulator models the fitter's final placement by following the

device's conventions for signal inversion for registers/latches.

To avoid check clashes during simulation for registers, use CHECK

statements according to the signal inversion conventions for registers

of the selected device.

a. In MACH4xx devices, all input registers power up in a low state.

(See pages 304-305 of the MACHXL revision 1.1 Software User's Guide.)

Depending upon how the input register is defined in the design

file, pin and node references to the input register may be

reported high or low. For example, if a node BIN is paired to

pin BIO as...

PIN /BIO IPAIR BIN

NODE ? BIN

BIN:= BIO

or

PIN BIO IPAIR BIN

NODE ? BIN

BIN:= /BIO

then all references to BIN will be high at power up. Similarly, after

power up, references to BIN will be high when BIO is low. This is

because there can be no inversion of BIO prior to the register.

Instead, inversion is after the register. Inversion is applied to

each reference to BIN as it is fed into its fanout destination

block via the central switch matrix.

b. MACH 3 and 4 devices do not have an inverter between the pin and

its output node. When writing check statements for nodes paired

to pins it is important to note that the simulator reports

physical values and not logical values. If the node has opposite

polarity than its output paired pin, then the physical value of

the node is inverted from the logic value of logic equation in

the PDS file. The true logical value of the node equation is

preserved by inverting the value of the node at each reference,

as pin polarity always takes precedence over node polarity.

c. Active-low buried nodes are implemented in MACH 3 and 4 devices

by inverting logic values at each reference. When writing check

statements for buried nodes, it is important to note that the

simulator reports physical values and not logical values. The

true logical value of the node equation is preserved by

inverting the value of the node at each reference.

7. ADDITIONAL WORDS ON INTERPRETING THE NUMBER OF

USED AND AVAILABLE MACROCELLS IN THE FITTER REPORT FILE, .RPT

Macrocells counted as "available" in the Device Resource Summary

and at the block level in the Block Partitioning Summary in the

.RPT file are indeed unused. However, the interpretation of the

other categories -- "> 1 PT Macrocells", "1 PT Macrocells", and

"Unusable Macrocells" -- in the Device Resource Summary are sensitive

to the "used" or "available" status of the macrocell being classified.

(See pages 241 to 244 of the MACHXL rev 1.1 Software User's Guide.)

a."> 1 PT Macrocells"

The "Used" column refers to the number of product term driven signals

that have more than 1 PT in the minimized primary equation.

The "Available" column refers to the number of unused macrocells.

These are macrocells which do not collect logic for an

already-placed product term driven signal and to which at least one

unused cluster can be steered. This is a coarse

measure of the potential to still fit a design

while adding more signals, and can overstate the

actual number of macrocells available. Because

these macrocells are counted and reported in a

mutually independent manner, it is not necessarily

true that more than one such macrocell can be used

if more product term signals are added, even if

there is more than one unused macrocell.

b. "1 PT Macrocells"

The "Used" column refers to signals having a

single product term and implemented via a XOR

product term. These macrocells may have the

potential to be expanded to more than one product

term.

The "Available" column refers to the number of

macrocells which do not collect logic for an

already-placed product term driven signal and from

which only a XOR product term can be steered to

implement a signal's primary equation without

changing the final placement. However, unlike the

"Available" column under "> 1 PT Macrocells", this

figure does not overstate the number of macrocells

simultaneously realizable.

c. "Unusable Macrocells"

These are the macrocells that are not used in the

final placement, and are not counted as available

under  "> 1 PT Macrocell" and "1 PT Macrocell".

This figure is reported only when partitioning

succeeds.