STANDARD MICROSYSTEMS CORPORATION

DOS ODI Driver

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to set up the SMC Ethernet parallel

port LAN adapter to communicate with both Novell NetWare 386 and IBM AS400

networks.

INSTALLATION PROCEDURE

Use the following procedure to enable a workstation to communicate

concurrently with both Novell NetWare 386 and IBM AS400 networks.

1. Create a directory on your hard drive called \NOVELL. For

example, to create this directory on the C drive, type the

following line and press the Enter key:

mkdir C:\NOVELL

2. Insert the SMC SuperDisk into an available floppy drive.

3. Copy the following files to the \NOVELL subdirectory:

LSL.COM

ODINSUP.COM

IPXODI.COM

NETX.EXE

8008ODI.COM

NET.CFG

For example, if the \NOVELL subdirectory is on drive C and

the SuperDisk is in drive A, type the following lines and press

the Enter key after typing each one:

copy a:\dos_odi\lsl.com c:\novell

copy a:\dos_odi\ipxodi.com c:\novell

copy a:\netx.exe c:\novell

copy a:\dos_odi\8008ODI.com c:\novell

copy a:\dos_odi\as400\net.cfg c:\novell

LSL.COM, IPXODI.COM, and NETX.COM are provided on the SuperDisk,

ODINSUP.COM may be acquired from Novell Technical Support or from

the Novell section on CompuServe (type GO NOVELL at the CompuServe

prompt).

4. Install the SMC (or IBM) LAN Support Program version 1.25 or later

on your workstation.

5. Copy the ODI driver, 8008ODI.COM, from your SMC SuperDisk to your

workstation. This file is located in the \DOS_ODI subdirectory on

the SuperDisk. For example:

copy a:\dos_odi\8008ODI.com c:\novell

6. Using a text editor, open the AUTOEXEC.BAT file on your workstation

and edit it to look like this:

lsl

8008ODI

ipxodi

odinsup

netbind

netx

f:

7. Reboot the workstation. The workstation displays the F: prompt.

Your workstation can now communicate with both Novell NetWare 386

and IBM AS400 networks.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.