STANDARD MICROSYSTEMS CORPORATION

Novell NetWare DOS ODI Workstation Driver

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to copy the Novell NetWare DOS ODI workstation driver

from the SuperDisk to your PC.

o  How to load the Novell NetWare DOS ODI driver and establish a

network connection.

o  How to print to a network printer from Windows.

Note: For custom installations involving multiple frame types, refer

to your Novell NetWare ODI Shell for DOS manual.

COPYING THE DRIVER FILES

Manual Method:

1. Copy the DOS ODI workstation driver, 8000ODI.COM, from the

\DOS_ODI subdirectory on the SuperDisk to the Destination

Directory on your PC's hard drive.

2. Copy the following files to the same subdirectory where the

8008ODI.COM file resides:

LSL.COM

IPXODI.COM

NETX.EXE

The first two files are located in the \DOS_ODI subdirectory on

the SuperDisk. The file NETX.EXE is located in the root directory.

LOADING THE DRIVER

To install the driver:

1. Follow the instructions in the Novell NetWare ODI Shell for DOS

manual.

2. Use any text editor to add the following lines to the

workstation's AUTOEXEC.BAT file:

CD \SMCNOV (or path to DOS ODI driver files)

LSL.COM

8008ODI.COM

IPXODI.COM

NETX.EXE

F:LOGIN

CD \

PRINTING TO A NETWORK PRINTER FROM WINDOWS

If you receive an error message when you try to print to a network

printer from Windows:

1. Exit Windows.

2. Run Windows Setup.

3. Look at the Network option. If this option is set to "Novell

NetWare (shell versions 3.26 and above)," select "Change System

Settings" from the Options menu and select "Novell NetWare

(shell versions 3.21 and above)" for the Network option.

4. Exit Windows Setup.

5. Start Windows and try to print again.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.