STANDARD MICROSYSTEMS CORPORATION

Novell LAN WorkPlace v4.1 for DOS Driver

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to copy the Novell LAN WorkPlace version 4.1 for DOS

driver from the SuperDisk to your PC.

o  How to load the Novell LAN WorkPlace for DOS driver and

establish a network connection.

COPYING THE DRIVER FILES

Copy the DOS ODI workstation file, 8008ODI.COM, from the \DOS_ODI

subdirectory on the SuperDisk to the root directory on your

computer's hard drive.

LOADING THE DRIVER

To install the driver:

1.  Insert the LAN WorkPlace Version 4.1 for DOS Install disk into

the disk drive.

2.  Move to the drive where you inserted the disk. Then type: INSTALL

3.  Press the Enter key.

4.  When prompted to gather information, such as IP address,

router address, subnet mask, BootP server usage, SLIP/PPP driver

usage, Domain server usage, Netware fileserver name, Netware

username, and Netware shell type, check with your System

Administrator to obtain this information.

5.  When prompted to install the DOS or the Windows portion of

LAN WorkPlace, select either "Yes" or "No". None of the choices

you select will affect your adapter installation.

6.  Press the Escape key.

7.  Select "Yes" to save your settings.

8.  Highlight the destination drive where you want to install LAN

WorkPlace and press the Enter key.

9.  Type the Destination Directory where you want to install LAN

Workplace and press the Enter key.

10. Select "No" when prompted to boot from server.

11. Select "Yes" to update your system files. Back-up copies will

be created.

12. Select the "Novell/Eagle NE2000" adapter from the option list.

13. Choose any values for the PORT and INT choices and press the

Escape key. (Your choices will have no effect on the adapter

installation.)

14. Follow the prompts on screen referring to the information you

gathered in step 4 above.

15. Select a frame type that IPXODI will use. Novell Netware 286

v2.x and Netware 386 v3.x default to an Ethernet_802.3 frame

type, Netware 386 v4.x uses an Ethernet_802.2 frame type.

16. Select a username for R-utilites. Usually, your Netware

username is sufficient.

17. After you complete the installation, exit to DOS.

18. Use any text editor to edit the NET.CFG file in the root

directory of your PC's hard drive. Remove the PORT and INT

lines from the Link Driver NE2000 section, and change any

instance of NE2000 to 8008ODI.

19. Save the NET.CFG file.

20. Use any text editor to edit the LANWP.BAT file in the root

directory of your PC's hard drive. Remove the NE2000.COM

statement and replace it with 8008ODI.COM.

21. Save the LANWP.BAT file.

22. Reboot your computer.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.