STANDARD MICROSYSTEMS CORPORATION

FTP PC/TCP v1.2 Driver for OS/2

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Copy the FP PC/TCP version 1.2 for OS/2 driver files from the

SuperDisk to your PC.

o  Prepare your PC's operating system so you can attach to the

network.

COPYING THE DRIVER FILES

1. Copy all the files from the \NDIS subdirectory on the SuperDisk

to a Destination Directory on your PC (the recommended Destination

Directory is C:\PCTCP).

2. Refer to the FTP PC/TCP 1.2 Installation Guide.

3. Copy the 8008NDIS.OS2 file in the SuperDisk \NDIS subdirectory to

the appropriate subdirectory on your computer's hard disk

(the default subdirectory is C:\PCTCP\DRV).

4. Use any text editor to modify your CONFIG.SYS file to include

the following device statement for 8008NDIS.OS2:

DEVICE=C:\PCTCP\DRV\8008NDIS.OS2

5. Use any text editor to make the following changes to the

PROTOCOL.INI file created by FTP in the C:\PCTCP\DRV

subdirectory:

[SOCKET]

Drivername = $socket

Bindings = SMC8008

[SMC8008]

Drivername = NDS8008$

PREPARING YOUR OPERATING SYSTEM

The PC/TCP setup program, TCPINST, is located on the PC/TCP Install

Disk 1. The chapter entitled "Installing PC/TCP" in the PC/TCP 1.2

Installation Guide contains complete instructions for using TCPINST.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.