STANDARD MICROSYSTEMS CORPORATION

IBM LAN Server v2.0 DOS Requester

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to install the IBM LAN

server version 2.0 DOS requester.

1. At the DOS prompt, insert the SMC or IBM LAN Support Program

(LSP) Version 1.30 diskette into your PC's disk drive.

2. Type: DXMAID

3. Press the Enter key three times.

The Environment Information screen appears.

4. Specify the following values for the fields on this screen:

Configuration for this computer?                Yes

Use existing configuration information?         No

Do you have adapter option diskettes?           Yes

Are you configuring for two adapters?           No

Source for LSP                                  A:\

Target for new configuration                    C:\lsp

5. Press Enter to store your choices.

The program prompts you to install the option diskette.

6. Insert the SuperDisk in your source drive and specify the

path: A:\NDIS. Then press Enter.

The necessary files are copied from the SuperDisk to your

target directory on the hard disk.

7. Press Enter three times.

8. Insert the LSP diskette and press Enter.

The Current Configuration screen appears with the option for

the SMC EtherNote ElitePort adapter and protocols already

selected ("IBM IEEE 802.2" and "IBM OS/2 NETBIOS").

9. Press F4. LSP is installed.

10. Use a text editor to specify at least 20 files in the

FILES= line of your CONFIG.SYS file; for example:

files=20

11. Save the CONFIG.SYS file.

12. Change to the \LSP subdirectory.

13. In the PROTOCOL.INI file, change the driver name in the

"[SMCDOSAT.MOD]" field from "SMCMAC$" to "NDS8008$". Then

delete any other statements in this field (such as

Save this file.

14. Copy the file 8008NDIS.DOS from the \NDIS subdirectory

on the SuperDisk to the \LSP subdirectory.

12. Reboot your computer.

13. Insert the DOS LAN Requester Install Diskette.

14. Type: INSTALL

15. Press Enter three times.

The program prompts you for the pathname of the DOS LAN

Requester directory.

16. Specify the pathname for the DOS LAN Requester directory

(C:\DOSLAN is the default). Press Enter.

The program asks which services you require.

17. Highlight either the "Send messages" or "Send, View, Edit, and

Log messages" option. (Normally, you select the second option.)

18. Press Enter to confirm your choice.

19. Specify the workstation's ID and the domain ID. The domain ID

must be the domain server ID and must be the same for all

workstations on the network.

20. Follow the screen prompts to complete the installation.

EtherCard PLUS, EtherNote, ElitePort, and SuperDisk are trademarks,

and SMC and Standard Microsystems are registered trademarks, of

Standard Microsystems Corporation. Other trademarks mentioned

herein belong to their respective companies.