STANDARD MICROSYSTEMS CORPORATION

IBM PC LAN Program Environment

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to install the IBM PC LAN

program environment driver on a workstation and a PC.

PC LAN PROGRAM WORKSTATION INSTALLATION

1. At the DOS prompt, insert the SMC or IBM LAN Support Program (LSP)

Version 1.30 diskette.

2. Type: DXMAID

3. Press the Enter key three times.

The Environment Information screen appears.

4. Specify the following values for the fields on this screen:

Configuration for this computer?                Yes

Use existing configuration information?         No

Do you have adapter option diskettes?           Yes

Are you configuring for two adapters?           No

Source for LSP                                  A:\

Target for new configuration                    C:\lsp

5. Press Enter to store your choices.

The program prompts you to install the option diskette.

6. Install the SuperDisk in your source drive and specify

the path: A:\NDIS.

7. Press Enter.

The necessary files are copied from the SuperDisk to your target

directory on your computer's hard disk.

8. Press Enter three times.

9. Insert the LSP diskette and press Enter.

The Current Configuration screen appears with the SMC Ethernet

adapter and protocols already selected.

10. Press F4. LSP is installed.

11. Use any text editor to edit the CONFIG.SYS file. Specify at least

20 files in the FILES= line; for example:

files=20

Once you save your CONFIG.SYS file, reset your computer.

12. Insert the PC LAN Install Diskette in your computer's diskette

drive.

13. Type: INSTALL

14. Press Enter.

15. Press Enter three more times.

16. Select the level of function for this workstation, either "Basic

Services" or "Extended Services".  Most of the time, select

the second option.

17. Specify "Workstation".

18. Specify either "Send messages" or "Send, receive, and log

messages" options. Most of the time, select the second option.

19. Specify the target path for the PC LAN files. The default

pathname is C:\DOS.

20. Specify the machine's ID and the domain ID. The domain ID must

be the domain server ID and must be the same for all workstations

on the network.

21. Follow the prompts to complete the installation until you reach

the Change System Configuration File screen. Usually, you should

accept the default for the system configuration file.

The Change System Batch File screen appears. Usually you should

accept the default.

PC LAN PROGRAM SERVER INSTALLATION

The following section provides instructions for installing PC LAN

on a server.  This procedure uses the LAN Support Program (LSP)

to facilitate the installation process.

1. At the DOS prompt, insert the LSP diskette.

2. Type: DXMAID

3. Press Enter.

4. Press Enter twice.

The Environment Information screen appears.

5. Specify the following values for the fields on this screen:

Configuration for this computer?                Yes

Use existing configuration information?         No

Do you have adapter option diskettes?           Yes

Are you configuring for two adapters?           No

Source for LSP                                  A:\

Target for new configuration                    C:\lsp

6. Press Enter to store your choices.

The program prompts you to install the option diskette.

7. Insert the SuperDisk in your source drive and specify the

path: A:\NDIS.

8. Press Enter.

The necessary files are copied from the SuperDisk to your

target directory on the computer's hard disk.

9. Press Enter three times.

10. Insert the LSP diskette into your computer's disk drive.

11. Press Enter.

The Current Configuration screen appears with the SMC Ethernet

adapter and protocols already selected.

12. Press F4. LSP is installed.

13. Use any text editor to edit the CONFIG.SYS file. Specify at

least 20 files in the FILES= line; for example:

files=20

Once you have saved the CONFIG.SYS file, reset the computer.

14. Insert the PC LAN Install Diskette in your computer's disk drive.

15. Type: INSTALL

16. Press Enter.

17. Press Enter three more times.

18. Select the level of function for this workstation: either "Basic

Services" or "Extended Services".  Most of the time, select the

second option.

19. Specify "Server".

20. Specify either "Send messages" or "Send, receive, and log

messages" options. Most of the time, select the second option.

21. Specify the target path for the PC LAN files. The default

pathname is C:\DOS.

22. Specify the machine's ID and the domain ID. The domain ID must

be the domain server ID and must be used by all workstations

on the network to communicate with the server.

23. Follow the prompts to complete the installation until you reach

the Change System Configuration File screen. Usually, you should

accept the default for the system configuration file.

The Change System Batch File screen appears. Usually, you should

accept the default.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.