STANDARD MICROSYSTEMS CORPORATION

Windows for Workgroups

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Install the Windows for Workgroups NDIS driver files from

the SuperDisk to your PC.

o  Prepare your PC's operating system so you can attach to the

network.

Note: The SuperDisk includes an OEMSETUP.INF file for Windows

for Workgroups. The OEMSETUP.INF file lets you use MS Windows

dialog boxes to install the SMC Ethernet parallel port LAN

adapter.

Installation procedures:

1. Proceed to install Windows for Workgroups. If you have already an

installed Windows for Workgroups, open the Control Panel Window to

change the network settings.

2. When Windows asks for the type of network adapter, it will list all

network adapters supported by Windows. Select "Unlisted or Updated

Network Adapter".

3. Windows will ask for an OEMSETUP.INF file. Since the SMC OEMSETUP.INF

file is included in the SuperDisk, insert the SuperDisk in drive A.

4. Enter the full path name where the SMC OEMSETUP.INF file is located.

For example: A:\NDIS

5. Press the Enter key.

6. You will see a Windows dialog box, listing all SMC network adapters

included in the OEMSETUP.INF file. Select the network adapter that

you will be using.

7. After selecting the type of SMC network adapter, Windows will look

for the NDIS driver of the network adapter you selected. The

SuperDisk should still be in drive A.

8. Enter the full path name where the SMC NDIS driver is located.

For example: A:\NDIS\DOS\

9. Press Enter.

10. Windows will copy the NDIS driver file and update the Windows network

system automatically. Be sure to configure the network adapter if you

are using an I/O port or IRQ other than the default.

11. After installation is completely finished, restart your system for the

updates to take effect.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.