STANDARD MICROSYSTEMS CORPORATION

Novell Netware IPX Shell

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Copy the Novell NetWare IPX Workstation driver files from the

SuperDisk to your PC.

o  Use SuperDisk with Novell's "WSGEN" process to create a

workstation driver.

o  Prepare your operating system so you can attach to the network.

o  Print to a network printer from Windows.

For your convenience, the SuperDisk includes the files IPX.COM

and NETX.COM. The IPX.COM file is a fully functional IPX driver for

the SMC Ethernet parallel port LAN adapter. You can load this file

directly into your system and bring up the SMC adapter immediately.

If you need to build a special IPX driver for your environment, the

following instructions apply.

COPYING THE DRIVER FILES

The following sections describe how to copy the driver files from

a floppy or hard disk.

Generating a Driver from a Floppy Disk

The following procedure describes how to generate a NetWare IPX

driver from a floppy drive.

Note that NetWare installation procedures require the driver files

to be located in the root directory of a floppy disk whose volume

label is LAN_DRV_SMC. If you copy these files to another floppy

disk, set the volume label of the new disk to LAN_DRV_SMC.

1. Type WSGEN from the floppy drive where you inserted the

WSGEN disk.

2. Press the Insert key on your keyboard.

3. When prompted to insert the LAN_DRV_??? disk, insert the

SuperDisk.

4. Press the Enter key.

5. After the files have been read, press Enter.

6. Select the SMC Ethernet 8008C/PP option and press Enter.

7. Select Yes to generate the IPX.COM file.

8. Copy the IPX.COM file from the WSGEN disk to the appropriate

location on your computer's boot disk.

Generating a Driver from a Hard Disk

To generate the driver from a hard disk:

1. Copy the Netware 2.2 floppies to the hard disk, as described in

the Netware 2.2 manual.

2. Create a directory named LAN_DRV_.SMC at the same level as the

LAN_DRV_.001 directory. Copy all the .OBJ files from the

SuperDisk root directory to this directory.

3. Copy the 8008CSH.LAN file to the subdirectory \WSGEN.

4. Run WSGEN.

5. Select the SMC Ethernet 8008C/PP option and press Enter.

6. Select Yes to generate the IPX.COM file.

7. Copy the IPX.COM file from the WSGEN directory to the

appropriate location on your computer's boot disk.

PREPARING YOUR OPERATING SYSTEM

1. Use any text editor to add the following lines to your

AUTOEXEC.BAT file:

IPX.COM

NETX.COM

2. Reboot your computer.

PRINTING TO A NETWORK PRINTER FROM WINDOWS

If you receive an error message when you try to print to a network

printer from Windows:

1. Exit Windows.

2. Run Windows Setup.

3. Look at the Network option. If this option is set to "Novell

NetWare (shell versions 3.26 and above)," select "Change System

Settings" from the Options menu and select "Novell NetWare

(shell versions 3.21 and above)" for the Network option.

4. Exit Windows Setup.

5. Start Windows and try to print again.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.