nuLIB02.DOC 			05-01-94

Document for nuLogic Library version v1.00

The new nuLib02.lib LARGE MEMORY MODEL library is for programmers who wish to build DOS applications. For those wishing to build to build Windows applications, our Motion02.DLL is the windows equivalent. See our documentation for Motion02.DLL in the DLL subdirectory of your pcRunner install disk.

In order to use this library, we recommend that you include our 'nulib02.h' include file in your source code, use the Large memory-model to compile, and link our library at compile time with your own object files.

This library supports the following types of boards:

board type 0:   pcControl board -- this is a board which was originally designed for 3 axes, not a depopulated 4-axis board

board type 1:   open-loop stepper board -- not a closed-loop board in open loop mode, but a true open-loop stepper board

board type 2:   closed-loop stepper board-- whether operating in open or closed loop mode, it remains a type 2 board

board type 3:   pcControl-4A -- this is nuLogic's more recent servo board

board type 6: pcPosition -- this is a pcControl-4A board without motion capabilities

You should know which type of board you're talking to before using these functions. the verifyaddr function can help to verify the presence of a board at the board address that you designate.

The board address must be a WORD (16 bit unsigned value) in the range of 200 hex to 3F8 hex, and care should be taken not to assign the board to an address that is already in use by one of your computer's peripherals. The address should also be evenly divisible by 8, for instance 200, 208, 210, 218 hex, etc. Please consult your own computer's manuals for more information on open addresses, and consult your nuLogic manual for information on how to set the dip switches.

Function Return Statuses

Every call to the driver has a function return value. This return status indicates whether or not the command completed without error. The following is a list of all the possible function return statuses and their descriptions:

status 0: Successful, no errors

status 1: Communication error, timeout on ready to receive

status 2: Command error on current packet

status 3: Communication error, no data available in the return data buffer

status 4: Communication error, return packet is not complete

status 5: Board failure

status 6: Application error, bad axis number passed to driver

status 7: Communication error, command-in-process bit stuck on

status 8: Command error occured prior to current packet

status 9: Unable to clear command error bit

status 10: Applications error, bad command ID passed to driver

status 11: Communication error, returned packet ID is corrupted

status 12: Application error, bad board ID or board ID not configured

status 13: Application error, bad word count passed to 'send_command'

status 14: Application error, return data does not match request

status 15: Communication error, unable to flush the return data buffer

status 16: Application error, servo command only

status 17: Application error, stepper command only

status 18: Application error, closed loop stepper command only

status 19: Application error, cannot find the board configuration file

status 20: Application error, steps/rev or encoder lines/rev not initialized

status 21: Communication error, system reset did not occur within allotted time

status 22: Application error, closed-loop command only

status 23: Application error, 4-Axis command issued to a 3-Axis board

status 24: Application error, motion command issued to a PCPosition board

status 25: Communication error, return data buffer is not empty

status 26: Application error, bad board address passed to the routine

status 27: Application error, bad board type passed to the routine

status 28: Application error, problem with contouring command

The Return Data Buffer

Most of the 'READ' commands, or any command that returns data other than the function return status, have a corresponding function that generates the data but does not immediately return it. In these cases the data is left in a buffer on the board to be read some time later. This buffer is called the 'Readback Buffer' or the 'Return Data Buffer' and can be read at any time using the 'read_rdb' function. The 'READ' functions that leave data in the Return Data Buffer use a naming convention that ends in '_rdb'. It should also be noted that any 'READ' function that does immediately return data requires that the Return Data Buffer be empty before execution.

The following functions make up the nuLib02.lib library that is included on this disk. These functions are completely stand-alone functions. you may use one, some, or all of these functions in any logical order for your application. A brief description follows each function as they are listed.

Board Level Commands

Most of the functions contained in this device driver correspond directly to the functions supported by nuLogic's motion control boards. These functions are described in the user manuals for the boards. The manuals should be used as a primary reference for programming these functions. The hexadecimal command ID is useful for referring to the user manual. Note that some of the commands pertain to only one type of board. Again, the appropriate user manual will list which functions pertain to a given type of board.

The following gives a list of function prototypes, the corresponding hex command ID and the command name in English as it appears in the appropriate user manual. Pertaining to the function prototypes, BYTE is an unsigned character, eight bits, and WORD is an unsigned int, sixteen bits.

Command ID 0x42: Read Limit Switch Status

int read_lim_stat(WORD boardaddr, WORD boardtype, WORD *switchstatptr);

int read_lim_stat_rdb(WORD boardaddr, WORD boardtype);

Command ID 0x43: Set Step Output Mode & Polarity

int set_step_mode_pol(WORD boardaddr, WORD boardtype, WORD mpdata);

Command ID 0x44: Set I/O Port Polarity & Direction

int set_io_pol(WORD boardaddr, WORD boardtype, WORD pddata);

Command ID 0x45: Set I/O Port Output

int set_io_output(WORD boardaddr, WORD boardtype, WORD podata);

Command ID 0x46: Enable I/O Port Trigger Inputs

int enable_io_trig(WORD boardaddr, WORD boardtype, WORD trigdata);

Command ID 0x47: Read I/O Port

int read_io_port(WORD boardaddr, WORD boardtype, WORD *iodataptr);

int read_io_port_rdb(WORD boardaddr, WORD boardtype);

Command ID 0x48: Trigger I/O Port #

int trigger_io(WORD boardaddr, WORD boardtype, WORD portnum);

Command ID 0x49: Read Per Axis H/W Status

int read_axis_stat(WORD boardaddr, WORD boardtype, BYTE axis, WORD *statptr);

int read_axis_stat_rdb(WORD boardaddr, WORD boardtype, BYTE axis);

Command ID 0x4A: Set Limit Switch Input Polarity

int set_lim_pol(WORD boardaddr, WORD boardtype, WORD polarity);

Command ID 0x4B: Enable Limit Switch Inputs

int enable_limits(WORD boardaddr, WORD boardtype, WORD enabledata);

Command ID 0x4C: Stop Motion

int stop_motion(WORD boardaddr, WORD boardtype, BYTE axis);

The stop_motion function stops the motor as defined in the set_stop_mode (0x55) function: either halt, decelerate, or kill.

Command ID 0x4D: Kill Motion

int kill_motion(WORD boardaddr, WORD boardtype, BYTE axis);

Command ID 0x4E: Load Target Position

int load_target_pos(WORD boardaddr, WORD boardtype, BYTE axis, DWORD position);

Command ID 0x4F: Load Velocity

int load_vel(WORD boardaddr, WORD boardtype, BYTE axis, DWORD velocity);

Command ID 0x50: Reset Position Counter to Zero

int reset_pos(WORD boardaddr, WORD boardtype, BYTE axis);

Command ID 0x51: Find Home

int find_home(WORD boardaddr, WORD boardtype, BYTE axis, WORD direction);

Command ID 0x52: Find Index

int find_index(WORD boardaddr, WORD boardtype, BYTE axis, int revdata, WORD direction, WORD offset, WORD *indexptr);

int find_index_rdb(WORD boardaddr, WORD boardtype, BYTE axis, int revdata, WORD direction, WORD offset);

Command ID 0x53: Read Present Position

int read_pos(WORD boardaddr, WORD boardtype, BYTE axis, DWORD *positionptr);

int read_pos_rdb(WORD boardaddr, WORD boardtype, BYTE axis);

Command ID 0x54: Read Steps/Sec (Velocity)

int read_steps_vel(WORD boardaddr, WORD boardtype, BYTE axis, DWORD *veloptr);

int read_vel_rdb(WORD boardaddr, WORD boardtype, BYTE axis);

Command ID 0x54: Read Present Velocity

int read_vel(WORD boardaddr, WORD boardtype, BYTE axis, WORD *velocityptr);

int read_vel_rdb(WORD boardaddr, WORD boardtype, BYTE axis);

Command ID 0x55: Set Stop Mode

int set_stop_mode(WORD boardaddr, WORD boardtype, BYTE axis, WORD mode);

Command ID 0x56: Load Following Error Count

int load_fol_err(WORD boardaddr, WORD boardtype, BYTE axis, DWORD errorcount);

Command ID 0x57: Load Steps and Lines Per Rev

int load_steps_lines(WORD boardaddr, WORD boardtype, BYTE axis, DWORD stepslines);

Command ID 0x57: Load Filter Derivative Sample Period

int load_deriv_per(WORD boardaddr, WORD boardtype, BYTE axis, WORD period);

Command ID 0x58: Load Filter Proportional Gain

int load_prop_gain(WORD boardaddr, WORD boardtype, BYTE axis, WORD gain);

Command ID 0x59: Read Encoder

int read_encoder(WORD boardaddr, WORD boardtype, BYTE axis, DWORD *encoderptr);

int read_encoder_rdb(WORD boardaddr, WORD boardtype, BYTE axis);

Command ID 0x59: Load Filter Integral Gain

int load_intg_gain(WORD boardaddr, WORD boardtype, BYTE axis, WORD gain);

Command ID 0x5A: Set Loop Mode

int set_loop_mode(WORD boardaddr, WORD boardtype, BYTE axis, WORD mode);

Command ID 0x5A: Load Filter Derivative Gain

int load_deriv_gain(WORD boardaddr, WORD boardtype, BYTE axis, WORD gain);

Command ID 0x5B: Set Base Velocity

int set_base_vel(WORD boardaddr, WORD boardtype, BYTE axis, WORD basevelo);

Command ID 0x5B: Load Filter Integration Limit

int load_intg_lim(WORD boardaddr, WORD boardtype, BYTE axis, WORD limit);

Command ID 0x5C: Multi-Axis Start

int multi_start(WORD boardaddr, WORD boardtype, WORD startdata);

Command ID 0x5D: Trigger Buffer Delimiter

int trig_buff_delim(WORD boardaddr, WORD boardtype);

Command ID 0x5E: Acquire Samples

int acquire_samples(WORD boardaddr, WORD boardtype, WORD numsamples);

Command ID 0x5F: Begin Trigger # Prestore

int begin_prestore(WORD boardaddr, WORD boardtype, WORD trigselect);

Command ID 0x60: End Trigger Prestore

int end_prestore(WORD boardaddr, WORD boardtype);

Command ID 0x61: Start Motion

int start_motion(WORD boardaddr, WORD boardtype, BYTE axis);

Command ID 0x62: Load Acceleration

int load_accel(WORD boardaddr, WORD boardtype, BYTE axis, DWORD acceleration);

Command ID 0x63: Load Acceleration Factor

int load_accel_fact(WORD boardaddr, WORD boardtype, BYTE axis, WORD accfactor);

Command ID 0x63: Set Direction

int set_direction(WORD boardaddr, WORD boardtype, BYTE axis, WORD direction);

Command ID 0x64: Set Position Mode or Set Operation Mode

int set_pos_mode(WORD boardaddr, WORD boardtype, BYTE axis, WORD posmode);

Command ID 0x65: Load Position Breakpoint

int load_pos_brk(WORD boardaddr, WORD boardtype, BYTE axis, DWORD breakpos);

Command ID 0x66: Enable Breakpoint Function

int enable_brk(WORD boardaddr, WORD boardtype, BYTE axis, WORD func);

Command ID 0x67: Load Anticipation Time Breakpoint

int load_time_brk(WORD boardaddr, WORD boardtype, BYTE axis, DWORD time);

Command ID 0x68: Load Position Reference Offset

int load_pos_ref(WORD boardaddr, WORD boardtype, BYTE axis, DWORD offset);

Command ID 0x69: Load Position Scale Factor

int load_pos_scale(WORD boardaddr, WORD boardtype, BYTE axis, DWORD scalefactor);

Command ID 0x6A: Set Scale Factor Sequence

int set_scale_seq(WORD boardaddr, WORD boardtype, BYTE axis, WORD sequence);

Command ID 0x6B: Set Run/Stop Status Pulse Width

int set_rs_pulse(WORD boardaddr, WORD boardtype, BYTE axis, WORD pulse);

Command ID 0x6C: Load Rotary Counts

int load_rot_counts(WORD boardaddr, WORD boardtype, BYTE axis, DWORD countdata);

Other Driver Functions

The following driver functions are not specific to any board level command.

Send Command

int send_command(WORD boardaddr, WORD boardtype, BYTE axis, BYTE cmd, BYTE wcount, DWORD cmddata);

'Send Command' can be used to send any command in the board's command set. The command requires board ID, axis number, the hex command ID, the command packet word count (2, 3 or 4), and 32 bits of command data.

Read the Return Data Buffer

int read_rdb(WORD boardaddr, WORD boardtype, WORD *numptr, WORD *axisptr, WORD *cmdptr, DWORD *rdbptr);

This function is used to read the Return Data Buffer. This function requires pointers to three arrays that will be used to return axis number, command ID and corresponding data for the number of packets specified. The number of packets to be read from the RDB is specified in *numptr. The number of packets actually read will also be returned in numptr.

Read The Communications Status Register

int read_csr(WORD boardaddr, WORD boardtype, WORD *csrptr);

This function returns the contents of the Commmunications Status Register in the location pointed to by *csrptr.

Flush the Return Data Buffer

int flush_rdb(WORD boardaddr, WORD boardtype);

This function will flush the RDB of the board specified in boardaddr.

In Position

int in_pos(WORD boardaddr, WORD boardtype, BYTE axis, DWORD target, DWORD deadband, DWORD *resultptr);

This function compares the current position to the position specified in 'target'. If the current position is within a given deadband from the target, specified in 'deadband', then zero is returned in *resultptr; otherwise, the difference between the current position and the target is returned in *resultptr.

Load RPM

int load_rpm(WORD boardaddr, WORD boardtype, BYTE axis, WORD revdata, double rpm);

This function will take an RPM and load a velocity value to the board that corresponds to that RPM. This function requires that the information containing the number of encoder lines per revolution for pcControl (servo) boards, or the number of steps per revolution for stepper boards has been previously loaded to the driver. These values can be loaded with the 'store_elc' and 'store_steps_rev' functions respectively. For stepper boards, the number of steps per revolution will also get saved when the function 'Load Steps & Lines Per Rev' is issued.

Read RPM

int read_rpm(WORD boardaddr, WORD boardtype, BYTE axis, WORD revdata, double *rpm);

Returns the present velocity in RPM in the location pointed to by *rpm. This requires the number of steps per rev or encoder lines per rev have been previously loaded (see 'Load RPM' above).

Load Revs Per Second Per Second

int load_rpsps(WORD boardaddr, WORD boardtype, BYTE axis, WORD revdata, double rpsps);

Takes an acceleration value in RPS/Sec and loads the appropriate acceleration value to the board. This requires the number of steps per rev or encoder lines per rev have been previously loaded (see 'Load RPM' above).

Verify Address

int verifyaddr(WORD boardaddr, WORD boardtype);

Attempts to communicate with a board at a specified hex I/O address. Returns a positive value if successful. The returned value represents the board type which the board most resembles. Returns a -1 if unsuccessful.