+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

-------------------------

ACER 386SX  M1209 chipset

-------------------------

Filename   Port      Description  386SX M1209

--------   -------   ---------------------------------------------------------

V058B300   ACER300   386SX chipset, no turbo-LED

V058B304   ACER304   x058B300 with turbo-LED active Lo  (TL-)

V058B305   ACER305   x058B300 with turbo-LED active Hi  (TL+)

V058B301   ACER301   no turbo-LED, with fast-A20 optimization  (*)

V058B306   ACER306   (TL-)         with fast-A20 optimization  (*)

V058B307   ACER307   (TL+)         with fast-A20 optimization  (*)

!  Cyrix 486SLC layout with #FLUSH and #A20M.  CCR0 = 14H.                 !

!                                                                          !

!  V058B308   ACER308   386SX chipset, no turbo-LED                        !

!  V058B30C   ACER30C   x058B300 with turbo-LED active Lo  (TL-)           !

!  V058B30D   ACER30D   x058B300 with turbo-LED active Hi  (TL+)           !

!                                                                          !

!  V058B309   ACER309   no turbo-LED, with fast-A20 optimization  (*)      !

!  V058B30E   ACER30E   (TL-)         with fast-A20 optimization  (*)      !

!  V058B30F   ACER30F   (TL+)         with fast-A20 optimization  (*)      !

(*) Board design with pin 61 "EN20/XDIR" pulled hi during

poweron/reset activate internal "fast-A20" mode that

bypasses slower 8042.  BIOS versions marked "*" will

assume "fast-A20" mode and give improved performance.

These "*" versions REQUIRE the "fast-A20" mode, and

will malfunction in board designs where the "fast-A20"

mode is not enabled.

The versions not marked "*" will operate correctly

in both designs, but cannot take full advantage of

this chipset's technology.  (However, the hardware

"fast-A20" still delivers improvement over 8042,

even when BIOS doesn't explicitly know about it).

To see the difference, run PC Magazine's Extended Memory

Test (or Bench20).  There is approximately a 25-30%

throughput improvement with "fast-A20" BIOS versions.

(!) Cyrix-layout versions require the M/Board connect 486SLC

pins #FLUSH and #A20M.  On such designs, use BIOS marked !

to obtain highest performance and best compatibility.

The normal i386SX versions (not marked !) still support

the 486SLC.  But performance is not peak because these

versions enable "BARB" and "NC0" in CCR0.  "BARB" flushes

the cache on each HOLD, and is needed to reduce chance that

DMA/Masters can make cache incoherent.  "NC0" prevents caching

64K blocks at 0M and 1M, and is needed since wraparound could

occur if A20-Gate is toggled on/off (especially by LOADHIGH

and HIMEM.SYS).

------------------------------------------------------------------------------

-------------------------

ACER 386SX  M1217 chipset

-------------------------

Filename   Port      Description  386SX M1217

--------   -------   ---------------------------------------------------------

V060B310   ACER310   386SX chipset, no turbo-LED

V060B314   ACER314   x060B310 with turbo-LED active Lo  (TL-)

V060B315   ACER315   x060B310 with turbo-LED active Hi  (TL+)

V060B311   ACER311   no turbo-LED, with fast-A20 optimization  (*)

V060B316   ACER316   (TL-)         with fast-A20 optimization  (*)

V060B317   ACER317   (TL+)         with fast-A20 optimization  (*)

!  Cyrix 486SLC layout with #FLUSH and #A20M.  CCR0 = 14H.                 !

!                                                                          !

!  V060B318   ACER318   386SX chipset, no turbo-LED                        !

!  V060B31C   ACER31C   x060B318 with turbo-LED active Lo  (TL-)           !

!  V060B31D   ACER31D   x060B318 with turbo-LED active Hi  (TL+)           !

!                                                                          !

!  V060B319   ACER319   no turbo-LED, with fast-A20 optimization  (*)      !

!  V060B31E   ACER31E   (TL-)         with fast-A20 optimization  (*)      !

!  V060B31F   ACER31F   (TL+)         with fast-A20 optimization  (*)      !

(*) Board design with pin 61 "EN20/XDIR" pulled hi during

poweron/reset activate internal "fast-A20" mode that

bypasses slower 8042.  BIOS versions marked "*" will

assume "fast-A20" mode and give improved performance.

These "*" versions REQUIRE the "fast-A20" mode, and

will malfunction in board designs where the "fast-A20"

mode is not enabled.

The versions not marked "*" will operate correctly

in both designs, but cannot take full advantage of

this chipset's technology.  (However, the hardware

"fast-A20" still delivers improvement over 8042,

even when BIOS doesn't explicitly know about it).

To see the difference, run PC Magazine's Extended Memory

Test (or Bench20).  There is approximately a 25-30%

throughput improvement with "fast-A20" BIOS versions.

(!) Cyrix-layout versions require the M/Board connect 486SLC

pins #FLUSH and #A20M.  On such designs, use BIOS marked !

to obtain highest performance and best compatibility.

The normal i386SX versions (not marked !) still support

the 486SLC.  But performance is not peak because these

versions enable "BARB" and "NC0" in CCR0.  "BARB" flushes

the cache on each HOLD, and is needed to reduce chance that

DMA/Masters can make cache incoherent.  "NC0" prevents caching

64K blocks at 0M and 1M, and is needed since wraparound could

occur if A20-Gate is toggled on/off (especially by LOADHIGH

and HIMEM.SYS).

------------------------------------------------------------------------------

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High