+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

--------------------------------------------------

Future Technology Devices International LTD - FTDI

--------------------------------------------------

Filename   Port      Description  82C3480-XM & XMVL  for 386,486,Cyrix

--------   -------   ---------------------------------------------------------

V088B418   FTDI418   386+486+Cyrix  82c3480-XM & XMVL  (upgraded from "AP")

V088B419   FTDI419   x088B418 (SMC/711) - with SMC or 82C711 combo I/O

V088B41B   FTDI41B   x088B418 (NSC 312) - with PC87311/2 combo I/O

V088B41C   FTDI41C   (*)  BL 2X-Osc (50/66 MHz), inverted 8042 IRQs

V088B41D   FTDI41D   (*)  BL 3X-Osc (75/99 MHz), inverted 8042 IRQs

V088B41E   FTDI41E   (*)  BL 2X-Osc (50/66 MHz)   (normal 8042 IRQs)

V088B41F   FTDI41F   (*)  BL 3X-Osc (75/99 MHz)   (normal 8042 IRQs)

(*) Versions '41C-F all contain support for:

(*)

(*)    - NSC-311/2

(*)    - Appian ADI2 VL-Bus IDE

(*)    - IBM Blue Lightning CPU

(*)

(*) Versions '41C and '41D are designed for the IBM "Gala" boards,

(*) or other boards using IBM 8042 with inverted IRQs (instead of

(*) PS/2 style IRQ Flip-Flops).

Filename   Port      Description  82C3480-AP  for 386,486 and 486DLC retrofit

--------   -------   ---------------------------------------------------------

V065B400   FTDI400   386+486  82c3480-AP (original chip)

V065B401   FTDI401   x065B400 (711) - with 82C711  Combo I/O chip

V065B402   FTDI402   x065B400 (NSC) - with PC87310 Super I/O chip

Note - Vers 400,401,402  fully support Cyrix 486DLC in 386 socket

without any special hardware considerations for it.  (To

maintain cache coherence, Cyrix CCR0 "BARB" is set, &tc).

Filename   Port      Description  82C3480  for 386,486 and 486DLC design-in

--------   -------   ---------------------------------------------------------

V065B408   FTDI408   x065B400  design-in Cyrix

V065B409   FTDI409   x065B408  design-in Cyrix (711)

V065B40A   FTDI40A   x065B408  design-in Cyrix (NSC)

Note - Vers 408,409,40A  require specific design-in considerations for

Cyrix 486DLC.  These pins must be connected: #FLUSH, #KEN, #A20M.

These versions give best Cyrix performance (compared with "retrofit"

versions), but will MALFUNCTION unless the board is designed for it.

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High