+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

----------------------

Symphony Labs "WAGNER"

----------------------

Filename   Port      Description  SL82C491 "Wagner"

--------   -------   ---------------------------------------------------------

V086B421   SYML421   SL82C491 Wagner

---------------------

Symphony Labs "HAYDN"

---------------------

Filename   Port      Description  Rev 1.1

--------   -------   ---------------------------------------------------------

V016B401   SYML401   SL82C46X  386sx/386DX/486/Cx

V016B402   SYML402   with  C&T 82C711 multi-IO

V016B403   SYML403   with  NSC PC87310 Super-IO

V016B404   SYML404   x016b401  (TL-)

V016B405   SYML405   x016b401  (TL+)

V016B406   SYML406   x016b402  (711) (TL-)

V016B407   SYML407   x016b402  (711) (TL+)

V016B408   SYML408   x016b403  (310) (TL-)

V016B409   SYML409   x016b403  (310) (TL+)

Filename   Port      Description  Rev 1.2

--------   -------   ---------------------------------------------------------

V071B411   SYML411   SL82C46X  386sx/386DX/486/Cx

V071B412   SYML412   with  C&T 82C711 multi-IO

V071B413   SYML413   with  NSC PC87310 Super-IO

V071B414   SYML414   x071b401  (TL-)

V071B415   SYML415   x071b401  (TL+)

V071B416   SYML416   x071b402  (711) (TL-)

V071B417   SYML417   x071b402  (711) (TL+)

V071B418   SYML418   x071b403  (310) (TL-)

V071B419   SYML419   x071b403  (310) (TL+)

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High