+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

------------------------------------------------------------------------------

----------------------

UMC  WriteBack  82C491

----------------------

Filename   Port      Description  491 Single Chip, 386/486/Cx486 combined

--------   -------   ---------------------------------------------------------

V079B410   UMC_410   386/486/Cx486 491

------------------------------------------------------------------------------

----------------------

UMC  WriteBack  82C48X

----------------------

Filename   Port      Description  REV 0

--------   -------   ---------------------------------------------------------

V037B301   UMC_301   386 WriteBack, Rev 0 *ONLY*

V037B401   UMC_401   486 WriteBack, Rev 0 *ONLY*

Filename   Port      Description  REV A & B

--------   -------   ---------------------------------------------------------

V037B302   UMC_302   386 WriteBack, Rev A,B *ONLY*  (Not REV-0 !)

V037B304   UMC_304   x037b302, for Cyrix layout with A20M and FLUSH

V037B402   UMC_402   486 WriteBack, Rev A,B *ONLY*

V037B40G   UMC_40G   x037B402, for L-Bus M/Boards that hang at PostCode 1D

(PAL bug causes Ready-Hang on WEITEK access)

V037B403   UMC_403   486 WriteBack, Rev A,B *ONLY*, NO Turbo (for 486SX 2X-OSC)

V037B404   UMC_404   x037B402  (TL-)

V037B405   UMC_405   x037B402  (TL+)

V037B406   UMC_406   x037B403  (CL-)  (NO turbo, for 486SX 2X-OSC)

V037B407   UMC_407   x037B403  (CL+)  (NO turbo, for 486SX 2X-OSC)

NOTE!  Some 486 '481 REV-B parts crash with  Cache-Write "1 WS"  option,

which UMC recommends at 16-33MHz.  (Part with problem observed

is marked UM82C481BF, whereas earlier parts marked UM82C481B

worked ok).

The following BIOS versions suggest "2 WS" instead of "1 WS"

for the default Cache-Write WS in the Setup Utility.

V037B40A   UMC_40A   486  REV-B (2WS)

V037B40B   UMC_40B   486  REV-B (2WS)  NO Turbo (for 486SX 2X-OSC)

V037B40C   UMC_40C   x037B40A  (2WS) (TL-)

V037B40D   UMC_40D   x037B40A  (2WS) (TL+)

V037B40E   UMC_40E   x037B40B  (2WS) (CL-)  (NO turbo, for 486SX 2X-OSC)

V037B40F   UMC_40F   x037B40B  (2WS) (CL+)  (NO turbo, for 486SX 2X-OSC)

------------------------------------------------------------------------------

----------------------------

UMC 386SX  UM82C330 Twinstar

----------------------------

Filename   Port      Description  386SX UM82C330 Twinstar

--------   -------   ---------------------------------------------------------

V057B310   UMC_310   386SX chipset

V057B314   UMC_314   X057b310  -  Turbo LED active Lo  (TL-)

V057B315   UMC_315   X057b310  -  Turbo LED active Hi  (TL+)

------------------------------------------------------------------------------

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High