F i l e    I n f o r m a t i o n

* DESCRIPTION

Documentation file for DIABLO.PAS.

* ASSOCIATED FILES

DIABLO.PAS

DIABLO.DOC

}

DIABLO.EXE

by

Rick Housh - CIS PIN 72466,212

Sets parameters for Diablo and compatible printers.

Syntax:  DIABLO [ R T C## L## P## S##]

R   Resets printer to switch defaults - cancels all current settings.

T   Top of form set at the current paper position.  Cancels SkipPerf.

C## Characters per inch, in ## range 10-17 cpi.

L## Left margin setting, in ## range 01-99 spaces.

P## Paper length setting in ## range 01-99 lines, at 6 lines/inch.

S## Skip perforation, range ## lines. (Cancelled by S00, T, or R).

This program is designed to work with Diablo and compatible printers

in much the same way as similar programs work to set up printing

parameters for dot matrix printers.  With it you may reset the

printer to its default switch settings, top of form (at the present

position), characters printed per inch (HMI), left margin (in spaces),

form (paper) length (in lines at 6 lines per inch), and the number

of lines to skip at the end of the page.

To set a left margin smaller than currently set, it is necessary to

reset the printer entirely, by turning it off and on again, or with

the R parameter as the first command on the line, in which case all

parameters must be reset.  Any of the above parameters may be used

in any combination, in any order, although obviously R will cancel

anything before it, and T will cancel any skip perforation command

before it. There must be a space between each parameter.

If you set no paper length at the same time you set skip-perf

the program assumes a paper length of 66 lines (ll inches).

You cannot read the printer switches to know how it is set.

Because of the way the Diablo sets most of these parameters, the

print head must be made to actually move to the column desired

(for setting margins) or the line on the page (for setting bottom

margin for skip-perf).  This is not a problem when the head is

moved, as the program simply issues a carriage return after setting

the margin, but it actually must reject a sheet of paper when

setting the bottom margin for the skip-perf function.  Sorry about

that.  The top margin is never set, so just roll the paper to where

you want to start printing when setting top of form and skip-perf.

The program is NOT menu driven.  All parameters must be issued

from the command line.  This allows its use in a batch file.

For example if you put DIABLO R T C10 L04 P66 S06 in your

AUTOEXEC.BAT file the printer will engage in some activity and

be set up for a page length of 11 inches, will skip 1 inch

at the end of each printed page, printing 60 lines per page,

and will print at 10 characters per inch, left margin 4 spaces.

I have found it very convenient for setting up the printer to

work much as a dot matrix at the DOS level.   With the above

settings a DIR > PRN works just fine, even with a sheet feeder.

Try it.

If you have a dumb mechanical sheet feeder (the kind which

must think the paper is 15 inches when it is really 11),

just use DIABLO P99 S39.  The printer will stop after

printing 60 lines, issue a form feed, and you will be at

the top of the next cut sheet.  Just like a tractor.

It also allows the printing of wide listings, such as assembly

language .LST files to the printer, with cpi settings of 15-17.

A setting of 15 allows printing 120 characters/line on 8 inch paper.

Of course, your print wheel should have a comparable pitch to

be very legible.  I use an Elite 12 at 15 CPI with fairly readable

results, but if anyone knows where I can get a 15 CPI ASCII wheel

I sure wish they would leave me a message at the above Compuserve

address.  It works great on a Brother with a 15 CPI wheel, but

of course they are not ASCII.  I need a Diablo wheel.

There is one undocumented parameter "Q", which I guess I am now

documenting.  On my CItoh F-10 clone, this adds an extra 20

milliseconds to the carriage settling time for higher quality

printing, although it does slow it down noticeably.  You can try

it if you wish, although I don't know whether it will work with

other printers.

DIABLO is written in Turbo Pascal, version 4.0, and the source will

be made available on the Borland Compuserve SIG (GO BORLAND).

"Diablo" is a trademark of the Xerox Corporation.

#