WELCOME TO DRIVE ROCKET DATA ACCELERATION SOFTWARE!

To install Drive Rocket, simply follow these steps:

1) Insert your DRIVE ROCKET diskette into drive A: and type

ROCKET.

Help is available for every option by pressing the F1 key.

2) When the installation has completed, remove the DRIVE ROCKET

diskette from drive A: and boot from the hard disk.

The Drive Rocket manual exists on the diskette and can be accessed

from the main menu or by pressing F1 twice from anywhere in the

program.

HOW DOES DRIVE ROCKET SOFTWARE WORK?

Drive Rocket software makes use of a new technology in most new

IDE (AT interface) hard drives called read-and-write-multiple

mode (also known as multiple block size or MBS).  By using this

new technique, overall data transfer rate (measured in kilo-bytes

per second, Kb/Sec) can be increased up to eighty percent.

The Drive Rocket software will determine if your hard drive(s)

support read-and-write-multiple mode.  Drive Rocket will then run

a series of non-destructive benchmarks on all multiple block size

(MBS) values supported, and I/O sizes from .5K to 64K, for three

seconds each, to determine which MBS value will give your hard

drive(s) the best overall data transfer rate.  When the benchmarks

have been completed, a graph showing a before-and-after

comparison of your hard drives performance will be displayed.

Drive Rocket software may increase your performance even if the

drive does not support Multiple Block Size, especially if your

system BIOS does not support shadowing. If shadowing is not

enabled, each I/O request is handled by the system ROM BIOS. When

Drive Rocket Software is installed, its device driver is handling

each I/O in RAM which is faster then ROM.

As always, there will be no charge to you for technical

support services.  Thank you!

There is a Windows bitmap file included on this diskette.  You can use

it as your desktop wallpaper.  Simply copy ROCKET.BMP to your Windows

directory by entering the following command at the DOS prompt:

copy a:rocket.bmp c:\windows <ENTER>

Refer to your Windows manual on how to choose your desktop wallpaper.

The Serial Nu,ber Is :89549   Strider[GNX]