====================================================================

Wireless Network Management Sample Programs

for NetWare Management System Version 1.1

(c) Copyright International Business Machines Corporation - 1994 -

All rights reserved

====================================================================

Contents

--------

A) Product Type and Conditions of Use

B) Prerequisites

C) Installation Procedure

D) IBM Wireless Network Management Sample Program Description

D.1) Basic Configuration

D.2) Sample Profiles (SNMP MIB Browser)

D.3) Event Configuration (Trap Customization)

E) Examples of Utilization

F) Appendix:  File List

A) Product Type and Conditions of Use

----------------------------------

The IBM Wireless LAN Utilities diskette contains the Wireless MIB, the

trap definition file, and a number of ready-to-use Wireless Profiles

built through the NMS SNMP MIB Browser (Edit Profile).  See the file

list in the Appendix.

The product is provided as a list of SAMPLE files shipped on a

3.5 inch diskette (DOS Format).

Since this software is provided as SAMPLE configuration files for

managing Wireless Workstations, there will be no associated service.

These files are distributed without any IBM warranty. However, they

were reviewed and tested as any regular IBM software product.

B) Prerequisites

-------------

The Wireless Network Management Sample Program requires no prerequisites

in addition to those required by the NetWare Management System.

Wireless devices, however, must already have been installed on your

network.

Installation can be completed only on a PS/2 where NetWare Management

System Version 1.1 is installed.

C) Installation Procedure

----------------------

This procedure is divided into three major steps:

1. Install the sample files:

a. Insert the IBM Wireless LAN Utilities diskette in the drive.

b. Go to the DOS command line.  If you are under the Windows

desktop, run the MS-DOS program item to get the command

line.                                                      ,

c. Change to the NMS drive and directory (as chosen during

installation of NMS), by typing, for example:

cd nms

d. Change the current drive, by typing:

a:

e. Change the current directory, by typing the two commands:

cd wlsample

cd nms

f. Run the installation batch file, by typing:

instnms a: c:

where a: is the drive where the IBM Wireless LAN Utilities

diskette is inserted, and c: is the NMS drive.

This script copies the sample files to the NetWare Management

System directories, and updates the configuration information.

You do not need to restart NMS to take account of changes.

But NMS must be running to continue the installation procedure.

2. Load the Wireless MIB into the NMS database:

a. In the NMS Main Window, select the SNMP MIB Compiler...

option from the Tools pull-down menu.

b. The SNMP MIB Compiler dialog box asks for your confirmation.

Click on the Yes pushbutton.

c. When the message that MIB compilation is complete appears,

press Q.  You will return to the NMS Main Window.

The wireless MIB is then stored in the NMS database.

3. Integrate the Wireless Traps into NMS:

a. In the NMS Main Window, select the SNMP Alarm Integrator...

option from the Tools pull-down menu.

b. The Alarm Integrator dialog box shows the list of trap

definition files present on your disk.  Select the file

IBMWLTRP.MIB and click on the OK pushbutton.

c. You will be asked for confirmation.  Click on the Yes

pushbutton.

d. When the Alarm Integrator completion message appears, click

on the OK pushbutton.

e. Close the Alarm Integrator dialog box.

The wireless traps are then stored in the NMS database.

IBM Wireless Network Management Sample Program installation is now complete.

You can now traverse the MIB tree and select objects from the Wireless MIB

when you use the SNMP MIB Browser (Edit profile operation).

New profiles appear in the profile list of the SNMP MIB Browser window.

Try one of these profiles by selecting it and clicking the OK pushbutton

in the MIB Browser window.

D) IBM Wireless Network Management Sample Program Description

----------------------------------------------------------

IBM Wireless Network Management Sample Programs let you display two

types of information, useful for managing the Wireless Network workstations:

- Configurations

- Statistics

Configuration for both wireless standard traps and specific traps is

also provided.

The purpose of these sample files is to offer basic Wireless Network

management using the standard NMS tools provided.

This complements the way Wireless MIB variables are retrieved using

the SNMP MIB Browser tool.

You may decide to build your own files, using these sample files as

a reference.

D.1) Basic Configuration

-------------------

* MIB

Note: It was decided to use the following MIB subtree as a repository for

the IBM Wireless objects:

1 . 3 . 6 .   1    .   4   .     1     . 2 .  6    . 10

(iso.org.dod.internet.private.enterprises.ibm.ibmProd.IBM Wireless LAN)

* Community Name

The community names must be correctly set between the SNMP agent of the

base stations you want to manage and the NMS platform.

For detailed information about this configuration, refer to:

- "IBM Wireless LAN Installing and Operating Your Network" (SA33-0232)

- "NetWare Management Map Installation Guide"

D.2) Sample Profiles (SNMP MIB Browser)

----------------------------------

These applications were built using the NMS integrated SNMP MIB Browser

(Edit Profile Tool).

You must first select a Wireless Base Workstation on the NMS Segment

View, or in the IP address list of the SNMP MIB Browser dialog box.

See the section Examples of Utilization for more details.

The Wireless sample programs let you display:

- Configuration of the selected wireless base station and its

associated wireless workstations.

- Vital Product Data for the selected wireless base station and

its associated wireless workstations.

- Statistics about the Radio Protocol, the modem, the traffic,

and so on.

Five profiles have been defined:

WLBSCFDS.PRF       Description of the Wireless Base Station:

------------

A form displays the Base Station Name, the Network

Name, and the IP Address of the base station

housing the Wireless Network Administration Program.

WLBSCFAD.PRF       Adapter Card Configuration:

------------

This table displays the configuration and the Vital

Product Data of each wireless interface plugged into

the selected base station.

WLBSST.PRF         Statistics on the Base Station:

----------

A graph displays the evolution of all Counters and

Gauges of the selected wireless interface.  See the

section Examples of Utilization, for information on

how to select an adapter through the Edit Profile

Dialog Box.

WLWKCF.PRF         Workstation Configuration:

----------

This table displays the configuration and the Vital

Product Data of each wireless workstation registered to

the selected base station.

WLWKST.PRF         Statistics on a Wireless Workstation:

----------

A graph displays the evolution of all Counters and Gauges

of the selected wireless workstation.  See the section

Examples of Utilization, for information on how to select

a workstation through the Edit Profile Dialog Box.

D.3) Event Configuration (Trap Customization)

----------------------------------------

Simple wireless traps have been defined in a trap MIB (according to

RFC-1215 and NMS syntax).

Wireless traps are therefore recognized by the NMS Alarm Monitor.

E) Examples of Utilization

-----------------------

Example 1

---------

If you want to see information about data received by a known

workstation (identified by its own name assigned through the

Wireless Network Administration Program during its installation),

follow this procedure:

1. Click on the SNMP MIB Browser... option on the Tools pull-down menu.

2. Select the wireless base station that the workstation is registered

under. To find this base station, you can use the IP address

list in the SNMP MIB Browser Dialog Box. This address is

usually known.

3. In the SNMP MIB Browser Dialog Box, select the profile

"WLWKCF.PRF ... Workstation Configuration" from the profile

group box.  Click on the OK pushbutton.

4. In the table displayed, you can see the list of all the workstations

registered to the selected base station.

5. Record the 'remIfIndex' and the 'remIndex' on the line where

the workstation name is displayed.  These parameters correspond to

the wireless interface number and the workstation number in the cell.

You will need this information to continue.

6. Close the window.

7. Click on the SNMP MIB Browser... option on the Tools pull-down menu.

8. In the SNMP MIB Browser Dialog Box, select the profile

"WLWKST.PRF ... Workstation Statistics" from the profile

group box.  Click on the Edit pushbutton.

9. In the SNMP Profile Editor Dialog Box, select "wrRemStatsEntry"

from the Attribute Selection group box.  Click on the

Row Select... pushbutton.

10. In the Row Selection Criteria Dialog Box, select

"remStatsIfIndex (Integer)" from the Index Columns group box.

In the Value entry field, enter the "remIfIndex" value that you

recorded in Step 5.

11. Still in the Row Selection Criteria Dialog Box, select

"remStatsIndex (Integer)" from the Index Columns group box.

In the Value entry field, enter the "remIndex" value that you

recorded in Step 5.

12. Click on the Add pushbutton of the Row Selection Criteria dialog

box.

13. Click on the Save pushbutton of the SNMP Profile Editor dialog

box.

14. Click on the OK pushbutton of the SNMP MIB Browser dialog box.

A graph of statistical information for the selected workstation is

displayed.

Example 2

---------

If you want to see information about data received by an adapter in

a base station, follow this procedure:

1. Click on the SNMP MIB Browser... option on the Tools pull-down menu.

2. Select the wireless base station.  Use the IP address

list in the SNMP MIB Browser Dialog Box. This address is

usually known.

3. In the SNMP MIB Browser Dialog Box, select the profile

"WLBSCFAD.PRF ... Wireless Base Adapters" from the profile

group box.  Click on the OK pushbutton.

4. Record the "baseIfIndex" on which you want statistics, as

displayed in the table.

5. Close the window.

6. Click on the SNMP MIB Browser... option on the Tools pull-down menu.

7. In the SNMP MIB Browser Dialog Box, select the profile

"WLBSST.PRF ... Wireless Base Statistics" from the profile

group box.  Click on the Edit pushbutton.

8. In the SNMP Profile Editor Dialog Box, select "wrRemStatsEntry"

from the Attribute Selection group box.  Click on the

Row Select... pushbutton.

9. In the Row Selection Criteria Dialog Box, in the Value entry

field, enter the "baseIfIndex" value that you recorded in Step 3.

10. Click on the Add pushbutton of the Row Selection Criteria dialog

box.

11. Click on the Save pushbutton of the SNMP Profile Editor dialog

box.

12. Click on the OK pushbutton of the SNMP MIB Browser dialog box.

A graph of statistical information for the adapter in the base station

is displayed.

F) Appendix:  File List

--------------------

-- Files and Directories ------------------- Contents --------------

->WLSAMPLE directory        Wireless Network Management Sample Programs

README.DOC                General Installation Information

->WLMIB directory         Wireless MIB

IBM-WLAN.MIB         Wireless MIB Definition File

->NMS directory           Samples for NMS

INSTNMS.DOC          This explanation file you're reading

INSTNMS.BAT          Installation Script (for DOS shell)

->WLPROFS directory    Sample Profiles

WLBSCFDS.PRF       Description of the Wireless Base Station

WLBSCFAD.PRF       Adapter Card Configuration

WLBSST.PRF         Statistics on Base Station

WLWKCF.PRF         Wireless Workstation Configuration

WLWKST.PRF         Statistics on Wireless Workstations

->WLTRAPS directory    Wireless Traps

IBMWLTRP.MIB       Wireless Traps Definition File