Chapter 1

This chapter describes technical issues related to network adapters and protocols for Windows 95, and also presents some technical notes and tips for networking.

Network Technical Discussion

Network Adapter Drivers and Protocols: The Basics    3

¨ In This Chapter

NDIS Driver Basics    3

Network Protocols Basics    4

Issues for Network Adapters and Protocols    5

Choosing Drivers for Network Adapters    6

Choosing the Protocols to Use    7

Installing Drivers and Protocols    9

Adding a Driver or Protocol after Setup    9

Installing Drivers and Protocols in Custom Setup Scripts    11

Network Adapters and Windows 95    13

Configuring Network Adapters    13

Setting LAN Adapter Numbers    19

Technical Notes on Network Adapters    20

IPX/SPX-Compatible Protocol    21

Configuring IPX/SPX-Compatible Protocol    21

Using NetBIOS Over IPX    24

IPX/SPX Technical Notes on NetWare Networks    25

TCP/IP Protocol    26

Configuring TCP/IP with DHCP    27

Configuring TCP/IP Settings Manually    28

Name Resolution for Windows Networking    31

Using WINS for Name Resolution    33

Using DNS for Name Resolution    35

Microsoft NetBEUI Protocol    39

Technical Notes for Windows 95 Networking    40

Using WinPopup to Broadcast a Pop-Up Message    40

Troubleshooting Protocol Problems    42

# Network Adapter Drivers and Protocols: The Basics

A network adapter (sometimes called a network interface card, or NIC) is a hardware card that is installed in a computer so it can function on a network. The network adapter provides one or more ports for the network cable to physically connect to, and it physically transmits data from the computer onto the network cable and vice versa.

Every network computer must have a network adapter driver, which is the software that controls the network adapter. Each network adapter driver is specifically configured to run with a certain type of network adapter.

In addition to the network adapter and its driver, a network computer must also have a protocol driver (sometimes called a transport protocol or just a protocol). The protocol driver works between the upper-level network software and the network adapter to package data that is to be sent on the network in a way that computers on the receiving end can understand. The process of associating a protocol driver with the network adapter and establishing a communication channel between the two is called binding.

For two computers to communicate on a network, they must use identical protocols. Sometimes, a computer is configured to use multiple protocols. In this case, two computers only need to have one protocol in common to communicate. For example, a computer running File and Printer Sharing for Microsoft Networks that uses both NetBEUI and TCP/IP can communicate with computers using only NetBEUI and with computers using only TCP/IP.

In Windows 95, all network adapter drivers and protocols are installed and configured using the Network option in Control Panel, rather than by manually editing configuration files. Configuration values are stored in the Registry.

## NDIS Driver Basics

Windows 95 supports the Network Device Interface Specification (NDIS) versions 2.x and 3.1 protocol and adapter drivers, and provides an NDIS 3.1 replacement for every version 3.0 driver. This section summarizes the benefits gained from NDIS drivers in Windows 95.

By using NDIS 3.1 drivers, Windows 95 can support a wide range of network media including Ethernet, Token Ring, and ArcNet®. The NDIS 3.1 specification accommodates Plug and Play features, so that in many cases network adapters can be added and removed dynamically while the computer is running. The related features and benefits are summarized in the following discussion.

Because of Plug and Play enhancements to the Protocol Manager and Media Access Control (MAC) layer, the operating system can automatically determine which adapters the protocol should bind to. If a Plug and Play event occurs, such as removing a PCMCIA network adapter from a portable computer, the NDIS 3.1 protocols and network adapter drivers remove themselves from memory automatically. (This Plug and Play capability is supported for most PCMCIA adapters, but not for most ISA adapters.)

###### Plug and Play support for network protocols and adapters.

The minidriver provided by the adapter manufacturer implements only the half of the MAC layer that is specific to the network adapter, which includes establishing communications with the adapter, turning on and off electrical isolation for Plug and Play, providing media detection, and enabling any value-added features the network adapter may contain.

###### New NDIS mini-driver model, dividing the MAC layer into two halves.

The NDIS wrapper provided in Windows 95 implements the half of the MAC functionality that is common to all NDIS drivers. The new minidrivers are faster and are roughly 40 percent smaller than earlier versions of NDIS 3.x network adapter drivers. The Windows 95 minidrivers are also binary-compatible with Windows NT 3.5 minidrivers, which means they can be used without recompiling. (You can recognize a minidriver by its .VXD or .SSS filename extension.)

For NDIS 2.x protocols under Windows 95:

·	An NDIS 2.x protocol driver must use an NDIS 2.x network adapter driver. Both protocol drivers and network adapter drivers must load and bind in real mode before launching Windows 95. Windows 95 uses values in PROTOCOL.INI to load the real-mode drivers, as described in Chapter 8, “Windows 95 on Microsoft Networks.”

·	When you run the real-mode network (for example, when using Safe Mode with Networking for system startup), Windows 95 uses NDIS 2 versions of NetBEUI and IPX/SPX protocols. These protocols are not intended for everyday use since Windows 95 supplies faster protected-mode versions of these protocols. Real-mode NDIS 2 NetBEUI and IPX/SPX-compatible protocols are provided for client computers that start from a floppy disk and run a shared copy of Windows 95 from a server.

Windows 95 also supports ODI drivers with Novell® NetWare®-compatible network clients. For information, see Chapter 9, “Windows 95 on NetWare Networks.”

## Network Protocols Basics

Protocols for networking components in Windows 95 are implemented as 32-bit, protected-mode components. Windows 95 can support multiple protocols simultaneously. Protocol stacks can be shared among the networks that are installed. For example, the Microsoft IPX/SPX-compatible protocol can serve both the needs of the Microsoft Client for NetWare Networks and Client for Microsoft Networks. These standard protocols are included with Windows 95:

This is the new default protocol for Windows 95 and is compatible with the Novell NetWare IPX/SPX implementation. This protocol can be used to communicate with servers on NetWare and Windows NT Server 3.5 networks. This protocol is routable and runs on most network bridges, routers, and so on, that are designed for IPX/SPX routing. The IPX/SPX-compatible protocol also supports for packet-burst mode to offer improved network performance, and supports both the Windows Sockets and NetBIOS programming interface.

###### Microsoft IPX/SPX-compatible protocol.

This protocol is a 32-bit VxD that offers high performance and uses no conventional memory. This is a full TCP/IP implementation that includes several of the more commonly used command line utilities, which include TELNET, FTP, ARP, PING, ROUTE, NETSTAT, NBSTAT, IPCONFIG, and TRACERT. Microsoft TCP/IP supports a NetBIOS programming interface and both 16-bit and 32-bit Windows Sockets for compatibility with Windows Sockets applications. Windows 95 also provides DHCP and WINS clients, to take advantage of the Dynamic Host Configuration Protocol (for automatic TCP/IP configuration using Windows NT DHCP servers) and Windows Internet Naming Service (for automatic IP-address-to-computer name resolution using Windows NT WINS servers).

###### Microsoft TCP/IP.

This protocol is compatible with existing networks that use NetBEUI, including Windows for Workgroups peer networks, Windows NT Server, LAN Manager, and other networks. A NetBIOS programming interface is also supported.

###### Microsoft NetBEUI.

All three protocols are Plug and Play-compliant. This means, for example, that if the network is unavailable either due to undocking a portable computer, or removal of a PCMCIA network adapter, Windows 95 continues to run. The protocol stacks unload themselves after having notified any dependent applications that they will be unloaded from the system. This also means protocols can automatically be loaded. For example, if a mobile user goes from a network attached to an infrared (IR) line-of-sight network, TCP/IP can be unloaded and the appropriate IR protocol loaded automatically.

Other protocols that appear under the Microsoft list in the Select Network Protocol dialog box have specific uses. The Microsoft DLC protocol is described in Chapter 10, “Windows 95 on Other Networks.”

# Issues for Network Adapters and Protocols

Windows 95 does not support NDIS 3.0 protocol drivers, because Windows 95 uses new conventions for calling drivers that match those used in Windows NT. Windows 95 includes an NDIS 3.1 replacement for all 3.0 drivers.

This section provides specific guidelines for choosing network adapter drivers and for choosing protocols to support your networking needs.

## Choosing Drivers for Network Adapters

When setting up Windows 95 for networking, you must choose which types of network adapter drivers and protocols to use. Because Windows 95 has an open architecture, you have a lot of flexibility in this decision. Windows 95 supports both the Network Driver Interface Specification (NDIS) and Transport Driver Interface (TDI) standards. These standards allow Windows 95 to communicate with many other networking products and make it possible for you to choose from a variety of network adapters and protocols for the network.

Network adapters have become exceptionally reliable and inexpensive. The low costs of Ethernet adapters, including new Plug and Play hardware, means the most cost-effective way to improve network performance is usually to replace an older network adapter with a new model. The cost for the new hardware is almost immediately offset by savings in support time and improved performance.

Tip

In choosing a new network adapter, you should also consider buying an adapter that matches the computer bus. For example, PCI network adapters are available for use in PCI computers.

### Choosing Separate Protocol and Adapter Drivers

With some networks, each computer’s network adapter driver and protocol are separate pieces of software. With other networks, a single piece of software called a monolithic protocol stack performs the functions of both the network adapter driver and the protocol. Microsoft recommends that you choose separate 32-bit protected-mode protocols and drivers rather than monolithic stacks that combine the protocol and adapter driver, because monolithic software runs only in real mode.

Windows 95 includes mapping technology for users who continue to use real-mode NDIS 2.x or ODI network adapter drivers.

### Choosing NDIS Drivers

Windows 95 supports NDIS versions 2.x and 3.1 (as described in “NDIS Driver Basics” earlier in this chapter), which lets you use separate protocols and network adapter drivers. All network adapter drivers and protocols shipped with Windows 95 conform to NDIS. You can use any combination of one or more protocol drivers that conform to NDIS along with any network adapter driver that conforms to NDIS.

It is probable that the computers on your network have different types of network adapters and thus require different network adapter drivers. Because of NDIS, you can use exactly the same protocol driver with each of these workstations and are not required to have a different version of the protocol for each type of network adapter, as would be required if you were using monolithic protocol stacks.

Also, NDIS allows multiple protocols to use a single network adapter. Usually, when you use a monolithic protocol, it monopolizes the network adapter, preventing you from using other protocols with that adapter.

### Choosing ODI Adapters

The Open Datalink Interface (ODI) specification was defined by Novell and Apple Computer to provide a protocol and a consistent API for communicating with a network adapter driver, and to support the use of multiple protocols by a network adapter driver.

To ensure the most flexibility in Windows 95 for supporting other protocols along with NetWare integration, Microsoft recommends that you use NDIS 3.1 drivers whenever possible if you are running Windows 95 on NetWare networks and using a Microsoft-supplied network client. If you are using a Novell-supplied network client, Novell recommends using ODI-based client software rather than monolithic IPX drivers. For information about ODI drivers, see Chapter 9, “Windows 95 on NetWare Networks.”

## Choosing the Protocols to Use

Windows 95 is based on the NDIS architecture and can support multiple network protocols simultaneously as described in Chapter 32, “Windows 95 Network Architecture.” Protocol software can be shared among the network providers that are installed. The protected-mode protocols included with Windows 95 are the Microsoft implementation of the IPX/SPX-compatible protocol, Microsoft TCP/IP, and NetBEUI, which are all separate virtual device drivers (VxDs). The real-mode protocols included with Windows 95 are IPX/SPX-compatible and NetBEUI. Technical details about each of the three protocols are provided in the following sections.

The protocols provided with Windows 95 (sometimes called protocol stacks) can be shared among the installed networks. For example, a single implementation of the IPX/SPX-compatible protocol can serve the needs of both Client for Microsoft Networks and Client for NetWare Networks.

Notice, however, that when choosing the network protocol, you can use more than one protocol on the network to ensure communication compatibility with all systems in the enterprise. However, multiple protocols can mean more network traffic and noise, more memory used on the local workstations, and more network delays. You may want to establish a single, standard protocol for use wherever possible.

All protected-mode protocols provided with Windows 95 are Plug and Play-compliant. This means that if the network is unavailable, either due to an undocking procedure or removal of a PCMCIA card, the operating system continues to run. The protocol software can unload itself after having notified any dependent applications that the protocol will be unloaded. Conversely, docking or adding a PCMCIA network adapter will cause the system to dynamically load the protocols.

The following sections discuss uses, advantages, and disadvantages of each protocol provided with Windows 95.

### Choosing IPX/SPX-Compatible Protocol

Microsoft provides an NDIS-compliant version of the Internetwork Packet Exchange (IPX/SPX) protocol that is used on Novell NetWare networks. This protocol appears in the Network option in Control Panel as IPX/SPX-Compatible Protocol.

This protocol is compatible with the Novell NetWare IPX/SPX protocol, and it provides 32-bit, protected-mode support for protected-mode network clients such as Microsoft Client for NetWare Networks and Client for Microsoft Networks. The IPX/SPX-compatible protocol in Windows 95 supports NetBIOS and Windows Sockets, but does not support TDI.

The IPX/SPX-compatible protocol provides the following benefits:

·	Provides support for any Novell NetWare-compatible network client

·	Provides 32-bit, protected-mode protocol for use with any 32-bit, protected mode network client

·	Allows a single protocol in combined NetWare and Windows NT network environments

·	Provides a routable protocol

### Choosing TCP/IP

TCP/IP stands for Transmission Control Protocol/Internet Protocol. It was developed in the late 1970s,as the result of work by the Defense Advanced Research Projects Agency (DARPA) on network interconnection. The major advantages of TCP/IP are that it is a standard, routable protocol, and it is the most complete and accepted protocol available. It provides communication across interconnected networks with different operating systems and hardware architectures, such as UNIX®, mainframes, and Microsoft networks.

TCP/IP also provides compatibility with the Internet, the collection of networks and gateways linking together many worldwide universities, corporations, government offices, and military installations.

Microsoft TCP/IP supports the Windows Sockets 1.1 interface, a cross-platform client-server framework that is ideal for developing client-server applications that can run with Windows Sockets-compliant stacks from other vendors. Microsoft TCP/IP uses the NetBIOS interface commonly known as NetBIOS over TCP/IP. Several TCP/IP utilities are also installed when you install Microsoft TCP/IP.

Microsoft TCP/IP provides the following benefits:

·	Provides connectivity across operating systems and hardware platforms

·	Provides Internet connectivity

·	Provides a routable protocol

·	Includes Windows Sockets 1.1 support

TCP/IP is generally known to require careful planning and management of the IP address space. However, this problem is vastly reduced when Windows NT DHCP servers are in place on the network to manage IP address assignment.

### Choosing Microsoft NetBEUI

NetBIOS extended user interface (NetBEUI) was first introduced by IBM in 1985. It is a small, efficient, and fast protocol.

When NetBEUI was developed in 1985, it was assumed that LANs would be segmented into workgroups of 20 to 200 computers and that gateways would be used to connect one LAN segment to other LAN segments or to a mainframe. NetBEUI is optimized for very high performance when used in departmental LANs or LAN segments. For traffic within a LAN segment, NetBEUI is the fastest of the protocols shipped with Windows 95. The version of NetBEUI shipping with Windows 95 is completely self-tuning and provides much better performance over slow links than did previous versions of NetBEUI.

Because NetBEUI is very fast for small LAN communications but is not a routable protocol, one recommended method for setting up a network is to use both NetBEUI and another protocol, such as TCP/IP, on each computer that may need to access computers across a router or on a WAN.

When you install both protocols on each computer and set NetBEUI as the first protocol to be used, Windows 95 uses NetBEUI for the communication between computers within each LAN segment, and uses TCP/IP for communication across routers and to other parts of your WAN.

# Installing Drivers and Protocols

Windows 95 supports up to four network adapters in a single computer. (Network adapters are also called network interface cards, or NICs.) Windows 95 Setup automatically detects most network adapters, installs the appropriate driver for the adapter, and provides appropriate default settings to configure the adapter.

## Adding a Driver or Protocol after Setup

When you install a network adapter in a computer, you can install the appropriate driver using the Network option in Control Panel. The network adapter driver is automatically bound to all NDIS protocols currently running on the computer, and if any protocols are added later, they will also be bound automatically to the network adapter driver. When you choose to add a network adapter, Windows 95 displays a list of supported network adapters.

For specific information about PCMCIA adapters, see Chapter 19, “Devices.”

¨	To add a driver for a network adapter

1.	In the Network option in Control Panel, click the Configuration tab, and then click the Add button. Double-click Adapter.

2.	In the Manufacturers list in the Select Device dialog box, select a network adapter manufacturer.

3.	In the Models list, select the appropriate model, and then click OK.

The following procedure shows how to add a network protocol provided by Microsoft. This procedure assumes that Windows 95 has already been installed with networking support, and that you need to add an additional protocol or want to change the protocol used on a particular computer.

¨	To add a network protocol provided by Microsoft

1.	In the Network option in Control Panel, click the Configuration tab, and then click the Add button.

2.	In the Select Network Component Type dialog box, double-click Protocol.

3.	In the Select Network Protocol dialog box, select Microsoft in the Manufacturers list. From the Models list, select the protocol you want. Then click OK.

## Installing Drivers and Protocols in Custom Setup Scripts

You can specify the network adapter drivers and protocols to be installed with Windows 95 in a custom setup script, as described in Chapter 5, “Custom, Automated, and Push Installations.” You can also specify configuration parameters in scripts for specific network adapter drivers and for Microsoft TCP/IP and the IPX/SPX-compatible protocols.

¨	To install network adapter drivers and protocols in custom setup scripts

·	In the [network] section of MSBATCH.INF, specify the values that define the network adapter drivers and protocols to be used, as described in the following list.

| [network] entry | Description |
| --- | --- |
| Protocols= | Specify a comma-separated list of protocols to be loaded, from the following values: NETBEUI (for Microsoft NetBEUI) NWLINK (for the IPX/SPX-compatible protocol) MSTCP (for Microsoft TCP/IP) |
| NetCards= | Specify a comma-separated list of network device IDs, as specified in the Windows 95 INF file. |
| IgnoreDetectedNetCards= | Specify a value of 1 if you do not want Windows 95 Setup to detect the computers network adapters. In this case, Setup will use the value of NetCards=. |

The following shows an example of a statement that installs the IPX/SPX-compatible protocol with an Intel® EtherExpress™ network adapter, without detecting the network adapter during Setup:

[network]
netcards=deviceID

protocols=nwlink
IgnoreDetectedNetcards=1

Some network adapter drivers have parameters that can be configured, as defined in the INF file for that network adapter driver. You can use the same syntax and values defined in the INF file to set values in a custom setup script. INF file contains the master information for detecting and configuring network adapters.

¨	To configure network adapter drivers in custom setup scripts

·	In the related [netcard_ID] section in MSBATCH.INF, add statements to define configuration values for the network adapter installed in a netcard= entry in the [network] section.

For example, the following shows the entries for the COMPAQ® Elite Ethernet Controller (*CPQA060 is the device ID for this adapter):

[*CPQA060]

IRQConfig=3,5,9,10,11

IOConfig=10@200-3FF%FFF0(3FF::)

For a list of entries for common network adapters, and information about how to find values for a specific adapter, see Appendix D, “MSBATCH.INF Parameters.”

¨	To configure Microsoft TCP/IP in custom setup scripts

·	In the [MSTCP] section of MSBATCH.INF, define values for this protocol’s parameters, as summarized in Appendix D, “MSBATCH.INF Parameters.”

For example, if your network uses DHCP to configure TCP/IP parameters, and uses WINS but not DNS, add the following section to MSBATCH.INF:

[mstcp]
dhcp=1
wins=dhcp

¨	To configure the IPX/SPX-compatible protocol in custom setup scripts

·	In the [NWLINK] section of MSBATCH.INF, define values for this protocol’s parameters, as summarized in Appendix D, “MSBATCH.INF Parameters.”

For example, if your network uses 802.3 frames and must support NetBIOS applications over IPX, add the following section to MSBATCH.INF:

[nwlink]
frame_type=0
netbios=1

# Network Adapters and Windows 95

This section provides technical details for the following topics:

·	Configuring network adapters

·	Setting LAN adapter numbers

·	Technical notes on network adapters

## Configuring Network Adapters

The following procedures show how to configure various properties of a network adapter driver.

¨	To configure properties for network adapter drivers

·	In the Network option in Control Panel, click the Configuration tab. Then select the driver in the list and click the Properties button.

¨	To specify the driver type for a selected network adapter

1.	On the property sheet for the network adapter, click the Driver Type tab.

2.	Click one of three options (if available for the specific adapter), as described in the following table.

| Network adapter driver type | Description |
| --- | --- |
| Enhanced mode (32-bit and 16-bit) NDIS driver | Installs an NDIS 3.1-compliant driver. |
| Real-mode NDIS driver | Installs an NDIS 2.x-compliant driver. |
| Real-mode (16-bit) ODI driver | Installs a real-mode driver created to support ODI for Windows 3.1. |

### Binding Network Adapter Drivers to Protocols

For a protocol to communicate with each network adapter on your computer, the network adapter driver must be bound to the protocol. The bindings define the relationships between networking software components. Windows 95 automatically binds the appropriate protocols to the network adapter.

You can change the bindings if, for example, you do not want to use a specific protocol with a particular network adapter. Or, as another example, if the computer is on a local area network and is also connected to the enterprise internetwork, you may not want your peer resource sharing services to be seen on the internetwork. In that case, you will want to disable the binding between the related network client and the adapter that connects the computer to the internetwork.

¨	To configure bindings for protocols

1.	On the property sheet for the selected network adapter, click the Bindings tab.

In the list, the protocols that are bound to the selected adapter are checked. If a particular protocol does not appear in the list, check that it is correctly installed by returning to the Configuration property sheet and reinstalling it.

2.	In the list of protocols, click a protocol so that it is not checked, if you do not want this network adapter to be bound to the selected protocol.

In this example, the IPX/SPX compatible and NetBEUI protocols are bound to the driver for the Intel EtherExpress 16 or 16TP adapter.

### Configuring Network Adapter Resource Settings

Windows 95 can determine hardware settings for most network adapters. When you install a network adapter, you should accept the proposed settings unless you are absolutely sure they are incorrect.

Most devices cannot share IRQ settings, memory buffer addresses, or ROM addresses. Where possible, Windows 95 identifies and resolves conflicts. However, if one of the supported devices does not seem to work, the problem may be the particular hardware configuration. Make sure there are no conflicts among network adapters or other peripherals, or between the system board and adapters. Sometimes these settings are set with software, sometimes with jumpers or switches on the hardware (refer to your hardware documentation for specific details). If settings for a network adapter can be configured through software, you can configure settings using the Network option in Control Panel.

¨	To configure hardware resources for a selected network adapter

1.	On the property sheet for the network adapter, click the Resources tab.

2.	If you are installing a new adapter and are unsure about the actual settings for this hardware, click the option titled Configure Hardware Settings Later.

– Or –

Click the option titled Use These Hardware Settings, and confirm values for the listed settings.

3.	To see and select from the available values for a setting, click the arrow beside the setting’s current value.

·	If the displayed value is the current setting, it is preceded by a hash (#) character.

·	If the value is preceded by an asterisk (*) character, there is a conflict with another device in the system. You should reconfigure one of the devices.

The possible settings displayed depend on the type of network adapter. The following table describes typical settings.

Sample Hardware Resource Settings for Network Adapters

| Setting type | Description |
| --- | --- |
| Configuration Type | Specifies a logical configuration type. Each configuration type refers to a hardware configuration for the adapter (rather than software settings for the driver). For some adapters, one logical configuration choice may be to use shared RAM. For more information about Configuration Types, see the discussion following this list. |
| I/O Address Range | Specifies the memory address range reserved for the network adapter to use for temporary storage of input and output data, expressed as a hexadecimal value. |
| Interrupt (IRQ) | Specifies the hardware line over which the device can send interrupts (requests for service) to the computer’s microprocessor. |

Alternate Configuration Types for a network adapter consist of the possible sets of hardware settings defined for the adapter by the manufacturer. For legacy network adapters, the settings that appear in the property sheet are only the possible settings for the adapter’s driver. However, many adapters have additional hardware settings that are programmed by running a configuration utility from the adapter’s manufacturer or by setting jumpers on the adapter itself. Although some of these settings do not appear in the adapter’s property sheet, the resources used could cause conflicts with other system components.

This also means that configuring most legacy network adapters in Windows 95 consists only of specifying the preprogrammed values for the adapter hardware. Changing values for most network adapters in the Network property sheets does not reprogram the adapter itself. At this writing, the Intel EtherExpress is the only adapter that is automatically reprogrammed to match the Registry settings.

The only ways you can verify hardware settings for network adapters are to run the hardware configuration program to view settings, or check the actual jumpers set on the adapter.

¨	To reprogram adapter settings

1.	Use the configuration disk that came with your adapter to reprogram it.

2.	Write down the settings made so that you can refer to them in the following step.

3.	In the Network option in Control Panel, change the settings to the ones you noted, and then shut down and restart the computer.

### Configuring Advanced Properties for Network Adapters

The options available on the Advanced property sheet vary, depending on the type of network adapter. For information about specific settings that appear for a selected network adapter, see the documentation provided by the manufacturer for the adapter and driver. The manufacturer will also provide guidelines for when to change the default values for advanced configuration options.

¨	To specify advanced settings for the network adapter

1.	On the property sheet for the selected network adapter, click the Advanced tab.

2.	To change these values, select an item in the Property list, and then select the value in the Value box.

3.	Click the OK button and restart the computer for the changes to take effect.

This example shows the advanced options for an Intel EtherExpress network adapter.

The following examples show some typical settings for general types of network adapters. For an explanation of the specific settings, see the documentation provided by the adapter manufacturer.

| Example setting type | Description |
| --- | --- |
| For Ethernet adapters: |  |
| Transceiver Type (cable connector) | The transceiver is the device that connects a computer to the network, defined as one of the following values. · Thick Net, for an AUI or DEC/Intel/Xerox (DIX) connection. · Thin Net, for a BNC or coaxial (COAX) connection. · Twisted Pair, for a TPE connection. |
| For Token Ring adapters: |  |
| Network Address | By default, hardware detection uses the network address burned into the adapter. To use another network address, type the network address in hexadecimal form, in the following format: XX-XX-XX-XX-XX-XX For example, 01-02-03-4E-2D-1F |

For example, some Token Ring network adapters can use a “local addressing” feature with Windows 95, which allows the network adapter’s internal network address to be bypassed and defined manually. To set the I/O address of an IBM® 4/16 Token Ring adapter, select either Primary or Secondary as the value for I/O Port Base Address on the adapter’s Advanced property sheet. In this case, the driver ignores the I/O settings on the Resource property sheet.

## Setting LAN Adapter Numbers

As described in Chapter 32, “Windows 95 Network Architecture,” NetBIOS defines the interface between the network client and the protocol layers through set of function calls that allow an application to use the services of a Transport-layer service provider. Because many network applications use NetBIOS to send commands to the protocol driver, the NetBIOS interface is supported by all three protocols provided with Windows 95.

Each combination of a NetBIOS network protocol and a network adapter forms a logical network over which computers can communicate with each other. For example, a computer can have a Token Ring adapter and an Ethernet adapter, and might use NetBEUI on the Token Ring network and both NetBEUI and TCP/IP on the Ethernet network. In this case, the computer is connected to three logical networks, each of which is assigned a NetBIOS LAN adapter (LANA) number that Windows 95 uses to communicate with them.

When Windows 95 uses multiple protocols, it transmits data first using one protocol, then again using the next protocol, and so on. When you install multiple protocols on a computer, you designate the order in which the computer will use them. The first protocol in this series is sometimes called the primary protocol.

On a computer running Windows 95, each binding of a protocol to a network adapter has a LAN adapter number assigned to it. (For example, one protocol bound to two network adapters requires two LAN adapter numbers; two protocols each bound to two adapters requires four LAN adapter numbers.)

In Windows 95, LANA numbers are assigned dynamically in sequence of binding order for the protocols, beginning with 7 and working downward. This accommodates dynamic Plug and Play events such as removing a network adapter while the computer is running. If you are running Windows 95 in such a dynamic environment, Windows 95 cannot guarantee that a given protocol will receive the same LANA number each time the system is started, although, if the computer’s network hardware never changes, the LANA numbers may not change at each startup. Also, the default protocol is always LANA 0.

You need to change a LAN adapter number only if you have a NetBIOS application that needs to know the LANA number. For example, Lotus Notes® requires that you enter the LANA number that Lotus Notes will use. To configure Windows 95 to use Lotus Notes, set the default protocol to be the NetBIOS-based protocol on which you want to run Lotus Notes. (Setting the default protocol makes it LANA 0.) This protocol can be NetBEUI, IPX/SPX-compatible with NetBIOS support, or TCP/IP with NetBIOS support.

¨	To select a default protocol for LANA settings

1.	In the Network option in Control Panel, click the Configuration tab.

2.	Double-click the protocol you want to be the default.

3.	On the property sheet, click the Advanced tab.

4.	Click the option named Set This Protocol To Be The Default Protocol so that the check box is checked. Then click OK, and shut down and restart the computer for the changes to take effect.

## Technical Notes on Network Adapters

This section provides some technical notes for specific network adapters and briefly describes changes to network adapter driver support from support provided under Windows for Workgroups 3.11.

As described earlier in “NDIS Driver Basics,” NDIS 3.0 network adapter drivers that worked with Windows for Workgroups 3.11 do not work under Windows 95. You must use an NDIS 2.x real-mode driver, an ODI driver, or an updated NDIS 3.1 protected-mode driver for your network adapter, plus a Windows 95 INF file. There are many real-mode drivers, updated protected-mode drivers, and supporting INF files included with Windows 95.

If your network adapter does not appear in the list of adapters in the Network option in Control Panel, you can use information for a Windows 3.x adapter to install it under Windows 95. If you have an OEMSETUP.INF file created for an earlier version of Windows to describe a network adapter driver, you can use that INF file by clicking the Have Disk button in the Select Network Adapter dialog box for installing network adapter drivers.

The major reason that the INF file format has changed for all types of Windows 95 network adapter drivers is that INF files are now used to add information to the Windows 95 Registry. The INF files created for earlier versions of Windows do not contain this kind of information. For information about INF files for network adapter drivers, see Appendix C, “Windows 95 INF Files.”

The following table provides technical notes for the few network adapters that have been tested and shown to require special consideration under Windows 95.

Network Adapters that Require Special Consideration under Windows 95

| Network adapter | Comment |
| --- | --- |
| DEC™ Etherworks 2 (DECPA) |  |
| 3Com® EtherLink II®-16 | This adapter uses shared RAM. |
| 3Com EtherLink® III |  |
| HP® Ethertwist® |  |
| IBM Token Ring card |  |
| NE2000+ | This adapter can use shared RAM, but its shared-RAM mode is not supported under Windows 95. |
| Ungermann-Bass |  |

# IPX/SPX-Compatible Protocol

Internet Packet Exchange/Sequential Packet Exchange (IPX/SPX) is a small, fast protocol that is routable and best suited for small to medium-sized LANs. The IPX/SPX-compatible protocol (NWLINK.VXD) is compatible with Novell NetWare IPX/SPX.

The Microsoft IPX/SPX-compatible protocol supports the 32-bit Windows Sockets programming interface, so that any Win32-based Windows Socket application can run on IPX/SPX with Windows 95. (There are no 16-bit Windows Sockets applications using IPX/SPX, so backward compatibility for 16-bit sockets for IPX/SPX is not an issue.)

The IPX/SPX-compatible protocol can be used by Client for NetWare Networks to communicate with NetWare 2.x, 3.x, or 4.x servers or a computer running File and Printer Sharing for NetWare Networks. IPX/SPX can also be used by Client for Microsoft Networks to communicate with computers running Windows for Workgroups 3.11 or Windows NT with the IPX/SPX-compatible protocol.

The Microsoft IPX/SPX-compatible protocol uses the NWNBLINK.VXD module to support computers that use NetBIOS over IPX and to support the NetBIOS programming interface.

The Microsoft IPX/SPX-compatible protocol is NDIS 3.1-compliant, allowing computers running Windows 95 to communicate over a routable IPX-compatible protocol. This protocol can use Novell NetWare servers configured as routers (and other IPX routers) to transfer its packets across LANs to access resources on other computers running Windows 95.

The IPX/SPX-compatible protocol is a 32-bit, protected-mode protocol that is Plug and Play-compliant. Because the IPX/SPX-compatible protocol provides virtualized services to all DOS VMs and applications, the Novell-supplied VIPX.386 driver is not needed.

## Configuring IPX/SPX-Compatible Protocol

The Microsoft IPX/SPX-compatible protocol is installed if you also install Client for NetWare Networks during Windows 95 Setup. You can also install this protocol to support other network clients, including Client for Microsoft Networks.

For information about installing and configuring this protocol in custom setup scripts, see “Installing Drivers and Protocols in Custom Setup Scripts” earlier in this chapter.

¨	To configure the IPX/SPX-compatible protocol

1.	On the Configuration sheet in the Network option in Control Panel, double-click IPX/SPX Compatible Protocol for Windows.

If your computer has multiple network adapters, there will be an instance of the IPX/SPX-compatible protocol for each network adapter. You must configure each adapter with its own settings.

2.	Click the Advanced tab.

3.	Most values have correct defaults in typical installations. If you need to change a value for a particular purpose, select the item in the Property list, and specify a value in the Value list, based on the settings in the following table.

| Property | Value |
| --- | --- |
| Force even-length IPX packets | Enabled only for Ethernet 802.3 on monolithic implementations that cannot handle odd-length packets. |
| Frame type | Specifies the frame type based on detection.1 This value is used for network adapters that support multiple frame types. The possible values are: · Auto-detect (recommended) · Ethernet 802.2 (default for NetWare 3.12 and later) · Ethernet 802.3 · Ethernet_II · SNAP (Token Ring networks can use the Regular or SNAP value.) |
| Maximum connections | Specifies the maximum number of connections that IPX will allow. Configured dynamically. |
| Maximum sockets | Specifies the maximum number of IPX sockets that IPX assigns. Configured dynamically. |
| Network address | Specifies the IPX network address as a four-byte value. Configured dynamically. |
| Source routing cache size | Specifies the cache size to use with source routing2. This parameter is used only on Token Ring networks, where it is used to turn on source routing. Important Cache size is determined by entry count, not byte count. The recommended value of 16 is the most efficient and best setting for most installations. |
| 1 Each time you start the computer, Windows 95 detects the frame type by sending a general RIP request in each frame format. Based on the responses received from routers, Windows 95 determines the most prevalent frame type used and sets that as the default frame type for the IPX/SPX-compatible protocol. 2 Source routing is a method of routing data across source-routing bridges. For NetWare networks, this means forwarding of NetWare frames across an IBM Token-Ring bridge. With NDIS protocols, source routing is done by the network protocol. (If you use Novell-supplied ODI-based protocols, source routing is configured with the network adapter driver.) |  |

You should not need to change bindings in most circumstances. However, you can disable the bindings for the protocol if you do not want other computers using that protocol to see this computer. At least one protocol, however, must be bound to the network client for the computer to be able to communicate with the network.

¨	To change bindings for the IPX/SPX-compatible protocol

1.	On the property sheet for IPX/SPX-Compatible Protocol, click the Bindings tab.

2.	Click any network component to change its bindings.

If the option is checked, it is bound to the protocol. If it is not checked, that network component is not using the IPX/SPX-compatible protocol. For more information, see “Configuring Network Adapters” earlier in this chapter.

Microsoft Client for NetWare Networks is always bound only to the IPX/SPX-Compatible Protocol. This network client cannot use another protocol.

Note

## Using NetBIOS Over IPX

NetBIOS is a high-level interface used by network applications to communicate with other NetBIOS-compliant applications. The NetBIOS interface is responsible for the following:

·	Establishing logical names on the network

·	Establishing connections (called sessions) between two logical names on the network

·	Transmitting data between network computers

Windows 95 provides a 32-bit, protected-mode NetBIOS driver to support NetBIOS services over IPX (NWNBLINK.VXD, which complements NWLINK). NetBIOS over IPX is compatible with the Novell NetBIOS support driver. Performance enhancements include acknowledgment of previous frames in response frames (called PiggyBackAck), plus a “sliding window” acknowledgment mechanism. These NetBIOS enhancements are used only when communicating with other computers using IPX over NetBIOS, such as other computers running Windows 95, Windows NT, or NetWare when running Lotus Notes or other NetBIOS applications.

Novell provides a TSR NetBIOS driver named NETBIOS.EXE, which is a Level 1 NetBIOS provider that consumes about 40K of conventional memory. This driver acknowledges each frame received, thus increasing the amount of traffic generated when NetBIOS is used. With the Microsoft implementation of NetBIOS over IPX, the real-mode NETBIOS.EXE TSR should be removed from the system.

NetBIOS over IPX is not necessary for computers running Windows 95 to communicate with each other. The redirector and server networking components in Windows 95 communicate with the IPX protocol directly without NetBIOS. NetBIOS over IPX is commonly required only when using any NetBIOS applications on a NetWare (or IPX) server, such as Lotus Notes.

A computer running Windows 95 that uses IPX without NetBIOS can connect to a Windows NT 3.5 server that uses IPX without NetBIOS. However, the Windows NT 3.5 workstation service (not Windows NT Workstation) can only connect to a computer running File and Printer Sharing for Microsoft Networks when the computer running Windows 95 is using NetBIOS over IPX.

Note

¨	To use the IPX/SPX-compatible protocol with NetBIOS on a computer

·	In the property sheet for the IPX/SPX-compatible protocol, click the NetBIOS tab. Then click the option named I Want To Enable NetBIOS Over IPX/SPX.

– Or –

In the Network option in Control Panel, click the Add button, and double-click Protocol. In the Manufacturers list in the Select dialog box, select Microsoft, and in the Models list, select the option named NetBIOS Support For IPX/SPX-Compatible Protocol. Then click OK.

## IPX/SPX Technical Notes on NetWare Networks

·	There is no need to enable Source Routing on Token Ring networks if the communication is on the same ring, even if one computer has it enabled.

·	SPX-II is a protocol definition for windowing and transmitting large packets over SPX. You can run any SPX-II application under Windows 95 using the IPX/SPX-compatible protocol. The related Novell-supplied DLL to support SPX-II must be available on the computer.

·	The IPX/SPX-compatible protocol in Windows 95 uses NET.CFG parameters for NetBIOS over IPX. However, configuring these parameters should not be necessary because most parameters are dynamic and self-adjusting. (When you install Windows 95, any NET.CFG and SHELL.CFG parameters are moved to the Registry.)

·	To determine the network ID (which is the network address in IPX packets), Windows 95 checks the wire for RIP packets and chooses the most likely ID. The network ID is dynamic and will change when a new network ID becomes more prevalent.

·	All Transport Layer Interface (TLI) libraries can run on the IPX/SPX-compatible protocol in Windows 95. TLI is similar to TDI in Microsoft networking as a layer between the protocol and network adapter driver; this implementation is similar to STREAMS and provides a STREAMS environment for NetWare, but Windows 95 uses the Windows Sockets specification instead.

# TCP/IP Protocol

Transmission Control Protocol/Internet Protocol (TCP/IP) is a networking protocol that provides communication across interconnected networks made up of computers with diverse hardware architectures and various operating systems. TCP/IP can be used to communicate with computers running Windows 95, with devices using other Microsoft networking products, or with non-Microsoft systems, such as UNIX.

The TCP/IP protocol family is a standard set of networking protocols. TCP/IP is used to connect to the Internet. These same protocols can be used in private internetworks that connect several local area networks.

The Microsoft implementation of TCP/IP in Windows 95 enables enterprise networking and connectivity on computers running the TCP/IP protocol. Adding TCP/IP to a computer running Windows 95 offers the following advantages:

·	A standard, routable enterprise networking protocol that is the most complete and accepted protocol available. All modern operating systems support TCP/IP, and most large networks rely on TCP/IP for much of their network traffic.

·	A technology for connecting dissimilar systems. Many standard connectivity utilities are available to transfer data between dissimilar systems, including File Transfer Protocol (FTP) and Terminal Emulation Protocol (Telnet).

With TCP/IP as a connectivity protocol, computers running Windows 95 can communicate with many non-Microsoft systems, including Internet hosts, Apple® Macintosh® systems, IBM mainframes, UNIX systems, and Open VMS™ systems.

·	A robust, scalable, cross-platform client-server framework. TCP/IP in Windows 95 supports the Windows Sockets 1.1 interface, which is ideal for developing client-server applications that can run Windows Sockets-compliant stacks from other vendors. Many public-domain Internet tools also use the Windows Sockets standard.

·	Client for Dynamic Host Configuration Protocol (DHCP), for dynamic resolution of IP addresses to host names on networks that have DHCP servers.

·	Client for Windows Internet Name Resolution (WINS), for dynamic resolution of IP addresses to computer names on networks that have WINS servers.

·	Point-to-Point protocol for asynchronous communication, as described in Chapter 28, “Dial-Up Networking and Mobile Computing.”

Microsoft TCP/IP provides all the following elements necessary to implement these protocols for networking:

·	Core TCP/IP protocols, including the Transmission Control Protocol (TCP), Internet Protocol (IP), User Datagram Protocol (UDP), Address Resolution Protocol (ARP), and Internet Control Message Protocol (ICMP). This suite of Internet protocols provides a set of standards for how computers communicate and how networks are interconnected.

·	Support for application interfaces, including Windows Sockets for network programming and NetBIOS for establishing sessions on the network.

·	Basic TCP/IP connectivity applications, including ftp and telnet. With these utilities, users can interact with and use resources on non-Microsoft hosts, such as UNIX workstations.

·	TCP/IP diagnostic tools to detect and resolve TCP/IP networking problems, including arp, nbtstat, netstat, route, and tracert.

For information about installing and configuring this protocol in custom setup scripts, see “Installing Drivers and Protocols in Custom Setup Scripts” earlier in this chapter. For a summary of the command-line options for TCP/IP utilities, see Appendix A, “Command-Line Commands Summary.”

## Configuring TCP/IP with DHCP

In an effort to make the implementation of the TCP/IP protocol more manageable, Microsoft, working with other industry leaders, has created a mechanism called Dynamic Host Configuration Protocol (DHCP) for the automatic allocation of IP addresses. DHCP is not a Microsoft standard, but a public Request for Comments (RFC) that Microsoft has implemented.

DHCP allows a network management utility to centrally establish a range of IP addresses per subnet. An individual IP address from the range is assigned to any Microsoft TCP/IP client requesting an address. DHCP also allows the network management utility to centrally establish a lease time or how long the allocated IP address is to remain valid.

A DHCP server runs as a process on a Windows NT 3.5 Server. Using DHCP company-wide, it is possible to move from subnet to subnet and always have a valid IP address. Microsoft TCP/IP includes a utility, IPCONFIG, that allows a user or administrator to quickly examine the current IP address allocated to the computer, the IP address lease time and other useful data about the DHCP allocation for troubleshooting purposes.

DHCP support can be specified during installation, or enabled using the Network option in Control Panel. If you want, a manually entered IP address can be used, and DHCP support can be disabled.

¨	To use DHCP for automatic configuration of TCP/IP settings

1.	On the Network property sheet, click the Configuration tab, and double-click Microsoft TCP/IP.

If your computer has multiple network adapters, there will be an entry for TCP/IP for each network adapter. You must configure each adapter with its own TCP/IP settings.

Note

2.	On the Microsoft TCP/IP property sheet, click the IP Address tab.

3.	Click the option named Obtain An IP Address From A DHCP Server, and then click OK.

When the IP address for your computer is configured using DHCP, you can use ipconfig at the MS-DOS Prompt to see the currently assigned IP address.

Tip

If Microsoft TCP/IP is configured to obtain an IP address from a DHCP server when a DHCP server is not available on the network, the next time Windows 95 starts, an error message announces that the DHCP client could not obtain an IP address from a DHCP server. To solve this problem, use the procedure described in the following section to configure TCP/IP manually.

A computer running Windows 95 cannot be a DHCP server. A computer running Microsoft Windows NT 3.5 Server can act as both a DHCP and a WINS server.

Note

## Configuring TCP/IP Settings Manually

If you cannot use DHCP for automatic configuration, the network administrator must provide these values so that individual users can configure TCP/IP manually:

·	The IP address and subnet mask for each network adapter installed on the computer. For more information, see Chapter 30, “Internet Access.”

·	The IP address for the default gateways (IP routers).

·	Whether the computer will use Domain Name System (DNS) and, if so, the IP addresses of the DNS servers on the internetwork.

·	WINS server addresses, if WINS servers are available on your network.

The following procedure describes the basic configuration options for TCP/IP. If you want to configure the computer to use WINS or DNS for name resolution, see the procedures in “Using WINS for Name Resolution” and “Using DNS for Name Resolution” later in this chapter.

¨	To configure the TCP/IP protocol manually

1.	In the Network option in Control Panel, click the Configuration tab, and double-click Microsoft TCP/IP.

If your computer has multiple network adapters, there will be an instance of TCP/IP for each network adapter. You must configure each adapter with its own TCP/IP settings.

Note

2.	On the Microsoft TCP/IP property sheet, click the IP Address tab.

3.	Type an IP address and subnet mask in the respective boxes.

The network administrator must provide these values for individual users, based on the network ID and the host ID plan for your site.

·	The value in the IP Address box identifies the IP address for the local computer or, if more than one network adapter is installed in the computer, for the network adapter selected in the Adapter box.

·	The value in the Subnet Mask box identifies the network membership for the selected network adapter and its host ID. This allows the computer to separate the IP address into host and network IDs. The subnet mask defaults to an appropriate value, as shown in the following list.

| Address class | Range of first octet in IP address | Subnet mask |
| --- | --- | --- |
| Class A | 1–126 | 255.0.0.0 |
| Class B | 128–191 | 255.255.0.0 |
| Class C | 192–223 | 255.255.255.0 |

4.	To view or specify which network clients (redirectors) are bound to the TCP/IP protocol, click the Bindings tab.

·	To keep a network client from using the TCP/IP protocol, click the client to remove the selection.

·	If the network client for which you want to use TCP/IP does not appear in this list, that client is not bound to the TCP/IP protocol.

For information, see “Configuring Network Adapters” earlier in this chapter.

The only network client provided with Windows 95 that can use Microsoft TCP/IP is the Client for Microsoft Networks. The Client for NetWare Networks does not use Microsoft TCP/IP.

Note

NetWare/IP from Novell allows the NCP request to be sent over an IP header. You can only use NetWare/IP with a Novell-provided, real-mode client.

5.	Click the Gateway tab, and then type at least one IP address for the default gateway (IP router) on the network.

6.	To specify an IP address for an additional gateway, type the IP address in the New Gateway box, and then click the Add button.

Up to 7 additional gateways can be specified to facilitate a connection to a computer that is not reachable through the default gateway. Windows 95 will attempt to connect to a remote computer using the default gateway, and then each additional gateway specified. Additional gateways also provide fault management in case the default gateway becomes inoperable.

The first gateway in the list is the default gateway. Gateway addresses can be prioritized by dragging the IP address in the list of Installed Gateways.

7.	Click OK, and then restart the computer for changes to take effect.

For more information about IP addresses on TCP/IP networks, see Chapter 30, “Internet Access.”

## Name Resolution for Windows Networking

For TCP/IP and the Internet, the computer name is the globally known system name plus a DNS domain name. On the local network, the computer name is the NetBIOS name that was defined during Windows 95 Setup.

Computers use IP addresses to identify each other, but users usually find it easier to work with computer names. A mechanism must be available on a TCP/IP network to resolve names to IP addresses. To ensure that both the name and the address are unique, the computer using Microsoft TCP/IP registers its name and IP address on the network during system startup. Computers running Microsoft TCP/IP on the network can use one or more of the following methods to ensure accurate name resolution in TCP/IP internetworks:

Computers running Microsoft TCP/IP can also use broadcast name resolution, which is a NetBIOS-over-TCP/IP mode of operation defined in RFC 1001/1002 as b-node. This method relies on a computer making IP-level broadcasts to register its name by announcing it on the network. Each computer in the broadcast area is responsible for challenging attempts to register a duplicate name and for responding to name queries for its registered name.

###### Broadcast name resolution

An LMHOSTS file specifies the NetBIOS computer name and IP address mappings; a HOSTS file specifies the DNS name and IP address. On a local computer, the HOSTS file (used by Windows Sockets applications to find TCP/IP host names) and LMHOSTS file (used by NetBIOS over TCP/IP to find Microsoft networking computer names) can be used to list known IP addresses mapped with corresponding computer names. LMHOSTS is still used for name resolution in Windows 95 for small-scale networks or remote subnets where WINS is not available.

###### LMHOSTS or HOSTS files

·	The HOSTS file is used as a local DNS equivalent to resolve host names to IP addresses.

·	The LMHOSTS file is used as a local WINS equivalent to resolve NetBIOS computer names to IP addresses.

Each of these files is also known as a host table. Sample versions of LMHOSTS (called LMHOSTS.SAM) and HOSTS files are added to the Windows directory when you install Windows 95 with TCP/IP support. These files can be edited using any ASCII editor, such as WordPad or Edit. For information about setting up and using HOSTS and LMHOSTS files, see Appendix G, “HOSTS and LMHOSTS Files for Windows 95.”

Computers running Microsoft TCP/IP can use WINS if one or more Windows NT Server computers configured as WINS servers are available, and those computers contain a dynamic database for mapping computer names to IP addresses. WINS can be used in conjunction with broadcast name resolution for an internetwork, where other name resolution methods are inadequate. WINS is a NetBIOS-over-TCP/IP mode of operation defined in RFC 1001/1002 as p-node. Notice that WINS replaces the functionality of the LMHOSTS file.

###### Windows Internet Name Service

The Domain Name System (DNS) provides a way to look up name mappings when connecting a computer to foreign hosts using NetBIOS over TCP/IP or Windows Sockets applications such as FTP. DNS is a distributed database designed to relieve the traffic problems that arose with the exploding growth of the Internet in the early 1980s. A DNS name server must be configured and available on the network. Notice that DNS replaces the functionality of the HOSTS file by providing a dynamic mapping of IP addresses to host names used by TCP/IP applications and utilities. For more information, see “Using DNS for Name Resolution” later in this chapter.

###### DNS name resolution

Windows 95 provides support for multiple DNS servers and up to two WINS servers. Support for either service can be configured in Windows 95 Setup or added later using the Network option in Control Panel.

## Using WINS for Name Resolution

WINS provides a distributed database for registering and querying dynamic computer name-to-IP address mappings in a routed network environment. If you are administering a routed network, WINS is your best choice for name resolution, because it is designed to solve the problems that occur with name resolution in more complex internetworks.

WINS reduces the use of local broadcasts for name resolution and allows users to easily locate computers on remote networks. Furthermore, when dynamic addressing through DHCP results in new IP addresses for computers that move between subnetworks, the changes are automatically updated in the WINS database. Neither the user nor the network administrator needs to make manual accommodations for name resolution in such a case.

The WINS protocol is based on and is compatible with the protocol defined for WINS server in Requests for Comments (RFCs) 1001 and 1002, so it is interoperable with any other implementations of these RFCs. A WINS server is a Windows NT Server 3.5 computer with WINS server software installed. When Microsoft TCP/IP is installed, WINS client software is installed automatically.

This section provides an overview of how WINS and name query broadcasts perform name resolution on Windows networks.

WINS consists of two components: the WINS server, which handles name queries and registrations, and the client software (NetBIOS over TCP/IP), which queries for computer name resolution.

Windows networking clients that are WINS-enabled (computers running Windows NT 3.5 or Windows 95) can use WINS directly. Non-WINS computers on the internetwork that are b-node – compatible, as described in RFCs 1001 and 1002, can access WINS through proxies, which are network WINS-enabled computers that listen to name query broadcasts and then respond for names that are not on the local subnet or are p-nodes.

On a Windows network, users can browse transparently across routers. To allow browsing without WINS, the network administrator must ensure that the users’ primary domain has Windows NT Server computers on both sides of the router to act as master browsers. These computers need correctly configured LMHOSTS files with entries for the domain controllers across the subnet.

With WINS, such strategies are not necessary because the WINS servers and proxies transparently provide the support necessary for browsing Windows NT domains across routers. For a technical discussion of how WINS works and how it can be set up on the network, see Windows NT Server 3.5 TCP/IP in the Windows NT Server 3.5 documentation set.

If there are WINS servers installed on your network, you can use WINS in combination with broadcast name queries to resolve NetBIOS computer names to IP addresses. If you do not use this option, Windows 95 uses name query broadcasts (b-node mode of NetBIOS over TCP/IP) plus the local LMHOSTS file to resolve computer names to IP addresses. Broadcast resolution is limited to the local network, as described earlier in this chapter.

¨	To configure a computer to use WINS for name resolution

1.	In the Network option in Control Panel, double-click Microsoft TCP/IP.

2.	Click the WINS Configuration tab.

3.	If a WINS server is not available, select Disable WINS Resolution.

– Or –

If a WINS server is available (or a secondary WINS server), select Enable WINS Resolution and type the IP addresses of the Primary and Secondary WINS servers. These values should be provided by the network administrator, based on the IP addresses assigned to these Windows NT Server computers.

4.	If WINS is enabled, in the Scope ID box, type the computer’s scope identifier, if required on an internetwork that uses NetBIOS over TCP/IP.

Usually this value is left blank. Scope IDs are used only for communication based on NetBIOS over TCP/IP. In such a case, all computers on a TCP/IP internetwork must have the same scope ID. A scope ID may be assigned to a group of computers if those computers communicate only with each other and not with computers outside the group. Such computers can find each other if their scope IDs are identical.

5.	If a DHCP server that is configured to provide information on available WINS servers is available , select Use DHCP for WINS Resolution.

If DHCP is used for automatic configuration, these parameters can be provided by the DHCP server.

## Using DNS for Name Resolution

The Domain Name System (DNS) is a distributed database, providing a hierarchical naming system for identifying hosts on the Internet. The specifications for DNS are defined in Requests for Comments (RFCs) 1034 and 1035.

Although DNS may seem similar to WINS, there is one major difference: DNS requires static configuration of IP addresses for name-to-address mapping. WINS can provide name-to-address mapping dynamically and requires far less administration.

The DNS database is a tree structure called the domain name space, where each node or domain is named and can contain subdomains. The domain name identifies the domain’s position in the database in relation to its parent domain, with a period (.) separating each part of the name for the network nodes of the DNS domain.

The root of the DNS database is managed by the Internet Network Information Center. The top-level domains were assigned organizationally and by country. These domain names follow the International Standard (ISO) 3166. Two-letter and three-letter abbreviations are used for countries, and various abbreviations are reserved for use by organizations, as shown in the following example.

| DNS domain name | Type of organization |
| --- | --- |
| com | Commercial (for example, microsoft.com) |
| edu | Educational (for example, mit.edu for Massachusetts Institute of Technology) |
| gov | Government (for example, nsf.gov for the National Science Foundation) |
| org | Noncommercial organizations (for example, fidonet.org for FidoNet) |
| net | Networking organizations (for example, nsf.net for NSFNET) |

DNS uses a client-server model, where the DNS servers contain information about a portion of the DNS database and make this information available to clients, called resolvers, that query the name server across the network. DNS name servers are programs that store information about parts of the domain namespace called zones. The administrator for a domain sets up name servers that contain the database files with all the resource records describing all hosts in their zones. DNS resolvers are clients that are trying to use name servers to gain information about the domain namespace.

Microsoft TCP/IP includes the DNS resolver functionality used by NetBIOS over TCP/IP and Windows Sockets connectivity applications such as ftp and telnet to query the name server and interpret the responses.

The key task for DNS is to present friendly names for users and then resolve those names to IP addresses, as required by the internetwork. Name resolution is provided through DNS by the name servers, which interpret the information in a fully qualified domain name (FQDN) to find its specific address. If a local name server doesn’t contain the data requested in a query, it sends back names and addresses of other name servers that could contain the information. The resolver then queries the other name servers until it finds the specific name and address it needs. This process is made faster because name servers continuously cache the information learned about the domain namespace as the result of queries.

All the resolver software necessary for using DNS on the Internet is installed with Microsoft TCP/IP.

Although TCP/IP uses IP addresses to identify and reach computers, users typically prefer to use host names. DNS is a naming service generally used in the UNIX networking community to provide standard naming conventions for IP workstations. TCP/IP utilities, such as ftp and telnet, can also use DNS, in addition to the HOSTS file, to find computer when connecting to foreign hosts or computers on your network.

The network administrator must determine whether users should configure their computers to use DNS. Usually you will use DNS if you are using TCP/IP to communicate over the Internet or if your private internetwork uses DNS to distribute host information.

Microsoft TCP/IP provides a DNS client for resolving Internet or UNIX system names. Microsoft Windows networking provides dynamic name resolution for NetBIOS computer names using WINS servers and NetBIOS over TCP/IP, as described in the previous section.

If you choose to use DNS, you must configure how the computer will use DNS and the HOSTS file. TCP/IP must be installed to manually set up the DNS connectivity options. DNS configuration is global for all network adapters installed on a computer.

If DHCP is used for automatic configuration, these parameters can be provided by the DHCP server.

¨	To configure a computer to use DNS for name resolution

1.	In the Network option in Control Panel, double-click Microsoft TCP/IP, and then click the DNS Configuration tab.

2.	If a DNS server is not available, click Disable DNS.

– Or –

If a DNS server is available, click Enable DNS. Then specify a host name and complete the other configuration information, as described in the following procedure.

The host name is used to identify the local computer by name for authentication by some utilities. Other TCP/IP-based utilities can use this value to learn the name of the local computer. Host names are stored on DNS servers in a table that maps names to IP addresses for use by DNS.

¨	To set the host name for DNS

·	Type a name in the Host Name box.

The name can be any combination of the letters A–Z, the numerals 0–9, and the hyphen (-), plus the period (.) character used as a separator. By default, this value is the Microsoft networking computer name, but the network administrator can assign another host name without affecting the computer name.

Some characters that can be used in computer names, especially the underscore, cannot be used in host names.

Note

The TCP Domain Name is used with the host name to create a fully qualified domain name (FQDN) for the computer. The FQDN is the host name followed by a period (.), followed by the domain name. For example, this could be johndoe.microsoft.com, where johndoe is the host name and microsoft.com is the domain name.

During DNS queries, the local domain name is appended to short names. A short name consists of just a host name, such as janedoe. When querying the DNS server for the IP address of janedoe, the domain name is appended to the short name and the DNS server is actually asked to resolve the FQDN of janedoe.microsoft.com. Notice that the FQDN of Jane Doe at Microsoft (janedoe.microsoft.com) is not the same as her Internet electronic mail address of janedoe@microsoft.com.

¨	To set the DNS domain name

·	Optionally, type a name in the Domain Name box.

This is usually an organization name followed by a period and an extension that indicates the type of organization, such as microsoft.com. The name can be any combination of the letters A–Z, the numerals 0–9, and the hyphen (-), plus the period (.) character used as a separator.

A DNS domain is not the same as a Windows NT or LAN Manager domain. A DNS domain is a hierarchical structure for organizing TCP/IP hosts and provides a naming scheme used in UNIX environments. A Windows NT or LAN Manager domain is a grouping of computers for security and administrative purposes.

Note

You can add up to three IP addresses for DNS servers. For a given DNS query, Windows 95 attempts to get DNS information from the first IP address in the list. If no response is sent, Windows 95 goes to the second server in the list, and so on. To change the order of the IP addresses, you must remove them and retype them in the order that you want the servers to be searched.

¨	To set the DNS server search order

1.	In the Domain Name System (DNS) Search Order box, type the IP address of a DNS server that will provide name resolution. Then click the Add button to add the IP address to the list.

The network administrator should provide the correct values for this parameter, based on the IP address assigned to the DNS server used at your site.

2.	To remove an IP address from the list, select it and click the Remove button.

The Domain Suffix Search Order specifies the DNS domain suffixes to be appended to host names during name resolution. You can add up to five domain suffixes. Domain suffixes are placed in the list in alphabetic order.

¨	To set the domain suffix search order

1.	In the Domain Suffix Search Order box, type the domain suffixes to add to your domain suffix search list, and then click the Add button.

2.	To remove a domain name from the list, select it and click the Remove button.

When attempting to resolve a fully qualified domain name (FQDN) from a short name, Windows 95 will first append the local domain name. If not successful, then Windows 95 will use the Domain Suffix list to create additional FQDNs and query DNS servers in the order listed.

# Microsoft NetBEUI Protocol

Windows 95 provides the NetBIOS extended user interface (NetBEUI) protocol for compatibility with existing networks that use NetBEUI, including Windows for Workgroups, Windows NT, LAN Manager, Microsoft Workgroup Add-On for MS-DOS, and other networks. NetBEUI is a small, efficient protocol designed for small networks of 20 to 200 computers, and is nonroutable. It is ideal for large I/O requirements such as copying files from network servers or launching applications from network servers. For enterprise-wide networks that require a routable protocol, TCP/IP or the IPX/SPX-compatible protocol should be used.

NetBEUI in Windows 95 supports a NetBIOS programming interface, conforms to the IBM NetBEUI specifications, and includes several performance enhancements.

The NetBEUI protocol is implemented as both a real-mode and a protected-mode protocol. The NetBEUI module, NETBEUI.VXD, is accessible through the NetBIOS interface.

¨	To manually configure NetBEUI

1.	In the Network option in Control Panel, click the Configuration tab. Then double-click Microsoft NetBEUI.

If your computer has multiple network adapters, there will be an instance of NetBEUI for each network adapter. You must configure each adapter with its own settings.

The Bindings tab shows which clients and services are currently using the NetBEUI protocol. For information about configuring bindings, see “Configuring Network Adapters” earlier in this chapter.

2.	Click the Advanced tab to modify settings for Maximum Sessions and NCBs for the real-mode NetBEUI. (The real-mode network runs, for example, when you use Safe Mode for system startup.)

| Option | Description |
| --- | --- |
| Maximum Sessions | Used to identify the maximum number of connections to remote computers that can be supported from the redirector. Equivalent to the sessions= parameter formerly specified in PROTOCOL.INI. |
| NCBS (network control blocks) | Used to identify the maximum number of NetBIOS commands that can be used. Equivalent to the ncbs= parameter formerly specified in PROTOCOL.INI. |

Protected-mode NetBEUI dynamically sets Maximum Sessions and NCBs, and therefore does not use these settings.

3.	Click OK. Then shut down and restart the computer.

# Technical Notes for Windows 95 Networking

## Using WinPopup to Broadcast a Pop-Up Message

You can use WinPopup to send a message to one person or to a whole workgroup. WinPopup can also display a message from someone else on your network or from a printer when your print job is done.

With WinPopup on your computer, you can send and receive messages and alerts from LAN Manager, Windows NT, and Windows 95 servers and clients.

On a NetWare network, you can also use WinPopup to send a message to a user on a computer running a Novell-supplied client if the targeted user is attached to your preferred server.

·	If you are running a NetWare-compatible client, you can receive pop-up messages from the server you are attached to, plus messages from other users running Novell-supplied network clients, if the message is sent to you on the server using NetWare utilities.

·	You can use WinPopup to send a message to a user on a computer running Client for NetWare Networks or a Novell-supplied client if that user is attached to your preferred server.

·	If you are running both Client for NetWare Networks and Client for Microsoft Networks, and if the message reaches the specified computer or user through Windows 95 networking, the message will not also be sent through the NetWare server.

You can install WinPopup from the ADMIN directory on the Windows 95 compact disk

¨	To install WinPopup

1.	In the Add/Remove Programs option in Control Panel, click the Install/Uninstall tab.

2.	Use the Install Program wizard to connect to the Windows 95 compact disk. You will find WinPopup in the ADMIN\APPTOOLS\WINPOPUP directory.

¨	To configure WinPopup on a client computer

1.	Place WINPOPUP.EXE in the Startup folder on each computer that you want to receive messages.

2.	On each computer, use the Options command on the Messages menu to specify choices for how WinPopup will present messages.

¨	To send a message using WinPopup

1.	Click the Send button on the toolbar.

Or, from the Messages menu, click Send.

2.	Click an option to specify whether the message will be sent to a specific user or computer, or to a workgroup. Then type the name for where the message is to be sent.

3.	Type a message, and click OK.

As an example of how this might be used in a workgroup, you might want to run WinPopup on a computer running File and Printer Sharing services. Every other computer in the workgroup that uses shared files and printers on this computer can also run WinPopup. On each client computer, WinPopup can report messages from the printer (such as notification that a print job has been completed). Or administrators can send messages to users and computers in the workgroup with pop-up notification.

## Troubleshooting Protocol Problems

This section contains information for troubleshooting problems related to network protocols. For general information about troubleshooting network installation, see Chapter 7, “Introduction to Windows 95 Networking.” For information about troubleshooting procedures and tools provided with Windows 95, see Chapter 35, “General Troubleshooting.”

##### You cannot connect using NetBEUI.

·	Use NET DIAG to test for NetBIOS connectivity over the LANA which NetBEUI is using. If it fails, check the transceiver type, cabling, and adapter.

·	Check the NetBEUI Protocol bindings.

·	Verify that routing is not involved.

NetBEUI is a nonroutable protocol which cannot cross routers, though it can cross bridges and source routing bridges. NetBEUI can also cross properly configured routers (bridging routers).

##### A NetBIOS application fails to start.

If a NetBIOS-based network application is not working, this may be because it is hard-coded to use the protocol on LANA 0 (such as Lotus Notes). You can force a particular protocol to always occupy LANA 0 by selecting it as the default protocol, as described in “Setting LAN Adapter Numbers” earlier in this chapter.

##### You cannot connect using the IPX/SPX-compatible protocol.

Check the following:

·	Check the Bindings tab on the protocol’s property sheet.

·	Check the Frame Type on the Advanced property sheet for the protocol.

Verify that both computers trying to connect are using the same frame type. The AUTO frame type only checks SAP broadcast traffic on the network and may be selecting an incorrect frame type in a mixed frame-type environment.

¨	To check IPX Frame Type

1.	In the Network option in Control Panel, double-click the IPX/SPX-compatible protocol, and then click the Advanced tab.

2.	Verify or set the Frame Type. (The recommended setting is Auto.)

3.	Check Type 20 Packets (NetBIOS packets) and IPX Routers.

When using IPX over NetBIOS, the IPX packet type is set to 14h (decimal 20). Manufacturers of routers may consider all NetBIOS traffic as being nonroutable LAN traffic even though it is being carried over the routable IPX protocol, and so, by default, will not pass Type 20 NetBIOS IPX packets. To enable NetBIOS over IPX connectivity, Type 20 packet passing must be enabled on the router.

4.	Check that Token Ring Source Routing is enabled, and set a cache size if needed.

¨	To check whether Token Ring Source Routing is enabled

1.	In the Network option in Control Panel, double-click the IPX/SPX-compatible protocol, and then click the Advanced tab.

2.	Verify the Source Routing Cache Size is set.

3.	Check the option named Force Even Length Packets.

NetWare servers with older NetWare Ethernet drivers or older IPX routers may require even-sized packets. Change this setting to make sure the computer only transmits even-length IPX frames.

4.	Use net diag to test for IPX connectivity over the LANA used by IPX over NetBIOS (NWNBLINK).

5.	Use System Monitor to view statistics on the IPX/SPX-compatible protocol. Then retry network operation and check the activity. If there is none, remove and reinstall the protocol, and then retry and retest the operation.

##### You cannot connect using TCP/IP.

Use the TCP/IP diagnostic utilities included with Windows 95 TCP/IP to isolate network hardware problems and incompatible configurations. The following list describes which utility helps to identify problems.

| Use this utility | To accomplish this action |
| --- | --- |
| ping | Check host name, host IP address, and TCP/IP configuration; verify physical connection and remote TCP/IP computer |
| arp | Detect invalid entries in the ARP table on the local computer |
| ipconfig | Display, update, or release TCP/IP configuration values |
| nbtstat | Check the state of NetBIOS over TCP/IP connections, update LMHOSTS cache, and determine registered name and scope ID |
| netstat | Display statistics and state of current TCP/IP connections |
| tracert | Check the route to a remote computer |

¨	To test TCP/IP using ping

Check the loopback address by typing ping 127.0.0.1 and pressing ENTER at the command prompt. The computer should respond immediately. (If using DHCP, use IPCONFIG to find the IP address.) To determine whether you configured IP properly, use ping with the IP address of your computer, your default gateway, and a remote host.

If you cannot use ping successfully at any point, verify:

·	The computer was restarted after TCP/IP was installed and configured.

·	The local computer’s IP address is valid and appears correctly in the TCP/IP Configuration dialog box.

·	The IP address of the default gateway and remote host are correct.

·	IP routing is enabled on the router and the link between routers is operational.

If you can use ping to connect to other computers running Windows 95 on a different subnetwork but cannot connect through Windows Explorer or with net use \\server\share, verify that:

·	The correct computer name was used.

·	The target host uses NetBIOS. If not, you must use FTP or Telnet to make a connection; in this case, the target host must be configured with the FTP server daemon or Telnet daemon, and you have correct permissions on the target host.

·	The scope ID on the target host is the same as the local computer.

·	A router path exists between your computer and the target computer.

·	LMHOSTS contains correct entries, so the computer name can be resolved.

·	The computer is configured to use WINS, the WINS server addresses are correct, and WINS servers are functioning.

##### The “Unable to connect to a server” message appears.

This message appears if name resolution fails for a particular computer name. If the computer is on the local subnetwork, confirm that the target server name is spelled correctly and that the target server is running TCP/IP. If the computer is not on the local subnetwork, be sure that its name and IP address mapping are available in the LMHOSTS file or the WINS database. If all TCP/IP elements appear to be installed properly, use ping with the remote computer to be sure that its TCP/IP software is working.

Use the nbtstat -n command to determine what name (or names) the server registered on the network. The nbtstat command can also display the cached entries for remote computers from either #PRE entries in LMHOSTS or recently resolved names. If the remote computers are using the same name for the server, and the other computers are on a remote subnetwork, be sure that they have the computer’s mapping in their LMHOSTS files.

##### IP address connects but host names do not.

Verify that the appropriate HOSTS file and DNS setup have been configured for the computer by checking the host name resolution configuration using the Network option in Control Panel and then choosing the DNS Configuration tab in the TCP/IP Properties.

·	If you are using a HOSTS file, verify that the name of the remote computer is identical — especially in terms of spelling and capitalization — to the name in the file and the application using it.

·	If you are using DNS, verify that the IP addresses of the DNS servers are correct and in proper order. Use ping with the remote computer and type both the host name and IP address to determine if the host name is resolved properly.

Use the netstat -a command to show the status of all activity on TCP and UDP ports on the local computer. A good TCP connection is usually established with 0 bytes in the send and receive queues. If data is blocked in either queue or if the state is irregular, there is probably a problem with the connection. If not, you are probably experiencing network or application delays.

##### Connect times are long after adding to LMHOSTS.

You may experience long connect times with a large LMHOSTS file if there is an entry at the end of the file. If so, mark the entry in LMHOSTS as a preloaded entry by following the mapping with the #PRE tag. Then use the nbtstat -r command to update the local name cache immediately, or place the mapping higher in the LMHOSTS file. The LMHOSTS file is parsed sequentially to locate entries without the #PRE keyword. You should place frequently used entries near the top of the file and place the #PRE entries near the bottom.