ASCII download  2-Dec-1994 10:18a

�������������������������������������������������������������������������������

1st-  You will need the files Fixit.com and Fixed.bin and run this

from the DOS prompt. (see attached files)

Example: C:\> fixit fixed.bin 2

Fixit.com is the program

Fixed.bin is a supporting file\

The number 2 represents the com port that your modem is currently

connected to. (1-4)

This should find the modem and begin the repair process. When it is

finished you will get a message telling you the modems firmware is

corrupted. You will now load the VFC code date of 5-25. This is

important. You won't be able to load the new V34 code to the modem

after you just ran fixit. The reason being FIXIT changes the boot

block and it won't be compatible with the new V34 SDL. The new V34 SDL

will get a CRC error and abort. Guess how I found that out? SO, start

with the 5-25 code and once that has successfully loaded you can then

upgrade the modem's firmware with the NEW and IMPROVED V34 SDL!! and

it should work. Your modem or the customer's will have been brought

back from the dead!! WOOO! \