AIOHST.NLM v1.0

Comtrol Corporation

(October 1993)

The AIOHST.NLM driver is a NetWare Loadable Module (NLM) that

provides asynchronous input/output (AIO) driver support for Comtrol

intelligent, ISA, multi-port serial controllers, including:

o	Hostess i (8 and 16 port)

o	Hostess 186 (4, 8, and 16 port)

The AIOHST.NLM driver is loaded onto the NetWare NACS server along

with AIO.NLM and the NetWare Asynchronous Communication Services

(NACS) software. You can obtain the NACS software from Novell.

AIOHST.NLM provides the low-level software interface between AIO and

the Comtrol hardware, which includes

o	Handling all enqueuing and dequeuing of serial data

o	Modem control signal functioning

o	Hardware and software flow control

o	Port configuration

o	Status information

Most of these operations are performed by the processor on the

controller, which considerably reduces system overhead on the NetWare

server, while at the same time, increasing serial I/O performance.

AIOHST.NLM supports up to four Comtrol intelligent controllers in one

server. The controllers can have any configuration of, from 4 to 16 ports

each, for a total of 64 ports. Different Comtrol intelligent controller

types may also be mixed in one server. For example, one Hostess i and

two Hostess 186 16's is a valid configuration.

Baud rates from 50 to 38,400 are supported on all ports. All ports

support software and hardware flow control. Because of hardware

limitations (lack of pins on connectors), some modem control signals are

not supported, for example:

o	No ports support the ring indicator (RI) and the AIOHST driver

always reports this signal as FALSE (deasserted).

o	No ports support DSR (data set ready) and AIOHST always reports

this signal with the same state as carrier detect (DCD).

o	Ports 11 and 12 (10 and 11 when counting from 0) on 16-port

controllers do not support DTR.

Installation

------------

Use the following information to configure and install your Comtrol

intelligent controller.

1.	Use the appropriate User's Guide to install the controller in your

NACS server.

2.	Choose a unique I/O address for each Hostess i or Hostess

186 controller.

3.	Set the I/O DIP switch according to the I/O address choice

you have made. AIOHST.NLM supports the following I/O

addresses for Comtrol ISA intelligent controllers (the

addresses are listed in hexadecimal):

218, 238, 23C, 338, 33C, 618, 61C, 638, 63C, 718, 71C, 738,

and 73C

The I/O address that you choose will later be supplied to

AIOHST.NLM.

Note:	Be sure to choose an address that does not conflict

with other hardware in your NACS server.

4.	Select a memory address. AIOHST.NLM also uses a memory

buffer that is configured under software control (there are

no switches to set for this).

You will provide this memory address to AIOHST.NLM when

it is loaded. Supported memory addresses are:

A0000, A4000, A8000, AC000, C0000, C4000, C8000, CC000,

D0000, D4000, D8000, DC000, E0000, E4000, E8000, and

EC000

The controller will occupy a 16K region from the specified

base memory address. For example, if you chose D0000 as

the memory address, the controller would occupy memory

from D0000 to D3FFF.

Notes:	AIOHST.NLM allows multiple controllers to share

the same memory buffer region with each other. So,

if you have more than one controller in your NACS

server, you may choose the same memory buffer

address for all of the controllers.

AIOHST.NLM does not share memory regions with

controllers that are not Comtrol intelligent

controllers.

Ethernet and ARCNET adapters, SCSI controllers,

and video cards can use memory buffers in these

regions, so be certain of your hardware

configuration and choose a non-conflicting

address.

The Comtrol Cnic series of Ethernet adapters are

bus-master controllers and do not use a shared

memory buffer .

AIOHST.NLM does not use any interrupt (IRQ) signals.

If you are not presently running the NACS software, you must install it

according to the instructions provided in the NetWare Asynchronous

Communications Services Administration Guide.

You may find the following information useful if you need to install the

NACS software:

o	You may want to save a copy of the original NACSPORT.CFG file just

after the NACS software is installed. For example, copy

SYS:\SYSTEM\NACSPORT.CFG to SYS:\SYSTEM\NACSPORT.ORG.

o	Whenever new hardware is installed for NACS, you must reset all

of the port settings so that the new hardware is properly

recognized. The NACS console does not provide a clean facility for

doing this. Restoring the original, unconfigured NACSPORT.CFG is a

reliable way of resetting the NACS configuration. To reset, simply

copy the NACSPORT.ORG file you saved at installation on top of

NACSPORT.CFG.

You can load the AIOHST.NLM driver either manually from the NetWare

console or automatically from the AUTOEXEC.NCF file when the server

boots.

The NACS Administration Guide provides detailed instructions on how

to modify AUTOEXEC.NCF to load AIO drivers. Follow those instructions

and load AIOHST in a similar manner, making appropriate changes in

the command line syntax as detailed in the following table.

AIOHST.NLM command line syntax is as follows:

load aiohst port=IOADDR mem=MEMADDR {16BIT} node=NNN {name=SSS}

AIOHST.NLM Parameters

---------------------

port=IOADDR

IOADDR specifies the hexadecimal I/O address of the

Comtrol intelligent controller. This must match the

setting you chose earlier when installing the controller.

mem=MEMADDR

MEMADDR specifies the hexadecimal memory address

for the Comtrol intelligent controller buffer. You may

choose the same MEMADDR for all Hostess i or Hostess

186 controllers running under AIOHST.NLM.

16BIT

This option configures AIOHST.NLM to use 16-bit

transfers (rather than 8-bit) to and from the memory

buffer region. There is a performance advantage to

using 16-bit transfers, however care must be used in

enabling this option.

Due to an ISA bus limitation, all peripherals in the same

128K region must use the same width transfers. So, if

you have an Ethernet card at C0000 doing 8-bit

transfers and you install your Hostess i or Hostess 186

controller at C8000, it must also use 8-bit transfers.

Similarly, a SCSI controller at D8000 doing 16-bit

transfers would require a Hostess i or Hostess 186

controller at D0000 to also use 16-bit transfers.

If you know the type of transfers that all of your shared

memory peripherals are using, then choose this option

accordingly.

The default is to do 8-bit transfers. If you load

AIOHST.NLM and it fails to recognize your controller, try

the 16BIT option.

Likewise, you may want to try the 16BIT option for

performance gain and if AIOHST.NLM fails to recognize

the controller, you will have to use 8-bit transfers at the

particular memory address specified.

A 128K region is fairly large, covering two segments.

For example, addresses in Cxxxx and Dxxxx are both in

the same 128K region. Axxxx and Cxxxx are not in the

same 128K region, nor are Cxxxx and Exxxx.

node=NNN

This option overrides the default assignment of board

numbers that are passed to AIO for this particular

board. Normally, the I/O address for the board is used

as its identification to AIO. Due to a bug in AIO, however,

these numbers must be overridden so that consecutive,

zero-based indexes are passed to AIO for the

controllers.

The first time AIOHST.NLM is loaded, specify NODE=0.

The next time you load it, specify NODE=1, and so forth.

node=NNN

The idea is to associate the NODE number with a

particular board so that all references AIO makes to a

particular board number will always get it to the same

physical board, independent of the order that the

AIOHST.NLM drivers were loaded. For example:

Hostess 186 at io=218

Hostess i   at io=21c and io=238

load aiohst port=218 mem=d0000 node=0

load aiohst port=21c mem=d0000 node=1

load aiohst port=238 mem=d0000 node=2

then, Hostess i at 21c is removed and all its ports are

wired to the one at 238.

In order to prevent applications from having to be

reconfigured, we just redirect all requests for the

removed controller (io=21c) to a different board

(io=238).

load aiohst port=218 mem=d0000 node=0

load aiohst port=238 mem=d0000 node=1

name=SSS

This option sets the textual name for the controller. If

omitted, the textual name is blank. Some NACS

applications require a board name and access boards

by this name rather than by number.

If AIOHST.NLM is loaded without any command line parameters, it will

prompt for required information.

In this case, however, an invalid NODE= number will be generated due to

the AIO bug mentioned above. In this case, NACS may fail to load. You

should then UNLOAD AIOHST and reload it specifying a NODE= parameter

on the command line.

Novell and NetWare are registered trademarks of Novell, Inc.