+---------------------------------------------------------------------------+

|                                                                           |

|                                CADSIM                                     |

|                                                                           |

|  This file contains information about CADSIM enhancements (for all CADSIM |

|  platforms) which has been changed since the last revision of the CADSIM  |

|  User's Manual.  The information appears in chronological order, newest   |

|  last.  Where possible, forward references have been inserted next to     |

|  items whose information has been superceded by new information later in  |

|  this file.                                                               |

|                                                                           |

|  Refer also to other companion read-me files:                             |

|    INSTALL.DOC - new installation notes.                                  |

|    MOUSE.DOC - new mouse installation notes.                              |

|    VIDEO.DOC - new video graphics formats supported.                      |

|    MACROS.DOC - enhancements to the CADSIM macro scripting commands.      |

|    MAKEHP.DOC - enhancements to the CADSIM MAKEHP.EXE utility.            |

|    MAKEWP.DOC - enhancements to the CADSIM MAKEWP.EXE utility.            |

|    MAKEACAD.DOC - enhancements to the CADSIM MAKEACAD.EXE utility.        |

|    MAKEIGDF.DOC - enhancements to the CADSIM MAKEIGDF.EXE utility.        |

|                                                                           |

+---------------------------------------------------------------------------+

----------------------- CADSIM Version 2.1 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.0

and Version 2.1:

- EXCHANGER, TRANSFER AND FLASH HAVE BEEN IMPROVED.

- CADSIM USER'S MANUAL ERRATA:

The following two errors appeared in the October 1989 printing, but have

been corrected in the February 1990 printing of the CADSIM User's Manual:

Page 93: The missing last line should read:

".GRP file.  Save the part file and test it by inserting it in a test

simulation drawing. "

Page 245: WATER and STEAM must also be stream variables for an EVAPORATOR.

----------------------- CADSIM Version 2.2 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.1

and Version 2.2:

- MULTI-VIDEO MODE VERSION OF CADSIM FOR DOS:

See enhancements in Version 2.4 notes in the VIDEO.DOC file for more

details about accessing the various video modes.

- CURSORS MADE BOLDER WITH NEW DOS VERSION:

Cursors have been made bolder or accented so that they are more visible

on the mono or grey-scale screens of laptop computers.  Therefore,

cursors have a slightly different appearance than that shown in the

manual.  Their functions remain the same.

- OS/2 VERSION OF CADSIM IS RELEASED:

An OS/2 version of CADSIM is available for EGA/VGA/8514A workstations.

General notes on the OS/2 CADSIM:

OS/2 CADSIM is a full Presentation Manager program and therefore

requires a Presentation Manager version of OS/2 (Version 1.1 or later).

All .DRA, .PAR, .GRP etc. files are identical with those produced by

other versions of CADSIM, including the Macintosh and all DOS versions.

CADSIM utility programs (MAKEHP.EXE, MAKEWP.EXE, MAKEACAD.EXE,

MAKEIGDF.EXE, TXTTOPAR.EXE) can be run from the OS/2 command line.

An OS/2 minimize button has been added to the Function Field (upper

right hand corner) of the CADSIM drawing screen.

Refer to INSTALL.DOC and MOUSE.DOC for OS/2 CADSIM installation notes.

----------------------- CADSIM Version 2.3 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.2

and Version 2.3:

- MASSBAL OPTIMIZE KEYWORD IS NOW SUPPORTED:

To perform an optimization, add a variable MINIMIZE (variable-type text)

to any polygon.  MINIMIZE must be all capital letters.  You will be

prompted to specify this MINIMIZE variable.  MINIMIZE should be set to

whatever you wish to minimize such as a LINEAR equation summing up all

costs and subtracting all product values.  (As MASSBAL only minimizes,

if you wish to maximize, set the MINIMIZE variable to minus the variable

or specification equation you wish to maximize.)  You will also be

prompted for the optimization algorithm and for variables that should be

constrained to a particular minimum or maximum.

A new option on the specification menu allows you to chose to vary a

variable as part of the optimization.  This replaces the normal fixed

setting or equation that would be given with the other types of

specifications.  You will be asked to provide a minimum and a maximum

limit for each variable that is to be varied with the optimization.

Note:  DOS-MASSBAL does not support the OPTIMIZE keyword as of yet.

- MASSBAL INTERCHANGE KEYWORD IMPROVED:

INTERCHANGE changed to remove PHASE query to match MASSBAL implementation.

----------------------- CADSIM Version 2.4 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.3

and Version 2.4:

- EXPANDED MEMORY SUPPORT IMPROVES PERFORMANCE OF DOS CADSIM:

Expanded Memory gives the DOS version a performance boost and size

enhancement.  If your computer can support expanded memory, and at least

64K of it is enabled (and free), CADSIM will use it to cache fonts and

menus.  There are several noticeable improvements when expanded memory

is used.  These include: redraw times are reduced by 50%, simulation

files can be about 30K larger, cached fonts will be substituted for

slower vector drawn fonts and the x and y coordinates are updated on the

fly as the cursor is moved.  This last item (coordinate update in Field 1)

will be the most recognizable indication that expanded memory is in use.

We recommend enabling at least 64K of expanded memory to get these DOS

version improvements.  There are some expanded memory compatibility notes

in the INSTALL.DOC file.

- NEW AUDIT TRAIL FEATURE IN CADSIM:

Audit Trail feature allows documentation of selected specifications.

Specifications can be selected to receive an audit trail.  Once selected,

or each time the specification is adjusted thereafter, an audit trail is

automatically produced.  This audit trail automatically records a

specification date stamp, and the specification type and parameters --

which then form a comment which is left open for the user to edit.

Such information as the user's initials and the assumptions or

calculations related to the specification can be added to the comment.

Old entries remain and new comments are added each time a specification

is changed, forming a trail that audits the changes.  Entries can be

edited, added or deleted.  To add or edit an audit trail on an existing

specification use the command Adjust, then Select its free variable

pointer and then select the "Trl" menubar item.  To add an audit trail to

a specification that is about to be created, select "add audit trail to

specification" from the control element menu before selecting the control

element icons.

- RE-SELECTION OF CONTROL/SPECIFICATION ICONS SUPPORTED:

If you change your mind about the control icons that you have selected...

don't despair!  A new item "REDO control element menu" appears at the

bottom of the menu for selecting specification type.

- SIMULATIONS WITH OPTIMIZATION ALLOW CONSTRAINTS TO BE EDITED:

Existing optimization constraints can be deleted and new ones added

without re-entering the entire optimization specification.  CADSIM prompts

for possible changes with any simulation containing optimization.

- CADSIM INSTALL PROGRAMS FOR DOS AND OS/2 HAVE BEEN UPGRADED:

More flexibility has been included for selection of source and target

drives.  CADSIM executables are now distributed in compressed form for

DOS.  The install program automatically expands the appropriate

executable during installation.

The DOS multimode version will install one of two CADSIM.EXE files

into the \BIN directory.  One of the CADSIM.EXE files will only run on a

computer with a math co-processor (it is smaller and faster).  The

other one is not co-processor specific.

- ENHANCEMENTS FOR OS/2 CADSIM:

OS/2 performance has been improved.  In addition, if your screen and the

OS/2 Presentation Manager have been set up to support a resolution

greater than 640 x 480 (VGA), the initial CADSIM window will be sized

640 x 480 -- allowing greater access to other PM functions without

minimizing or leaving CADSIM.  This CADSIM window can also be maximized

to the maximum resolution using the new maximize button in the CADSIM

menubar or minimized to an icon using the minimize button.

- NEW SCREEN CURSORS FOR THE DOS AND OS/2 VERSIONS OF CADSIM:

For some functions, the small cross-hair cursor has been replaced with a

large arrow for better visibility.  A new drawing mode "pencil" cursor

has been introduced.  This cursor appears during drawing and text entry.

- MACINTOSH IMPROVEMENTS:

Macintosh performance has been improved.  In addition, extended keyboard

function keys F1, F2 and F3 now emulate the 3 mouse button functions as

described in the CADSIM manual for DOS-type computers.

- ENHANCED MULTI-VIDEO MODE VERSION OF CADSIM FOR DOS:

A new version of CADSIM supports multiple graphic standards from a single

version.  This should make changing between hardware easy. The standards

supported are:  CGA, EGA, VGA, Super VGA, 8514A, VESA, TIGA, Hercules and

Toshiba.  Resolutions up to 1280 x 1024 are supported.  CADSIM uses

auto-sensing.  For most hardware it will automatically select the highest

resolution and number of colors (max 16) supported by your graphics card.

Refer to the VIDEO.DOC file for a list of graphics cards supported and

for information on how to override CADSIM's auto-selection process.

----------------------- CADSIM Version 2.5 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.4

and Version 2.5:

- CADSIM PRINTER AND PLOTTING PROGRAMS HAVE BEEN UPGRADED.

- OS/2 v2.0 AND PRINTING FROM CADSIM:

In order for CADSIM to print automatically under OS/2 2.0, disable

the Print Spooler so that CADSIM can print directly to the Printer

Device.  The Print Spooler can be disabled by selecting the OS/2 System

Folder; select System Setup; select the spooler icon with the middle

(or right) mouse button and select disable.  Remember, CADSIM always

generates a print (.PRN) file when printing, even if automatic spooling

fails.  (There have been changes to this in version 2.6  -- see notes below

in that section).  PRN files can be printed directly by dragging them in

icon form and dropping them onto the Printer Device icon or by issuing the

command:

PRINT [filename].PRN

at either the DOS or OS/2 command prompts.

- EDIT PROGRAM IS NO LONGER SHIPPED WITH MACINTOSH CADSIM:

The Apple program Edit no longer runs under Macintosh System 7.1 with

a large number of fonts loaded; is no longer a supported program from

Apple; and is therefore no longer being shipped with CADSIM.  All support

files which come with CADSIM, which were editable with Edit can now be

edited with the Apple program TeachText, which is included with System 6

and System 7.  If the user does not have a copy of TeachText, it can be

obtained from an Apple dealer or reseller.  Most of the functions which

were provided by Edit are available with TeachText.  Some files may be

too large for TeachText.  In this case the user may wish to obtain a

text editor program from a Macintosh software vendor.

- NEW COPY PROTECTION DEVICES FOR CADSIM:                 -- March 30, 1993

The serial Protector Box (Black Box) that has been used as the copy

protection device for all copies of CADSIM on all platforms (DOS,

OS/2 and Macintosh) will no longer be shipped.  New orders of CADSIM

will be shipped with a new and smaller Parallel Protector (printer port)

for DOS and OS/2 and a new ADB Protector for Macintosh clients.

Parallel Protector devices are distinguishable from the older serial port

Protector Boxes by their shape, size and colour.  The new parallel port

devices are approx. 1" X 2" X 1/2" thick and are white in colour.

The old serial port devices are approx. 2" X 4" X 1" thick and are black.

Refer to INSTALL.DOC for installation details.

- HPGL PLOTTER EMULATION ON LASER PRINTERS:                -- April 4, 1993

Some laser printers (including many H.P. LaserJet printers) will emulate

an H.P. pen plotter.  Refer to the MAKEHP.DOC file for more details.

----------------------- CADSIM Version 2.6 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.5

and Version 2.6:

---- CHANGES TO TEXT SUPPORT ----

- SOLITARY TEXT:                                      -- (new) August, 1993

A new entity, solitary text, can stand on its own without being attached

to any other entity (line, polygon, part etc).  Solitary text can only be

entered on a drawing in nonprocess mode -- use Set Nonprocess.  Select

Text from the nonprocess drawing mode menubar and then "Text (solitary)".

Solitary text can be deleted without deleting any other elements.

- SOURCE, TIME AND DATE STAMP SIMULATION RESULTS:     -- (new) August, 1993

Special codes that start with a "?" followed by a descriptor will turn

solitary text into a type of nonprocess format that allows these codes to

be interpreted and returned with the simulation results.  This only works

with solitary text.  Similar layering rules as FORMAT text types apply.

The available codes and range of results are:

?min             00 - 59

?hr              00 - 23

?day             1 - 31

?month_name      January - December

?month           1 - 12

?year            1993 - 2099

?filename        drawing file root name (before the .DRA extension)

Examples:  The solitary text "?MONTH_NAME ?DAY, ?YEAR" might return

"August 17, 1993" as part of the results file.  The solitary text

"DRAWING FILE: ?FILENAME.DRA" might return "DRAWING FILE: CLEANING.DRA".

The solitary text "?day/?month/?year ?hr:?min" might return the result

"17/8/1993 11:05".

- BETTER THAN A MAGNIFYING GLASS:                     -- (new) August, 1993

Text input is now echoed in the message field #2 at the top of the screen

as well as on the drawing itself.  So if the text you are typing in is too

small to read on the drawing, look in message field #2.

---- NEW DRAWING FEATURES ----

- ELLIPSES:                                           -- (new) August, 1993

A special form of circles/arcs has been added where they can be elongated

or shortened in the vertical direction. This is accomplished with an

aspect ratio of vertical over horizontal radius.  Select the Aspect

command when a new circle is being drawn to adjust it into an ellipse.

Circles have an aspect that is 1.0.

NOTE:  ellipses are not supported by HPGL -- MAKEHP and MAKEWP convert them

to circles/arcs.  Unclosed ellipses (an ellipse arc) are not supported by

AutoCAD -- MAKEACAD converts them to circular arcs.

- STYLED CIRCLES:                                     -- (new) August, 1993

Circles, arcs and ellipses now pick up the current nonprocess line style

(dashes, dots etc.)

- LINE PATTERNS ALLOW AREA FILLS, SHADING, EFFECTS:   -- (new) August, 1993

Line patterns are a collection of nonprocess lines all having the same

style, color, layer, thickness and no arrow heads and can have a pattern

that repeats.  Their advantage over individual nonprocess lines is much

smaller storage size and faster drawing performance.  Line patterns are

made by collecting existing nonprocess lines into a line pattern.  Use

command Adjust Window (window all nonprocess lines to be collected) Make

Line pattern.  You will be prompted to select an example nonprocess line

-- select one and all similarly styled lines within the window will be

collected into the pattern.  If a pattern repeats between successive lines

then a repeat number avoids storage of individual lines.

Line patterns are stored as a starting line (with its style attributes)

followed by a series of pattern entries each having: a delta x and delta y

to get from the start of the current line to the start of the next line,

a delta x and delta y to get from the end of the current line to the end

of the next line, plus a repeat number if this sub pattern of deltas is

repeated.

Understanding how these pattern repetitions are stored can help you draw

more effectively.  For example, to fill a shape such as a box, do not draw

all the lines that would fill it with stripes -- rather, just draw two

vertical nonprocess lines, the first at the left hand edge of the box and

the second very close but to the right of the first.  Make a line pattern

of these two lines using Adjust Window, Make Line pattern.  Then fill the

rest of the box by using Adjust, (selecting the line pattern just made)

and increase the repeat number to fill the box.

The whole line pattern can be Adjusted or Deleted as a single entity.  It

can also be expanded back into individual nonprocess lines (to work on

individual sections or sub-patterns) by using Adjust and selecting Expand

to lines.  Individual line patterns (such as two overlaid line patterns

that from a cross hatch) can be joined together by expanding them both to

lines and then making a single line pattern of the combined sets of lines.

- GOODBYE TO ZOOM USING ONLY PRECISE, FINE, MEDIUM, COURSE:    -- Aug, 1993

Formerly, it was only easy to access the zoom scales:

Precise (0.1 mm/pixel), Fine (0.25 mm/pixel), Medium (0.5 mm/pixel) or

Course (1 mm/pixel) which were compatible with the fundamental pixel grid

on the screen.  Drawing at an arbitrary scales (of say 0.39746 and then

0.124 mm/pixel) would (and still does) result in lines that were not

perfectly horizontal or vertical (regardless of guideline/snap settings)

because the scales were no even multiples of a fraction of a mm/pixel.

It has been made easier to get at the many intermediate, larger and

smaller zoom scales which are multiples of 0.05 mm/pixel.  In addition

to the zoom menubar selections Precise, Fine, Medium and Coarse, using the

arrow keys or the menubar selections Extent and "Nearest grid" now also

leave the drawing in a pixel grid compatible scale.  Zoom Window does not

leave the drawing in a pixel grid compatible scale, but you can follow up

with "Nearest grid".

Warning messages have also changed:  Whenever entering drawing mode, you

may get a pop up message "Drawing scale is not grid compatible" with a

number of choices.  Select "Use nearest compatible zoom" if you wish to

do any drawing.

- DRAG ONLY ON OPTION:                                -- (new) August, 1993

Formerly, accidental mouse movements could give unwanted misalignments

of drawing elements because they could be automatically dragged with the

mouse.  Throughout CADSIM, we have changed the philosophy:  drag is no

longer automatic, it must be specifically requested by "Drag enable".

Drag can be toggled on and off, but the default is off.  This affects part

insertion, and repositioning commands like Adjust Window Stretch-move

and Adjust Position.

- MORE INFORMATIVE BLINKING:                          -- (new) August, 1993

When one element of many is selected during Adjust or What, all items are

first blinked followed by several blinks of just the selected element.

This blinking pattern is repeated and helps identify the specific element

selected.  Two examples where this is useful:  1) Adjust of one piece of

text associated with an item with many pieces of text; and 2) Adjust of

one line of a Polygon.

- KEYBOARD CONTROL FOR POP UP MENUS:                  -- (new) August, 1993

Hitting any keyboard letter will accelerate the pop up menu selection

down to first menu item starting with that letter.  Successive hits of

the same letter cycle through all selections starting with that letter.

This can make insertion from a long list of parts faster.  The Enter key

can now also be used to select a highlighted pop up menu item.

- ADDITIONAL WARNING MESSAGES:                        -- (new) August, 1993

Close to zero length nonprocess lines:  Nonprocess lines which are too

short are warned/ignored when a drawing file is loaded.

- DEFINE STANDARD LINE THICKNESS, STYLE, COLOR ETC.:     -- (new) Jan, 1993

A set of standard lines (style, color, layer, thickness, arrow length) can

be defined in the CADSIM.INI file, along with a name by which they can be

refered.  See the section "CADSIM.INI FILE CHANGES" below.  To set line

settings to match those of one of the standard defined lines, use the

command Set Line Menu.  A pull down menu listing the names of the standard

lines, as well as a preview screen showing a sample, will appear for your

sellection.  CADSIM version 2.6 has a CADSIM.INI file that contains

standard line style example definitions to demonstrate this capability.

- CUSTOMIZE LINE STYLE PATTERNS:                     -- (new) January, 1993

The patterns that make up the dots or dashes or solid line styles can be

customized with entries in the CADSIM.INI file.  See the section

"CADSIM.INI FILE CHANGES" below.

- COPY FUNCTION:                                     -- (new) January, 1993

A copy function allows one section of a drawing to be copied to one or

more other locations.  Use the commands Adjust Window (select items in

part) Make Copy.  You will be asked only for the origin of the copy and

then for the first location for a copy.  If you do not want several

repated copies, use the Quit command next.  An intermediate file COPY.DRA

is produced which can be used with subsequent Insert commands.

- NON-PROCESS LINE SEGMENT REMOVAL:                  -- (new) January, 1993

Segments of non-process lines can be removed to form a hidden section.

Use Adjust (select a non-process line) and select "Erase segment".  You

will be asked to digitize the locations for segment removal.  The line will

become two shorter, alligned, but unconnected lines.

- KEYBOARD MACRO RECORD, PLAY AND PROGRAMMING:       -- (new) January, 1993

Keystrokes and mouse motions can be recorded in keyboard macros by hitting

Ctrl-K on the keyboard.  Each macro can be named by a letter (K is the

default).  The macro can be played back by hitting Alt-x (where x is K for

the default, or other letters for permanently stored macros).  Logic and

programming can be added to macros.  A complete list of macro commands

is documented in a file called MACROS.DOC in the README directory.

A macro can be launched automatically when CADSIM is started by including

the switch:

CADSIM [filename] -Sx

where "filename" could be any CADSIM drawing file and "x" could be any

letter which represents a previously recorded macro.  A good example of

how this can be set up is the DEMO macro which resides in a subdirectory

(or sub-folder) of CADSIM called DEMO.

---- NEW SIMULATION FEATURES ----

- ALTERNATIVES; USING "DORMANT PROCESS" LAYERS:       -- (new) August, 1993

Process lines, Polygons and Controls on layers that are made Dormant

cannot take part in the process or the simulation.  Process elements on

these layers are essentially considered nonprocess elements.  Use the

commands Set "Drawing layers" "Dormant process" then select "Layer" and

enter the layer number.  The layer can be toggled active again by

selecting "Active process" instead of "Dormant process".  Use is similar

to the Invisible and Visible commands.

Note:  the Invisible and Visible commands are now one sub menu down under

Set "Drawing layers".  Use "Set" "Drawing layers" "Invisible" "Layer" and

enter the layer number.

By toggling layers between Active and Dormant, the scope and function of a

simulation problem can be changed without losing the configuration and

specification of the alternate (currently dormant) process elements.

- RELAXED NAMING; USING "UNIQUE NAME" LAYERS:         -- (new) August, 1993

Parts will require names to be entered on insertion only if it has

Polygons that are on "unique name" layers.  Otherwise, no name will be

required on insertion of a part.  The reason this is possible is that

Polygons on "unique name" layers will be forced automatically to have

a unique name (using unique point ID numbers).  This also implies that

several Polygons on a unique name layer can have the same given name.

This is the case as all Polygons on a unique name layer will be forced to

have unique names.

Use the commands Set "Drawing layers" "Unique names" then select "Layer"

and enter the layer number.  The layer can be toggled conventional again by

selecting "Conventional names" instead of "Unique names".  Use is similar

to the "Invisible"/"Visible" and the "Dormant process"/"Active process"

commands.

- SIMULATION 'MEASUREMENT'S SUPPORTED:                -- (new) August, 1993

Any specification can be replaced with one or more MASSBAL MEASUREMENT

type of specifications.  MASSBAL will do a best fit through the

overconstrained weighted process measurements given.  To add a measurement,

use Adjust during drawing mode and select a control free variable pointer

icon, then select More then Measurement and place the measurement icon on

some line, select a variable and enter the measurement value and a

significant weighting factor.  The weighting factor gives the estimated

confidence in the measurement.  Measurements can be deleted, added or

adjusted separately from the parent specification.  The parent

specification is not lost, you can select the type of MASSBAL data file

to be made from either "normal specifications", or "use measurements".

- ADDER SUPPORTS MULTIPLE STREAM TYPES AS INPUT:      -- (new) August, 1993

The ADDER unit can now accept inlet streams with different stream

chemistry definition.

- REVISED PROMPTS, FREE VARIABLES FOR STREAMS WITH PHASES:  -- August, 1993

Formerly, units with vapour liquid equilibrium streams prompted for

LIQUID-VAPOUR as a free variable and then QUALITY rather than TEMPERATURE

if the phase was specified as MIXED.  This caused confusion for many users

(and in some cases to CADSIM as well).  These free variables have been

changed to have more meaningful names.  For example a SOURCE unit having

WATER, STEAM, TEMPERATURE and PRESSURE as stream variables will first

prompt for PHASE and then depending on that phase specification it will

prompt for the following outlet stream free variables:

Phase      New free variable prompts

LIQUID     WATER TEMPERATURE PRESSURE

MIXED      WATER STEAM       PRESSURE

VAPOUR     STEAM TEMPERATURE PRESSURE

NOTE:  Since the free variable names have changed, some specifications in

existing files will no longer be recognized (CADSIM will prompt for the

new free variable name and ask that the old specification be deleted).

To avoid having to re-enter specifications, old drawing files can be

edited with a text editor to replace the free variable names as follows:

Replace any occurrence of LIQUID-VAPOUR with the liquid component name such

as WATER on lines starting with "C ".  For example:

C      50.000    120.000  2.000  2.000    0.00  4 201  4 LIQUID-VAPOUR

becomes

C      50.000    120.000  2.000  2.000    0.00  4 201  4 WATER

Similarly, replace any occurrence of QUALITY with the vapour component name

such as STEAM on lines starting with "C ".  Make sure the line starts with

"C " and not " S" as QUALITY may be a valid variable as part of some

specification.

- ADDITION OF PHASE SPEC AND NAME CHANGE ON FLASH UNITS:    -- August, 1993

A specification for the phase of the VAP stream from a FLASH unit has been

added to indicate whether that stream is to be considered saturated (the

default) or superheated (the vapour leaving the flash is the same

temperature as the liquid leaving such as would be the case with a liquid

with a boiling point rise).

Also, a recent change to MASSBAL 2 requires that the inlet stream for a

FLASH unit now be name "IN".  This was not previously required.  This

note is an addendum to the CADSIM User's Manual, page 246, FLASH

requirements.

- ALTERED ZONE OF SENSITIVITY AROUND CONTROL ICONS:         -- August, 1993

Formerly, it was hard to select a line hidden under a number of control

icons.  Control icons are now sensed using a 'box' surrounding the icon,

but shifted slightly to the side of the line that the icon is on.  This

allows discriminating between the line and between icons on opposite sides

of the line.

- CONTROL ICONS TRANSPORTED AROUND CORNERS:           -- (new) August, 1993

Select "Jump over elbow" from the menubar when placing a free variable

icon pointer and it will jump over (one-in/one-out) line connections.

This gives more room for placement of specifications -- especially useful

when the original line is very short and requires multiple specifications.

- CONTROL ICONS SMARTER TOO:                          -- (new) August, 1993

On units with multiple outlet streams, the free-variable pointer control

icon will skip over streams which already have a specification for the

unit outlet free variable currently being prompted for specification.

In addition, if the "Next" outlet stream is off the screen, a repaint is

now forced with the center of the screen at the start of the line

representing that stream.

- ADDITIONAL WARNING MESSAGES:                        -- (new) August, 1993

Orphaned formats:  If text which is of FORMAT type becomes orphaned from

the starting segment of a process line that is being bisected by a joining

line, a warning message is given.  The format will belong to the segment

of the line that started the original line, but is closer to the new

ending segment which has been created by the bisection.  Move the format

text to a more reasonable location.

---- CHANGES TO PARTS ----

- PARTS CAN BE ROTATED ON INSERTION:                  -- (new) August, 1993

Insertion of a part (.PAR extension only) allows the optional entry of a

Rotation angle.  Text is rotated, but may be re-oriented within its

boundaries to avoid upside down text.

- INSERTING PARTS; IMPROVED MIRRORING, POSITIONING:         -- August, 1993

Horizontal mirror and Vertical mirror menubar options have been added for

direct mirroring of inserted parts.  Scale (formerly Rescale) can still

be used with negative multipliers to mirror as well.  In keeping with our

new philosophy of allowing dragging of items only by option, inserted

parts can be dragged only if Drag enable is selected.  Removing the

former automatic drag feature should avoid accidental misalignments

due to a slight mouse movement.

- REPEATED INSERTION OF THE SAME PART:                -- (new) August, 1993

Easy multiple insertions of the same part can be set up by selecting

"Insert this part repeatedly" from the menubar during insertion.  If this

was selected, you will not returned to drawing mode when the part has been

placed.  Instead, repeated repositions with the mouse followed by Insert

will automatically select the same part name.  Quit returns to drawing

mode.

- NEW METHOD TO DELETE PARTS:                         -- (new) August, 1993

When any element within a part is selected for deletion, confirming the

deletion by selecting the "Part partname (delete full part)" menubar item

will delete the whole part.  This new option has been made necessary

partly since the requirement to name parts on insertion has been relaxed

in some cases.  Parts now only require names entered at insertion time if

they contain polygons that are on layers that are not unique name layers.

Formerly the only way to delete an entire part was by deleting its name

which was entered at insertion.

- PARTS CAN ORIENT THEMSELVES TO A LINE ON INSERTION:    -- (new) Oct, 1993

Parts can now orient themselves to the angle of a line and can chop a

segment out of a line when they are inserted.  To give a part this

capability, CADSIM the .PAR file.  Select Orientation from the menubar.

Locate the Orientation point which defines the orientation relative to

the origin of a part.  If no line segment(s) are to be chopped from the

process line(s) over which a part is being inserted, select Quit.

Otherwise, locate the Start point and the End point of each line segment

that should be chopped on insertion.  Save the .PAR file making sure to

redefine the part-name position and the carry-on drawing position with

the cursor anchor and active points.  A part thus modified will then

automatically orient itself over a process line during insertion and it

will delete any line segments that align with the segments defined for

delete.

- PREVIEW PARTS WITH HIERARCHIAL POP UP MENUS:       -- (new) October, 1993

Parts from a library of parts can be previewed in special hierarchial

pop up menus.  The preview shows the part in a small window of the menu,

and shows the location of the origin.  These hierarchial pop up menus can

be defined as sub menus of the main Insert menu using a command in the

CADSIM.INI file as described in the next section.  Parts libraries can thus

reside in a central disk location (parts do not have to be in the current

directory) with the CADSIM.INI file describing their location.  Group

(.GRP) files may also reside in a different directory.  CADSIM version 2.6

parts have been arranged to demonstrate this capability.

All Insert menus now have two new entries "cancel" and "type in name".

This avoids the older obscure method of hitting a mouse button with no

part selected to get these alternatives.

- ADDITIONAL WARNING MESSAGES:                       -- (new) October, 1993

Overwrite existing part?:  When making a part (Adjust Window, Make Part),

if the given part name exists, a warning asks if it is to be overwritten.

---- CADSIM ORGANIZATIONAL CHANGES ----

- CADSIM.INI FILE CHANGES:  PARTS LIBRARIES AND MORE...  -- (new) Oct, 1993

Older CADSIM.INI files can still be read, but a new format described

below gives new initialization features.  Each entry in the CADSIM.INI

is now described starting with a one-word command as follows:

MM_PER_PIXEL number

This number describes your computer screen's pitch in mm/pixel.

GRAPHICS_MODE mode

DOS only.  This number describes the graphics mode CADSIM should use.

For a list of mode numbers refer to the VIDEO.DOC file.

COLOR  color_number   %red   %green   %blue

This command can be repeated for color_number from 0 through 15 to

define the 16 CADSIM screen colors in percentages.

INVERT_COLOR   %red   %green   %blue

OS/2 only.  This defines the color to use for inverting menu

selections.

PARTS_LIBRARY library_name path

This describes the library name for a sub menu item in the Insert pop

up menu.  The path describes the full path where the library of parts

are located.  See the "preview parts with hierarchial menus" item above.

Files with the extensions .PAR, .BOR or .DRA are placed in the menu.

GROUPS_LIBRARY path

This describes the library path for GROUPs (.GRP).  The path describes

the full path where the library of GROUPs are located.  If this command

is missing GROUPs will be looked for in the current working directory.

MACROS_LIBRARY path

This describes the library path for macros (.mac).  The path describes

the full path where the library of macros are located.  If this command

is missing macros will be looked for in the current working directory.

One exception, the default Keyboard macro ALT-K.MAC is always local.

LINE_MENU line_name style color layer thickness arrow_length

This command adds a line style to a standard line selection menu which

can be used to set line settings using the drawing command Set Line

Menu.  The line_name will be used to refer to that line style in the

menu.

LINE_STYLE style_number pattern

This command changes the defined line style pattern (dots/dashes).

The pattern can be any combination of 16 spaces ' ' and dashes '-'

inside double quotes that represents the required line pattern.

(Default line patterns may perform faster on some systems.)

MOUSE_BUTTONS left middle right

This command changes the mapping of the mouse buttons.  The normal

mapping is left = 1, middle = 2 and right = 3.

A new CADSIM.INI file might look like this:

MM_PER_PIXEL 0.26

COLOR  0   0   0   0

COLOR  1 100   0   0

COLOR  2 100 100   0

COLOR  3   0  90   0

COLOR  4  66  66 100

COLOR  5 100   0 100

COLOR  6   0 100 100

COLOR  7  66  66  66

COLOR  8  33  66  33

COLOR  9  33  33  33

COLOR 10 100  66   0

COLOR 11  66  33   0

COLOR 12  33  66   0

COLOR 13   0 100  66

COLOR 14 100  66 100

COLOR 15 100 100 100

INVERT_COLOR 100 10 100

PARTS_LIBRARY borders c:\cadsim\borders\

PARTS_LIBRARY stream_defs c:\cadsim\sdefs\

PARTS_LIBRARY heating c:\cadsim\heating\

PARTS_LIBRARY pressure c:\cadsim\pressure\

PARTS_LIBRARY pulp c:\cadsim\pulp\

PARTS_LIBRARY vapor c:\cadsim\vapor\

PARTS_LIBRARY process c:\cadsim\process\

PARTS_LIBRARY nonprocess c:\cadsim\nonproc\

PARTS_LIBRARY groups c:\cadsim\groups\

GROUPS_LIBRARY c:\cadsim\groups\

MACROS_LIBRARY c:\cadsim\macros\

LINE_MENU major_process    1  6   1 0.500  5.00

LINE_MENU minor_process    1  6   1 0.100  3.00

LINE_MENU major_equipment  1  1   3 0.750  0.00

LINE_MENU minor_equipment  1  1   3 0.100  0.00

LINE_STYLE 2 "        --------"

LINE_STYLE 3 "------  ------  "

LINE_STYLE 4 "----    ----    "

LINE_STYLE 5 "----    --------"

LINE_STYLE 6 "  --  --  ------"

LINE_STYLE 7 "   --   --------"

LINE_STYLE 8 "        --    --"

MOUSE_BUTTONS 1 2 3

Comments can be included at the end of each command line if separated by

a space or a tab.

- SYSTEM RESOURCES DIR AND SORT NO LONGER USED:            -- October, 1993

DOS, OS/2 Users -->

Formerly, CADSIM used the system DIR and SORT programs to build a list of

parts that could be inserted.  This gave problems if SORT was missing and

with HPFS file formats.  These services are now written into CADSIM and

in addition to resolving the problems, give much faster load times.

- PATH IS STRIPPED FROM DRAWING FILE NAME:                 -- October, 1993

DOS shell, Windows, OS/2 Users -->

Double clicking on a cadsim drawing file (assuming .DRA has been

associated with a CADSIM executable or batch file) will likely pass the

drawing name along with the drive and full path to that file.  CADSIM now

parses the input file name and strips off this extra information.  This

extra information was confusing CADSIM about the real file name.  Because

of the need to find related .GRP, .PAR and .RES files, CADSIM still

assumes the drawing file is in the current working directory.  This change

allows CADSIM to be launched by shell programs, by double clicking on a

drawing and drag and drop launching under shells such as the OS/2 WP shell.

- MASSBAL INTERFACE MODIFICATIONS:                         -- October, 1993

All Users -->

Two exit pop up menus "Execute MASSBAL" and "Quit CADSIM" replace the

former single "Execute MASSBAL" menu item.  The "Execute MASSBAL" menu

item may be greyed out if MASSBAL is not present on the computer.

DOS Users -->

MASSBAL is now launched with MASSBAL.BAT (this was formerly DOMARKII.BAT).

OS/2 Users -->

CADSIM is now always launched with a CADSIM.CMD or CS.CMD file.  This

enhances control over interface with MASSBAL.  MASSBAL is now launched

with MASSBAL.CMD (this was formerly DOMARKII.CMD).

---- PRINTING AND PLOTTING ----

- PRINTER PLOTTING; PRINT DIRECTLY, SPOOL OR PRINT TO FILE:    -- Oct, 1993

DOS, OS/2 Users -->

When making a printer plot, you will be asked to select the plot printing

method.  Formerly, CADSIM would plot to a file and try to spool it using

PRINT.  There are now tree options as follows:

Make plot file and spool with PRINT

Make plot file for later printing

Send directly to printer without spooling

The second option is useful if there is currently no printer connected to

your computer.  The third option is the fastest, but requires a printer to

be connected and turned on as the output is sent directly to 'PRN' and no

plot file is made.  These options should be a great help to OS/2 users

without connected printers, or with unique spooling set-ups.

- LASER PRINTER PLOTTING SUPPORTED:                  -- (new) October, 1993

Print plotting support has been developed for laser printers that are

compatible with the HP LaserJet+ and HP LaserJet II printers.  Either a

"Print screen", or a "Multiple page plot" can be made.  Since laser

printers are page plotters and not continuous paper plotters, the plot has

to be a multiple page plot if it does not fit on one page.  Either

portrait or landscape multiple page plots can be selected from the

following paper sizes:  Letter 8.5" x 11", Legal 8.5" x 14",

Ledger 11" x 17", A4 210mm x 297mm, or A3 297mm x 420mm.  Not all paper

sizes have been tested:  Letter 8.5" x 11" has been tested on an HP

LaserJet II and an HP LaserJet 4; Legal 8.5" x 14" has been tested on an

HP LaserJet 4.

Please note: CADSIM Laser printer drivers are set up to print at a

resolution of 300 D.P.I.  Consequently, it will be necessary for your

laser printer to have a minimum of one to one and a half megabytes of

memory in order to print a full 8.5 x 11 inch page of CADSIM graphics.

If your laser printer lacks sufficient memory, you may find that it will

eject pages that only contain half a page or less of graphics.

---- CHANGES TO CADSIM UTILITIES ----

- MAKEHP/WP DRAWING FILES WITH A SINGLE PEN COLOR:   -- (new) October, 1993

Refer to the MAKEHP.DOC file for more information.

- MAKEACAD.INI FILE CONTROLS AUTOCAD CONVERSION:     -- (new) October, 1993

Refer to the MAKEACAD.DOC file for more information.

---- OS/2 SUPPORT  ----

- COPY PROTECTION AND PLATFORM SUPPORT:           -- (new) February 4, 1994

OS/2 USERS -->

Version 2.6 of CADSIM for OS/2 is a 32 bit Workplace Shell program.

Consequently, it requires OS/2 version 2.0 or 2.1 in order to run.

32 bit CADSIM performs some functions, such as file reading and writing

at least twice as fast as older 16 bit (OS/2 1.x) versions of CADSIM.

Unfortunately, the old style CADSIM Protector Box (serial port copy

protection device) can no longer be supported by OS/2 versions 2.x.

Any user who has upgraded to v2.6 of OS2-CADSIM from a previous

version will be upgraded to the new style parallel port copy protection

device at no extra charge.

- CADSIM TUTORIAL - ADDENDUM TO CHAP. 2 - CADSIM USER'S MANUAL -- Feb.4 1994

The reorganization of the CADSIM directory allows the use of part

libraries which reside outside of the current working directory.

The distribution version of CADSIM has relocated some of the parts

which are needed to do the Chapter 2 Tutorial in the CADSIM User's

Manual.  If the tutorial is done in the CADSIM directory, parts are now

relocated as follows:

- PULP_AM.PAR is now located in the Insert library called "SDEFS"

- SCREEN.PAR is now located in the Insert library called "PULP"