+---------------------------------------------------------------------------+

|                                                                           |

|                            MAKEACAD utility                               |

|                                                                           |

|  This file contains information about changes, and enhancements to the    |

|  MAKEACAD utility (for all CADSIM platforms) which has been changed since |

|  the last revision of the CADSIM User's Manual.  MAKEACAD converts CADSIM |

|  drawings to an AutoCAD script file for import into AutoCAD.  Information |

|  appears in chronological order, newest last.  Where possible, forward    |

|  references have been inserted next to items whose information has been   |

|  superceded by new information later in this file.                        |

|                                                                           |

+---------------------------------------------------------------------------+

----------------------- CADSIM Version 2.1 ----------------------------------

(no MAKEACAD changes)

----------------------- CADSIM Version 2.2 ----------------------------------

(no MAKEACAD changes)

----------------------- CADSIM Version 2.3 ----------------------------------

(no MAKEACAD changes)

----------------------- CADSIM Version 2.4 ----------------------------------

(no MAKEACAD changes)

----------------------- CADSIM Version 2.5 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.4

and Version 2.5:

- CADSIM PRINTER AND PLOTTING PROGRAMS HAVE BEEN UPGRADED.

----------------- CADSIM Version 2.6 / COACH Version 0.9 --------------------

The following notes outline the changes between CADSIM Release Version 2.5

and Version 2.6:

- ELLIPSES:                                           -- (new) August, 1993

NOTE:  Unclosed ellipses (an ellipse arc) are not supported by AutoCAD --

MAKEACAD converts elliptical arcs to circular arcs.

- MAKEACAD SUPPORTS NEW GRAPHICS FEATURES OF CADSIM V2.6 -- (new) Oct, 1993

- MAKEACAD.INI FILE CONTROLS AUTOCAD CONVERSION:     -- (new) October, 1993

An initialization file, MAKEACAD.INI, can be created and placed in the

working directory (where the drawings are located) to add options to the

conversion of a CADSIM drawing to an AutoCAD drawing.  The new options

allow selected:

1) use of LINEs (with line thickness defined by color -- for plots by pen)

instead of PLINEs,

2) use of Left justified text instead of Fit justified text,

3) use of an optional font,

4) use of alternate Linestyle translations,

5) definition of drawing layers to be made invisible,

6) rescaling of the drawing, and

7) substitution of native AutoCAD parts for CADSIM parts.

Seven commands in the MAKEACAD.INI define these seven options as follows:

LINE_THICKNESS low_thick high_thick pen_color

Any line with a thickness greater than low_thick and less than or equal

to high_thick will become a LINE with the color pen_color.  This

command is optional but can be repeated to cover any or all ranges of

line thicknesses.  Any line which has a thickness that falls outside

any of the defined thickness ranges given by these commands will

continue to be converted to PLINE.  This command is useful for those

that wish to "plot by pen color" or to adopt use of the simpler

AutoCAD LINE entities instead of PLINE.

LEFT_JUSTIFY width_to_height pen_color

Any text with the given width_to_height ratio will become left justified

text with the same height as in CADSIM, but with the AutoCAD default

text width.  This command is optional and can be used only once.  If

this command is not present, or if an item of text does not have the

given height_to_width ratio, it will be justified with the AutoCAD Fit

command so that it appears exactly the same height and width as in

the CADSIM drawing.  The reason for this command is that left justified

text is easier to edit within AutoCAD than text that is Fit justified.

Text that has the given width_to_height ratio will assume a color

pen_color if the optional pen_color parameter is given, otherwise text

color will not be changed.

FONT fontname

This command requests a different global fontname for text.  The

default font is now MONOTXT.  This optional command can be used only

once.

LINESTYLE cadsim_style autocad_style

Any line with the style cadsim_style will be given the style

autocad_style.  This command is optional and can be repeated for any

or all of the CADSIM line styles.  This allows conversion of line styles

to either the default AutoCAD styles or to the users own custom styles.

Note that loading the CADSIM.LIN styles with the script file LOADLIN.SCR

before loading the drawing script file may now be optional if all line

styles used in the CADSIM drawing are converted to valid AutoCAD styles.

INVISIBLE invisible_layer

Any entity on the layer, invisible_layer, will not be converted to the

AutoCAD script file.  This command is optional and can be repeated for

additional invisible layers.

RESCALE multiply_factor

This allows all dimensions in the drawing to be scaled by

multiply_factor.  The above LINE_THICKNESS command references line

thicknesses before scaling.  Any remaining PLINE entitities will have

their thickness rescaled.  If this command does not exist,

multiply_factor defaults to 1.0.

PART cadsim_part_name autocad_part_path+name ATTRIBUTES

This command requests AutoCAD parts to be substituted for selected

CADSIM parts.  The CADSIM part to be replaced is given by

cadsim_part_name.  The AutoCAD part to be used is given with a full

path to find it as autocad_part_path+name.  Obviously the native

AutoCAD parts will have to have been drawn to the same scale and have

the same origin.  All rotations, flips and rescaling applied to the

CADSIM part when it is inserted will be applied to the substituted

AutoCAD part.  This command is optional and can be repeated for

additional parts.  An optional qualifier ATTRIBUTES should be used if

the AutoCAD part has attributes.  This puts a space at the end of the

INSERT command line in the script file so AutoCAD does not get

confused.

An example MAKEACAD.INI file might look as follows:

LINE_THICKNESS 0.0  0.1   1

LINE_THICKNESS 0.1  0.2   2

LINE_THICKNESS 0.2  0.5   3

LINE_THICKNESS 0.5  0.75  4

LINE_THICKNESS 0.75 1.0   5

LINE_THICKNESS 1.0  2.0   6

LINE_THICKNESS 2.0  999.0 7

LEFT_JUSTIFY 0.8 2

LINESTYLE 1 CONTINUOUS

LINESTYLE 2 HIDDEN

LINESTYLE 3 DASHED

LINESTYLE 4 DOT

LINESTYLE 5 DASHDOT

LINESTYLE 6 PHANTOM

LINESTYLE 7 CENTER

LINESTYLE 8 DIVIDE

INVISIBLE 101

INVISIBLE 102

INVISIBLE 103

INVISIBLE 104

RESCALE 0.5

PART VALVE C:\ACAD\ACVALVE

PART PUMP C:\ACAD\ACPUMP ATTRIBUTES